3GPP TSG-RAN3 NR AdHoc 1801	R3-180503

Sophia Antipolis, France, 22-26 January 2018

Agenda Item:	10.1.2

Source:	China Telecom

Title:	TP for optional Reference QoS Profile

Document for:	Approval

Introduction

In the following we propose a TP to capture the proposals in R3-180502.

Text Proposal for TS 38.413

9.3.1.23	QoS Flow Level Parameters

Editor’s Note:	Further details FFS.

This IE defines the QoS to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M (FFS) |  | INTEGER (0..255) | 5G QoS Indicator defined in TS 23.501 [9]. Logical range and coding specified in TS 23.501 [9]. eNote: Coding is FFS. Presence is FFS (probably not needed if the Non standardised QoS Flow descriptor is present. Could also be realised as a choice with the Non standardised QoS Flow descriptor.) | - |  |
| Allocation and Retention Priority | M |  | <ref> | eNote: Details are FFS, but expected similar to 4G Note: presence needs to be checked with latest SA2 status. | - |  |
| Non-standardised QoS Flow descriptor | O |  | <ref> | eNote: The detailed definition of this IE will be included once stage 2 is stable | - |  |
| GBR QoS Flow Information | O |  | <ref> | This IE applies to GBR bearers only and shall be ignored otherwise. | - |  |
| Notification Control | O |  | ENUMERATED (notification requested, ...) | eNote: Details to be checked with SA2 progress. | - |  |
| RQA | O |  | ENUMERATED(subject to, …) | Reflective QoS Attribute;indicates that certain traffic on this QoS flow may be subject to NAS reflective QoS. This IE applies to non-GBR bearers only and shall be ignored otherwise. | - |  |
| Reference QoS profile | O |  | INTEGER (0,1) | Value ‘1’ Indicates the QoS profile served as “reference” for the PDU session. | - |  |

Conclusion and proposal

Proposal: agree the above text proposal for TS 38.413 to support the optional indication of the reference QoS profile to gNB for the PDU session.

References

- 3GPP TS 23.501, Architecture for Next Generation System, SA2.

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies, RAN.

- 3GPP TR 38.801, Study on New Radio Access Technology: Radio Access Architecture and Interface, RAN3.

- R3-173760, Text Proposal for Reference QoS Profile, Nokia, Nokia Shanghai Bell, Intel Corporation, LG Electronics Inc.

- R3-180502, On the reference QoS profile for the default DRBs, China Telecom.