3GPP TSG-RAN WG3 Meeting #Ad-Hoc 1807	R3-184108

# Montreal, Canada, 2nd- 6th July  2018

Agenda Item:	31.3.1.11

Source:	Ericsson

Title: 	Correction on NGAP

Document for:	Discussion and Approval

# 1	Introduction

This paper provides the necessary correction to NGAP.

# 2	Discussion

## 2.1	Correction to HANDOVER COMMAND

Currently, the Handover Command Transfer IE is Optional present when at least one PDU Session Resource Subject to Forwarding Item IE is present.

Note that the PDU Session Resource Subject to Forwarding Item IEs is Optional present in the message. However, the “Handover Command Transfer” should be Mandatory present instead of  Optional when there is PDU Session Resource Subject to Forwarding Item presented. Otherwise the data forwarding would not work if only PDU session ID is presented.

Proposal 1	RAN3 to agree to change the presence of Handover Command Transfer IE to Mandatory in HAND OVER COMMAND.

## 2.1	Correction to PATH SWITCH REQUEST ACKNOWLEDGE

It is specified that “after all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU session resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.”

That is to say, at least one PDU session resouce should be included in the

Currently, the PATH SWITCH REQUEST ACKNOWLEDGE message has PDU Session Resource to be Switched in Uplink List IE as defined as 0..1, that is to say, it is Optional present.

In our opinion, the PDU Session Resource to be Switched in Uplink List IE should be defined as 1, that is to say, mandatory present. And then at least one PDU session is sucessfully compelted with path switch.

Proposal 2	RAN3 to agree to change to PDU Session Resource to be Switched in Uplink List IE to 1 to show it is Mandatory present in PATH SWITCH REQUEST ACKNOWLEDGE.

# 3	Conclusion

Proposal 1	RAN3 to agree to change the presence of Handover Command Transfer IE to Mandatory in HAND OVER COMMAND.

Proposal 2	RAN3 to agree to change to PDU Session Resource to be Switched in Uplink List IE to 1 to show it is Mandatory present in PATH SWITCH REQUEST ACKNOWLEDGE.