3GPP TSG-RAN WG3 Meeting #Ad-Hoc 1807	R3-184110

# Montreal, Canada, 2nd- 6th July  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PATH SWITCH REQUEST ACKNOWLEDGE. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The “PDU Session Resource to be Switched in Uplink List IE” should be Mandatory present instead of Optional. Change the Path Switch Request Acknowledge Transfer from a SMF IE to a normal defined IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the PDU Session Resource to be Switched in Uplink List IE from range 0..1 to 1; Modify the Path Switch Request Acknowledge Transfer IE. Correct ASN.1 error. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.9; 9.3.4.9, 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| UE Security Capabilities | O |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| KAMF Change Indicator | O |  | 9.3.1.55 |  | YES | reject |
| PDU Session Resource to be Switched in Uplink List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource to be Switched in Uplink Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request AcknowledgeTransfer | M |  | 9.3.4.9 |  | - |  |
| PDU Session Resource Released List | O |  | PDU Session List 9.3.1.11 |  | YES | ignore |
| RRC Inactive Assistance Information | O |  | 9.3.1.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

--------------------------------

Skip unchanged text

---------------------------------

#### 9.3.4.9	Path Switch Request Acknowledge Transfer

This IE is contains the acknowledge information for the successfully completed path switch for the PDU session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. |
| Security Indication | O |  | 9.3.1.27 |  |

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

--------------------------------

Skip unchanged text

---------------------------------

-- **************************************************************

--

##### -- PATH SWITCH REQUEST ACKNOWLEDGE

--

-- **************************************************************

PathSwitchRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestAcknowledgeIEs} },

...

}

PathSwitchRequestAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities						CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE optional		}|

{ ID id-SecurityContext								CRITICALITY reject	TYPE SecurityContext									PRESENCE mandatory	}|

{ ID id-KamfChangeInd								CRITICALITY reject	TYPE KamfChangeInd										PRESENCE optional		}|

{ ID id-PDUSessionResourceToBeSwitchedULList		CRITICALITY ignore	TYPE PDUSessionResourceToBeSwitchedULList		PRESENCE mandatory		}|

{ ID id-PDUSessionResourceReleasedList				CRITICALITY ignore	TYPE PDUSessionList										PRESENCE optional		}|

{ ID id-RRCInactiveAssistanceInformation			CRITICALITY ignore	TYPE RRCInactiveAssistanceInformation			PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

PDUSessionResourceToBeSwitchedULList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF ProtocolIE-SingleContainer { {PDUSessionResourceToBeSwitchedULItemIEs} }

PDUSessionResourceToBeSwitchedULItemIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourceToBeSwitchedULItem		CRITICALITY reject	TYPE PDUSessionResourceToBeSwitchedULItem		PRESENCE mandatory	},

...

}

PDUSessionResourceToBeSwitchedULItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

pathSwitchRequestAcknowledgeTransfer		PathSwitchRequestAcknowledgeTransfer,

iE-Extensions		ProtocolExtensionContainer { { PDUSessionResourceToBeSwitchedULItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToBeSwitchedULItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}