RA3GPP TSG-RAN WG3 NR AH 1807	Tdoc R3-184243

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.1.5

Source:	Ericsson

Title:	AMBR enhancements: summary of offline discussions

Document for:	Discussion, Decision

# Introduction

This contribution summarizes the offline discussion on possible enhancements to the solution on AMBR.

# Discussion

The current solution for monitoring and enforcing UL AMBR in DC and CU-DU split consists of:

- For DC: the MN provides a portion of the UL AMBR to the SN; the SN monitors and enforces the portion of UL AMBR received by the MN at MAC level.

- For CU-DU split: the CU provides a portion of the UL AMBR to the DU; the DU monitors and enforces the portion of UL AMBR received by the CU at MAC level.

According to the offline discussion, it was confirmed that the current solution does not address the issues identified in the meetings in the past [Reference to R3-182369].

On the other hand, it was acknowledged that solutions proposed so far that allows for updating the UL AMBR dynamically according to e.g., radio conditions, comes with a cost in terms of network signalling and complexity.

Therefore, at this meeting, the details of the proposed enhancements were not studied and the discussion on the solutions addressing the identified issues may continue in future (work as always is contribution driven).

Conclusion: At this meeting, RAN3 does not consider any enhancements to the UE AMBR, even if they are not excluded for the future.