3GPP TSG-RAN WG3 # Ad-hoc	R3-170072

Spokane, Washington, USA, Jan. 17th – 19th, 2017

Agenda item:		10.6.1

Source:	LG Electronics Inc.

Title:	Consideration on Xx Interface

Document for:		Discussion

1. Introduction

In the past meetings, we have agreed many aspects on option 3/3a. It is time to review all the agreements and investigate, on whether we call it X2 or Xn interface. This paper is taking this as a goal to achieve.

2. Discussion

The starting point could be to review the agreements that we have achieved.

Firstly, in last RAN3 meeting, the following agreement was achieved on QoS in TR 38.801 for option 3/3a.

10.1.2.5	QoS aspect

For this option, the LTE QoS framework defined in TS 36.300 [12] applies:

-	E-RAB (and concerning S1-U bearer) is established between EPC and gNB for SCG bearer option

-	Xx-U is established between LTE eNB and gNB for split bearer option

-	DRB is established between gNB and UE according to SCG bearer option or split bearer option

The first bullet on SCG bearer option, that is, E-RAB is established between EPC and gNB for this option. The LTE QoS framework is applied to this option. Basically, the LTE QoS framework includes the bearer concept, i.e., the EPS/E-RAB/DRB. Of course, the MCG bearer also follows the legacy model on S1 bearer and QoS concept.

The second bullet is on split bearer option, which should also adopt the legacy LTE QoS framework, i.e., the E-RAB/DRB  concept. This means that the Xx-U interface is very similar to legacy X2-U interface, which is much different from the newly to be designed Xn-U model.

Secondly, in last RAN2 meeting, the following agreements were achieved on option 3a:

- EPC-NR PDCP interaction for tight interworking

- In Scenario 3A, the PDCP layer for SCG Bearers is NR-PDCP.

- In scenario 3A, there is a 1:1 mapping between S1 bearer and DRB for SCG bearers.

The second bullet means that for option 3a the S1 bearer concept is kept to be used, which is aligned with the RAN3 agreement above, i.e., on S1 interface the legacy bearer concept and QoS model will be kept to use.

Observation 1): Xx-U interface is very similar to legacy X2-U interface since the LTE QoS framework (EPS/E-RAB/DRB based) defined in TS 36.300 is used, which is much different from new QoS framework based Xn interface.

Generally speaking, the X2 interface is a horizontal interface, which should follow the principle/concept of vertical S1 interface. In this way, the mobility for a UE can be easier on the handover related parameters transferring from a node to another, which also applies to dual connectivity.

In addition, in the past RAN3 meetings, we have also agreed the procedures on option 3/3a given as follows:

10.1.2.3	Procedural aspects

The procedures defined under section 10.1.2.8 (Dual Connectivity operation) in TS 36.300 [12] listed below apply. In this list, LTE eNB and gNB connected via Xx are considered to have the role similar to MeNB (or eNB without DC) and SeNB, respectively. In this context, the Xx-U has all the functionality of X2-U for LTE DC operation. The DC procedures defined in TS 36.423 [13] are used as baseline.

-	SeNB Addition

-	SeNB Modification (MeNB initiated SeNB Modification)

-	SeNB Modification (SeNB initiated SeNB Modification)

…

It can be seen that the Xx-U should have all the functionality of X2-U of LTE-DC operation. On the control plane procedures, they are almost the same as those of the legacy DC procedures and only small changes are needed such as UE capability coordination, UE measurement.

Observation 2): It is convenient on mobility/offloading if the model of horizontal interface Xx is aligned with vertical S1 interface from the bearer point of view.

Observation 3): Existing procedures defined in TS 36.300 on DC can be reused without any procedural change, but with minor update to adapt the message information to NR.

Based on the observations above, the following proposal is suggested to RAN3:

Proposal 1): To take a working assumption on Xx interface for option 3/3a is X2 interface.

Proposal 2): To turn the interface name “Xx” into “X2” in section 10.1.2 of TR 38.801.

3. Conclusion

In this contribution, the discussion on Xx was performed. The following proposals are suggested to RAN3:

Observation 1): Xx-U interface is very similar to legacy X2-U interface since the LTE QoS framework (EPS/E-RAB/DRB based) defined in TS 36.300 is used, which is much different from new QoS framework based Xn interface.

Observation 2): It is convenient on mobility/offloading if the model of horizontal interface Xx is aligned with vertical S1 interface from the bearer point of view.

Observation 3): Existing procedures defined in TS 36.300 on DC can be reused without any procedural change, but with minor update to adapt the message information to NR.

Proposal 1): To take a working assumption on Xx interface for option 3/3a is X2 interface.

Proposal 2): To turn the interface name “Xx” into “X2” in section 10.1.2 of TR 38.801.

4. References

- RP-160576, “New SID Proposal: Study on New Radio Access Technology”

- TR 38.801

- R3-170003, “TP for tight interworking between new RAT and LTE”, LG Electronics