3GPP TSG-RAN WG3 Meeting NR Ad hoc	R3-172045

Qingdao, China, June 27th – 29th, 2017

Agenda item:		10.8.1

Source:	LG Electronics Inc.

Title:	Stage 3 TP for SN change procedure

Document for:		Discussion and Decision

1. Introduction

This paper is a stage 3 TP for SN change procedure, which is a correction to the baseline CR (R3-171993) p-approved in last meeting.

1. Text Proposal

----------------Start of the First Change---------------

8.X.5	SgNB Change

8.X.5.1	General

This procedure is used by the SgNB to change to another SgNB.

The procedure uses UE-associated signalling.

8.X.5.2	Successful Operation

Figure 8.X.5.2-1: SgNB Change, successful operation.

The SgNB initiates the procedure by sending the SGNB CHANGE REQUIRED message to the MeNB. When the SgNB sends the SGNB CHANGE REQUIRED message, it shall start the timer TDCoverall.

The SGNB CHANGE REQUIRED message may contain

-	the SgNB to MeNB Container IE (the container is FFS).

-	the candidate target cell ID list

If the MeNB is able to perform the change requested by the SgNB, the MeNB shall send the SGNB CHANGE CONFIRM message to the SgNB. For each E-RAB configured with the SCG bearer option or SCG split bearer, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer. For each E-RAB configured with the split bearer option, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of downlink packets to be performed for that bearer.

The SgNB may start data forwarding and stop providing user data to the UE and shall stop the timer TDCoverall upon reception of the SGNB CHANGE CONFIRM message.

----------------End of the First Change---------------

----------------Start of the Second Change---------------

9.1.X.15	SGNB CHANGE REQUIRED

This message is sent by the SgNB to the MeNB to request the change of SgNB for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Editor’s note: the container is FFS | YES | reject |
| Candidate Cell ID List |  | 1 |  |  | GLOBAL | ignore |
| >gNB Cell ID | M | 1 .. <maxCellingNB> | FFS | Definition is FFS | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNB | Maximum no. cells that can be served by a gNB. Value is FFS. |

----------------End of the Second Change---------------