3GPP TSG-RAN WG3 AH #2	Tdoc R3-172517

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	10.10.1

Source:	Ericsson

Title:	TP for system information in disaggregated gNB

Document for:	pCR

# Introduction

In the following we propose a pCR to capture the proposals in R3-172516.

# pCR for TS 38.401

Start of Text Proposal for TS 38.401

### 10.3.2.2.4	System Information management function

The system information parameters are partly controlled by the gNB-CU and partly controlled by the gNB-DU.

NOTE: With reference to the system information parameters in LTE:

- The parameters in the MIB are controlled by the gNB-DU;

- The gNB-DU controls the parameters in SIB1 related to frequency bands and radio configuration. The gNB-CU controls the parameters in SIB1 related to core network, cell (re)selection, and frequency priority;

- The gNB-DU controls the parameters in SIB2 related to radio configuration and access barring (when related to radio resources). The gNB-CU controls the parameters in SIB2 related to core network, UE identification and access barring (when related to non-radio resources);

- The parameters contained in the other SIBs are controlled by the gNB-CU

The gNB-CU configures its own system information parameters and sends these parameters to the gNB-DU over the F1-C interface. The gNB-DU encodes the system information parameters and transmits over the air. The gNB-CU can retrieve the system information parameters that are configured by the gNB-DU by sending a specific request over the F1-C interface.

The gNB-DU can provide on-demand the “other SI” to the UEs in RRC_IDLE or RRC_INACTIVE without involving the gNB-CU.

End of Text Proposal for TS 38.401