3GPP TSG-RAN WG3 AH #2	Tdoc R3-172522

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	10.10.3

Source:	Ericsson

Title:	SgNB addition in disaggregated gNB

Document for:	Discussion, Decision

# Introduction

During RAN3 #96 the procedures over the F1 interface were discussed. It was noted that the stage 2 procedures should be agreed first. Afterwards, we can start discussing the stage 3 procedures and the encoding of the messages. In this contribution, we discuss the stage 2 SgNB addition procedure in a disaggregated gNB.

# Discussion

The call-flow for the SgNB addition is shown in Fig. 1.

The following terminology is utilized to describe the entities involved in the procedure:

- MeNB: master eNB

- SgNB: secondary gNB

- DU: secondary gNB-DU

- CU: secondary gNB-CU

The UE is initially connected to LTE. The call-flow for adding the secondary gNB node encompasses the fowling steps.

- UE sends a RRC MEASUREMENT REPORT to MeNB. The RRC MEASUREMENT REPORT indicates the presence of an NR cell with good radio quality. The event that triggers the measurement report depends on the UE measurement configuration.

- MeNB sends the X2AP SgNB ADDITION REQUEST message to SgNB (CU-CP). MeNB requests radio resources for specific E-RABs, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type). The MeNB can provide the latest measurement results for the SCG cell(s) requested to be added.

- CU performs admission control, creates a UE context, and identifies DU. Then, CU sends the F1-AP CONTEXT SETUP REQUEST message to DU, including UE context information and CU-UL-TEID for data radio bearers.

- DU performs admission control, configures the lower-layers, and creates a local UE context. Then, DU sends the F1-AP CONTEXT SETUP RESPONSE message to CU, including lower-layers configuration, and DU-DL-TEID for data radio bearers.

- SgNB (CU-CP) sends the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message to MeNB. For SCG and SCG split bearers, the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message may provide S1 DL TNL address information for the respective E-RAB, for split bearers together with X2 DL TNL address information.

- MeNB sends the RRC CONNECTION RECONFIGURATION message to UE.

- UE applies the new configuration and replies with the RRC CONNECTION RECONFIGURATION COMPLETE message to MeNB.

- MeNB sends the SeNB ADDITION RECONFIGURATION COMPLETE message to SgNB (CU) to indicate the success of the procedure.

- UE sends a RANDOM ACCESS preamble to DU.

- DU replies with a RANDOM ACCESS RESPONSE to UE.

- MeNB sends the X2AP SN STATUS TRANSFER message to SgNB (CU) for data forwarding (if applicable).

- For SCG and SCG split bearers, MeNB sends the E-RAB MODIFICATION INDICATION message to the MME to update the UP path.

- MME confirms the new UP path by sending the E-RAB MODIFICATION CONFIRMATION message to MeNB.

- and 15: UL and DL data transmission.

- Fig. 1: SgNB addition in option 3 with a disaggregated gNB deployment.

# Conclusion

We conclude with the following proposals:

Proposal 1	RAN3 is kindly asked to agree with the proposed call-flow for SgNB addition in disaggregated gNB deployment.

Proposal 2	RAN3 is kindly asked to agree with the TP in R3-172523 [1]

# References

R3-172523, “TP for SgNB addition in disaggregated gNB”, Ericsson