## A.6.6	Measurement procedure

### A.6.6.1	Intra-frequency Measurements

#### A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.1.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.1.1.1-1 and A.6.6.1.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.1.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.2	SA event triggered reporting tests without gap under DRX

##### A.6.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.1.2.2-1, A.6.6.1.2.2-2 and A.6.6.1.2.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.2.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.2.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX

##### A.6.6.1.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.3.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.1.3.1-1 and A.6.6.1.3.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.3.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.3.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX

##### A.6.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.4.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.1.4.2-1, A.6.6.1.4.2-2 and A.6.6.1.4.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.4.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.4.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: Table A.6.6.1.4.2-1The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Table A.6.6.1.4.2-1Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Table A.6.6.1.4.2-1SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.1.4.2-4: Void

Table A.6.6.1.4.2-5: Void

##### A.6.6.1.4.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading

##### A.6.6.1.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.5.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in table A.6.6.1.5.2-1 and A.6.6.1.5.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.5.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.5.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading

##### A.6.6.1.6.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.6.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in table A.6.6.1.6.2-1 and A.6.6.1.6.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.6.2-2: General test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.6.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16

##### A.6.6.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.7.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.1.7.2-1, A.6.6.1.7.2-2 and A.6.6.1.7.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.7.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.7 | 640 ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

Table A.6.6.1.7.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity |  | -94 |
|  |  | 2 | -94 | -94 | -Infinity |  | -94 |
|  |  | 3 | -91 | -91 | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN 1944 Hz Note 4 |  |
|  |  | 3 | AWGN |  |  | AWGN 3334 Hz Note 5 |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. NOTE 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |

##### A.6.6.1.7.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17

##### A.6.6.1.8.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasCA-Scell-r17. This test will partly verify the intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.8.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), a FR1 deactivated SCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2). The test parameters for PCell are given in table A.6.6.1.8.21, A.6.6.1.8.2-2 and A.6.6.1.8.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.8.2-1: Supported PCell test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with the smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 6.6.1.8.4.1-1A: Supported SCell test configurations

| ConfigurationSCell | Description |
| --- | --- |
| 1A | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2A | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3A | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. NOTE 3: NR Cell 3 has the same SCS, BW and duplex mode as NR Cell 2. |  |

Table A.6.6.1.8.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for deactivated SCell in FR1 with DRX highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Deactivated SCell |  | 1A, 2A, 3A | Cell 2 |  |
| Neighbour cell |  | 1A, 2A, 3A | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and 3 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640 ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3 ms later than the timing of Cell 2. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.6.1.8.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for deactivated SCell in FR1 with DRX highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  | 1,1A | N/A |  | N/A |  | N/A |  |
|  |  | 2,2A | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3,3A | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,1A | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3,3A | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1,1A | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3,3A | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,1A | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3,3A | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3,1A,2A,3A | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1,1A | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3,3A | TRS.1.2 TDD |  | N/A |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3,1A,2A,3A | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3,1A,2A,3A | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1,1A | -98 |  |  |  |  |  |
|  |  | 2,2A | -98 |  |  |  |  |  |
|  |  | 3,3A | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1,1A | -98 |  |  |  |  |  |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
|  | dB | 1,1A | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
|  | dB | 1,1A | 4 | 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
| SSB_RP Note 3 | dBm/SCS kHz | 1,1A | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2,2A | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3,3A | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,1A | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2,2A | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3,3A | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1,2,1A,2A | AWGN |  |  |  | AWGN 1944 Hz Note 4 |  |
|  |  | 3,3A | AWGN |  |  |  | AWGN 3334 Hz Note 5 |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. NOTE 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |

##### A.6.6.1.8.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 5760 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.9	SA event triggered reporting tests with MUSIM gap configured

##### A.6.6.1.9.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event on an intra-frequency layer based on measurement performed without measurement gaps when the UE is also configured with MUSIM gaps. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.9.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.1.9.2-1 and A.6.6.1.9.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.9.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.9.2.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| MUSIM gap Pattern ID |  | 1,2,3 | 0 |  |
| MUSIM gap offset | ms | 1,2,3 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.9.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.1.9.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.10	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP

##### A.6.6.1.10.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of an event when CD-SSB is outside active BWP. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

The test environment is the same as in clause A.6.6.1.1 with following exceptions in table Table A.6.6.1.1.2-3.

Table A.6.6.1.1.2-3: Test environment exceptions

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | - |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | - |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | - |  |

##### A.6.6.1.10.2	Test Requirements

The test requirements are the same as in clause A.6.6.1.1.3.

#### A.6.6.1.11	SA event triggered reporting tests without gap under non-DRX with NCD-SSB

##### A.6.6.1.11.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements when NCD-SSB is configured in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.11.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.1.11.1-1 and A.6.6.1.11.1-2 below. The CD-SSB is configured outside active DL BWP and NCD-SSB is configured fully within active DL BWP of FR1 PCell. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.11.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.11.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| NCD-SSB configuration |  | 1 | SSB.9 FR1 |  |
|  |  | 2 | SSB.9 FR1 |  |
|  |  | 3 | SSB.10 FR1 |  |
| SMTC configuration for NCD-SSB |  | 1 | SMTC.11 |  |
|  |  | 2 | SMTC.10 |  |
|  |  | 3 | SMTC.10 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: The starting PRB index of the CD-SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP. NOTE 2: The starting PRB index of the NCD-SSB can be the same as the starting PRB index for DL active BWP. |  |  |  |  |

Table A.6.6.1.11.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | NCD-SSB |  | NCD-SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.1.11.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB

##### A.6.6.1.12.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 makes correct reporting of an event when the intra-frequency target cell is transmitting 12 PRB SSB. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2 for 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is not used and with 15 PRB PDCCH configuration.

##### A.6.6.1.12.2	Test parameters

The test procedure in clause A.6.6.1.5.2 applies for this test. Supported test configurations are specified in table A.6.6.1.12.2-1. General test parameters as specified in table A.6.6.1.5.2-2 with config 1 apply to this test, except those specified in table A.6.6.1.12.2-2. Cell specific test parameters as specified in table A.6.6.1.5.2-3 with config 1 apply to this test, except those specified in table A.6.6.1.12.2-3.

Table A.6.6.1.12.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.12.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| SSB configuration |  | 1 | SSB.13 FR1 |  |

Table A.6.6.1.12.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 1 | 3: NPRB,c = 15 |  | 3: NPRB,c = 15 |  |
| PDSCH RMC configuration |  | 1 | SR.1.2 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.3 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.7 FDD |  | N/A |  |
| Io | dBm/2.7 MHz | 1 | -70.00 | -67.65 | -70.00 | -67.65 |

##### A.6.6.1.12.3	Test Requirements

The test requirements in clause A.6.6.1.5.3 applies to this test, except that the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 940 ms from the beginning of time period T2.

#### A.6.6.1.13	SA event triggered reporting tests without gap under Cell DTX

##### A.6.6.1.13.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2 under Cell DTX configuration.

##### A.6.6.1.13.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.1.13.2-1 and A.6.6.1.13.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. During time duration T2, NW triggers the Cell DTX.

UE is allocated with PUSCH resource at every Cell DTX cycle.

Table A.6.6.1.13.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE fulfils the requirements in this Cell DTX test case can skip the test case in A.6.6.1.2 DRX.1 configuration. |  |

Table A.6.6.1.13.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 with Cell DTX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 |  |
| Cell DTX |  | 1,2,3 | DTX.1 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.13.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1 with Cell DTX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.1.13.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 + 160 = 1080 ms from the beginning of time period T2, while 160 ms is the Cell DTX cycle length, which is the delay uncertainty for next available PUSCH for L3 MR reporting during cell DTX. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.2	Inter-frequency Measurements

#### A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.6.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.2.1.1-1, A.6.6.2.1.1-2 and A.6.6.2.1.1-3.

Measurement gap pattern configuration is defined in table A.6.6.2.1.1-2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Table A.6.6.2.1.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.6.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.2.2.1-1, A.6.6.2.2.1-2 and A.6.6.2.2.1-3.

Measurement gap pattern configuration is defined in table A.6.6.2.2.1-2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 |  |

Table A.6.6.2.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR.2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | Config 1,2,3 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.6.6.2.2.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.6.6.2.2.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.3	Void

#### A.6.6.2.4	Void

#### A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.6.6.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.2.5.1-1, A.6.6.2.5.1-2 and A.6.6.2.5.1-3.

Measurement gap pattern configuration is defined in table A.6.6.2.5.1-2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Table A.6.6.2.5.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1.1 |  |

Table A.6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16 MHz | Config 3 | -58.4 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.5.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.6.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.2.6.1-1, A.6.6.2.6.1-2 and A.6.6.2.6.1-3.

Measurement gap pattern configuration is defined in table A.6.6.2.6.1-2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.6.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 |  |

Table A.6.6.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.7	Void

#### A.6.6.2.8	Void

#### A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern

##### A.6.6.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when mandatory gap pattern with 3 ms MGL is configured.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.2.9.1-1, A.6.6.2.9.1-2 and A.6.6.2.9.1-3.

Measurement gap pattern configuration defined in table A.6.6.2.9.1-2 does not depend on per-FR gap capability of UE.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Table A.6.6.2.9.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.9.1-2: General test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 3 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 12 |  |
|  |  | Config 2,3 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.9.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  | SMTC.9 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.9.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used

##### A.6.6.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in tables A.6.6.2.10.1-1, A.6.6.2.10.1-2 and A.6.6.2.10.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.10.1-1: SA event triggered reporting tests when DRX is used for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.10.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.10.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | Config 1,2,3 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.6.6.2.10.1-4: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.10.2	Test Requirements

In test config 1, UE is required to report SSB time index. UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used

##### A.6.6.2.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.9.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in tables A.6.6.2.11.1-1, A.6.6.2.11.1-2 and A.6.6.2.11.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Table A.6.6.2.11.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.11.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.11.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.11.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17

##### A.6.6.2.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when UE is configured with highSpeedMeasInterFreq-r17. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.2.12.1-1, A.6.6.2.12.1-2 and A.6.6.2.12.1-3.

Measurement gap pattern configuration is defined in table A.6.6.2.12.1-2. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.12.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.12.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.14 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 2.3 |  |

Table A.6.6.2.12.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection for UE configured with highSpeedMeasInterFreq-r17

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR.2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | Config 1,2,3 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN 1944 Hz Note 5 |  |
|  |  |  | Config 3 | AWGN |  | AWGN 3334 Hz Note 6 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. NOTE 6: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |

##### A.6.6.2.12.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.13	SA event triggered reporting tests for FR1 with measurement gap with priority and periodic MUSIM gap configured

##### A.6.6.2.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event on an inter-frequency layer based on measurement performed within measurement gap when UE is also configured with MUSIM gap. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4. In addition, the test will verify that UE makes correct transmission/reception during occasions of measurement gaps and MUSIM gaps that are dropped due to collision handling as defined in clause 9.1.10.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.2.13.1-1, A.6.6.2.13.1-2 and A.6.6.2.13.1-3.

Measurement gap and MUSIM gap pattern configurations defined in table A.6.6.2.13.1-2 are provided to the UE.

NOTE:	The signaling procedure to trigger the UE to request MUSIM gaps before the test equipment configures MUSIM gaps to the UE is left to comformance test implementation.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively. During T1 and T3, the UE shall not have any timing information of NR Cell 2. During T1 and T2, MUSIM gap is configured with lower priority than the measurement gap. At the beginning of T3, test equipment reconfigures the priority of MUSIM gap such that MUSIM gap is configured with higher priority than the measurement gap during T3 and T4.

The TE schedules continuous DL data on PCell throughout the test.

Table A.6.6.2.13.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.13.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Measurement gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 9 |  |
| MUSIM gap Pattern Id |  | Config 1,2,3 | 1 |  |
| MUSIM gap offset | ms | Config 1,2,3 | 2 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1.3 |  |

Table A.6.6.2.13.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1, T3 | T2, T4 | T1, T3 | T2, T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.13.2	Test Requirements

During T1 and T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the measurement gap occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T3 and T4, after UE sends RRC ReconfigurationComplete message for the reconfiguration of MUSIM gap priority, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with

-	MUSIM gap occasions, and

-	measurement gap occasions that are not colliding with MUSIM gap.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T4. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.14	SA event triggered reporting tests for FR1 with measurement gap without priority and periodic MUSIM gapconfigured

##### A.6.6.2.14.1	Test Purpose and Environment

The purpose of this test is to verify that the MUSIM gap capable UE makes correct reporting of an event when MUSIM gap collides with measurement gaps without assigned priority. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4. This test is conducted assuming that the measurement gap is configured as Gap(s) configured via GapConfig without suffix, and the MUSIM gap parameters are configured as requested by UE.

NOTE:	The signaling procedure to trigger the UE to request MUSIM gaps before the test equipment configures MUSIM gaps to the UE is left to comformance test implementation.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.2.14.1-1, A.6.6.2.14.1-2 and A.6.6.2.14.1-3.

Measurement gap pattern configuration defined in table A.6.6.2.14.1-2 is provided for a UE.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Table A.6.6.2.14.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.14.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 measurement gap and periodic MUSIM gap with partially partial overlapping scenario for SSB-based measurements in inter-frequency layers

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 1 | As specified in table 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 4 |  |
| MUSIM Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in table 9.1.10-1 |
| MUSIM gap offset |  | Config 1,2,3 | 39 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.14.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 measurement gap and periodic MUSIM gap with partially partial overlapping scenario for SSB-based measurements in inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.7 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.14.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.15	SA event triggered reporting tests for FR1 with 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used

##### A.6.6.2.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4 for 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used and with 15 PRB PDCCH configuration.

Supported test configurations are specified in table A.6.6.2.15.1-1. General test parameters as specified in table A.6.6.2.2.1-2 with config 1 apply to this test. Cell specific test parameters as specified in table A.6.6.2.2.1-3 with config 1 apply except those specified in table A.6.6.1.13.1-2. DRX-Configuration and TimeAlignmentTimer -Configuration specified in table A.6.6.2.2.1-4 and table A.6.6.2.2.1-5, respectively, apply to this test case.

The test environment specified in A.6.6.2.2.1 applies to this test.

Table A.6.6.2.15.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| NOTE 1: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.15.1-2: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode |  | Config 1 | FDD |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  | NA |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.3 FDD |  | NA |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.7 FDD |  | NA |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  | SSB.13 FR1 |  |
| IoNote3 | dBm/3 MHz | Config 1 | -69.9 | -69.9 | -75.4 | -67.6 |

##### A.6.6.2.15.2	Test Requirements

Test requirements specified in clause A.6.6.2.2.2 apply to this test, except that the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1920 ms and 14720 ms from the beginning of time period T2 in test 1 and test, respectively.

### A.6.6.3	Inter-RAT Measurements

#### A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1

##### A.6.6.3.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clauses 9.4.2 and 9.4.3.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.3.1.1-1. General test parameters are provided in table A.6.6.3.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.3.1.1-3 and A.6.6.3.1.1-4, respectively.

Table A.6.6.3.1.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.1.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.3.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.1.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.3.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1

##### A.6.6.3.2.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in clauses 9.4.2 and 9.4.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indctated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.2.1-1. General test parameters are provided in table A.6.6.3.2.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.3.2.1-3 and A.6.6.3.2.1-4, respectively.

Table A.6.6.3.2.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.2.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.2.1-2 and A.6.6.3.2.1-3. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX. 7 | DRX cycle configurations DRX.1 and DRX. 7 are defined in table A.3.3.1-1 and table A.3.3. 7-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 15 |  |
| NOTE 1: Values are defined in table A.6.6.3.2.1-3 |  |  |  |  |

Table A.6.6.3.2.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.2.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.2.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16

##### A.6.6.3.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements for UE configured with highSpeedMeasFlag-r16 in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in clauses 9.4.2 and 9.4.3.

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

The UE shall be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.3.1-1. General test parameters are provided in table A.6.6.3.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.3.3.1-3 and A.6.6.3.3.1-4, respectively.

Table A.6.6.3.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.3.1-3 |  |  |  |

Table A.6.6.3.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NPRB,c /50) | -59.16+10log (NPRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.3.2	Test Requirements

In the test, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.4	L1-RSRP measurement for beam reporting

#### A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used

##### A.6.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in table A.6.6.4.1.1-1.

Table A.6.6.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.1.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.1.2-1 and table A.6.6.4.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used

##### A.6.6.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in table A.6.6.4.2.1-1.

Table A.6.6.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.2.2-1 and table A.6.6.4.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used

##### A.6.6.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in table A.6.6.4.3.1-1.

Table A.6.6.4.3.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.3.2-1 and table A.6.6.4.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80 ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.4.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15 kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.3.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at the 8th slot from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used

##### A.6.6.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in table A.6.6.4.4.1-1.

Table A.6.6.4.4.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.4.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.4.2-1 and table A.6.6.4.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80 ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.4.4.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.4.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15 kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.4.3	Test Requirements

After 80 ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8  from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16

##### A.6.6.4.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement when UE is configured with highSpeedMeasFlag-r16. This test will partly verify the L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in clause 9.5.4.1, with the testing configurations for NR cells in table A.6.6.4.5.1-1.

Table A.6.6.4.5.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.5.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.5.2-1 and table A.6.6.4.5.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.5.2-1: General test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.5.2-2: SSB specific test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Propagation conditions | 1,2 |  | AWGN |  | AWGN 1944 Hz |  |
|  | 3 |  | AWGN |  | AWGN 3334 Hz |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.5.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used

##### A.6.6.4.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement for SSB of a cell with additional PCI. This test will partly verify the L1-RSRP measurement requirements in clause 9.13.4.1, with the testing configurations for NR serving cells in table A.6.6.4.6.1-1.

Table A.6.6.4.6.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.6.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) and a cell with additional PCI (Cell 2). The test parameters for the Cell 1 are given in table A.6.6.4.6.2-1. The test parameters for Cell 2 are given in table A.6.6.4.6.2-2 below.

SSB#0 and SSB#1 are transmitted on Cell 1 and Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSB#0 and report measurement results periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#1 starts transmission and the UE is configured for L1-RSRP measurement on SSB#1. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the the SSB#0 for Cell 1.

Table A.6.6.4.6.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active cell | 1~3 |  | Cell 1 |
| A cell with additional PCI | 1~3 |  | Cell 2 |
| RF Channel Number | 1~3 |  | 1: Cell 1 and Cell 2 |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  | 2 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  | 3 |  | SSB.4 FR1 for Cell 1 and Cell 2 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| CP length | 1~3 |  | Normal |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Time offset between serving and a cell with additional PCI | 1 |  | < CP |
|  | 2 |  | < CP |
|  | 3 |  | < CP |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.6.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 of Cell 1 |  | SSB#1 of Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.6.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than X ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of SSB#1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2, where X is

-	680 for Config 1&2

-	660 for Config 3

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

##### A.6.6.4.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting bwpOperationMeasWithoutInterrupt-r18 makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in table A.6.6.4.1.1-1.

The test environment is the same as in A.6.6.4.1 with following exceptions in table A.6.6.4.1.2-1.

Table A.6.6.4.1.2-1

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.2 ULBWP.1.2 |

##### A.6.6.4.7.2	Test Requirements

The test requirements are the same as in A.6.6.4.1.3.

#### A.6.6.4.8	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP

##### A.6.6.4.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement when CD-SSB is outside active BWP. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in table A.6.6.4.3.1-1.

The test is for UE supporting rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 and the UE is not required past legacy test in A.6.6.4.3.

The test environment is the same as in A.6.6.4.3.2 with following exceptions in table Table A.6.6.4.3.2-1.

The value of parameter “Dedicated BWP configuration” is DLBWP.1.2 and ULBWP.1.2.

NOTE:	The starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP.

The test requirements are the same as in A.6.6.4.3.3.

#### A.6.6.4.9	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used

##### A.6.6.4.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in table A.6.6.4.9.1-1.

Table A.6.6.4.9.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.9.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.9.2-1 and table A.6.6.4.9.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured. During time duration T1, the UE shall not have any timing information of NR Cell 2.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.9.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| CD-SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| NCD-SSB configuration | 1 |  | [SSB.9 FR1] |
|  | 2 |  | [SSB.9 FR1] |
|  | 3 |  | [SSB.10 FR1] |
| OCNG Patterns | 1~3 |  | OP.1 |
| SMTC configuration for NCD-SSB | 1~3 |  | [SMTC.2 RedCap] |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | [DLBWP.1.1 RedCap] [ULBWP.1.1 RedCap] |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2:NCD-SSB is configured within dedicated DL BWP. |  |  |  |

Table A.6.6.4.9.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/18.36 MHz | -60.77 | -60.77 | -63.78 | -59.01 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.9.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than [620]ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.5	Inter-RAT UTRAN FDD measurements

#### A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1

##### A.6.6.5.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN FDD measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clause 9.4.6.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.5.1.1-1. General test parameters are provided in table A.6.6.5.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4, respectively.

Table A.6.6.5.1.1-1: Supported test configurations in SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRA FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRA FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRA FDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.5.1.1-2: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.5.1.1-3 |  |  |  |

Table A.6.6.5.1.1-3: PCell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.5.1.1-4: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| NOTE 1: The DPCH level is controlled by the power control loop. NOTE 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

##### A.6.6.5.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4 s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.6	CLI measurements

#### A.6.6.6.1	SRS-RSRP measurement with DRX

##### A.6.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 9.7.2.5 with the testing configurations for NR cells in table A.6.6.6.1.1-1.

Table A.6.6.6.1.1-1: Applicable NR configurations for FR1 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.1.2	Test Parameters

One cell is deployed in the test, which is FR1 PCell (Cell 1). The test parameters for PCell is given in table A.6.6.6.1.2-1 and A.6.6.6.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in table A.6.6.6.1.2-4 and the test parameters for the (virtual) neighbour cell UE in table A. 6.6.6.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.6.6.1.2-1: General test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.6.1.2-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.6.6.6.1.2-2: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.6.6.6.1.2-3: NR Cell specific test parameters for SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94 |
|  |  | 2 | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 2 | -63.96 | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.6.1.2-4: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.6.6.6.1.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.6.2	CLI-RSSI measurement with DRX

##### A.6.6.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of CLI-RSSI measurement. This test will verify the CLI-RSSI measurement requirements in clause 9.7.3.5 with the testing configurations for NR cells in table A.6.6.6.2.1-1.

Table A.6.6.6.2.1-1: Applicable NR configurations for FR1 CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.2.2	Test Parameters

One cell is deployed in the test, which are FR1 PCell (Cell 1). The test parameters for PCell is given in table A.6.6.6.2.2-1 and A.6.6.6.2.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI measurement resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in table A.6.6.6.2.2-3.

Table A.6.6.6.2.2-1: General test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | NR Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| CLI-RSSI configuration |  | 1 | CLI-RSSIConf.1 | Table A.6.6.6.2.2-3 |
|  |  | 2 | CLI-RSSIConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -93 |  |
|  |  | 2 | -93 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 2 |  |

Table A.6.6.6.2.2-2: NR Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
|  |  | 2 |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -116 | -108 |
|  |  | 2 |  |  |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -116 | -108 |
|  |  | 2 | -113 | -105 |
| Io on CLI-RSSI measurement resource | dBm/9.36 MHz | 1 | -88.05 | -80.05 |
|  | dBm/38.16 MHz | 2 | -81.96 | -74.00 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -97.43 | -89.43 |
|  | dBm/1.08 MHz | 2 | -97.44 | -89.44 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.6.6.6.2.2-3: CLI-RSSI measurement resource configuration for measurement reporting

|  | Field | CLI-RSSIConf.1 | CLI-RSSIConf.2 |
| --- | --- | --- | --- |
| RSSI-Resource | rssi-ResourceId | 0 | 0 |
|  | rssi-SCS | 15 | 30 |
|  | startPRB | 0 | 0 |
|  | nrofPRBs | 52 | 106 |
|  | startPosition | 3 | 3 |
|  | nrofSymbols | 11 | 11 |
|  | rssi-PeriodicityAndOffset | sl640, 4 | sl640, 9 |

##### A.6.6.6.2.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 640 ms from the beginning of time period T2. The nominal RSSI used to evaluate the requirement shall be based on Io.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.7	NR measurements with autonomous gaps

#### A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1

##### A.6.6.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in clause 9.11.

##### A.6.6.7.1.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.7.1.1-2 and A.6.6.7.1.3-2 below. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable.  A measurement object is configured for the frequency of the PCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is expected to detect and send a measurement report with Event A3.

A new RRC message triggering CGI identification shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to setup. The start of T3 is the instant when the last TTI containing the RRC message implying CGI identification is sent to the UE.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell during T3 untill a measurement report with CGI is sent.

Table A.6.6.71.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.7.1.2-2: General test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| T3 | s | 1, 2, 3 | 5 |  |

Table A.6.6.7.1.2-3: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 |  | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -1.46 |  | -Infinity | -1.46 | -1.46 |  |
|  |  | 2 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | 4 |  | -Infinity | 4 | 4 |  |
|  |  | 2 |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 |  | -Infinity | -94 | -94 |  |
|  |  | 2 | -94 | -94 | -94 |  | -Infinity | -94 | -94 |  |
|  |  | 3 | -91 | -91 | -91 |  | -Infinity | -91 | -91 |  |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -62.25 |  | -64.60 | -62.25 | -62.25 |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -62.25 |  | -64.60 | -62.25 | -62.25 |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -56.16 |  | -58.50 | -56.16 | -56.16 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.7.1.3	Test Requirements

The UE shall send a measurement report containing the CGI of Cell 2 within 255 ms from the start of time period T3.

Test requirement 	= RRC Procedure delay + Tidentify_CGI + TTI insertion uncertainty.

= 10 + 240 + 2 ms from the start of T3

= 252 ms, allow 255 ms

The UE shall be scheduled continuously throughout the test. From the start of T3 until 252 ms, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.14.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

##### A.6.6.7.2.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in clause 9.4.7.

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells as given in tables A.6.6.7.2.1-1, A.6.6.7.2.1-2, A.6.6.7.2.1-3 and A.6.6.7.2.1-4. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to setup. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.6.6.7.2.1-1: Supported test configurations of inter-RAT E-UTRAN cell using autonomous gap in SA

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.7.2.1-2: General test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.3.7.2.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | setup | As specified in clause 5.5.3.1 in TS 38.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.6.6.7.2.1-3: PCell specific test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR

| Parameter |  |  | Unit | Configuration | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |  |  |
|  |  |  |  | 4, 5, 6 | TDD |  |  |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |  |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |  |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |  |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |
| b2-Threshold1 |  |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |  |  |
|  |  |  |  | 3, 6 | -95 |  |  |  |
| b2-Threshold2EUTRA |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -109 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |  |  |
|  |  |  |  | 3, 6 | -103 |  |  |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 |  | -75.92 |  |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 |  | -69.82 |  |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.6.7.2.1-4: Cell specific test parameters for inter-RAT E-UTRAN cell for identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 10 MHz: NPRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.6.7.2.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within 200 milliseconds from the start of T3.

Test requirement = RRC Procedure delay with additional margin + Tidentify_CGI,E-UTRAN + TTI insertion uncertainty.

= 15 + 30 + 150 + 2 ms from the start of T3

= 197 ms, allow 200 ms.

-	The UE shall be scheduled continuously throughout the test, and from the start of T3 until 200 ms at least the number of ACK/NACK specified in NOTE 2 shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of Cell 2, where X is defined in table 8.2.2.2.15-1. Secondly, given that continuous DL data allocation, additional 43, 14 and 34 ACK/NACK shall be sent for FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of Cell 2.

### A.6.6.8	L1-SINR measurement for beam reporting

#### A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used

##### A.6.6.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.1, with the testing configurations for NR cells in table A.6.6.8.1.1-1.

Table A.6.6.8.1.1-1: Applicable NR configurations for FR1 CSI-RS based L1-SINR test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.1.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.8.1.2-1 and table A.6.6.8.1.2-2 below.

In the CSI-RS measurement configuration, UE is indicated to perform L1-SINR measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. After 80 ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.8.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.8.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS.1.3 FDD |
|  | 2 |  | CSI-RS.1.3TDD |
|  | 3 |  | CSI-RS.2.3TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.1.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15 kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.6.8.1.3	Test Requirements

After 80 ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.27.1.1 and relative accuracy requirement in clause 10.1.27.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

##### A.6.6.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.2, with the testing configurations for NR cells in table A.6.6.8.2.1-1.

Table A.6.6.8.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.8.2.2-1 and table A.6.6.8.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-RS resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.6.8.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.8.2.2-3: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.8.2.3	Test Requirements

The UE shall send L1-SINR report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 while meeting the accuracy requirement in clause 10.1.27.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used

##### A.6.6.8.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements with CSI-RS based CMR and dedicated IMR cofigured in clause 9.8.4.3, with the testing configurations for NR cells in table A.6.6.8.3.1-1.

Table A.6.6.8.3.1-1: Applicable NR configurations for FR1 L1-SINR test with CMR and dedicated IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.8.3.2-1 and table A.6.6.8.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the configured CSI-RS as CMR and an associated CSI-IM as IMR, and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. UE is also configured to measure L1-SINR based on SSB. After 80 ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in table A.6.6.8.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs, and UE is configured to perform L1-SINR measurement based on the CSI-RS as CMR and the CSI-IM as IMR.

Table A.6.6.8.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| CSI-IM configuration | 1 |  | CSI-IM.1.2 FDD |
|  | 2 |  | CSI-IM.1.2 TDD |
|  | 3 |  | CSI-IM.2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

A.6.6.8.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15 kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.8.3.3	Test Requirements

After 80 ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR while meeting the accuracy requirement in clause 10.1.27.3.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.9	Idle Mode CA/DC Measurements

#### A.6.6.9.1	SA Idle mode CA/DC measurement for FR1

##### A.6.6.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the Idle mode CA/DC measurements requirements in clause 4.4.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.9.1.1-1, A.6.6.9.1.1-2, A.6.6.9.1.1-3 and A.6.6.9.1.1-4.

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting with channel 2. Beam level reporting for early measurements is not configured. The connection is released at the end of T1. T2 starts when the connection is released. During the time periods T2 UE is in Idle mode. At T3 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.6.6.9.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.1.1-2: General test parameters for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 300 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 11.52 |  |
| T3 | s | Config 1,2,3 | 10 |  |

Table A.6.6.9.1.1-3: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  |  | Config 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
|  |  | dB | Config 1,2,3,4,5,6 | 7 | 7 | 7 | - infinity | 0 | 0 |
|  |  | dB | Config 1,2,3 | 7 | 7 | 7 | infinity | 0 | 0 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | Config 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

Table A.6.6.9.1.1-4: Cell specific test parameters for idle mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -infinity | 12 | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 7 | 7 | 7 | -infinity | 0 | 0 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| ÊsIot | dB | 1, 2, 3 | 7 | 7 | 7 | -infinity | 0 | 0 |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
| Io | dBm/9.36 MHz | 1 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/9.36 MHz | 2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/38.16 MHz | 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  |  | Not sent |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.9.1.2	Test Requirements

The UE behaviour during time durations T2 and T3 shall be as follows:

During the time period T2 the UE is in Idle mode and the signal level of Cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to section 4.4.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in section 10.1.4B for SS-RSRP and in section 10.1.8B for SS-RSRQ and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.9.2		Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting

##### A.6.6.9.2.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7 when the UE supports measValidationReportEMR-r18. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measIdleValidityDuration-r18 is configured for the test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.9.2.1-1, A.6.6.9.2.1-2, A.6.6.9.2.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting for Cell 2 in measIdleCarrierListNR-r16. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shall not be performed. Signal level of Cell 2 is set to the value given in table A.6.6.9.2.1-3. The time when T331 timer expires defines the ending point of T2.

At the beginning of T3, the signal level of the neighbour cell is set to turned off.  The duration of the T3 equals to measIdleValidityDuration-r18.

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall not send measurement report.

Table A.6.6.9.2.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.2.1-2: General test parameters for Idle mode fast CA/DC eEMR measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration-r18 | s |  | 10 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 10 |  |
| T4 | s | Config 1,2,3 | 1 |  |

Table A.6.6.9.2.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC eEMR measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  |  |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  | -infinity | -98 | infinity | infinity |
|  |  |  | Config 3 | -88 |  |  |  | -infinity | -95 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | - infinity | 0 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | infinity | 0 | infinity | infinity |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -62.26 |  |  |  | -70.05 | -67.04 | -70.05 | -70.05 |
|  |  | dBm/38.16 MHz | Config 3 | -56.15 |  |  |  | -63.96 | -60.93 | -63.96 | -63.96 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.9.2.2	Test Requirements

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.9.3	Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting

##### A.6.6.9.3.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7 when the UE supports measValidationReportReselectionMeasurements-r18. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when  measReselectionValidityDuration-r18 is configured for the test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.9.3.1-1, A.6.6.9.3.1-2, A.6.6.9.3.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with inter-frequency measurement reporting for Cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shall not be performed. Signal level of Cell 2 is set to the value given in table A.6.6.9.3.1-3. The duration of T2 is set to fixed value according to the Table A.6.6.9.3.1-2.

At the beginning of T3, the signal level of Cell 2 is set to turned off. The duration of the T3 equals to measReselectionValidityDuration-r18.

The time when TE sends the paging message defined as the starting point of T4. During T4, in this test the UE shall not send measurement report.

Table A.6.6.9.3.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.3.1-2: General test parameters for Idle mode fast CA/DC cell-reselection measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1 |  |

Table A.6.6.9.3.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC cell re-selection measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  | -infinity | -98 | infinity | infinity |
|  |  |  | Config 3 | -88 |  |  |  | -infinity | -95 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | - infinity | 0 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | infinity | 0 | infinity | infinity |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -62.26 |  |  |  | -70.05 | -67.04 | -70.05 | -70.05 |
|  |  | dBm/38.16 MHz | Config 3 | -56.15 |  |  |  | -63.96 | -60.93 | -63.96 | -63.96 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.9.3.2	Test Requirements

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.9.4	Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting

##### A.6.6.9.4.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7 when the UE supports measValidationReportReselectionMeasurements-r18 . This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measReselectionValidityDuration-r18 is configured for the test case when there are measurement results to report at RRC connection setup.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in tables A.6.6.9.4.1-1, A.6.6.9.4.1-2, A.6.6.9.4.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with inter-frequency measurement reporting for Cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shall not be performed. Signal level of Cell 2 is set to the value given in table A.6.6.9.4.1-3. The duration of T2 is set to fixed value according to the Table A.6.6.9.4.1-2.

At the beginning of T3, the signal level of Cell 2 is set to another value according to the Table A.6.6.9.4.1-3. The duration of T3 equals to measReselectionValidityDuration-r18 .

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4.

Table A.6.6.9.4.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.4.1-2: General test parameters for Idle mode fast CA/DC cell-reselection measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1 |  |

Table A.6.6.9.4.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC cell re-selection measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -88 |  |  |  | -infinity | -91 | -100 | -100 |
|  |  |  | Config 3 | -85 |  |  |  | -infinity | -88 | -97 | -97 |
|  |  | dB | Config 1,2,3,4, | 10 |  |  |  | - infinity | 7 | -2 | -2 |
|  |  | dB | Config 1,2,3 | 10 |  |  |  | -infinity | 7 | -2 | -2 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -59.63 |  |  |  | -70.5 | -62.26 | -67.92 | -67.92 |
|  |  | dBm/38.16 MHz | Config 3 | -53.54 |  |  |  | -63.96 | -56.17 | -61.83 | -61.83 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.9.4.2	Test Requirements

During the period T2 and T3 the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall send early measurement report to the PCell with valid measurement results.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in section 10.1.4B for SS-RSRP and in section 10.1.8B for SS-RSRQ and test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to Cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.10	CSI-RS based intra-frequency Measurements

#### A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.10.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 intra-frequency requirements in clauses 9.10.2.

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.10.1.1-1 and A.6.6.10.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.10.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.10.1.1-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified and measured. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS configuration for RRM |  | 1 | CSI-RS.RRM.FR1.1 FDD |  |
|  |  | 2 | CSI-RS.RRM.FR1.1 TDD |  |
|  |  | 3 | CSI-RS.RRM.FR1.2 TDD |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells | s | 1 | 4.7 | The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | 2 | 4.7 |  |
|  |  | 3 | 2.35 |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 2 |  |

Table A.6.6.10.1.1-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for SSB | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for CSI-RS | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for SSB | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| for CSI-RS | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| CSI-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.10.1.2	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1600 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.11	CSI-RS based inter-frequency Measurements

#### A.6.6.11.1	 SA event triggered reporting tests with gap under DRX

##### A.6.6.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 inter-frequency measurement requirements in clause 9.10.3.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in tables A.6.6.11.1.1-1, A.6.6.11.1.1-2 and A.6.6.11.1.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in table A.6.6.11.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in table A.6.6.11.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE is allocated with PUSCH resource at every DRX cycle.

NOTE:	TAT= infinite based on the DRX configuration used in test.

Table A.6.6.11.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.11.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | 19 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.5 | DRX.5 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells | s | Config 1 | 4.7 |  | The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | Config 2 | 4.7 |  |  |
|  |  | Config 3 | 2.35 |  |  |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 21 | 21 |  |

Table A.6.6.11.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| CSI-RS configuration for RRM |  |  | Config 1 | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |
|  |  |  | Config 2 | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |
|  |  |  | Config 3 | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| CSI-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| for SSB |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| for CSI-RS |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| for SSB |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| for CSI-RS |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.11.1.2	Test Requirements

In test 1 with per-UE gap and test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 20480 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is required to read the SSB index indicated by associatedSSB in the neighbour cell in this test

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.12	RSTD measurements

#### A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA

##### A.6.6.12.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.6.12.1.1-1.

Table A.6.6.12.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in table A.6.6.12.1.1-2, and cell specific test parameters are listed in table A.6.6.12.1.1-3.

Table A.6.6.12.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| SSB | Config 1~3 | dB | 10 | 3 | 3 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 19.08 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88 MHz | -61.44 | -61.44 | -61.44 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -95 | -95 |
|  | Config 2 | dBm/SCS | -88 | -95 | -95 |
|  | Config 3 | dBm/SCS | -85 | -92 | -92 |
| PRS |  | dB | -6.00 | -12.98 | -12.98 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |  |

##### A.6.6.12.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049

#### A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA

##### A.6.6.12.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations are specified in table A.6.6.12.2.1-1.

Table A.6.6.12.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 3 is on a different RF channel with Cell 1 and Cell 2.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in table A.6.6.12.2.1-2, and cell specific test parameters are listed in table A.6.6.12.2.1-3.

Table A.6.6.12.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first place in the respective neighbour cell list in the DL-TDOA assistance data for two positioning frequency layers. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -66.96 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | --66.96 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -62.97 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |  |

Table A.6.6.12.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.7 | -11.9 | -13 |
|  | Config 2 | dB | -5.7 | -11.9 | -13 |
|  | Config 3 | dB | -5.7 | -11.9 | -13 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -65.70 | -65.70 | -66.74 |
|  | Config 2 | dBm/ 19.08 MHz | -65.70 | -65.70 | -66.74 |
|  | Config 3 | dBm/ 47.88 MHz | -61.72 | -61.72 | -62.76 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |  |

##### A.6.6.12.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

#### A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA

##### A.6.6.12.3.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement with the reduced samples meets the requirements specified in clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.6.12.3.1-1.

Table A.6.6.12.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

NOTE: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request. UE can support supportedDL-PRS-ProcessingSamples-RRC-CONNECTED, and the LMF indicates the UE to perform positioning measurements with reduced number of samples.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in table A.6.6.12.3.1-2, and cell specific test parameters are listed in table A.6.6.12.3.1-3.

Table A.6.6.12.3.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.2 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.3.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.3.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | 1 | -1.2 | -1.2 |
|  | Config 2 | dB | 1 | -1.2 | -1.2 |
|  | Config 3 | dB | 1 | -1.2 | -1.2 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -61.18 | -61.18 | -61.18 |
|  | Config 2 | dBm/ 19.08 MHz | -61.18 | -61.18 | -61.18 |
|  | Config 3 | dBm/ 47.88 MHz | -57.20 | -57.20 | -57.20 |
| PRS |  | dB | -3 | -6 | -6 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.3.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 9.9.2.5

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration X1 specified in section 9.9.2.5 starting from the beginning of time interval T2, where X1 is 320 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap

##### A.6.6.12.4.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the gapless RSTD measurement period requirement specified in clause 9.9.2.7 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured. Reporting delay test for gapless PRS measurement is conducted assuming that the PRS has higher priority, i.e., state 1, than all other DL signals/channels and is transmitted within active DL BWP of UE. Two sub-tests are defined, sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. For sub-test 2 LMF indicates UE to perform PRS measurement with Nsample = 1. The cell specific parameters for sub-test 1 and sub-test 2 are defined in table A.6.6.12.4.1-4.

The supported test configurations are specified in table A.6.6.12.4.1-1.

Table A.6.6.12.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first PPW instance containing the PRS resources.

The UE is configured with PPW before start of T2.

The general test parameters are listed in table A.6.6.12.4.1-2, and cell specific test parameters are listed in table A.6.6.12.4.1-3.

Table A.6.6.12.4.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| BWchannel | Config 1 | MHz | 20: NPRB,c = 106 |  |
|  | Config 2 |  | 20: NPRB,c = 106 |  |
|  | Config 3 |  | 50: NPRB,c = 133 |  |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 | Cell 2 and cell 3 also transmit PRS on active DL BWP configured to the UE |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.4 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.4 FR1 |  |
|  | Config 3 |  | PRS.2.4 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| PPW configuration |  |  | PPW.1 | As defined in A.3.33 |
| Time offset between serving and neighbour cells |  | s | set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs 3µs otherwise | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | 0 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | Same as time offset | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | ms | 10 | The length of the time interval that follows after time interval T1 and processing time of assistance data. In this test UE is configured to measure single PFL within the configured PPW. |

Table A.6.6.12.4.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.4.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| NR RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 |  | OP.1 |  | OP.1 |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 |  |
| Note 3 | Config 1,2 | dBm/SCS | -98 |  | -98 |  | -98 |  |
|  | Config 3 | dBm/SCS | -95 |  | -95 |  | -95 |  |
| PRS | Config 1,2,3 | dB | -5.45 | 1 | -11.67 | -1.2 | -11.67 | -1.2 |
| Io Note 4 | Config 1,2 | dBm/ 19.08 MHz | -65.43 | -61.18 | -65.43 | -61.18 | -65.43 | -61.18 |
|  | Config 3 | dBm/ 47.88 MHz | -61.44 | -57.20 | -61.44 | -57.20 | -61.44 | -57.20 |
| PRS | Config 1,2,3 | dB | -6 | -3 | -13 | -6 | -13 | -6 |
| PRS-RSRPNote 4 | Config 1,2 | dBm/SCS | -103.45 | -97 | -109.67 | -99.2 | -109.67 | -99.2 |
|  | Config 3 |  | -100.45 | -94 | -106.67 | -96.2 | -106.67 | -96.2 |
| SS-RSRPNote 4 | Config 1,2 | dBm/SCS | -88 | -97 | -88 | -99.2 | -88 | -99.2 |
|  | Config 3 |  | -85 | -94 | -85 | -96.2 | -85 | -96.2 |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.12.4.2	Test Requirements

The RSTD measurement time fulfils the gapless RSTD measurement period requirements specified in clause 9.9.2.7.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.7 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG

##### A.6.6.12.5.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the Rx TEG based measurement period requirements specified in clause 9.9.2.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.6.12.5.1-1.

Table A.6.6.12.5.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The test applies to the UE supporting Rx TEG indicated via NR-UE-TEG-Capability and is requested to provide the Rx TEG in the test via nr-UE-RxTEG-Request-r17 in NR-TDOA-RequestLocationInformation. In the location request measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 is set to n0. The UE shall perform and optionally report the Rx TEG based RSTD measurements.

The general test parameters are listed in table A.6.6.12.5.1-2, and cell specific test parameters are listed in table A.6.6.12.5.1-3.

Table A.6.6.12.5.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| BWchannel | Config 1 | MHz | 20: NPRB,c = 106 |  |
|  | Config 2 |  | 20: NPRB,c = 106 |  |
|  | Config 3 |  | 50: NPRB,c = 133 |  |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.4 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.4 FR1 |  |
|  | Config 3 |  | PRS.2.4 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [1.28*kmultiTEG]Note 1 | The length of the time interval that follows immediately after time interval T1 |
| NOTE 1: kmultiTEG=NTEGkTEG,simul if UE is capable of receiving the same DL PRS resource from the same TRP simultaneously from multiple Rx TEGs, where NTEG is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource, and kTEG,simul is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul. |  |  |  |  |

Table A.6.6.12.5.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.5.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 19.08 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88 MHz | -61.44 | -61.44 | -61.44 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.5.2	Test Requirements

The RSTD measurement time fulfils the Rx TEG based RSTD measurement period requirements specified in clause 9.9.2.5. The UE shall perform and report the Rx TEG based RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.6.12.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode

##### A.6.6.12.6.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement by aggregating PRS resources from two positioning frequency layers (PFLs) meets the measurement period requirements specified in clause 9.9.2.10 in an environment with AWGN propagation conditions in FR1.

The supported test configurations are specified in table A.6.6.12.6.1-1.

Table A.6.6.12.6.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth per PFL, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth per PFL, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth per PFL, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Each cell is associated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, all cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS resources on two positioning frequency layers during T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.10, shall be provided to the UE during T1. The UE is capable of performing RSTD measurements by aggregating PRS resources from two PFLs and is configured by the LMF to perform measurements by aggregating the PRS resources from two positioning frequency layers via nr-DL-PRS-JointMeasurementRequestedPFL-List. The NR-DL-TDOA-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first measurement gap instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or measurement gap pattern ID # 0 before T2.

The general test parameters are listed in table A.6.6.12.6.1-2, and cell specific test parameters are listed in table A.6.6.12.6.1-3.

Table A.6.6.12.6.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first place in the respective neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration per PFL | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31. |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.2 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2) mod 6=0 and (PCI of Cell 1 – PCI of Cell 3) mod 6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expected RSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to NR-MutingPattern defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.6.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS EsNoc |  | dB | -Infinity | -Infinity | -Infinity |
| SSB EsNoc |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -66.96 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | --66.96 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -62.97 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.6.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number Note 5 |  |  | 1,2 | 1,2 | 1,2 |
| Positiong frequency layer (PFL) |  |  | 1,2 | 1,2 | 1,2 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in clause A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| NocNote 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS EsNoc | Config 1 | dB | -5.7 | -11.9 | -13 |
|  | Config 2 | dB | -5.7 | -11.9 | -13 |
|  | Config 3 | dB | -5.7 | -11.9 | -13 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz per PFL | -65.70 | -65.70 | -66.74 |
|  | Config 2 | dBm/ 19.08MHz per PFL | -65.70 | -65.70 | -66.74 |
|  | Config 3 | dBm/ 47.88MHz per PFL | -61.72 | -61.72 | -62.76 |
| PRS EsIot |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. Note 5: Carriers are intra-band and contiguous. |  |  |  |  |  |  |

##### A.6.6.12.6.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 9.9.2.10.

The UE shall perform and report the RSTD measurements by aggregating PRS resources from multiple positioning frequency layers for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.10 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23A.3, i.e., between RSTD_000000000 and RSTD_126083073.

### A.6.6.13 PRS-RSRP measurements

#### A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer

##### A.6.6.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.6.13.1.1-1.

Table A.6.6.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.13.1.1-2, and cell specific test parameters are listed in table A.6.6.13.1.1-3.

Table A.6.6.13.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time limit above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer

##### A.6.6.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.6.13.2.1-1.

Table A.6.6.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.13.2.1-2, and cell specific test parameters are listed in table A.6.6.13.2.1-3.

Table A.6.6.13.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 | Cell 1 and Cell 2 are on differnet positioning frequency layers |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io | dBm/19.08 MHz | 1 | N/A | -65.19 | N/A | -66.74 |
|  | dBm/19.08 MHz | 2 |  | -65.19 |  | -66.74 |
|  | dBm/47.88 MHz | 3 |  | -61.21 |  | -62.76 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.13.2.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time limit above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples

##### A.6.6.13.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements for reduced number of samples specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.6.13.3.1-1.

Table A.6.6.13.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.13.3.1-2, and cell specific test parameters are listed in table A.6.6.13.3.1-3.

Table A.6.6.13.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| Io | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.6.13.3.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit for reduced number of samples specified in clause 9.9.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG

##### A.6.6.13.4.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement outside MG meets the delay requirements specified in clause 9.9.3.6 in an environment with AWGN propagation conditions. There are two sub-tests in the test, sub-test 1 is to verify the delay requirements with Nsample=1, and sub-test 2 is to verify the delay requirements with Nsample=4.

The supported test configurations are specified in table A.6.6.13.4.1-1.

Table A.6.6.13.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In sub-test 1, reducedDL-PRS-ProcessingSamples shall be included in the location information request and set to ‘requested’.

During T1, a PPW shall be configured for the PCell and be activated via DL MAC CE. The last PDSCH containing the MAC CE shall be transmitted before slot #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first PPW instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.13.4.1-2, and cell specific test parameters during T2 are listed in table A.6.6.13.4.1-3.

Table A.6.6.13.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1 | PPW.1 | As defined in A.3.33 |
|  |  | 2 | PPW.1 | As defined in A.3.33 |
|  |  | 3 | PPW.1 | As defined in A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1 | 3 |  |
|  |  | 2 | 3 |  |
|  |  | 3 | 3 or 2.34 Note 1 |  |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset between serving and neighbour cells |  |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: If UE indicates support of CP length for the receive time difference threshold, the time offset is set to 2.34 us, otherwise 3 us. |  |  |  |  |

Table A.6.6.13.4.1-3: Cell specific test parameters during T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 0 | -2.41 | -6 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 2.23 | -2 | -1.73 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -100 | -99.73 | -108 |
|  |  | 2 | -95.77 | -100 | -99.73 | -108 |
|  |  | 3 | -92.77 | -97 | -96.73 | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -101 | -101 | -104 | -108 |
|  |  | 2 | -101 | -101 | -104 | -108 |
|  |  | 3 | -98 | -98 | -101 | -105 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/19.08 MHz | 2 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/47.88 MHz | 3 | -57.73 | -60.59 | -57.73 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.4.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.6, starting from the beginning of time interval T2, with Nsample=1 for sub-test 1 and Nsample=4 for sub-test 2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.14	UE Rx-Tx time difference measurements

#### A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.6.14.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.6.14.1.1-1.

Table A.6.6.14.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.1.1-2 and table A.6.6.14.1.1-3 respectively.

Table A.6.6.14.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.1.1-4: Void

##### A.6.6.14.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA

##### A.6.6.14.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations in listed in table A.6.6.9.2.1-1.

Table A.6.6.14.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Cell 1 and Cell 2 are on different RF channels in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.2.1-2 and table A.6.6.14.2.1-3 respectively.

Table A.6.6.14.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 2 | For Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io | dBm/19.08 MHz | 1 | N/A | -65.19 | N/A | -66.74 |
|  | dBm/19.08 MHz | 2 |  | -65.19 |  | -66.74 |
|  | dBm/47.88 MHz | 3 |  | -61.21 |  | -62.76 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.2.1-4: Void

##### A.6.6.14.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number

##### A.6.6.14.3.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 with Nsample = 1 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.6.14.3.1-1.

Table A.6.6.14.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. reducedDL-PRS-ProcessingSamples shall be included in the location information request and set to ‘requested’. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.3.1-2 and table A.6.6.14.3.1-3 respectively.

Table A.6.6.14.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -Infinity | -104 |
|  |  | 2 | -100 | -100 | -Infinity | -104 |
|  |  | 3 | -97 | -97 | -Infinity | -101 |
| Io | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.3.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5 with Nsample=1.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA

##### A.6.6.14.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.6 in AWGN propagation condition in FR1 in standalone scenario. There are two sub-tests in the test, sub-test 1 is to verify the delay requirements with Nsample=1, and sub-test 2 is to verify the delay requirements with Nsample=4.

The supported test configurations in listed in table A.6.6.14.4.1-1.

Table A.6.6.14.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of PRS processing window containing the PRS resources.

The UE is configured with PRS processing window before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.4.1-2 and table A.6.6.14.4.1-3 respectively.

Table A.6.6.14.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PRS processing window |  | 1,2,3 | PPW.1 | As defined in A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 Note1 |  |
|  |  | 2 | 3 Note1 |  |
|  |  | 3 | 3 or 2.34 Note1 |  |
| Expected RSTD | s | 1,2,3 | 0 |  |
| Expected RSTD uncertainty | s | 1,2,3 | Same as time offset between serving and neighbour cells |  |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: The value is up to the UE capability. The possible UE capability value: (1/4 symbol, 1/2 symbol, CP length, half of slot). When the UE reported value is > 3 us, the time offset between serving and neighbour cells is set to 3 us; when the UE reported value is < 3 us, the time offset between serving and neighbour cells is set to the UE reported value. |  |  |  |  |

Table A.6.6.14.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-tets 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 0 | -2.41 | -6 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | 2.23 | -2 | -1.73 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -100 | -99.73 | -108 |
|  |  | 2 | -95.77 | -100 | -99.73 | -108 |
|  |  | 3 | -92.77 | -97 | -96.73 | -105 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/19.08 MHz | 2 | -61.71 | -64.57 | -61.71 | -64.57 |
|  | dBm/47.88 MHz | 3 | -57.73 | -60.59 | -57.73 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.6.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs

##### A.6.6.14.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured, and when UE is requested to measure a PRS resource with multiple RxTx TEGs.

The supported test configurations in listed in table A.6.6.14.4.1-1.

Table A.6.6.14.4.11: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request. In nr-Multi-RTT-RequestLocationInformation, measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 shall be set to ‘n2’.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.4.1-2 and table A.6.6.14.4.1-3 respectively.

Table A.6.6.14.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Number of cells provided in assistance data |  | 1, 2, 3 | 4 | Including the reference cell |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.14.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5, with kmultiTEG=2 if UE does not support or indicate value ‘n1’ for measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul, and  kmultiTEG=1 otherwise.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA

##### A.6.6.14.6.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 9.9.4.9 for UE Rx-Tx measurements with PRS bandwidth aggregation. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform UE Rx-Tx measurements by aggregating two intra-band contiguous positioning frequency layers (PFLs) in FR1.

The supported test configurations in listed in table A.6.6.14.6.1-1.

Table A.6.6.14.6.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: Cell 1 (PCell) and Cell 2 (neighbor cell). Each cell is sssociated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, both cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 transmit PRS only during the second time interval of duration T2. Similarly, the UE is configured to transmit positioning SRS during only during the second time interval of duration T2.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12, shall be provided to the UE during T1. The last TTI of the last message shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The NR-Multi-RTT-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The NR-Multi-RTT-RequestLocationInformation message provided to the UE must request bandwidth aggregated measurements via jointMeasurementsReq and nr-DL-PRS-JointMeasurementRequestedPFL-List.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The general test parameters and cell specific test parameters are as given in table A.6.6.14.6.1-2 and table A.6.6.14.6.1-3, respectively.

Table A.6.6.14.6.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2, all signals/channels except PRS in PFL2 are allocated within this channel. |
|  |  |  | 2 | For both Cell 1 and Cell 2, only PRS in PFL2 is allocated within this channel. |
| BWchannel | MHz | 1 | 20: NRB,c = 106 | For both RF channels |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells/TRPs |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if the UE supports it, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.6.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns Note 1 |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1,4 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1, 4 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration (for both PFL1 and PFL2) |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info (dl-PRS-MutingOption1 for both PFL1 and PFL2) |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/19.08 MHz | 1 | N/A | -65.05 | N/A | -65.05 |
|  | dBm/19.08 MHz | 2 |  | -65.05 |  | -65.05 |
|  | dBm/47.88 MHz | 3 |  | -61.06 |  | -61.06 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. NOTE 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.6.14.6.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.9.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25A.3.

### A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting

#### A.6.6.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode, when the UE has entered Idle mode. Additionally, test that the UE performs the required measurements on the serving cell and the configured inter-RAT carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in clause 4.4. In the test, connected mode DRX configuration is not configured in either PCell or PSCell.

Additionally, the purpose of this test is to verify that the SS-RSRP, SS-RSRQ, RSRP and RSRQ measurement accuracy is within the specified limits. This test will verify the accuracy requirements in sections 10.1.2B and 10.1.7B for intra-frequency measurements and section 10.2.2 and 10.2.3 for the inter-RAT measurements for the supported test configurations in tables A.6.6.15.1-4 and A.6.6.15.1-5.

The supported test configurations are given in table A.6.6.15.1-1. The test parameters are given in tables A.6.6.15.1-2, A.6.6.15.1-3, A.6.6.15.1-4 and A.6.6.15.1-5 below. In the test there are two cells, Cell 1, which is the PCell in connected, and serving cell in idle mode, on radio channel 1 in FR1, and Cell 2, which is the PSCell in connected, and measured LTE inter-RAT cell in idle mode, on radio channel 2 in LTE.

For the purpose of testing absolute accuracy in idle mode in this set of test cases the cells in idle mode are on different carrier frequencies (NR FR1 and LTE).  The absolute accuracy of RSRP and RSRQ inter-RAT measurements are tested by using the parameters in table A.6.6.15.1-4 and table A.6.6.15.1-5. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. During T1 Cell 2, the PSCell, shall be configured.

Time duration T2 starts when UE has transmitted random access preamble on the PSCell. During T2, the UE is configured with idle mode CA measurements with the PSCell carrier as the target carrier. The connection is released 500 ms after T2 when the UE has sent random access preamble on the PSCell.

T3 starts when the connection is released. During the time periods T3 and T4 the UE is in Idle mode with the serving cell on the FR1 carrier. The UE is configured to perform inter-RAT idle mode CA/DC measurements on Cell 2 carrier. After the connection release and during T3, 1000 ms after T3 is started, the signal level of the inter-RAT carrier configured for idle mode CA/DC measurements is changed at which time T4 starts. T5 starts 65 s after T4, when the UE is paged for connection setup and UE is requested by the network to report idle mode CA/DC measurements.

Table A.6.6.15.1-1: Supported test configurations for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15 kHz BW 10 MHz – LTE FDD 10 MHz |
| 2 | FR1 FDD SSB SCS 15 kHz BW 10 MHz – LTE TDD 10 MHz |
| 3 | FR1 TDD SSB SCS 30 kHz BW 40 MHz – LTE FDD 10 MHz |
| 4 | FR1 TDD SSB SCS 30 kHz BW 40 MHz – LTE TDD 10 MHz |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.15.1-2: General test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| PSCell |  | Cell 2 | PSCell on RF channel number 2 in LTE. Cell 2 is of higher priority. |
| DRX |  | OFF | For both PCell and PSCell once configured |
| PRACH configuration in Cell 2 |  | PRACH_2CE | PRACH configuration as specified in clause A.3.16 in TS 36.133 |
| T331 | s | 300 |  |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | 2 |  |
| T1 | s | 0.5 | During this time the PCell is known and PSCell is configured. |
| T2 | s | 0.5 | PSCell access. |
| T3 + T4 | s | 66 | During this time the UE is configured to perform inter-RAT measurements in idle mode on the PSCell carrier. |
| T5 | s | 0.5 | UE is paged and connection is setup. Network requests measurement report from the UE. |

Table A.6.6.15.1-3: Cell specific test parameters for NR cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | 1,2,3,4 | N/A |  |  |  |  |
| Assumption for UE beams Note 5 R: Rough |  | 1,2,3,4 | N/A | N/A | R | R | N/A |
| Frequency Range |  | 1,2,3,4 | FR1 |  |  |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |  |  |
|  |  | 3, 4 | TDD |  |  |  |  |
| TDD Confguration 1: TDDConf.1.1 2: TDDConf.2.1 |  | 1,2 | - |  |  |  |  |
|  |  | 3,4 | 1 | 1 | 2 | 2 | 1 |
| BWchannel 1: 10: NPRB,c = 52 2: 40: NPRB,c = 106 | MHz | 1, 2 | 1 | 1 | - | - | 1 |
|  |  | 3, 4 | 2 | 2 | - | - | 2 |
| Initial Downlink BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  |  |  |
| Dedicated Downlink BWP configuration 1: DLBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - | - | 1 |
| Dedicated Uplink BWP configuration 1: ULBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - | - | 1 |
| PDSCH Reference Measurement Channel 1: SR.1.1 FDD 2: SR.2.1 TDD | FDD | 1,2 | 1 | 1 | 1 | 1 | 1 |
|  | TDD | 3,4 | 2 | 2 | 2 | 2 | 2 |
| TRS configuration |  | 1,2,3,4 | - |  |  |  |  |
| TCI state |  | 1,2,3,4 | - |  |  |  |  |
| RMSI CORESET parameters | FDD | 1,2 | CR.1.1 FDD |  |  |  |  |
|  | TDD | 3,4 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters | FDD | 1,2 | CCR.1.1 FDD |  |  |  |  |
|  | TDD | 3,4 | CCR.2.1 TDD |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3,4 | OP.1 defined in A.3.2.1 |  |  |  |  |
| SSB configuration 1: SSB.1 FR1 2: SSB.2 FR1 |  | 1,2 | 1 |  |  |  |  |
|  |  | 3,4 | 2 |  |  |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.2 |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 | - | - | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDSCH to PDSCH |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of OCNG DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| Noc Note2 | dBm/ 15 kHz | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 | -95 | -95 | -95 | -95 | -95 |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | 5 | 5 | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | 5 | 5 | 5 |
| SS-RSRPNote3,4 | dBm/SCS | 1,2 | -93 | -93 | -93 | -93 | -93 |
|  |  | 3,4 | -90 | -90 | -90 | -90 | -90 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2 | -63.85 | -63.85 | -63.85 | -63.85 | -63.85 |
|  | dBm/ 38.16 MHz | 3,4 | -57.76 | -57.76 | -57.76 | -57.76 | -57.76 |
| Qrxlevmin | dBm/SCS | 1, 2 | - | - | -140 |  | - |
|  |  | 3, 4 | - | - | -137 |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | SS-RSRP |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not Sent |  | - |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not Sent |  | - |
| Threshx, high | dB | 1,2,3,4 | - |  | 48 |  | - |
| Threshserving, low | dB | 1,2,3,4 | - |  | 44 |  | - |
| Threshx, low | dB | 1,2,3,4 | - |  | 50 |  | - |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.6.6.15.1-4: Cell specific test parameters for LTE cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 |  | T5 |
| Frequency Range |  | 1,2,3,4 | LTE |  |  |  |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |  |  |  |
|  |  | 2, 4 | TDD |  |  |  |  |  |
| BWchannel | MHz | 1,2,3,4 | 10 | 10 | - | - |  | 10 |
| Measurement bandwidth |  | 1,2,3,4 | - | - | 22-27 | 22-27 |  | - |
| PDSCH Reference Measurement Channel 1: R.1 FDD 2: R.1 TDD | FDD | 1,3 | 1 | 1 | - | - |  | 1 |
|  | TDD | 2,4 | 2 | 2 | - | - |  | 2 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 in 36.133 1: R.6 FDD 2: R.6 TDD |  | 1,3 | 1 | 1 | - | - |  | 1 |
|  |  | 2,4 | 2 | 2 | - | - |  | 2 |
| OCNG Patterns defined in A.3.2.1.1 (OP.2 FDD) and A.3.2.1.2 (OP.2 TDD) in 36.133 1: OP.2 FDD 2: OP.2 TDD |  | 1,3 | 1 |  |  |  |  |  |
|  |  | 2,4 | 2 |  |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |  |
| PBCH_RA | dB | 1,2,3,4 | N/A | N/A | 0 | 0 |  | N/A |
| PBCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PSS_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| SSS_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PCFICH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PHICH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PHICH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDCCH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDCCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDSCH_RA |  |  | N/A | N/A | 0 | 0 |  | N/A |
| PDSCH_RB |  |  | N/A | N/A | 0 | 0 |  | N/A |
| OCNG_RANote 1 |  |  | N/A | N/A | 0 | 0 |  | N/A |
| OCNG_RBNote 1 |  |  | N/A | N/A | 0 | 0 |  | N/A |
| Noc Note2 | dBm/ 15 kHz | 1,2 | -98 | -98 | -98 | -98 |  | -98 |
|  |  | 3,4 |  |  |  |  |  |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | -3 | 8 |  | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | -3 | 8 |  | 5 |
| SS-RSRPNote3,4 | dBm/SCS | 1,2,3,4 | -93 | -93 | -101 | -90 |  | -93 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2, 3, 4 | -61.01 | -61.01 | -65.44 | -58.57 |  | -61.01 |
| Qrxlevmin | dBm/SCS | 1,2, 3, 4 | - | - | -140 |  |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 |  | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | RSRP and RSRQ |  |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |  |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not Sent |  | - |  |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not Sent |  | - |  |
| Threshx, high | dB | 1,2,3,4 | - |  | 48 |  | - |  |
| Threshserving, low | dB | 1,2,3,4 | - |  | 44 |  | - |  |
| Threshx, low | dB | 1,2,3,4 | - |  | 50 |  | - |  |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

Table A.6.6.15.1-5: General idle mode test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell |  | 1, 2, 3, 4 | Cell 1 | The UE camps on Cell 1 which is the former PCell. |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 | The UE shall perform inter-RAT measurements on Cell 2 which is the former PSCell. |
| RF Channel Number |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  | 1, 2, 3, 4 | 3 s | Synchronous cells |
| Access Barring Information | - | 1, 2, 3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  | 1, 2 | SSB.1 FR1 | Serving cell |
|  |  | 3, 4 | SSB.2 FR1 | Serving cell |
| SMTC configuration Serving cell |  | 1, 2, 3, 4 | SMTC.2 |  |
| DRX cycle length | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  | 1, 2, 3, 4 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  | 1, 2, 3, 4 | Not configured |  |
| T3 | s | 1, 2, 3, 4 | 0.5 | T3 needs to be defined so that cell measurement time is taken into account. |
| T4 | s | 1, 2, 3, 4 | 65 | T4 needs to be defined so that cell measurement time is taken into account. |

#### A.6.6.15.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During time durations T1 the UE shall start transmitting preamble on PSCell. During T2 the UE perform intra-frequency measurements on the PCell and the PSCell.

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 and T4 the UE is camped in Idle mode and at T4 the signal level of Cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to section 4.4.

At the start of T5 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell including idle mode CA/DC measurement from Cell 2.

After receiving the requested early measurement report, the test equipment verifies that the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in sections 10.1.2B and 10.1.7B and sections 10.2.4 and 10.2.5, respectively and test ends.

### A.6.6.16	PRS-RSRPP measurements

#### A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.6.16.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 9.9.6.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.6.16.1.1-1.

Table A.6.6.16.1.11: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.16.1.1-2, and cell specific test parameters are listed in table A.6.6.16.1.1-3.

Table A.6.6.16.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.16.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.1.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.6.16.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the reduced sample measurement delay requirements specified in clause 9.9.6.5 in an environment with AWGN propagation conditions for reduced number of samples. In this test UE that supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in table A.6.6.16.2.1-1.

Table A.6.6.16.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.6.16.2.1-2, and cell specific test parameters are listed in table A.6.6.16.2.1-3.

Table A.6.6.16.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.16.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -Infinity | -104 |
|  |  | 2 | -100 | -100 | -Infinity | -104 |
|  |  | 3 | -97 | -97 | -Infinity | -101 |
| Io | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.2.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap

##### A.6.6.16.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 9.9.6.6 in an environment with AWGN propagation conditions. Reporting delay test for gapless PRS measurement is conducted assuming that the PRS has higher priority, i.e., state 1, than all other DL signals/channels and is transmitted within active DL BWP of UE. Two sub-tests are defined, sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. For sub-test 2 LMF indicates UE to perform PRS measurement with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. The cell specific parameters for sub-test 1 and sub-test 2 are defined in table A.6.6.16.3.1-3.

The supported test configurations are specified in table A.6.6.16.3.1-1.

Table A.6.6.16.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first PRS processing window instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The UE is configured with PPW before start of T2.

The general test parameters are listed in table A.6.6.16.3.1-2, and cell specific test parameters are listed in table A.6.6.16.3.1-3.

Table A.6.6.16.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1,2,3 | PPW.1 | As defined in A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | - set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs - 3µs otherwise | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | ms | 1, 2, 3 | 10 | In this test UE is configured to measure single PFL within the configured PPW. |

Table A.6.6.16.3.1-3: Cell specific test parameters T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | 0 | -12.12 | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | 2.23 | -10 | -1.72 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -95.77 | -108 | -99.73 |
|  |  | 2 | -100 | -95.77 | -108 | -99.73 |
|  |  | 3 | -97 | -92.77 | -105 | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -104 | -104 |
|  |  | 2 | -100 | -100 | -104 | -104 |
|  |  | 3 | -97 | -97 | -101 | -101 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -61.71 | -64.57 | -61.71 |
|  | dBm/19.08 MHz | 2 | -64.57 | -61.71 | -64.57 | -61.71 |
|  | dBm/47.88 MHz | 3 | -60.59 | -57.73 | -60.59 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.16.3.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.6.6, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.17	SA event triggered reporting tests with Pre-MG

#### A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG

##### A.6.6.17.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3. And this test will also jointly verify Pre-configured measurement gap activation/deactivation delay in clause 8.19.2.

##### A.6.6.17.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.17.1.2-1 and A.6.6.17.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively.

During the duration of T1,

-	UE is configured with 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2, before starting the test.

-	BWP-1 includes bandwidth of the initial DL BWP and SSB. UE is expected to deactivate the Pre-MG when this BWP is active.

-	BWP-2 does not include bandwidth of the initial DL BWP and SSB. UE is expected to activate the Pre-MG when this BWP is active.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	At the start of time duration T2, the serving gNB can trigger Pre-MG activation starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. And UE is expected to complete the Pre-MG activation within T2.

-	At the start of time duration T3, the UE may not have any timing information of neighbor cell to be measured (e.g. Cell 2). And UE was expected to complete the measurements of SSBs with the activated Pre-MG within T3.

There are two BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1.

Table A.6.6.17.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.17.1.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.2 |  |
| T3 | s | 1,2, 3 | 5 |  |
| bwp-InactivityTimer | ms |  | 500 |  |

Table A.6.6.17.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| Intial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  |  | N/A |  |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  |  | N/A |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  | SSB |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 |  | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | 4 |  | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 |  | -Infinity |  | -94 |
|  |  | 2 | -94 | -94 |  | -Infinity |  | -94 |
|  |  | 3 | -91 | -91 |  | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 |  | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.17.1.3	Test Requirements

During T1 the UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1 st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5 ms as defined in clause 8.19.2.

During T3, the UE shall NOT be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T3.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation

##### A.6.6.17.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the pre-MG activation and deactivation delay requirements in clause 8.19.2 and the intra-frequency cell search requirements in clause 9.2.6.2 and 9.3.4.

##### A.6.6.17.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The supported test configurations are listed in table A.6.6.17.2.2-1, general test parameters are listed in table A.6.6.17.2.2-2, and cell specific test parameters are listed in table A.6.6.17.2.2-3.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. A pre-MG is configured before the test.

The UE is configured with 2 dedicated BWPs, BWP-1 and BWP-2. BWP-1 includes bandwidth of the SSB, and preConfGapStatus for BWP-1 is set to ‘0’; BWP-2 does not include bandwidth of the SSB, and preConfGapStatus for BWP-2 is set to ‘1’.

The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Before the test starts, UE is switched to BWP-1.

The time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2 and complete pre-MG activation during T2.

During T3, UE shall perform intra-frequency measurement with pre-MG activated.

During time duration T1 and T2, the UE shall not have any timing information of Cell 2.

Table A.6.6.17.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.17.2.2-2: General test parameters for SA intra-frequency event triggered reporting with with pre-configured measurement gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.1 |  |
| T3 | s | 1, 2, 3 | 5 |  |

Table A.6.6.17.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with with pre-configured measurement gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | N/A |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-1 |  | 1, 2, 3 | DLBWP.1.3 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-1 |  | 1, 2, 3 | ULBWP.1.3 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-2 |  | 1, 2, 3 | DLBWP.1.2 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-2 |  | 1, 2, 3 | ULBWP.1.2 |  |  |  | N/A |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  |  | N/A |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1,2,3 | 4 | 4 |  | -1.46 | -Infinity | -Infinity | -1.46 |
|  | dB | 1,2,3 | 4 | 4 |  | 4 | -Infinity | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1,2 | -94 | -94 |  | -94 | -Infinity | -Infinity | -94 |
|  |  | 3 | -91 | -91 |  | -91 | -Infinity | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2 | -64.60 | -64.60 |  | -62.25 | -64.60 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 |  | -56.16 | -58.50 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2 and T4. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.6.6.17.2.3	Test Requirements

During T1, UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T3. During T3, UE is not required to report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.17.3	Void

##### A.6.6.17.3.1	Void

##### A.6.6.17.3.2	Void

##### A.6.6.17.3.3	Void

### A.6.6.18	SA event triggered reporting tests with concurrent gaps

#### A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers

##### A.6.6.18.1.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gaps capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR cell 3 as neighbour cell in FR1 on NR RF channel 3.  The test parameters are given in tables A.6.6.18.1.1-1, A.6.6.18.1.1-2 and A.6.6.18.1.1-3.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in table A.6.6.18.1.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR cell 3.

Table A.6.6.18.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.18.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR Cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour Cell 2 |  | Config 1, 2, 3 | 5 ms |  | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

Table A.6.6.18.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 920 ms for Cell 2 and 1280 ms for cell 3 from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers

##### A.6.6.18.2.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gap capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR cell 3 as neighbour cell in FR1 on NR RF channel 3.  The test parameters are given in tables A.6.6.18.2.1-1, A.6.6.18.2.1-2 and A.6.6.18.2.1-3. The TE schedules continuous DL data on PCell throughout the test.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in table A.6.6.18.2.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR cell 3.

Table A.6.6.18.2.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.18.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR Cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 39 for MeasGapId #1 4 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour Cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour Cell 2 |  | Config 1, 2, 3 | 5 ms |  | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

Table A.6.6.18.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

##### A.6.6.18.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of concurrent inter-RAT E-UTRAN and NR FR1 measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clauses 9.4.2, 9.4.3, 9.3.4 and 9.3.5.

In each test there are three cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the NR PCell, Cell 2 is an Inter-frequency NR FR1 neighbour cell on NR RF channel 2 and Cell 3 is an inter-RAT E-UTRAN neighbour cell on LTE RF channel 3.

In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used for the E-UTRAN cell (cell 3). In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used for the NR FR1 cell (Cell 2).

Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2 and Cell 3.

In the test two concurrent per-UE measurement gap pattern configurations # 0 as defined in table A.6.6.18.3.1-2 are provided for a UE. Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #1 as defined in table A.6.6.18.3.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for NR RF channel numbers #2 and LTE RF channel #3, respectively.

Supported test configurations are shown in table A.6.6.18.3.1-1. General test parameters are provided in table A.6.6.18.3.1-2 below. Test parameters for Cell 1, Cell 2 and Cell 3, valid for both time duration T1 and T2, are provided in tables A.6.6.18.3.1-3 and A.6.6.18.3.1-4, respectively.

The test parameters and configurations are given in tables A.6.6.18.3.1-1, A.6.6.18.3.1-2, and A.6.6.18.3.1-3.

Table A.6.6.18.3.1-1: Supported test configurations in SA concurrent inter-RAT E-UTRAN and NR FR1 inter-frequency event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE: FDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE: FDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE: FDD. NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE: TDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE: TDD. NR: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE: TDD. NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.6.6.18.3.1-2: General test parameters for SA concurrent inter-RAT E-UTRAN and NR FR1 inter-frequency event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 - 6 | 1, 2 | 2 NR carrier frequency is used in the test. |
| LTE RF Channel Number |  |  | 3 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | 1 - 6 | As specified in tables A.6.6.18.3.1-3 and A.6.6.18.3.1-4. |  |
| Active cell |  | 1 - 6 | NR Cell 1 (PCell in FR1) | Cell 1 is on NR RF channel number 1 |
| Neighbour Cell 1 |  | 1 - 6 | Cell 2 (NR FR1) | Cell 2 is on NR RF channel number 2 |
| Neighbour Cell 2 |  | 1 - 6 | Cell 3 (LTE) | Cell 3 is on LTE RF channel number 3 |
| Gap Pattern Id |  | 1 - 6 | 1 for MeasGapId #1 1 for MeasGapId #2 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| Measurement gap offset | ms | 1 – 6 | 19 for MeasGapId #1 39 for MeasGapId #2 |  |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 1 and Cell 2 |
| Inter-RAT E-UTRAN measurement quantity |  |  | RSRP | Measurement quantity for Cell 3 |
| b2-Threshold1 | dBm |  | As specified in table A.6. 6.18.3.1-3 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm |  | -101 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| Hysteresis | dB | 1 – 6 | 0 |  |
| TimeToTrigger | s | 1 – 6 | 0 |  |
| Filter coefficient |  | 1 – 6 | 0 | L3 filtering is not used |
| offsetMO | dB | 1 - 6 | 6 | NR Cell 2 |
| a4-Threshold | dBm | 1 – 6 | -105 | NR Cell 2 |
| DRX |  |  | OFF | N.A |
| Time offset between serving and neighbour cells |  | 1, 4 | 3 ms | Asynchronous cells NR cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous NR cells. |
| T1 | s | 1 – 6 | 5 | for LTE Cell 3 and NR FR1 Cell 2 |
| T2 | s | 1 - 6 | 5 | for LTE Cell 3 |
|  |  |  | 10.3 for PC1 and PC5; 6.5 for other PC | for NR FR1 Cell 2 |

Table A.6.6.18.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1-6 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1, 4 | FDD |  | FDD |  |  |
|  |  |  | Config 2, 3, 5, 6 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1, 4 | Not Applicable |  | Not Applicable |  |  |
|  |  |  | Config 2, 5 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  |  | Config 3, 6 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | Config 1, 4 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |  |
|  |  |  | Config 2, 5 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |  |
|  |  |  | Config 3, 6 | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1-6 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
| CSI-RS for tracking parameters on NR RF Channel 1 |  |  | Config 1, 4 | TRS.1.1 FDD |  |  |  |  |
|  |  |  | Config 2, 5 | TRS.1.1 TDD |  |  |  |  |
|  |  |  | Config 3, 6 | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  | Config 1-6 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference |  |  | Config 1, 4 | SR.1.1 FDD |  | - |  |  |
| measurement channel |  |  | Config 2, 5 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3, 6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1, 4 | CR.1.1 FDD |  | - |  |  |
| Channel |  |  | Config 2, 5 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3, 6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1, 4 | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2, 5 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3, 6 | CCR.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | Config 1, 4 | SMTC.2 |  | SMTC.2 |  |  |
|  |  |  | Config 2, 3, 5, 6 | SMTC.1 |  | SMTC.1 |  |  |
| SSB configuration |  |  | Config 1, 4 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  |  | Config 2, 5 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  |  | Config 3, 6 | SSB.2 FR1 |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1, 2, 4, 5 | 15 |  | 15 |  |  |
|  |  |  | Config 3, 6 | 30 |  | 30 |  |  |
| b2-Threshold1 |  | dBm | Config 1, 2, 4, 5 | -81 |  | - |  |  |
|  |  |  | Config 3, 6 | -78 |  | - |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1-6 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Noc note2 |  | dBm/15 KHz | 1-6 | -98 |  |  |  |  |
| Noc note2 |  | dBm/SCS | Config 1, 2, 4, 5 | -98 |  |  |  |  |
|  |  |  | Config 3, 6 | -95 |  |  |  |  |
| Ês/Noc |  | dB | Config 1 - 6 | 4 |  | -Infinity |  | 4 |
| Ês/Iot note3 |  | dB | Config 1 - 6 | 4 |  | -Infinity |  | 4 |
| SSB_RP note3 |  | dBm/SCS | Config 1, 2, 4, 5 | -94 |  | -Infinity |  | -94 |
|  |  |  | Config 3, 6 | -91 |  | -Infinity |  | -91 |
| Io note3 |  | dBm/9.36 MHz | Config 1, 2, 4, 5 | -64.59 |  | -70.05 |  | -64.59 |
|  |  | dBm/38.16 MHz | Config 3, 6 | -58.49 |  | -63.94 |  | -58.49 |
| Propagation condition |  |  | Config 1 - 6 | AWGN |  |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  | Config 1 - 6 | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void |  |  |  |  |  |  |  |  |

Table A.6.6.18.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 3 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NPRB,c /50) | -59.13+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.18.3.2	Test Requirements

In this test with per-UE gap, the UE shall send one Event A4 triggered measurement report for Cell 2, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

10240 for UE supporting power class 1 and 5, or

6400 for UE supporting other power class.

The UE shall send one Event B2 triggered measurement report for Cell 3 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

##### A.6.6.18.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA NR measurements with concurrent gaps requirements in clause 9.2.6(when one of concurrent gaps in same frequency layer of serving cells), 9.3.6(when one of concurrent gaps in the different frequency layer of serving cells) and 9.9.2(when one of concurrent gaps used for PRS measurement).

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR cell 3 as neighbor cell in FR1 on NR RF channel 1.  The test parameters are given in tables A.6.6.18.4.1-1, A.6.6.18.4.1-2 and A.6.6.18.4.1-3.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #24 as defined in table A.6.6.18.4.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #1 and #2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #2 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

Table A.6.6.18.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.18.4.1-2: General test parameters for SA inter-frequency event triggered reporting for concurrent gaps with partially partial overalpping scenario for SSB-based measurements and PRS measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two FR1 carrier frequencies are used for the NR cells. |
| Active cell |  | 1,2,3 | NR Cell 1 (PCell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1,2,3 | NR Cell 2, NR cell 3 | Cell 2 is an inter-frequency cell neighbor cell Cell 3 is an intra-frequency neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | 1,2,3 | 0 for MeasGapId #1 24 for MeasGapId #2 |  |
| Measurement gap offset | ms | 1,2,3 | 7 for MeasGapId #1 11 for MeasGapId #2 |  |
| A3-Offset | dB | 1, 2, 3 | -9 |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.18.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overalpping scenario for SSB-based measurements and PRS measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
|  |  |  | 3 | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
|  |  |  | 3 | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| SSB parameters |  |  | 1 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | 1 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  | PRS.2.4 FR1 |  |
| PRS muting configuation |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  | ‘01’ |  |
| PDSCH/PDCCH |  | KHz | 1, 2 | 15 |  |  |  |  |  |
| subcarrier spacing |  |  | 3 | 30 |  |  |  |  |  |
| Note2 |  | dBm/15 kHz Note5 | 1 | -98 |  | -98 |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  | -98 |  |
|  |  |  | 3 | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS Note4 | 1 | -98 |  | -98 |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -94 | -94 | -Infinity | -94 | N/A | N/A |
|  |  |  | 2 | -94 | -94 | -Infinity | -94 | N/A | N/A |
|  |  |  | 3 | -91 | -91 | -Infinity | -94 | N/A | N/A |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -Infinity | -101 | N/A | N/A | -Infinity | -108 |
|  |  |  | 2 | -Infinity | -101 | N/A | N/A | -Infinity | -108 |
|  |  |  | 3 | -Infinity | -98 | N/A | N/A | -Infinity | -105 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A | N/A | -Infinity | -10 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/9.36 MHz | 1 | -64.59 |  | -70.05 | -64.59 | -64.59 |  |
|  |  | dBm/9.36 MHz | 2 | -64.59 |  | -70.05 | -64.59 | -64.59 |  |
|  |  | dBm/38.16 MHz | 3 | -58.49 |  | -63.94 | -58.49 | -58.49 |  |
| Propagation Condition |  |  | 1,2,3 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io values are derived without considering PRS-RSRP. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.4.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 2, with a measurement reporting delay less than 1840 ms from the beginning of time period T2.

The PRS RSRP measurement time fulfils the requirements specified in clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.19	SA event triggered reporting tests with NCSG

#### A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1

##### A.6.6.19.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.7.1 and 9.2.7.2, and also verify the scheduling availability during intra-frequency measurement with NCSG in clause 9.2.7.3.

The serving frequency should be selected for which UE reports ‘ncsg’.

##### A.6.6.19.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.19.1.2-1, A.6.6.19.1.2-2 and A.6.6.19.1.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

The UE is capable of NCSG and report ‘ncsg’ through NeedForGapNCSG-InfoNR for PCell.

NCSG pattern configuration # 0 as defined in table A.6.6.19.1.2-2 is provided for UE regardless of UE capable or incapable of supporting per-FR NCSG.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.19.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.19.1.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| NCSG type |  | 1, 2, 3 | Per-UE | Per-UE NCSG patter is configured. |
| NCSG pattern |  | 1, 2, 3 | 0 |  |
| Visible interruption repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement length | ms | 1, 2, 3 | 5 |  |
| NCSG offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 1 |  |

Table A.6.6.19.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with NCSG for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.19.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

During T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement

##### A.6.6.19.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.10.

The serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

##### A.6.6.19.2.2	Test parameters

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘ncsg’ for the target frequency on NR RF channel 2.

The test parameters are given in tables A.6.6.19.2.2-1, A.6.6.19.2.2-2 and A.6.6.19.2.2-3.

NCSG pattern configuration # 0 as defined in table A.6.6.19.2.2-2 is provided for UE regardless of UE capable or incapable of supporting per-FR NCSG.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.19.2.2-1: SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.19.2.2-2: General test parameters for event triggered reporting for FR1 with NCSG for inter-frequency measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| NCSG Pattern Id |  | Config 1,2,3 | 0 | As specified in table 9.1.9.3-1. |
| NCSG offset |  | Config 1,2,3 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.19.2.2-3: Cell specific test parameters for SA event triggered reporting for FR1 with NCSG for inter-frequency measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.19.2.3	Test Requirements

In test 1 and 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG

##### A.6.6.19.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements based on NCSG when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clauses 9.4.2 and 9.4.3.

The serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

##### A.6.6.19.3.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.19.3.2-1. General test parameters are provided in table A.6.6.19.3.2-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.19.3.2-3 and A.6.6.19.3.2-4, respectively.

Table A.6.6.19.3.2-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.19.3.2-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on LTE RF channel number 2 |
| NCSG Pattern Id |  | 0 | As specified in clause Table 9.1.9.3-1. |
| NCSG offset | ms | 39 |  |
| NCSG mgta | ms | 0 |  |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | As specified in table A.6.3.1.4-3 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.19.3.2-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | --53.84 | --66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.19.3.2-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NPRB,c /50) | -59.13+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.19.3.3	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX

##### A.6.6.19.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the cell search requirements on SCC with deactivated SCell in clauses 9.2.7.1 and 9.2.7.2.

##### A.6.6.19.4.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 SCell (Cell 2) and FR1 neighbour cell (Cell 3) on the same frequency as the SCell. The SCell is deactivated during the test. The test parameters for PCell, the SCell and the neighbour cell are given in table A.6.6.19.4.2-1 and A.6.6.19.4.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2.

Table A.6.6.19.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.19.4.2-2: General test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Configured deactivated SCell |  | 1, 2, 3 | Cell 2 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 |  |
| RF Channel Number |  | 1, 2, 3 | 2: Cell 2 and Cell 3 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE NCSG |  |
| NCSG pattern |  | 1, 2, 3 | ID # 0 | Defined in table 9.1.9.3-1 |
| MGTA | ms | 1, 2, 5 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1, 2, 3 | 160 |  |
| Cell 2 timing offset to Cell 1 | s | 1, 2, 3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1, 2, 3 | ≤ Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.19.4.2-3: NR Cell specific test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |  |  |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.19.4.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1600 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For a test to be considered successful requirements on both Event A6 detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation

#### A.6.6.20.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in clause 9.12.4.1 for measurement delay and clause 10.1.39.2 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in table A.6.6.20.1-1.

Table A.6.6.20.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. If the test is based on PRS, the Cell 1 mutes PRS transmission during T1 and transmits PRS during T2.

If the test is based on PRS, the MeasObjectRxTxDiff-r17 with prs-Ref-r17 , measObject  with measObjectRxTxDiff-17, and NR-DL-PRS-PDC-Info as defined in TS 38.331 shall be provided to the UE during T1.

The last TTI containing the RRC configuration shall be provided to the UE T ms before the start of T2, where T = 10 ms.

The beginning of the time interval T2 shall be aligned with the beginning of the first PRS resources.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.20.1-2. The test parameters for PRS are given Table A.6.6.20.1-3.

Table A.6.6.20.1-2: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NPRB,c = 52 |
|  |  | 2 | 10: NPRB,c = 52 |
|  |  | 3 | 40: NPRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 5 |
| T2 | s | 1, 2, 3 | 10 |

Table A.6.6.20.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  |
| SRS configuration |  | 1 | PDC-SRS.1 |  |
|  |  | 2 | PDC-SRS.1 |  |
|  |  | 3 | PDC-SRS.2 |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -98 |  |
| PRS | dB | 1 | -Infinity | -2.41 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -Infinity | -2 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 |
|  |  | 2 | -Infinity | -100 |
|  |  | 3 | -Infinity | -97 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: The noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.6.6.20.2	Test requirements

If the test is based on PRS, the UE Rx-Tx time difference measurement time fulfils the requirements specified in 9.12.4.1.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.39.2 for Cell 1.

### A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA

#### A.6.6.21.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.39.3 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in table A.6.6.21.1-1.

Table A.6.6.21.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There is a single cell in the test: PCell (Cell 1) on RF channel 1 in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in TS 38.331 [2], shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.6.21.1-2 and table A.6.6.21.1-3 respectively. The test consists two sub-tests with different TRS BW.

Table A.6.6.21.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell |
| RF Channel Number |  | 1, 2, 3 | 1 |  |
| BWchannel | MHz | 1 | 10: NPRB,c = 52 |  |
|  |  | 2 | 10: NPRB,c = 52 |  |
|  |  | 3 | 40: NPRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| T1 | s | 1, 2, 3 | 1 |  |
| T2 | s | 1, 2, 3 | 1 |  |

Table A.6.6.21.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Values |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | PDC-SRS.1 |  |
|  |  | 2 | PDC-SRS.1 |  |
|  |  | 3 | PDC-SRS.2 |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 |
|  |  | 2 | -Infinity | -101 |
|  |  | 3 | -Infinity | -98 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: TRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.6.6.21.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.12.4.2.

The UE shall perform and report the UE Rx-Tx time difference measurement for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.39.3

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%.

### A.6.6.22	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG

#### A.6.6.22.1	SA event triggered reporting tests for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers

##### A.6.6.22.1.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gap with Pre-MG capable UE makes correct reporting of events. This test will partly verify the SA intra-frequency and inter-frequency NR cell search requirements in clauses 9.2.6 and 9.3.4/9.3.5, respectively. Also, this test will also jointly verify pre-configured measurement gap autonomous activation/deactivation delay in clause 8.19.2.

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as intra-frequency neighbour cell in FR1 on the same frequency as the PCell, and NR cell 3 as neighbour cell in FR1on NR RF channel 2. There are two BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1. The test parameters are given in tables A.6.6.22.1.1-1, A.6.6.22.1.1-2 and A.6.6.22.1.1-3. The TE schedules continuous DL data on PCell throughout the test.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. Before the test starts, the UE shall not have any timing information of NR Cell 2 or NR Cell 3.

Before the test starts,

-	For Cell 1, the UE is configured with 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2, before starting the test.

-	BWP-1 includes bandwidth of the initial DL BWP and SSB. UE is expected to deactivate the Pre-MG when this BWP is active.

-	BWP-2 does not include bandwidth of the initial DL BWP and SSB. UE is expected to activate the Pre-MG when this BWP is active.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

During T1, UE active DL BWP is BWP-1, and the pre-configured gap (MeasGapId #1) is deactivated. Cell 3 is switched ON from the beginning of T1, and UE is expected to search for Cell 3 in MeasGapId #2.

At the start of time duration T2, Cell 3 is switched OFF and the serving gNB can trigger Pre-MG activation starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. The UE is expected to complete the Pre-MG activation within T2. Cell 2 is switched ON from the beginning of T2, and UE is expected to search for Cell 2 in MeasGapId #1.

Two measurement gap patterns (MeasGapId #1 (Pre-MG) and MeasGapId #2) are configured with the gap pattern ID #1 and #0 as defined in table A.6.6.22.1.1-2. MeasGapId #1 is configured with a higher priority than MeasGapId #2. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #1 and #2, respectively.

Table A.6.6.22.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.22.1.1-2: General test parameters for SA intra-frequency and inter-frequency event triggered reporting for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 1. NR Cell 3 is on the NR RF channel 2. |
| Gap Pattern Id |  | Config 1,2,3 | 1 for MeasGapId #1 (80 ms MGRP) 0 for MeasGapId #2 (40 ms MGRP) | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 79 for MeasGapId #1 4 for MeasGapId #2 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cell |  | Config 1, 2, 3 | 3s | The timing of Cell 2 is 3s later than the timing of Cell 1. |
|  |  | Config 1, 2, 3 | 5 ms | The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 2 |  |
| T2 | s | Config 1,2,3 | 2 |  |

Table A.6.6.22.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with Pre-MG with partially-partial overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  | Config 1,2,3 | 1 |  | 1 |  | 2 |  |  |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |  |
| BWP BW | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |  |
| Initial BWP Configuration |  | Config 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | NA |  | NA |  |  |
| BWP-1 Configuration |  | Config 1, 2, 3 | DLBWP.1.3 ULBWP.1.3 |  | NA |  | NA |  |  |
| BWP-2 Configuration |  | Config 1, 2, 3 | DLBWP.1.2 ULBWP.1.2 |  | NA |  | NA |  |  |
| TRS configuration |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.1 FDD |  | NA |  | NA |  |  |
|  |  | Config 2 | SR.1.1 TDD |  | NA |  | NA |  |  |
|  |  | Config 3 | SR.2.1 TDD |  | NA |  | NA |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.1 FDD |  | NA |  | NA |  |  |
|  |  | Config 2 | CR.1.1 TDD |  | NA |  | NA |  |  |
|  |  | Config 3 | CR.2.1 TDD |  | NA |  | NA |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.1 FDD |  | NA |  | NA |  |  |
|  |  | Config 2 | CCR.1.1 TDD |  | NA |  | NA |  |  |
|  |  | Config 3 | CCR.2.1 TDD |  | NA |  | NA |  |  |
| SSB parameters |  | Config 1,2 | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration defined in A.3.11 |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.7 |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz |  | -98 |  | -98 |  | -98 |  |  |
| Note2 | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |  |
|  |  | Config 3 | -95 |  | -95 |  | -95 |  |  |
| SS-RSRP Note 3 | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -94 | -91 | -Infinity |  |
|  |  | Config 3 | -91 | -91 | -Infinity | -91 | -88 | -Infinity |  |
|  | dB | Config 1,2,3 | 4 | -1.46 | -Infinity | -1.46 | 7 | -Infinity |  |
|  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 4 | 7 | -Infinity |  |
| IoNote3 | dBm/9.36 MHz | Config 1,2 | -64.59 | -62.25 | -64.59 | -62.26 | -62.26 | -70.05 |  |
|  | dBm/38.16 MHz | Config 3 | -58.49 | -56.16 | -58.49 | -56.15 | -56.15 | -63.94 |  |
| Propagation Condition |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.22.1.2	Test Requirements

For UE supporting dynamicCollision-r18:

During T1, the UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the MeasGapId#2 occasions. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms for cell 3 from the beginning of time period T1. The measurement reporting delay is derived based on the requirements for inter-frequency measurement in clause 9.3.4 and 9.3.5.

For UE not supporting dynamicCollision-r18:

During T1, the UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-dropped MeasGapId#2 occasions. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms for cell 3 from the beginning of time period T1. The measurement reporting delay is derived based on the requirements for inter-frequency measurement in clause 9.3.4 and 9.3.5.

For both UE supporting dynamicCollision-r18 and not supporting dynamicCollision-r18:

During T2, the UE shall report ACK/NACK for PDSCHs scheduled in the slots that are not overlapped with the MeasGapId #1 occasions or non-dropped MeasGapId #2 occasions after MeasGapId #1 is activated, i.e. starting from the 1 st complete MeasGapId #1 occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5NR slot length as defined in clause 8.19.2.

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1080 ms for Cell 2 from the beginning of time period T2. The measurement reporting delay is derived based on the requirements for intra-frequency measurement in clause 9.2.6 plus 80 ms, considering that the frist MeasGapId #1 occasion in T2 may collide with the pre-configured gap activation delay.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. UE is not required to report SSB time index.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.22.2	SA event triggered reporting tests for concurrent gap with pre-configured gaps and network-controlled activation/deactivation

##### A.6.6.22.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MGs and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the multiple Pre-MG activation/deactivation delay in clause 8.19.5.2, the intra-frequency cell search requirements in clause 9.2.6 and inter-frequency cell search requirements in clause 9.3.4 and 9.3.5.

##### A.6.6.22.2.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) in FR1 on NR RF channel 1 and a neighbour cell (Cell 2) in FR1 on NR RF channel 1 and a neighbour cell (Cell 3) in FR1 on NR RF channel 2. The supported test configurations are listed in table A.6.6.22.2.2-1, general test parameters are listed in table A.6.6.22.2.2-2, and cell specific test parameters are listed in table A.6.6.22.2.2-3.

Two Pre-MG gaps (MeasGapId #1 and MeasGapId #2) are configured with the Pre-MG gap pattern ID #0 and #1 as defined in table A.6.6.22.2.2-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1.

In the measurement control information, two measurement objects (MOs) are configured, the measurement object #1 (MO1) for NR RF channel 1 is associated with MeasGapId #1, and measurement object #2 (MO2) for NR RF channel 2 is associated with MeasGapId #2. And it is indicated to the UE that event-triggered reporting with Event A3 is used.

Before the test, UE is connected to Cell 1 (PCell) on radio channel 1. The UE is configured with two dedicated BWPs for Cell 1 (PCell), BWP-1 and BWP-2. BWP-1 includes bandwidth of the SSB, and preConfGapStatus of the pre-MG (MeasGapId #1) for measurements on BWP-1 is set to ‘0’, preConfGapStatus of the pre-MG (MeasGapId #2) for measurements on BWP-1 is set to ‘1’; BWP-2 does not include bandwidth of the SSB, and preConfGapStatus of the pre-MG (MeasGapId #1) for measurements on BWP-2 is set to ‘1’, preConfGapStatus of the pre-MG (MeasGapId #2) for measurements on BWP-2 is set to ‘0’.

The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively.

During T1, UE active DL BWP is BWP-1, and the pre-configured gap (MeasGapId #1) is deactivated, pre-configured gap (MeasGapId #2) is activated. Cell 3 is switched ON from the beginning of T1, and UE is supposed to search Cell 3 in MeasGapId #2.

The time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its DL active BWP from BWP-1 to BWP-2, and the pre-configured gap (MeasGapId #1) is activated and pre-configured gap (MeasGapId #2) is deactivated. Cell 3 is switched OFF.

At the beginning of T3, Cell 2 is switched ON, and UE is supposed to search Cell 2 in MeasGapId#1.

During T1, UE shall perform inter-frequency measurement with pre-MG (MeasGapId #2) activated.

During T3, UE shall perform intra-frequency measurement with pre-MG (MeasGapId #1) activated.

The TE schedules continuous DL data on PCell throughout the test.

Table A.6.6.22.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.22.2.2-2: General test parameters for SA intra-frequency event triggered reporting with  concurrent gap with pre-configured gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2, Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2, 2: Cell 3 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE Pre-MG gaps |  |
| Pre-MG Gap Pattern Id |  | 1, 2, 3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  |
| Measurement gap offset |  | 1, 2, 3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 | Same configuration for MO1 and MO2 |
|  |  | 2 | SMTC.1 | Same configuration for MO1 and MO2 |
|  |  | 3 | SMTC.1 | Same configuration for MO1 and MO2 |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 0.1 |  |
| T3 | s | 1, 2, 3 | 5 |  |

Table A.6.6.22.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with concurrent gap with pre-configured gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | N/A |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-1 |  | 1, 2, 3 | DLBWP.1.3 |  |  | N/A |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-1 |  | 1, 2, 3 | ULBWP.1.3 |  |  | N/A |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-2 |  | 1, 2, 3 | DLBWP.1.2 |  |  | N/A |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-2 |  | 1, 2, 3 | ULBWP.1.2 |  |  | N/A |  |  | N/A |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  | N/A |  |  | N/A |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  | -98 |  |  | -98 |  |  |
|  |  | 2 | -98 |  |  | -98 |  |  | -98 |  |  |
|  |  | 3 | -95 |  |  | -95 |  |  | -95 |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  | -98 |  |  | -98 |  |  |
|  |  | 2 | -98 |  |  | -98 |  |  | -98 |  |  |
|  |  | 3 | -95 |  |  | -95 |  |  | -95 |  |  |
|  | dB | 1,2,3 | 4 |  | -1.46 | -Infinity |  | -1.46 | 4 | -Infinity |  |
|  | dB | 1,2,3 | 4 |  | 4 | -Infinity |  | 4 | 4 | -Infinity |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1,2 | -94 |  | -94 | -Infinity |  | -94 | -94 | -Infinity |  |
|  |  | 3 | -91 |  | -91 | -Infinity |  | -91 | -91 | -Infinity |  |
| Io | dBm/9.36 MHz | 1,2 | -64.60 |  | -62.25 | -Infinity |  | -62.25 | -64.60 | -Infinity |  |
|  | dBm/38.16 MHz | 3 | -58.50 |  | -56.16 | -Infinity |  | -56.16 | -58.50 | -Infinity |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2 and T4. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.22.2.3	Test Requirements

During T1, UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #2; and

The UE shall send one Event A3 triggered measurement report of cell 3 on RF channel 2, with a measurement reporting delay less than 1280 ms from the beginning of time period T1.

During T2 and starting from the 1 st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 7NR slot length as defined in clause 8.19.5.2,  the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE shall send one Event A3 triggered measurement report of Cell 2 on RF channel 1, with a measurement reporting delay less than 800 ms from the beginning of time period T3.

During T3, the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.23	SA event triggered reporting tests for concurrent measurement gaps with NCSG

#### A.6.6.23.1	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [one MG + one NCSG]

##### A.6.6.23.1.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.10.

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR Cell 3 as neighbour cell in FR1 on NR RF channel 3. The test parameters are given in tables A.6.6.23.1.1-1, A.6.6.23.1.1-2 and A.6.6.23.1.1-3.

One measurement gap and one NCSG are configured to UE with measurement gap pattern #0 and NCSG pattern #1 respectively. Measurement gap with pattern #0 is associated with inter-frequency measurement on NR Cell 2, and NCSG with pattern #1 is associated with inter-frequency measurement on NR Cell 3 as defined in table A.6.6.23.1.1-2.

NCSG is configured with higher priority than measurement gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR Cell 3.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.23.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.23.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent NCSGs with partially partially overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cells 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Measurement Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in Table 9.1.2-1. |
| Measurement Gap offset |  | Config 1,2,3 | 39 |  |
| NCSG Pattern Id |  | Config 1,2,3 | 1 | As specified in Table 9.1.9.3-1. |
| NCSG offset |  | Config 1,2,3 | 4 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | Config 1, 2, 3 | 3s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | Config 1, 2, 3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1.5 |  |

Table A.6.6.23.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 Con-NCSG gaps with partially partially overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.23.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1280 ms for Cell 2 and 1280 ms for Cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and the non-dropped measurement gap occasions after considering the collision between NCSG and measurement gap.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.23.2	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [two NCSG]

##### A.6.6.23.2.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.10.

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR Cell 3 as neighbour cell in FR1 on NR RF channel 3. The test parameters are given in tables A.6.6.23.2.1-1, A.6.6.23.2.1-2 and A.6.6.23.2.1-3.

Two NCSG patterns (NCSGId #0 and NCSGId #1) are configured with the NCSG pattern ID #0 and #1 as defined in table A.6.6.23.2.1-2. NCSGId #1 is configured with a higher priority than NCSGId #0.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR Cell 3.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.23.2.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.23.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent NCSGs with partially partially overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| NCSG Pattern Id |  | Config 1,2,3 | 0 for NCSGId #0 1 for NCSGId #1 | As specified in Table 9.1.9.3-1. |
| NCSG offset |  | Config 1,2,3 | 39 for NCSGId #0 4 for NCSGId #1 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | Config 1, 2, 3 | 3s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | Config 1, 2, 3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1.5 |  |

Table A.6.6.23.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 Con-NCSG gaps with partially partially overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.23.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1280 ms for Cell 2 and 1280 ms for cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of non-dropped NCSG patterns after considering the collision between NCSGs.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.23.3	Event triggered reporting on SCC with deactivated SCell test with per-UE Con-NCSG under non-DRX

##### A.6.6.23.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the cell search requirements on SCC with deactivated SCell in clauses 9.2.7.1 and 9.2.7.2.

##### A.6.6.23.3.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on NR RF channel 1, FR1 SCell (Cell 2) and FR1 neighbour cell (Cell 3) on the same frequency as the SCell on NR RF channel 2.

The SCell is deactivated during the test. The test parameters for PCell, the SCell and the neighbour cell are given in table A.6.6.23.3.2-1 and A.6.6.23.3.2-2 below.

One measurement gap patterns (MeasGapId #1) and one NCSG pattern (NCSGId #1) are configured with the gap pattern ID #0 and NCSG pattern ID #0 as defined in table A.6.6.x.1.1-2. [NCSGId #1 is associated with the MO for RF channel numbers #2, respectively.]

In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2.

Table A.6.6.23.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.23.3.2-2: General test parameters for event triggered reporting on SCC with deactivated SCell with per-UE Con-NCSG for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Configured deactivated SCell |  | 1, 2, 3 | Cell 2 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 |  |
| RF Channel Number |  | 1, 2, 3 | 2: Cell 2 and Cell 3 |  |
| NCSG type |  | 1, 2, 3 | Per-UE NCSG |  |
| NCSG pattern Id |  | 1, 2, 3 | ID # 1 | Defined in table 9.1.9.3-1 |
| NCSG offset |  | 1, 2, 3 | 39 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE MG |  |
| MG pattern Id |  | 1, 2, 3 | ID #1 | Defined in table 9.1.2-1 |
| Measurement gap offset |  | 1, 2, 3 | 19 |  |
| MGTA | ms | 1, 2, 3 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1, 2, 3 | 640 |  |
| Cell 2 timing offset to Cell 1 | s | 1, 2, 3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1, 2, 3 | ≤ Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.23.3.2-3: NR Cell specific test parameters for event triggered reporting on SCC with deactivated SCell with per-UE Con-NCSG for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |  |  |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1, 2, 3 | 4 | 4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.23.3.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than [6400] ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and MGL of MG pattern.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For a test to be considered successful requirements on both Event A6 detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.24	SA event triggered reporting tests with NeedForGap in FR1

#### A.6.6.24.1	SA event triggered reporting tests without gaps, with interruptions, under non-DRX

##### A.6.6.24.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event, and to verify that the interruption ratio does not exceed the limits for the NR PCell during the measurement without gaps and with interruptions. This test will partly verify the cell search requirements in clauses 9.2.5.1 and 9.2.5.2 for measurements performed without gaps and with interruptions. This test will also verify the interruption ratio for PCell in standalone NR specified in clause 8.2.2.2.19. The test will measure that the measurement delay is within the specified boundaries.

The serving frequency should be selected for which UE reports ‘no-gap’ in NeedForGapsIntraFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18.

##### A.6.6.24.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in table A.6.6.24.1.2-1, A.6.6.24.1.2-2 and A.6.6.24.1.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The UE is capable of measurements without gaps with interruption and report ‘no-gap’ through NeedForGapsIntraFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18 for PCell. UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment.

Table A.6.6.24.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.24.1.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 without DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.6.24.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 without DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.2 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.24.1.3	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1 and T2.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

During 1000 ms from the beginning of time period T2, the UE shall transmit ACK/NACK in PCell and the rate of missed ACK/NACK shall no more than 2.5%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.24.2	SA event triggered reporting tests for FR1 without gap with interruption for inter-frequency measurement with SSB time index detection when DRX is not used

##### A.6.6.24.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.9 and interruption requirements during measurement without gap in clause 8.2.2.2.19.

The serving frequency and the target frequency should be selected such that UE reports ‘no-gap’ via needForGapsInfoNR-r16 and ‘no-gap-with-interruption’ via NeedForInterruptionNR-r18 for the target frequency given the serving frequency.

##### A.6.6.24.2.2	Test parameters

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘no-gap-with-interruption’ for the target frequency on NR RF channel 2. The test parameters are given in tables A.6.6.24.2.2-1, A.6.6.24.2.2-2 and A.6.6.24.2.2-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.24.2.2-1: SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.24.2.2-2: General test parameters for SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 2 |  |

Table A.6.6.24.2.2-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.24.2.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1520 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

During 1520 ms from the beginning of time period T2, the UE shall transmit ACK/NACK in PCell and the rate of missed ACK/NACKs shall no more than 2.5%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.24.3	SA event triggered reporting tests for FR1 with ‘no-gap-with-interruption’, without measurement gap or DRX

##### A.6.6.24.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

The serving frequency and the target frequency should be selected such that UE reports ‘no-gap-with-interruption’ for the target frequency given the serving frequency.

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. Supported test configurations are shown in table A.6.6.24.3.1-1. The general test parameters are given in tables A.6.6.24.3.1-2, and cell specific test parameters are given in table A.6.6.24.3.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

The TE schedules continuous DL data on PCell during the test duration.

Table A.6.6.24.3.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.24.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC for NR RF channel number 2 |  | Config 1,2,3 | SMTC.3 | As specified in clause A.3.11 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.24.3.1-3: Cell specific test parameters for inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.24.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2560 ms from the beginning of time period T2, where 2560 ms is derived based on inter-frequency measurement requirements in clause 9.3.4.

In the test, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots with an interruption ratio between number of interrupted slot over the total number of slots no larger than 1.25%.

In the test UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.24.4	SA event triggered reporting tests for FR1 NeedForGaps without gap without interruption when DRX is not used

##### A.6.6.24.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.9 and also verify the interruption during inter-frequency measurement with NeedForGaps.

The serving frequency and the target frequency should be selected for which UE supports NeedForInterruptionInfoNR-R18 measurements and indicates ‘no-gap-no-interruption’ for the target frequency given the serving frequency.

##### A.6.6.24.4.2	Test parameters

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘no-gap-no-interruption’ for the target frequency on NR RF channel 2.

The test parameters are given in tables A.6.6.24.4.1-1, A.6.6.24.4.1-2 and A.6.6.24.4.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

During T2, the UE is continuously scheduled with data on the PCell.

Table A.6.6.24.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.24.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with NeedForInterruption ‘no-gap-no-interruption’

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.24.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with NeedForInterruption ‘no-gap-no-interruption’

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NPRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NPRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16 MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.24.4.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.24.5	SA event triggered reporting tests without gap under non-DRX for UE indicating no-gap-no-interruption

##### A.6.6.24.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports ‘no-gap’ makes correct reporting of an event and the UE performs intra-frequency measurement without gap without interuption when the UE indicates ‘no-gap’ via intraFreq-needForGap and the UE indicates no-gap-no-interruption via NeedForInterruptionInfoNR-r18. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.24.5.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in table A.6.6.24.5.2-2 and A.6.6.24.5.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.24.5.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.24.5.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.24.5.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.2 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.24.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

UE is not allowed to cause interruption during intra-frequency measurement without gap.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps

#### A.6.6.25.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1

##### A.6.6.25.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that if UE reports “nogap-noncsg” via NeedForGapNCSG-InfoEUTRA-r17, the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clause 9.4.8.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.25.1.1-1. General test parameters are provided in table A.6.6.25.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.25.1.1-3 and A.6.6.25.1.1-4, respectively.

Table A.6.6.25.1.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.25.1.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.25.1.1-1 and A.6.6.25.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Effective Measurement Window (EMW) Id |  | 0 | As specified in table 9.4.8.2-1 |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.25.1.1-3 |  |  |  |

Table A.6.6.25.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.25.1.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.25.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

During the T2, UE shall be able to report ACK/NACK for all slots with PDCCH/PDSCH on PCell excluding those symbles as defined in 9.4.8.3.5 or 9.4.8.4.5.

The rate of correct events observed during repeated tests shall be at least 90%

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.25.2	SA NR - E-UTRAN event-triggered reporting without gap under non-DRX in FR1

##### A.6.6.25.2.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements without gap as there are vacant RF chains available for UE measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clause 9.4.8, and also verify the scheduling availability during inter-RAT E-UTRAN measurement without gap in clause 9.4.8.3.5 and 9.4.8.4.5.

The serving frequency and the target frequency should be selected such that UE reports ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA-r17 for the target frequency given the serving frequency and ‘eutra-MeasEMW-r18’.

##### A.6.6.25.2.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT E-UTRAN neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are given in Table A.6.6.25.1.1-1 and for EMW configuration in table A.6.6.25.2.2-1 with EMW configuration specified in table 9.4.8.2-1. General test parameters are provided in Table A.6.6.25.2.2-2. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A. A.6.6.25.1.1-3 and A. A.6.6.25.1.1-4, respectively.

Table A.6.6.25.2.2-1: EMW configuration test parameters for SA inter-RAT E-UTRA without gap event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | EMW confiutation |
| --- | --- | --- | --- |
| EMW configuration |  | 1, 2, 3, 4, 5, 6 | 0 |

Table A.6.6.25.2.2-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.2.1-2 and A.6.6.3.2.1-3. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for RSRP measurement on cell 2 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | OFF |  | Non-DRX |
| T1 | s | 5 |  |  |
| T2 | s | 5 |  |  |
| NOTE 1: Values are defined in table A.6.6.3.2.1-3 |  |  |  |  |

##### A.6.6.25.2.3	Test Requirements

The actual overall measurement delay test requirements can refer to the requirements in A.6.6.3.1.2. And during T2, for the two subtests regarding to EMW configuration, UE shall or not send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell based on the requirements defined in clauses 9.4.8.3.5 and 9.4.8.4.5.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.25.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for UE capable of inter-RAT EUTRAN measurement without gap when CRS is contained within UE’s active DL BWP

##### A.6.6.25.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE which supports inter-RAT E-UTRAN measurement without gap when CRS is contained within UE’s active DL BWP makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in clause 9.4.8, and also verify the scheduling availability during inter-RAT E-UTRAN measurement in clauses  9.4.8.3.5 and 9.4.8.4.5.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The CRS of Cell 2 is completely within UE’s active BWP BW.

In the measurement control information from the PCell, it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT E-UTRAN neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.25.3.1-1. General test parameters are provided in table A.6.6.25.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in tables A.6.6.25.3.1-3 and A.6.6.25.3.1-4, respectively.

Table A.6.6.25.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.25.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Effective Measurement Window (EMW) Id |  | 0 | As specified in table 9.4.8.2-1 |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.25.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | - 96 |  |
|  |  |  |  | 3, 6 | - 93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.25.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.25.3.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

During T2, UE does not cause interruption due to inter-RAT E-UTRAN measurement without gap.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.26	LTM Intra-frequency L1-RSRP measurement

#### A.6.6.26.1	Intra-frequency SSB based L1-RSRP measurement in FR1

##### A.6.6.26.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SSB based intra-frequency L1-RSRP measurement on neighbor cell in FR1. This test will partly verify the requirements for SSB based intra-frequency L1-RSRP measurement on neighbor cell specified in clause 9.14, with the testing configurations for NR cells in table A.6.6.26.1.1-1.

Table A.6.6.26.1.1-1: Applicable NR configurations for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.26.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Measurement period [and measurement accuracy] is tested by using the parameters in table A.6.6.26.1.2-1, and A.6.6.26.1.2-2.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

If a UE does not support multiCellL1-measRTD-greaterThan-CP-r18, it is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different.  No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an odd SFN.

Table A.6.6.26.1.2-1: General test parameters for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 2 s | 20s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 20 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

Table A.6.6.26.1.2-2: Cell specific test parameters for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| SSB ARFCN |  |  |  | freq1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | KHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | KHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -101 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -101 |  |  |  |  |
|  | Config 3 |  |  | -98 |  |  |  |  |
|  |  |  | dB | -1.76 | -0.19 |  | -1.76 | -0.19 |
|  |  |  | dB | 3 | 13.5 |  | 3 | 13.5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -98 | -87.5 |  | -98 | -87.5 |
|  | Config 3 |  | dBm/SCS | -95 | -84.5 |  | -95 | -84.5 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36 MHz | -66.07 | -56.44 |  | -66.07 | -56.44 |
|  | Config 3 |  | dBm/ 38.16 MHz | -59.96 | -50.34 |  | -59.96 | -50.34 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.26.1.3	Test Requirements

The UE shall send L1-RSRP report every 20 slots. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 20 ms plus 20 slots from the beginning of time period T2. UE shall send L1-RSRP report including results of Cell 2 while meeting the L1-RSRP absolute accuracy requirement in clause 10.1.19D.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap

#### A.6.6.27.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap

##### A.6.6.27.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter-frequency L1-RSRP measurement with measurement gap on candidate neighbour cell. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.5, with the testing configurations for NR cells in table A.6.6.27.1.1-1.

Table A.6.6.27.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

##### A.6.6.27.1.2	Test parameters

There are two carriers and one cell on each carrier in the test, NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for the Cell 1 and Cell 2 are given in table A.6.6.27.1.2-1 and table A.6.6.27.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively.

Measurement gap pattern configuration defined in table A.6.6.27.1.2-1 is provided.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1.

Table A.6.6.27.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -6 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 160 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled | Not relevant to this test case |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |
| T2 |  | s | 0.5 |  |

Table A.6.6.27.1.2-2: Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB ARFCN | 1~3 |  | freq1 |  | freq2 |  |
| Duplex mode | 1 |  | FDD |  |  |  |
|  | 2 |  | TDD |  |  |  |
|  | 3 |  | TDD |  |  |  |
| TDD Configuration | 1 |  | N/A |  |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |  |  |  |
|  | 2 |  | 10: NPRB,c = 52 |  |  |  |
|  | 3 |  | 40: NPRB,c = 106 |  |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  | N/A |  |
|  | 2 |  | SR.1.1 TDD |  | N/A |  |
|  | 3 |  | SR.2.1 TDD |  | N/A |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  | N/A |  |
|  | 2 |  | CR.1.1 TDD |  | N/A |  |
|  | 3 |  | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  | N/A |  |
|  | 2 |  | CCR.1.1 TDD |  | N/A |  |
|  | 3 |  | CCR.2.1 TDD |  | N/A |  |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  |  |  |
| TRS Configuration | 1 |  | TRS.1.1 FDD |  | N/A |  |
|  | 2 |  | TRS.1.1 TDD |  | N/A |  |
|  | 3 |  | TRS.1.2 TDD |  | N/A |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | 0 | 8 |
|  | 1~3 | dB | 0 | 0 | 0 | 8 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -94.65 | -86.65 |
|  | 3 |  | -91.65 | -91.65 | -91.65 | -83.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -63.69 | -58.06 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -57.59 | -51.97 |
| Propagation condition | 1~3 |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves.. |  |  |  |  |  |  |

##### A.6.6.27.1.3	Test Requirements

During T1 The UE shall send inter-frequency L1-RSRP report every 80 slots. No later than 80 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report of Cell 2. The RSRP report during T2 shall be larger than that during T1. These reported measurement report shall meet the absolute accuracy requirement in clause 10.1.19E. The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap

#### A.6.6.28.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap

##### A.6.6.28.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting inter-frequency L1-RSRP measurements without gap makes correct reporting of inter-frequency L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.6, with the testing configurations for NR serving cells in table A.6.6.28.1.1-1.

Table A.6.6.28.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

##### A.6.6.28.1.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) on NR RF channel 1 and Cell 2 as neighbour cell in FR1 on NR RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in table A.6.6.28.1.2-1 and table A.6.6.28.1.2-2 below.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

UE not capable of multiCellL1-measRTD-greaterThan-CP-r18 is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. There is no measurement gap configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an even SFN.

Table A.6.6.28.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell 2 is candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |  |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| Time offset between cells |  | s | 2 | 20 |  |
| DRX |  |  | OFF |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 20 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

Table A.6.6.28.1.2-2: Cell specific test parameters  for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |  |
| SSB ARFCN |  |  |  | freq1 |  | freq2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  | SMTC.5 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  | SMTC.4 |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  | SSB.5 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.6 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  | NA |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  | NA |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  | NA |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  | NA |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -101 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -101 |  |  |  |  |
|  | Config 3 |  |  | -98 |  |  |  |  |
|  |  |  | dB | -1.76 | -0.19 |  | -1.76 | -0.19 |
|  |  |  | dB | 3 | 14.5 |  | 3 | 14.5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -98 | -87.5 |  | -98 | -87.5 |
|  | Config 3 |  | dBm/SCS | -95 | -84.5 |  | -95 | -84.5 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36 MHz | -66.07 | -56.44 |  | -66.07 | -56.44 |
|  | Config 3 |  | dBm/ 38.16 MHz | -59.96 | -50.34 |  | -59.96 | -50.34 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.28.1.3	Test Requirements

The UE shall send L1-RSRP report every 20 slots. No later than 20 ms plus 20 slots from the beginning of time period T2, UE shall send L1-RSRP report of Cell 2 while meeting the absolute accuracy requirement in clause 10.1.19E.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.29	RSCPD Measurements

#### A.6.6.29.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state

##### A.6.6.29.1.1	Test Purpose and Environment

The purpose of the test is to verify that the DL RSCPD measurement reported together with the RSTD measurement meets the requirements specified in clause 9.9.7 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured to the UE.

The supported test configurations are specified in table A.6.6.29.1.1-1.

Table A.6.6.29.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and NR-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.10, shall be provided to the UE during T1. In NR-DL-TDOA-RequestLocationInformation, the UE is configured to perform DL RSCPD measurement via dl-PRS-RSCPD-Request. The UE is configured to perform both RSCPD and RSTD measurements within the time window indicated to UE via nr-DL-PRS-MeasurementTimeWindowsConfig. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources to be measured within the configured time window.

The UE is configured with measurement gap pattern ID # 24 or # 0 before T2.

The general test parameters are listed in table A.6.6.29.1.1-2, and cell specific test parameters are listed in table A.6.6.29.1.1-3.

Table A.6.6.29.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSCPD and RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2) mod 6=0 and (PCI of Cell 1 – PCI of Cell 3) mod 6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Time window configuration |  |  | MTW.1 | As specified in clause A.3.38 |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to NR-MutingPattern defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.29.1.1-3: Cell-specific test parameters for RSCPD with RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in clause A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Noc Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS EsNoc |  | dB | -Infinity | -Infinity | -Infinity |
| SSB EsNoc |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.29.1.1-4: Cell-specific test parameters for RSCPD with RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in clause A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Noc Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS EsNoc | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| SSB EsNoc | Config 1,2,3 | dB | 10 | 3 | 3 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88MHz | -61.44 | -61.44 | -61.44 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -95 | -95 |
|  | Config 2 | dBm/SCS | -88 | -95 | -95 |
|  | Config 3 | dBm/SCS | -85 | -92 | -92 |
| PRS EsIot |  | dB | -6.00 | -12.98 | -12.98 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |  |

##### A.6.6.29.1.2	Test Requirements

The RSCPD reported together with RSTD measurement time fulfils the requirements specified in the clause 9.9.7.

The UE shall perform and report the RSCPD and RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.7 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for DL RSCPD measurement for each neighbour cell observed during the repeated tests shall be at least 90%. The reported DL RSCPD measurement shall be within the DL RSCPD reporting range specified in the clause 10.1.43 and the reported RSTD measurement shall be within the RSTD reporting range specified in the clause 10.1.23.

### A.6.6.30	RSCP Measurements

#### A.6.6.30.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.6.30.1.1	Test purpose and environment

The purpose of the test is to verify that the DL RSCP and UE Rx-Tx time difference measurements meet the requirements specified in clause 9.9.8.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured for both DL RSCP measurement and UE Rx-Tx time difference measurement.

The supported test configurations are listed in table A.6.6.30.1.1-1.

Table A.6.6.30.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE during T1. In NR-Multi-RTT-RequestLocationInformation, the UE is configured to perform DL RSCP measurement via nr-DL-PRS-RSCP-Request. The UE is configured to perform both DL RSCP and UE Rx-Tx time difference measurements within the time window indicated to UE via nr-DL-PRS-MeasurementTimeWindowsConfig. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources to be measured within the configured time window.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are listed in table A.6.6.30.1.1-2 and table A.6.6.30.1.1-3.

Table A.6.6.30.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Time window configuration |  | 1, 2, 3 | MTW.1 | As specified in clause A.3.38 |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.30.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.30.1.2	Test requirements

The DL RSCP with UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.8 with Nsample=4 for UE Rx-Tx time difference.

The UE shall perform and report the DL RSCP and UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified DL RSCP with UE Rx-Tx time difference measurement time specified in clause 9.9.8 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%. The reported DL RSCP measurement shall be within the DL RSCP reporting range specified in clause 10.1.44 and the reported UE Rx-Tx measurement shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.

## A.6.7	Measurement Performance requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in clause 10 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in clause 10 for at least 90% of the reported cases.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

### A.6.7.1	SS-RSRP

#### A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.1.1 and 10.1.2.1.2 for intra-frequency measurements.

##### A.6.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.1.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -106 |  | -88 |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -94 |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15 kHz |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -91 |  | -111 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -104.5 |  |
|  |  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -100 | -105 | -82 | -87 | -111.00 | -114.00 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -110.50 | -113.50 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -110.00 | -113.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.50 | -112.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109.00 | -112.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.50 | -111.50 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -104.50 | -107.50 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -90 | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -107.00 | -110.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -106.50 | -109.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -106.00 | -109.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -105.50 | -108.50 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -105.00 | -108.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -104.50 | -107.50 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -101.50 | -104.50 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -70.09 |  | -52.09 |  | -80.03 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -79.53 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -79.03 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -78.53 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -78.03 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -77.53 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -77.03 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -73.53 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | Not applicableNote 5- |  | -51.99 |  | -73.94 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -73.44 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -72.94 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -72.44 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -71.94 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -71.44 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -70.94 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -70.44 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -67.44 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30 kHz SSB SCS. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil absolute requirement in clause 10.1.2.1.1 and relative requirement in clause 10.1.2.1.2.

#### A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations for NR cells in table A.6.7.1.2.1-1.

Table A.6.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in table A.6.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in table A.6.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 |  |  | 10: NPRB,c = 52 |  |  |
|  |  | 2 |  | 10: NPRB,c = 52 |  |  | 10: NPRB,c = 52 |  |  |
|  |  | 3 |  | 40: NPRB,c = 106 |  |  | 40: NPRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD |  | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 TDD |  | - | CR.2.1 TDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD |  | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1 | ms | - | 3 |  | - | 3 |  |
|  |  | 2,3 | s | - | 3 |  | - | 3 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 |  |  | ( for Channel 2 +8 dB) |  | -115 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8 dB) |  | -115 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  | ( for Channel 2 +8 dB) |  | -112.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -109.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -109.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -108.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -105.50 |
|  |  | 1~3 | dB | 10 |  | 10 | 13 |  | -3 |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +24 dB) |  | -118.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -117.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  | (RSRP for Cell 2 +24 dB) |  | -115.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36 MHz | -56.28 |  |  | (Io for Channel 2 +19.45 dB) |  | -85.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -84.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -78.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16 MHz | -50.19 |  |  | (Io for Channel 2 +19.45 dB) |  | -79.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -78.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -75.69 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -72.69 |
|  |  | 1~3 | dB | 10 |  | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

#### A.6.7.1.3	Void

### A.6.7.2	SS-RSRQ

#### A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.7.1.1.

##### A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.6.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in table A.6.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell 2 |  |  |  | Cell 1 |  | Cell 1 |  | Cell 2 |  | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 |  | 0 |  | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |  | - |  | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
|  |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5..46 | -5.46 |
|  |  |  | dB | 3 | 3 | -2.9 |  | -2.9 |  | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  | -103.9 |  | -118 | -118 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  | - |  | -115 | -115 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  | -16.76 |  | -17.34 | -17.34 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -50 |  | - |  |  |  | -77.4 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -76.9 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -76.4 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -75.9 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -75.4 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -74.9 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -74.4 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -73.9 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -70.9 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  | AWGN |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.1.1.

#### A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.9.1.1 and 10.1.9.1.2.

##### A.6.7.2.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.6.7.2.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in table A.6.7.2.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.2.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 |  | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  | 0 |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR2.1 TDD |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 | - |  | 3 |  |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - |  | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC pattern 2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC pattern 1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB pattern 1 in FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB pattern 2 in FR1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -86.27 |  | -113 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  | -110 |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -110 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  |  |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 | -107.75 | -107.75 |  | -113 |  | -117.75 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -112.5 |  | -117.25 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  | -116.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  | -116.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  | -115.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  | -115.2 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 | -111.75 | -111.75 |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -109 |  | -113.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -108.5 |  | -113.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -108 |  | -112.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -107.5 |  | -112.2 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -107 |  | -111.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -103.5 |  | -108.25 |
| SS-RSRQNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -14.77 | -14.77 |  | 12.56 |  | 14.76 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  | -75.83 |  |  | -83.28 |  | -85.83 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -82.78 |  | -85.33 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -82.28 |  | -84.83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -81.78 |  | -84.33 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -81.28 |  | -83.83 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -80.78 |  | -83.33 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -80.28 |  | -82.83 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -79.78 |  | -82.33 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -76.78 |  | -79.33 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  | -76.73 |  |  | -77.19 |  | -79.73 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -76.69 |  | -79.23 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -76.19 |  | -78.73 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -75.69 |  | -78.23 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -75.19 |  | -77.73 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -74.69 |  | -77.23 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -74.19 |  | -76.73 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -73.69 |  | -76.53 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -70.69 |  | -73.53 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.1.1 and 10.1.9.1.2.

### A.6.7.3	SS-SINR

#### A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.1.1.

##### A.6.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.6.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in table A.6.7.3.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |
|  |  | Config 2,3 | ms | - | 3 | - | 3 |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -93 |  | -116 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -115.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -115 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -114.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  | -109.5 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -93 |  | Same as Noc for 15 kHz |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  | -113 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  | -106.5 |  |
|  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 | -120 | -120 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 | -119.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 | -119 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 | -118.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 | -118 |
|  |  | NR_FDD_FR1_F |  |  |  | -117.5 | -117.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_H |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_N |  |  |  | -113.5 | -113.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 | -117 | -117 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_F |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_H |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_N |  |  |  | -110.5 | -110.5 |
| SS-SINR Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -57.5 |  | -85.51 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -85.01 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -84.51 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.01 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -83.51 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -83.01 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -82.51 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -82.01 |  |
|  |  | NR_FDD_FR1_N |  |  |  | -79.01 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -51.41 |  | -79.41 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -78.91 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -78.41 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -77.91 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.41 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -76.91 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -76.41 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -75.91 |  |
|  |  | NR_FDD_FR1_N |  |  |  | -72.91 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.1.1.

#### A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.1.1 and 10.1.14.1.2.

##### A.6.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.6.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in table A.6.7.3.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.2.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 |  | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 | - | 0 | - | 0 |  | - |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 | - |  | 3 |  |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - |  | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC pattern 2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC pattern 1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -88 |  | -108.5 |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -113 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -88 |  | -108.5 |  |  | Same as Noc for 15 kHz |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  | -105.5 |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -110 |  |  |
|  |  |  | dB | -1.75 | -1.75 | 20 | 20 |  | -4.0 |  | -4.0 |
|  |  |  | dB | -1.75 |  | 20 |  |  | -4.0 |  |  |
| SS-RSRP Note3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  | -88.5 |  |  | -123.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -123 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -122.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -122 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -121.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -121 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -117 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  | -85.5 |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -114 |  |  |
| SS-SINRNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  | 20 |  |  | -4.0 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -57.83 |  | -60.5 |  |  | -90.09 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -89.59 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -89.09 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -88.59 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -88.09 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -87.59 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -87.09 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -86.59 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -83.59 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -51.73 |  | -54.41 |  |  | -84 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -83.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -83 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -82.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -82 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -81.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -81 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -80.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -77.5 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.1.1 and 10.1.14.1.2.

### A.6.7.4	L1-RSRP measurement for beam reporting

#### A.6.7.4.1	SSB based L1-RSRP measurement

##### A.6.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.2 and clause 10.1.19.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in table A.6.7.4.1.1-1.

Table A.6.7.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.1.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in table A.6.7.4.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | -3 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -86.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -80.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.1.3	Test Requirements

The L1-RSRP measurement accuracy for SSB resource reported by UE in L1-RSRP report (SSB#0 or SSB#1) of Cell 2 shall fulfil the requirements in clauses 10.1.19.1.

#### A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off

##### A.6.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.3 and clause 10.1.19.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in table A.6.7.4.2.1-1.

Table A.6.7.4.2.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.2.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.4.2.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in table A.6.7.4.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.4.2.2-1: FR1 CSI-RS based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | cri-RSRP | cri-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -86.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -80.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.2.3	Test Requirements

The L1-RSRP measurement accuracy for CSI-RS resource reported by UE in L1-RSRP report (CSI-RS#0 or CSI-RS#1) of Cell 1 shall fulfil the requirements in clause 10.1.19.2.

### A.6.7.5	E-UTRAN RSRP

#### A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.2 for SA inter-RAT E-UTRAN RSRP measurements.

##### A.6.7.5.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.5.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRP are tested by using the parameters in A.6.7.5.1.2-2 and A.6.7.5.1.2-3.

Table A.6.7.5.1.2-1: Inter-RAT E-UTRAN RSRP supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.5.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NPRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NPRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NPRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.5.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -91.65 | -117 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -116.5 |
|  | Bands FDD_C, TDD_C |  |  | -116 |
|  | Bands FDD_D |  |  | -115.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -115 |
|  | Bands FDD_G Note 8 |  |  | -114 |
|  | Bands FDD_H |  |  | -113.5 |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| SCH_RPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.45 + 10log(NPRB,c/50) | -87.76 + 10log(NPRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -87.26 + 10log(NPRB,c/50) |
|  | Bands FDD_C, TDD_C |  |  | -86.76 + 10log(NPRB,c/50) |
|  | Bands FDD_D |  |  | -86.26 + 10log(NPRB,c/50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -85.76 + 10log(NPRB,c/50) |
|  | Bands FDD_G Note 8 |  |  | -84.76 + 10log(NPRB,c/50) |
|  | Bands FDD_H |  |  | -84.26 + 10log(NPRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. NOTE 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. NOTE 8: Except Band 29. NOTE 9: Except Band 32, Band 75 and Band 76. NOTE 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

##### A.6.7.5.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRP measurement accuracy for Cell 2 shall fulfil absolute requirement in clause 10.2.2.

### A.6.7.6	E-UTRAN RSRQ

#### A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.3 for SA inter-RAT E-UTRAN RSRQ measurements.

##### A.6.7.6.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.6.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRQ are tested by using the parameters in A.6.7.6.1.2-2 and A.6.7.6.1.2-3.

Table A.6.7.6.1.2-1: Inter-RAT E-UTRAN RSRQ supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.6.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NPRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NPRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NPRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | dB |
| Ês/IotNote3 |  | dB | dB |
| SS-RSRQNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRQ, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.6.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -83 | -104.70 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| Ês/Noc |  | dB | -1.75 | -4.0 | -4.0 |
| Ês/IotNote5 |  | dB | -1.75 | -4.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -84.75 | -108.70 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RSRQNote5 | Bands FDD_A Note 9, TDD_A | dB | -14.76 | -16.25 | -16.25 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53 + 10log(NPRB,c /50) | -75.46 + 10log(NPRB,c /50) | -90.26 + 10log(NPRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -89.76 + 10log(NPRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -89.26 + 10log(NPRB,c /50) |
|  | Bands FDD_D |  |  |  | -88.76 + 10log(NPRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -88.26 + 10log(NPRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -87.26 + 10log(NPRB,c /50) |
|  | Bands FDD_H |  |  |  | -86.76 + 10log(NPRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. NOTE 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. NOTE 8: Except Band 29. NOTE 9: Except Band 32, Band 75 and Band 76. NOTE 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.6.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRQ measurement accuracy for Cell 2 shall fulfil absolute requirement in clause 10.2.3.

### A.6.7.7	E-UTRAN RS-SINR

#### A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.4 for SA inter-RAT E-UTRAN RS-SINR measurements.

##### A.6.7.7.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.7.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RS-SINR are tested by using the parameters in A.6.7.7.1.2-2 and A.6.7.7.1.2-3.

Table A.6.7.7.1.2-1: Inter-RAT E-UTRAN RS-SINR supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.7.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NPRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NPRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NPRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RS-SINRNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RS-SINR, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.7.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -88 | -108.50 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -4.0 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15 kHz | -89.75 | -88.50 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RS-SINRNote5 | Bands FDD_A Note 9, TDD_A | dB | -1.75 | 20 | -4.0 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.79 + 10log(NPRB,c /50) | -60.56 + 10log(NPRB,c /50) | -93.48 + 10log(NPRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -92.98 + 10log(NPRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -92.48 + 10log(NPRB,c /50) |
|  | Bands FDD_D |  |  |  | -91.98 + 10log(NPRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -91.48 + 10log(NPRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -90.48 + 10log(NPRB,c /50) |
|  | Bands FDD_H |  |  |  | -89.98 + 10log(NPRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4a: Void. NOTE 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. NOTE 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. NOTE 8: Except Band 29. NOTE 9: Except Band 32, Band 75 and Band 76. NOTE 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.7.1.3	Test Requirements

The SA inter-RAT E-UTRAN RS-SINR measurement accuracy for Cell 2 shall fulfil absolute requirement in clause 10.2.4.

### A.6.7.8	CLI measurements

#### A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell

##### A.6.7.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.22.1.1 with the testing configurations for NR cells in table A.6.7.8.1.1-1.

Table A.6.7.8.1.1-1: Applicable NR configurations for FR1 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.1.2	Test parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.8.1.2-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in table A.6.7.8.1.2-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in table A.6.7.8.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.7.8.1.2-1: FR1 test parameters for SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15 kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 4: Test 1 is not used when testing with 30 kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-2: FR1 test parameters for SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15 kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -105 | -87 | -113 |
|  | NR_TDD_FR1_C |  |  |  |  | -112 |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -111 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 6: Test 1 is not used when testing with 30 kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-3: SRS configuration parameters for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

#### A.6.7.8.1.3	Test Requirements

The SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.22.1.1.

#### A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell

##### A.6.7.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.22.2.1 with the testing configurations for NR cells in table A.6.7.8.2.1-1.

Table A.6.7.8.2.1-1: Applicable NR configurations for FR1 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.2.2	Test parameters

In this set of test cases there is one cell in the test, the FR1 PSCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.8.2.2-1 below.

Before the test UE is configured to perform CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in table A.6.7.8.2.2-2.

Table A.6.7.8.2.2-1: FR1 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD configuration | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 TDD |
|  | 2 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 TDD |
|  | 2 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 TDD |
|  | 2 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR1 |
|  | 2 |  | SSB.2 FR1 |
| OCNG Patterns Note6 | 1~2 |  | OP.1 |
| TRS configuration | 1 |  | TRS.1.1 TDD |
|  | 2 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1~2 | s | 17.67 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/15 kHz | -106 |
|  | 2 |  | -106 |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/ BWP SCS | -106 |
|  | 2 |  | -103 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note3 | 1~2 | dBm/ BWP SCS | -Infinity |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/9.36 MHz | -78.05 |
|  | 2 | dBm/38.16 MHz | -71.96 |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/1.08 MHz | -87.43 |
|  | 2 |  | -87.44 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| Propagation condition | 1~2 |  | AWGN |
| Antenna configuration | 1~2 |  | 1x2 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 6: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |

Table A.6.7.8.2.2-2: CLI-RSSI measurement resource configuration for FR1 CLI-RSSI accuracy

|  | Field | Config | SRSConf.1 |
| --- | --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 1~2 | 0 |
|  | rssi-SCS | 1 | 15 kHz |
|  |  | 2 | 30 kHz |
|  | startPRB | 1~2 | 0 |
|  | nrofPRBs | 1 | 52 |
|  |  | 2 | 106 |
|  | startPosition | 1~2 | 3 |
|  | nrofSymbols | 1~2 | 11 |
|  | rssi-PeriodicityAndOffset | 1 | sl20, 9 |
|  |  | 2 | sl40, 19 |

##### A.6.7.8.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the requirements in clauses 10.1.22.2.1.

### A.6.7.9	L1-SINR measurement for beam reporting

#### A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

##### A.6.7.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.1 and clause 10.1.27.1 for L1-SINR measurements based on CSI-RS with the testing configurations for NR cells in table A.6.7.9.1.1-1.

Table A.6.7.9.1.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.1.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.9.1.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in table A.6.7.9.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.9.1.2-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -86.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -80.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.1.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.27.1.

#### A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR

##### A.6.7.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.2 and clause 10.1.27.2 for L1-SINR measurements with SSB based CMR and dedicated CSI-RS based IMR, with the testing configurations for NR cells in table A.6.7.9.2.1-1.

Table A.6.7.9.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.2.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.9.2.2-1 below. The absolute accuracy of L1-SINR measurements are tested by using the parameters in table A.6.7.9.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-RS resource set with two CSI-RS resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.7.9.2.2-1: FR1 SSB based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| CSI-RS configuration |  | 1 |  | CSI-RS 1.1A FDD | CSI-RS 1.1A FDD |
|  |  | 2 |  | CSI-RS 1.1A TDD | CSI-RS 1.1A TDD |
|  |  | 3 |  | CSI-RS 2.1A TDD | CSI-RS 2.1A TDD |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | ssb-Index-SINR-r16 | ssb-Index-SINR-r16 |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | 0 |
|  |  | 1~6 | dB | 10 | 0 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -86.04 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -85.54 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -85.04 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.54 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -84.04 |
|  | NR_FDD_FR1_F |  |  |  | -83.54 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -83.04 |
|  | NR_FDD_FR1_H |  |  |  | -82.54 |
|  | NR_FDD_FR1_N |  |  |  | -79.54 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -79.94 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -79.44 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -78.94 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -78.44 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.94 |
|  | NR_FDD_FR1_F |  |  |  | -77.44 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -76.94 |
|  | NR_FDD_FR1_H |  |  |  | -76.44 |
|  | NR_FDD_FR1_N |  |  |  | -73.44 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.2.3	Test Requirements

The L1-SINR measurement accuracy for SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 of Cell 1 shall fulfil the requirements in clauses 10.1.27.2.

#### A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR

##### A.6.7.9.3.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will partly verify the requirements in clauses 9.8.4.3 and clause 10.1.27.3 for L1-SINR measurements based on CSI-RS as CMR and CSI-IM as IMR with the testing configurations for NR cells in table A.6.7.9.3.1-1.

Table A.6.7.9.3.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.3.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.9.3.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in table A.6.7.9.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR.

Table A.6.7.9.3.2-1: FR2 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS configuration as CMR |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| CSI-IM configuration as IMR |  | 1 |  | CSI-IM 1.3 FDD | CSI-IM 1.3 FDD |
|  |  | 2 |  | CSI-IM 1.3 TDD | CSI-IM 1.3 TDD |
|  |  | 3 |  | CSI-IM 2.3 TDD | CSI-IM 2.3 TDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -113.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -86.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -80.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.3.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM# of Cell 1 shall fulfil the requirements in clause 10.1.27.3.

### A.6.7.10	CSI-RSRP

#### A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.10.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.3.1 and 10.1.2.3.2 for CSI-RS intra-frequency measurements.

##### A.6.7.10.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.10.1.2-1. Both absolute and relative accuracy of CSI-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.10.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.10.1.2-1: CSI-RSRP intra frequency supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.10.1.2-2: CSI-RSRP intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 4.7 | - | 4.7 | - | 4.7 |
|  |  | Config 3 | s | - | 2.35 | - | 2.35 | - | 2.35 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| CSI-RS configuration for RRM |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -106 |  | -88 |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -94 |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15 kHz |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -91 |  | -111 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -104.5 |  |
| for SSB |  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | -0.01 | -4.76 |
| for CSI-RS |  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | -0.01 | -4.76 |
| for SSB |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| for CSI-RS |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -100 | -105 | -82 | -87 | -111.00 | -114.00 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -110.50 | -113.50 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -110.00 | -113.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.50 | -112.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109.00 | -112.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.50 | -111.50 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -104.50 | -107.50 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -90 | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -107.00 | -110.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -106.50 | -109.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -106.00 | -109.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -105.50 | -108.50 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -105.00 | -108.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -104.50 | -107.50 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -101.50 | -104.50 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -70.09 |  | -52.09 |  | -80.03 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -79.53 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -79.03 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -78.53 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -78.03 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -77.53 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -77.03 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -73.53 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | Not applicableNote 5- |  | -51.99 |  | -73.94 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  | -73.44 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  | -72.94 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -72.44 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -71.94 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -71.44 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  | -70.94 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -70.44 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  | -67.44 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30 kHz SSB and CSI-RS SCS. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

##### A.6.7.10.1.3	Test Requirements

The CSI-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil absolute requirement in clause 10.1.2.3.1 and relative requirement in clause 10.1.2.3.2.

#### A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.10.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.3.1 and 10.1.4.3.2 for CSI-RS inter-frequency measurements with the testing configurations for NR cells in table A.6.7.9.2.1-1.

Table A.6.7.10.2.1-1: Applicable NR configurations for FR1 inter-frequency CSI-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.10.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in table A.6.7.10.2.2-1 below. Both absolute and relative accuracy of CSI-RSRP inter-frequency measurements are tested by using the parameters in table A.6.7.10.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.10.2.2-1: CSI-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |  |
|  |  | 2 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |  |
|  |  | 3 |  | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |  |
|  |  | 2 |  | TDD |  | TDD |  |  |
|  |  | 3 |  | TDD |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,2 | s | - | 4.7 | - | 4.7 |  |
|  |  | 3 | s | - | 2.35 | - | 2.35 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |  |
| CSI-RS configuration for RRM |  | 1 |  | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |  |
|  |  | 2 |  | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |  |
|  |  | 3 |  | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 |  | ( for Channel 2 +8 dB) |  | -115 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -108.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  | ( for Channel 2 +8 dB) |  | -115 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -108,5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  | ( for Channel 2 +8 dB) |  | -112.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -107.50 |
| for SSB |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| for CSI-RS |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  | (RSRP for Cell 2 +24dB) |  | -118.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -117.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  | (RSRP for Cell 2 +24dB) |  | -115.00 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -108.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36 MHz | -56.28 |  | Io for Channel 2 +19.45dB |  | -85.28 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -84.78 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -78.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16 MHz | -50.19 |  | Io for Channel 2 +19.45dB |  | -79.19 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -78.69 |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -75.69 |
|  | NR_FDD_FR1_N |  |  |  |  |  |  | -72.69 |
| for SSB |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| for CSI-RS |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

##### A.6.7.10.2.3	Test Requirements

The CSI-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.3.1 and relative requirement in clause 10.1.4.3.2.

### A.6.7.11	CSI-RSRQ

#### A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.7.2.

##### A.6.7.11.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.6.7.11.1.2-1. The absolute accuracy of CSI-RSRQ intra-frequency measurement is tested by using the parameters in table A.6.7.11.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.11.1.2-1: Intra frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.1.2-2: CSI-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  |  | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  | 0 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  |  | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 4.7 |  | - | 4.7 |  | - | 4.7 |
|  |  | Config 2,3 | s | - | 2.35 |  | - | 2.35 |  | - | 2.35 |
| CSI-RS configuration for RRM |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  |  | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
| for SSB |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5.46 | -5.46 |
| for CSI-RS |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5.46 | -5.46 |
| for SSB |  |  | dB | 3 | 3 | -2.9 |  |  | -2.9 | -4 | -4 |
| for CSI-RS |  |  | dB | 3 | 3 | -2.9 |  |  | -2.9 | -4 | -4 |
| SS-RSRP/ CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  |  | -103.9 | -118 | -118 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  |  | - | -115 | -115 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ/CSI-RSRQ Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  |  | -16.76 | -17.34 | -17.34 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -50 |  | - |  |  |  | -77.4 |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  | -76.9 |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  | -76.4 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -75.9 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -75.4 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -74.9 |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  | -74.4 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -73.9 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -70.9 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  |  | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |  |  | 1x2 | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.1.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.2.

#### A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.9.2.1 and 10.1.9.2.2.

##### A.6.7.11.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.6.7.11.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-RSRQ inter-frequency measurement are tested by using test parameters in table A.6.7.11.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.11.2.2-1: CSI-RSRQ Inter frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.2.2-2: CSI-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  | 0 |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NPRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 4.7 | - | 4.7 | - |  | 4.7 |  |
|  |  | Config 3 | s | - | 2.35 | - | 2.35 | - |  | 2.35 |  |
| CSI-RS configuration for RRM |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -86.27 |  | -113 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  | -110 |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -110 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  |  |
| for SSB |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| for CSI-RS |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| for SSB |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| for CSI-RS |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| SS-RSRP/CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 | -107.75 | -107.75 |  | -113 |  | -117.75 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -112.5 |  | -117.25 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  | -116.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  | -116.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  | -115.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  | -115.2 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 | -111.75 | -111.75 |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -109 |  | -113.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -108.5 |  | -113.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -108 |  | -112.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -107.5 |  | -112.2 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -107 |  | -111.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -103.5 |  | -108.25 |
| SS-RSRQ/CSI-RSRQNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -40.59 | -40.59 |  | 12.56T |  | 14.76T |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  | -75.83 |  |  | -83.28 |  | -85.83 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -82.78 |  | -85.33 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -82.28 |  | -84.83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -81.78 |  | -84.33 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -81.28 |  | -83.83 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -80.78 |  | -83.33 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -80.28 |  | -82.83 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -79.78 |  | -82.33 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -76.78 |  | -79.33 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  | -76.73 |  |  | -77.19 |  | -79.73 |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -76.69 |  | -79.23 |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -76.19 |  | -78.73 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -75.69 |  | -78.23 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -75.19 |  | -77.73 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -74.69 |  | -77.23 |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -74.19 |  | -76.73 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -73.69 |  | -76.53 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -70.69 |  | -73.53 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.2.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.2.1 and 10.1.9.2.2.

### A.6.7.12	CSI-SINR

#### A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.2.1.

##### A.6.7.12.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in table A.6.7.11.1.2-1. The absolute accuracy of CSI-SINR intra-frequency measurement is tested by using the parameters in table A.6.7.11.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.12.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.1.2-2: CSI-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| CSI-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 |  | s | - | 2.35 | - | 2.35 |
|  |  | Config 3 |  | s | - | 1.17 | - | 1.17 |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  | Config 1 |  | SMTC.2 |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |
| CSI-RS for mobility |  |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -93 |  | -116 |  |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -115.5 |  |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -113.5 |  |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -109.5 |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -93 |  | Same as Noc for 15 kHz |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  | -113 |  |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -110.5 |  |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -110 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -109.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -106.5 |  |
| for SSB |  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| for CSI-RS |  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| for SSB |  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| for CSI-RS |  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 | -120 | -120 |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -119.5 | -119.5 |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -119 | -119 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 | -118.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 | -118 |
|  |  |  | NR_FDD_FR1_F |  |  |  | -117.5 | -117.5 |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -117 | -117 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  | -113.5 | -113.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 | -117 | -117 |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -116 | -116 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 | -115.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 | -115 |
|  |  |  | NR_FDD_FR1_F |  |  |  | -114.5 | -114.5 |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -114 | -114 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -113.5 | -113.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  | -113.5 | -113.5 |
| CSI-SINR Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -110.5 | -110.5 |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -57.5 |  | -85.51 |  |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -85.01 |  |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -84.51 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.01 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -83.51 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -83.01 |  |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -82.51 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -82.01 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -79.01 |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -51.41 |  | -79.41 |  |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  | -78.91 |  |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  | -78.41 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -77.91 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.41 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -76.91 |  |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  | -76.41 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -75.91 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -72.91 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

##### A.6.7.12.1.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.2.1.

#### A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.2.1 and 10.1.14.2.2.

##### A.6.7.12.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in table A.6.7.12.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-SINR inter-frequency measurement are tested by using test parameters in table A.6.7.12.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.12.2.2-1: CSI-SINR Inter frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.2.2-2: CSI-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 |  | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 2.35 | - | 2.35 | - |  | 2.35 |  |
|  |  | Config 3 | s | - | 1.17 | - | 1.17 | - |  | 1.17 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS for mobility |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15 kHz | -88 |  | -108.5 |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -113 |  |  |
| Note2 | Config 1,2 N |  | dBm/SCS | -88 |  | -108.5 |  |  | Same as Noc for 15 kHz T |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  | -105.5 |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -110 |  |  |
| for SSB |  |  | dB | -1.75 | -1.75 | 15 | 15 |  | -4.0 |  | -4.0 |
| for CSI-RS |  |  | dB | -1.75 | -1.75 | 15 | 15 |  | -4.0 |  | -4.0 |
| for SSB |  |  | dB | -1.75 |  | 15 |  |  | -4.0 |  |  |
| for CSI-RS |  |  | dB | -1.75 |  | 15 |  |  | -4.0 |  |  |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  | -93.5 |  |  | -123.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -123 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -122.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -122 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -121.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -121 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -117 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  | -90.5 |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -114 |  |  |
| CSI-SINRNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  | 15 |  |  | -4.0 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36 MHz | -57.83 |  | -65.4 |  |  | -90.09 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -89.59 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -89.09 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -88.59 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -88.09 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -87.59 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -87.09 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -86.59 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -83.59 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16 MHz | -51.73 |  | -59.3 |  |  | -84 |  |  |
|  |  | NR_FDD_FR1_B, NR_TDD_FR1_B |  |  |  |  |  |  | -83.5 |  |  |
|  |  | NR_FDD_FR1_C, NR_TDD_FR1_C |  |  |  |  |  |  | -83 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -82.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -82 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -81.5 |  |  |
|  |  | NR_FDD_FR1_G, NR_TDD_FR1_G |  |  |  |  |  |  | -81 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -80.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -77.5 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: NR operating band groups are as defined in clause 3.5.2. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.12.2.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.2.1 and 10.1.14.2.2.

### A.6.7.13	RSTD measurements

#### A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer

##### A.6.7.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.7.13.1.1-1.

Table A.6.7.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 2 | 0 | 2 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -65.70 | -65.70 | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88 MHz | -61.72 | -61.72 | -61.72 | -61.72 |
| PRS | 1~3 | dB | -65.70 | -65.70 | -65.70 | -65.70 |
| SSB | 1~3 | dB | -5.7 |  | -11.9 |  |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer

##### A.6.7.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.7.13.2.1-1.

Table A.6.7.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 2 | 0 | 2 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -104 | -111 | -104 | -111 |
|  | 3 |  | -101 | -108 | -101 | -108 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -65.98 | -66.74 | -65.98 | -66.74 |
|  | 3 | dBm/ 47.88 MHz | -62.00 | -62.76 | -62.00 | -62.76 |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state

##### A.6.7.13.3.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements for reduced number of samples specified in clause 10.1.23.2 in an environment with AWGN propagation conditions. In this test UE that supports supportedDL-PRS-ProcessingSamples is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in table A.6.7.13.3.1-1.

Table A.6.7.13.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be longer than the UE measurement period as defined in clause 9.9.2.7.

Table A.6.7.13.3.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.3 FR1 |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.3 FR1 |  | PRS.2.4 FR1 |  |
| PRS BW | 1,2 |  | 52 PRBs |  | - |  |
|  | 3 |  | 48 PRBs |  | - |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
|  | 1~3 | dB | -3 | -6 | -3 | -6 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -99.44 | -101.65 | -99.44 | -101.65 |
|  | 3 |  | -96.44 | -98.65 | -96.44 | -98.65 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -99.44 | -101.65 | -99.44 | -101.65 |
|  | 3 |  | -96.44 | -98.65 | -96.44 | -98.65 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -63.63 | -63.63 | -63.63 | -63.63 |
|  | 3 | dBm/ 47.88 MHz | -59.65 | -59.65 | -59.65 | -59.65 |
|  | 1~3 | dB | -1.44 | -3.65 | -1.44 | -3.65 |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.3.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG

##### A.6.7.13.4.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement when the measurements of reference cell and neighbor cell are within the same Rx TEG meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.7.13.4.1-1.

Table A.6.7.13.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports GP#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

The UE is requested to provide the Rx TEG in the test via nr-UE-RxTEG-Request-r17 in NR-TDOA-RequestLocationInformation.

The test applies to the UE supporting Rx TEG defiend in NR-UE-TEG-Capability and reporting the same Rx TEG for the measurements of reference cell and neighbour cell.

Table A.6.7.13.4.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 | - | 3 |
|  | 2,3 |  | - | 3 | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  | PRS.1.2 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | 1~3 | slot | 0 | 2 | 0 | 2 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | N/A | 5 |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
| PRS | 1~3 | dB | -6 | -13 | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -65.70 | -65.70 | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88 MHz | -61.72 | -61.72 | -61.72 | -61.72 |
| PRS | 1~3 | dB | -5.7 | -11.9 | -5.7 | -11.9 |
| Propagation condition | 1~3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.7.13.4.2	Test Requirements

The RSTD measurement for Cell 1 and Cell 2 should fulfil the absolute accuracy requirements with same Rx TEG in clause 10.1.23.2.

#### A.6.7.13.5	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode

##### A.6.7.13.5.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement by aggregating PRS resoureces from two positioning frequency layers (PFLs) meets the measurement accuracy requirements specified in clause 10.1.23A.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.7.13.5.1-1.

Table A.6.7.13.5.1-1: Supported test configurations for PRS aggregation

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth per PFL, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth per PFL, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth per PFL, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Each cell is associated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, all cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE before the start of the test. The UE is capable of performing RSTD measurements by aggregating PRS resources from two PFLs and is configured by the LMF to perform measurements by aggregating the PRS resources from two positioning frequency layers via nr-DL-PRS-JointMeasurementRequestedPFL-List. The NR-DL-TDOA-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.5.1-2: RSTD accuracy test parameters for PRS aggregation

| Parameter | Config | Unit | Cell 1 | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
| PRS ARFCN | 1,2,3 |  | freq1, freq2 | freq1, freq2 |  |
| BWchannel per carrier | 1 | MHz | 20: NRB,c = 106 |  |  |
|  | 2 |  | 20: NRB,c = 106 |  |  |
|  | 3 |  | 50: NRB,c = 133 |  |  |
| Duplex mode | 1 |  | FDD |  |  |
|  | 2 |  | TDD |  |  |
|  | 3 |  | TDD |  |  |
| TDD configuration | 1 |  | N/A |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  | - |
|  | 2 |  | SR.1.1 TDD |  |  |
|  | 3 |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  | - |
|  | 2 |  | CR.1.1 TDD |  | - |
|  | 3 |  | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  | - |
|  | 2 |  | CCR.1.1 TDD |  | - |
|  | 3 |  | CCR.2.1 TDD |  | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - |
|  | 2 |  | TRS.1.1 TDD |  |  |
|  | 3 |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 | 1 | s | - |  | 3 |
|  | 2,3 |  | - |  | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  |
|  | 2,3 |  | SMTC.1 |  |  |
| PRS configuration | 1 |  | PRS.1.2 FR1 |  |  |
|  | 2 |  | PRS.1.2 FR1 |  |  |
|  | 3 |  | PRS.2.2 FR1 |  |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 |  | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A |  | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A |  | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3 | dB | 0 |  | 0 |
| NocNote2 | 1,2 | dBm/ SCS | -98 |  |  |
|  | 3 |  | -95 |  |  |
| PRS EsIot | 1,2,3 | dB | -6 |  | -13 |
| PRP | 1,2 | dBm/SCS | -103.7 |  | -109.9 |
|  | 3 |  | -100.7 |  | -106.9 |
| Io | 1,2 | dBm/ 19.08MHz per PFL | -65.70 |  | -65.70 |
|  | 3 | dBm/ 47.88MHz per PFL | -61.72 |  | -61.72 |
| PRS EsNoc | 1,2,3 | dB | -65.70 |  | -65.70 |
| SSB EsNoc | 1,2,3 | dB | -5.7 |  |  |
| Propagation condition | 1,2,3 | - | AWGN |  |  |

##### A.6.7.13.5.2	Test Requirements

The RSTD measurement accuracy shall fulfil the requirements defined in clause 10.1.23A.2.

### A.6.7.14	PRS-RSRP measurements

#### A.6.7.14.1	SA: measurement accuracy with PRS in FR1

##### A.6.7.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.7.14.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.14.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.7.14.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.14.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.14.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 2 | 0 | 2 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | s | - | 3 | - | 3 |  |
|  |  | Config 2,3 |  | - | 3 | - | 3 |  |
| Expected RSTD |  | Config 1,2,3 | s | 3 |  |  |  |  |
| Expected RSTD uncertainty |  | Config 1,2,3 | s | 5 |  |  |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  | -95 |  |  |
| PRS |  |  | dB | -2.41 | -12.12 | -2.41 |  | -12.12 |
| PRS |  |  | dB | -2 | -10 | -2 |  | -10 |
| PRP Note3 | Config 1, 2 |  | dBm/SCS | -100 | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -64.57 |  | -64.57 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -60.59 |  | -60.59 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. NOTE 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |  |  |

##### A.6.7.14.1.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number

##### A.6.7.14.2.1	Test Purpose and Environment

The purpose of this test is to verify that the accuracy of PRS-RSRP measurement with reduced sample number is within the specified limits provided that PRS is transmitted within the active BWP of the UE. UE can support supportedDL-PRS-ProcessingSamples-RRC-CONNECTED, and the LMF indicates the UE to perform positioning measurements with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.7.14.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.14.2.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.7.14.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.14.2.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.14.2.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 | PRS.2.4 FR1 |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 2 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 |
|  |  | Config 2,3 | s | - | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |
|  |  | Config 2,3 |  | SMTC.1 |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  |
|  | Config 3 |  |  | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 0 | -6 |
|  |  |  | dB | 2.23 | -1.73 |
| SS-RSRP |  |  | dB | Same as PRS-RSRP |  |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -95.77 | -99.73 |
|  | Config 3 |  |  | -92.77 | -96.73 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -61.71 |  |
|  | Config 3 |  | dBm/47.88 MHz | -57.73 |  |
| Propagation condition |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. NOTE 6: Void. NOTE 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |

##### A.6.7.14.2.3	Test Requirements

In the test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.7.14.3	Void

##### A.6.7.14.3.1	Void

##### A.6.7.14.3.2	Void

##### A.6.7.14.3.3	Void

### A.6.7.15	UE Rx-Tx time difference measurements

#### A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA

##### A.6.7.15.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.7.15.1.1-1.

Table A.6.7.15.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.15.1.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in table A.6.7.15.1.2-1.

Table A.6.7.15.1.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 2 | 0 | 2 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | -12.12 | -2.41 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | -10 | -2 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -100 | -108 | -100 | -108 |
|  |  | 2 | -100 | -108 | -100 | -108 |
|  |  | 3 | -97 | -105 | -97 | -105 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/19.08 MHz | 2 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/47.88 MHz | 3 | -60.59 | -60.59 | -60.59 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| NOTE 1: Void. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

Table A.6.7.15.1.2-2: Void

##### A.6.7.15.1.3	Test requirements

The UE Rx-Tx time difference measurement fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

#### A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA

##### A.6.7.15.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.7.15.2.1-1.

Table A.6.7.15.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation during the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.15.2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in table A.6.7.15.2.2-1.

Table A.6.7.15.2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 52 PRBs | 52 PRBs |
|  |  | 2 | 52 PRBs | 52 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 2 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -6 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 2.23 | -1.73 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -99.73 |
|  |  | 2 | -95.77 | -99.73 |
|  |  | 3 | -92.77 | -96.73 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -61.71 |
|  | dBm/19.08 MHz | 2 | -61.71 | -61.71 |
|  | dBm/47.88 MHz | 3 | -57.73 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

##### A.6.7.15.2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples fulfils the UE Rx-Tx measurement accuracy.

#### A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG

##### A.6.7.15.3.1	Test purpose and environment

The purpose of the test is to verify that the relative UE Rx-Tx time difference measurement accuracy when the two measurements are within the same RxTx TEG is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations is listed in table A.6.7.15.3.1-1.

Table A.6.7.15.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is requested to provide the RxTx TEG in the test via nr-UE-RxTxTEG-Request-r17 in NR-Multi-RTT-RequestLocationInformation.

The test applies to the UE supporting RxTx TEG defiend in NR-UE-TEG-Capability and reporting the same RxTx TEG for the two UE Rx-Tx measurements.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The UE Rx-Tx time difference is derived by the difference of the receiving timing and the transmit timing for each cell.

##### A.6.7.15.3.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in table A.6.7.15.3.2-1. y requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

Table A.6.7.15.3.2-2: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 2 | 0 | 2 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | -12.12 | -2.41 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | -10 | -2 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -100 | -108 | -100 | -108 |
|  |  | 2 | -100 | -108 | -100 | -108 |
|  |  | 3 | -97 | -105 | -97 | -105 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/19.08 MHz | 2 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/47.88 MHz | 3 | -60.59 | -60.59 | -60.59 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| NOTE 1: Void. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

##### A.6.7.15.3.3	Test requirements

The relative accuracy is derived by the difference of the UE Rx-Tx measurements on the two cells.

The UE Rx-Tx time difference measurements for Cell 1 and Cell 2 fulfil the relative UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.x.

#### A.6.7.15.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA

##### A.6.7.15.4.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25A. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform UE Rx-Tx measurements by aggregating two intra-band contiguous positioning frequency layers (PFLs) in FR1.

The supported test configurations are listed in table A.6.7.15.4.1-1.

Table A.6.7.15.4.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: Cell 1 (PCell) and Cell 2 (neighbor cell). Each cell is sssociated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData 37.355 [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, both cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2.. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12, shall be provided to the UE before the start of the test.

The NR-Multi-RTT-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The NR-Multi-RTT-RequestLocationInformation message provided to the UE must request bandwidth aggregated measurements via jointMeasurementsReq and nr-DL-PRS-JointMeasurementRequestedPFL-List.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

The general test parameters and cell specific test parameters are as given in table A.6.7.15.4.1-2 and table A.6.7.15.4.1-3 respectively.

Table A.6.7.15.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2, all signals/channels except PRS in PFL2 are allocated within this channel. |
|  |  |  | 2 | For both Cell 1 and Cell 2, only PRS in PFL2 is allocated within this channel. |
| BWchannel | MHz | 1 | 20: NRB,c = 106 | For both RF channels |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells/TRPs |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| Note 1: GP#24 is configured if the UE supports it, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.7.15.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns Note 1 |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active BWP configuration |  | 1, 2, 3 | DLBWP.1.1 ULBWP.1.1 | N/A |
| PRS configuration (for both PFL1 and PFL2) |  | 1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset (for both PFL1 and PFL2) | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1, 2, 3 | -3 | -13 |
| PRS | dB | 1, 2, 3 | -3 | -13 |
| PRP Note 3 | dBm/SCS kHz | 1 | -101 | -111 |
|  |  | 2 | -101 | -111 |
|  |  | 3 | -98 | -108 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/19.08 MHz | 1 | -65.05 | -65.05 |
|  | dBm/19.08 MHz | 2 | -65.05 | -65.05 |
|  | dBm/47.88 MHz | 3 | -61.06 | -61.06 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. |  |  |  |  |

##### A.6.7.15.4.2	Test requirements

The UE Rx-Tx time difference measurement fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25A for both Cell 1 and Cell 2.

### A.6.7.16	PRS-RSRPP measurements

#### A.6.7.16.1	SA: measurement accuracy with PRS in FR1

##### A.6.7.16.1.1	Test Purpose and Environment

The purpose of this test is to verify that the accuracy of PRS-RSRPP measurement in RRC_CONNECTED is within the specified limits. This test will verify the requirements in clauses 10.1.38.2.

##### A.6.7.16.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.16.1.2-1. Both absolute accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.7.16.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.16.1.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.16.1.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 2 | 0 | 2 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  | -95 |  |  |
|  |  |  | dB | -2.41 | -12.12 | -2.41 |  | -12.12 |
|  |  |  | dB | -2 | -10 | -2 |  | -10 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -100 | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -64.57 |  | -64.57 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -60.59 |  | -60.59 |  |  |
| Propagation condition |  |  |  | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRPP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. NOTE 6: Void. NOTE 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |  |  |

##### A.6.7.16.1.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.38.2.

#### A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1

##### A.6.7.16.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy with Nsample = 1 in FR1 is within the specified limits. This test will verify the requirements in clauses 10.1.38.2.

The UE under test should support supportedDL-PRS-ProcessingSamples-RRC-CONNECTED, and the TE indicates the UE to perform positioning measurements with reduced number of samples. The PRS bandwidth is contained within the active BWP and the power difference between the serving cell SS-RSRP and neighbour cell PRS-RSRP is within 6 dB, so that Nsample= 1 is assumed.

##### A.6.7.16.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.16.2.2-1. In all test cases, Cell 1 is the PCell.

Table A.6.7.16.2.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.16.2.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 5 |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 |  | PRS.2.4 FR1 |
| PRS bandwidth |  | Config 1,2,3 |  | 52 PRBs |  | 52 PRBs |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 2 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |
|  | Config 3 |  |  | -95 |  |  |
|  |  |  | dB | 0 |  | -6 |
|  |  |  | dB | 2.23 |  | -1.73 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -95.77 |  | -99.73 |
|  | Config 3 |  |  | -92.77 |  | -96.73 |
| SS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -95.77 |  | -99.73 |
|  | Config 3 |  |  | -92.77 |  | -96.73 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -61.71 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -57.73 |  |  |
| Propagation condition |  |  |  | Two-tap channel Note6 |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. NOTE 6: The two-tap channel model is defined in 38.101-4 annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |

##### A.6.7.16.2.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.38.2.

### A.6.7.17	LTM L1-RSRP measurement

#### A.6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1

##### A.6.7.17.1.1	Test Purpose and Environment

The purpose of this test is to verify that the inter-frequency L1-RSRP measurement accuracy on neigbor cell is within the specified limits. This test will verify the requirements in clause 9.15.5 and clause 10.1.19E for inter-frequency L1-RSRP measurements based on SSB with the testing configurations for NR cells in table A.6.7.17.1.1-1.

Table A.6.7.17.1.1-1: Applicable NR configurations for FR1 SSB based inte-frequency L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations in each supported band NOTE 2: Target NR cell has the same SCS, BW and deplex mode as NR serving cell. |  |

##### A.6.7.17.1.2	Test parameters

In this set of test cases there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for the Cell 2 are given in table A.6.7.17.1.2-1 below. The absolute accuracy of L1-RSRP measurements are tested by using the parameters in table A.6.7.17.1.2-1.

Measurement gap pattern configuration defined in table A.6.7.17.1.2-1 is provided. Before the test,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC)

-	UE is configured one SSB resource set with one SSB resource on Cell 2

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE to reprort periodica reporting with with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

Table A.6.7.17.1.2-1: FR1 inter-frequency SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |
|  |  | 2 |  | TDD |  | TDD |  |
|  |  | 3 |  | TDD |  | TDD |  |
| Gap Pattern Id |  | 1~3 |  | 0 |  |  |  |
| Measurement gap offset |  | 1~3 |  | 39 |  |  |  |
| TDD Configuration |  | 1 |  | N/A |  | N/A |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
|  |  | 2 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
|  |  | 3 |  | 40: NPRB,c = 106 |  | 40: NPRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | 2 |  | SR.1.1 TDD | - | SR.1.1 TDD | - |
|  |  | 3 |  | SR.2.1 TDD | - | SR.2.1 TDD | - |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  |  | 3 |  | CR.2.1 TDD | - | CR.2.1 TDD | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  |  | 3 |  | TRS.1.2 TDD | - | TRS.1.2 TDD | - |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| reportConfigType |  | 1~3 |  | periodic |  | periodic |  |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS |  | 1~3 |  | 1 |  | 1 |  |
| L1-RSRP reporting period |  | 1~3 |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15 kHz | -94.65 |  | ( for Channel 2 +8 dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 |  | ( for Channel 2 +8 dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 |  | ( for Channel 2 +8 dB) | -114 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | 10 | 13 | -3 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 |  | (RSRP for Cell 2 +24 dB) | -120 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 |  | (RSRP for Cell 2 +24 dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 |  | (Io for Channel 2 +19.75 dB) | -87.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 |  | (Io for Channel 2 +19.75 dB) | -81.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | 10 | 13 | -3 |
| Propagation condition |  | 1~3 |  | AWGN |  | AWGN |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

##### A.6.7.17.1.3	Test Requirements

The inter-frequency L1-RSRP measurement accuracy for SSB resource reported by UE in L1-RSRP report (SSB#0 of Cell 2) shall fulfil the requirements in clauses 10.1.19E.

### A.6.7.18	TDCP amplitude measurement accuracy

#### A.6.7.18.1	TDCP amplitude measurement accuracy in FR1

##### A.6.7.18.1.1	Test Purpose and Environment

The purpose of this test is to verify that the TRS based TDCP amplitude measurement accuracy is within the specified limits in the test requirements section. The cofigurations for the test are specified in table A.6.7.18.1.1-1.

The test consists of two tests, Test 1 and Test 2. Each test further consists of two subtests Test 1A, 1B and Test 2A, 2B.

Test 1A: 10 Hz doppler + 15 kHz SCS FDD + 20 dB SNR

Test 1B: 10 Hz doppler + 30 kHz SCS TDD + 20 dB SNR

Test 2A: 300 Hz doppler + 15 kHz SCS FDD + 10 dB SNR

Test 2B: 300 Hz doppler + 30 kHz SCS TDD + 10 dB SNR

Relevant parmeters for each test are provided in the table A.6.7.18.1.2-1. UE needs to pass Test 1A, 1B, 2A, 2B.

Table A.6.7.18.1.1-1: Applicable NR configurations for FR1 TRS based TDCP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

##### A.6.7.18.1.2	Test parameters

In this set of test cases there is one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.18.1.2-1. Ampliutude of TDCP is tested by using the parameters in table A.6.7.18.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured with 1 TRS set with the TRS resources in the set are configured in adjacent slot. UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.18.1.2-1: FR1 TRS based TDCP test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1 |  | FDD | FDD |
|  | 2 |  | TDD | TDD |
| TDD Configuration | 1 |  | N/A | N/A |
|  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 | 10: NPRB,c = 52 |
|  | 2 |  | 40: NPRB,c = 106 | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | 2 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | 2 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  | 2 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration | 1 |  | TRS.1.5 FDD | TRS.1.5 FDD |
|  | 2 |  | TRS.1.5 TDD | TRS.1.5 TDD |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Noc | dBm/15 kHz | 1 | -98 | -98 |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 | -98 |
|  |  | 2 | -95 | -95 |
|  | 1~2 | dB | TBD | TBD |
|  | 1~2 | dB | 10 | 20 |
| SSB_RP | 1 |  | -88 | -78 |
| Propagation condition | 1~2 |  | TDL-A 30 ns 10 Hz | TDL-A 30 ns 300 Hz |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

##### A.6.7.18.1.3	Test Requirements

For Test 1A, the reported TDCP index shall be smaller than or equal to 6 for the 80% of the times over repeated tests

For Test 1B: the reported TDCP index shall be smaller than or equal to 5 for the 80% of the times over repeated tests

For Test 2A: the reported TDCP index shall be lrager than 8 for the 80% of the times over repeated tests

For Test 2B: the reported TDCP index shall be larger than  6 for the 80% of the times over repeated tests

### A.6.7.19	RSCPD Measurements

#### A.6.7.19.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_CONNECTED

##### A.6.7.19.1.1	Test purpose and environment

The purpose of the test is to verify that accuracy of RSCPD measurement reported with RSTD measurement is within the specified limits. This test will verify the requirements in clause 10.1.43.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are listed in table A.6.7.19.1.1-1.

Table A.6.7.19.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation with dl-PRS-RSCPD-Request from LMF via LPP [34] as defined in TS 37.355 [34], clause 6.5.12, to enable UE to perform and report RSCPD in RRC CONNECTED, shall be provided to the UE before the start of the test. The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

##### A.6.7.19.1.2	Test parameters

The RSCPD with RSTD accuracy test parameters are given in table A.6.7.19.1.2-1.

Table A.6.7.19.1.2-1: RSCPD with UE RSTD measurement accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1,2,3 |  | Freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| Measurement gap | 1, 2, 3 |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |
|  | 2,3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3 | dB | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |
|  | 3 |  | -95 |  |
| PRS | 1,2,3 | dB | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |
| PRS | 1,2,3 | dB | -5.7 | -11.9 |
| SSB | 1,2,3 | dB | -5.7 |  |
| Propagation condition | 1,2,3 | - | AWGN |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |

##### A.6.7.19.1.3	Test requirements

The RSCPD reported together with RSTD fulfils RSCPD measurement accuracy requirements specified in clause 10.1.43.2 for Cell 2.

### A.6.7.20	RSCP Measurements

#### A.6.7.20.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA

##### A.6.7.20.1.1	Test purpose and environment

The purpose of the test is to verify that the accuracy of RSCP measurement with UE Rx-Tx time difference measurement is within the specified limits. This test will verify the requirements in clause 10.1.44.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.7.20.1.1-1.

Table A.6.7.20.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData , NR-Multi-RTT-RequestLocationInformation with nr-DL-PRS-RSCP-Request from LMF via LPP and NR-Multi-RTT-MeasurementCapability as defined in TS 37.355 [34], clause 6.5.12, to enable UE to perform and report RSCP in RRC CONNECTED, shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.20.1.2	Test parameters

The RSCP with UE Rx-Tx time difference accuracy test parameters are given in table A.6.7.20. 1.2-1.

Table A.6.7.20.1.2-1: RSCP with UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  |
| DRX |  | 1,2,3 | OFF |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 52 PRBs | 52 PRBs |
|  |  | 2 | 52 PRBs | 52 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -6 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 2.23 | -1.73 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -99.73 |
|  |  | 2 | -95.77 | -99.73 |
|  |  | 3 | -92.77 | -96.73 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -61.71 |
|  | dBm/19.08 MHz | 2 | -61.71 | -61.71 |
|  | dBm/47.88 MHz | 3 | -57.73 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

##### A.6.7.20.1.3	Test requirements

The RSCP reported with UE Rx-Tx time difference measurement fulfils RSCP measurement accuracy specified in clause 10.1.44.2 for both Cell 1 and Cell 2.

## A.6.8	Measurement procedure in RRC_INACTIVE

### A.6.8.1	RSTD measurements

#### A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state

##### A.6.8.1.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in clause 5.6.2.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.8.1.1.1-1.

Table A.6.8.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2 and Cell 3. During T2 UE shall be in RRC_INACTIVE state and all three cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the first DRX cycle containing a DL PRS resource(s).

The UE is configured with DRX cycle of 1.28 s.

The general test parameters are listed in table A.6.8.1.1.1-2, and cell specific test parameters are listed in table A.6.8.1.1.1-3 and table A.6.8.1.1.1-4.

Table A.6.8.1.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.8.1.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08 MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88 MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.8.1.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 96.48 MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88 MHz | -61.44 | -61.44 | -61.44 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cells (all, except Cell 3 in T2) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. NOTE 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.8.1.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 5.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 5.6.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049.

#### A.6.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA

##### A.6.8.1.2.1	Test Purpose and Environment

##### A.6.8.1.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in clause 5.6.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single-sample measurements are requested by the LMF. This test is applicable to UEs that support supportedDL-PRS-ProcessingSamples-RRC-Inactive.

The supported test configurations are specified in table A.6.8.1.2.1-1.

Table A.6.8.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2. PRS resources from all three cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

NOTE:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The nr-DL-TDOA-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to requested.

The general test parameters are listed in table A.6.8.1.2.1-2, and cell specific test parameters are listed in table A.6.8.1.2.1-3 and table A.6.8.1.2.1-4.

Table A.6.8.1.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.2 FR1 | As specified in clause A.3.31. |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | Config 1, 2, 3 | slots | Cell 1, 2, 3: 0 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX cycle length |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 4 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.8.1.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 0 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -63.94 | -63.94 | -63.94 |
|  | Config 2 | dBm/ 19.08 MHz | -63.94 | -63.94 | -63.94 |
|  | Config 3 | dBm/ 47.88 MHz | -59.96 | -59.96 | -59.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -98 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -98 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -95 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.8.1.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | 1 | -1.2 | -1.2 |
|  | Config 2 | dB | 1 | -1.2 | -1.2 |
|  | Config 3 | dB | 1 | -1.2 | -1.2 |
| Io Note 4 | Config 1 | dBm/ 19.08 MHz | -61.18 | -61.18 | -61.18 |
|  | Config 2 | dBm/ 19.08 MHz | -61.18 | -61.18 | -61.18 |
|  | Config 3 | dBm/ 47.88 MHz | -57.20 | -57.20 | -57.20 |
| PRS |  | dB | -3 | -6 | -6 |
| Propagation Condition |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 4: Io levels apply only for symbols where PRS is transmitted. |  |  |  |  |  |

##### A.6.8.1.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 5.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 5.6.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

#### A.6.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_INACTIVE state

##### A.6.8.1.3.1	Test purpose and environment

The purpose of the test is to verify that the RSTD measurement with PRS aggregation in RRC_INACTIVE state meets the requirements specified in clause 5.6.2.6 in AWGN propagation condition in FR1 in standalone scenario when two intra-band contiguous positioning frequency layers (PFL) are configured.

The supported test configurations are listed in table A.6.8.1.3.1-1.

Table A.6.8.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are 6 synchronous cells in the test: Cell 1, Cell 2, Cell 3 Cell 4, Cell 5 and Cell 6. Cell 1 is the PCell on NR RF channel 1 in FR1. Cell 2 and Cell 3 are neighbour cells on the same RF channel as Cell 1. Cell 4, Cell 5 and Cell 6 are the neighbour cells on a different NR RF channel, i.e., RF channel 2, in FR1. Cell 1 and Cell 4, Cell 2 and Cell 5, Cell 3 and Cell 6 are respectively intra-band contiguous and PRS resources are transmitted by the same Tx chain for each combination.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. During T2 UE shall be in RRC_INACTIVE state and all 6 cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12.1, shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

In NR-TDOA-ProvideAssistanceData, there are three NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList. The first list indicates aggregation of PRS resource sets from Cell 1 and Cell 4, and the second list indicates aggregation of PRS resource sets from Cell 2 and Cell 5. The third list indicates aggregation of PRS resource sets from Cell 3 and Cell 6. In NR-TDOA-RequestLocationInformation, the IE nr-DL-PRS-JointMeasurementRequestedPFL-List is included and indicates aggregation of PFLs on RF channel 1 and RF channel 2.

The beginning of the time interval T2 shall be aligned with the first DRX cycle containing a DL PRS resource(s).

The UE is configured with DRX cycle of 1.28s.

The general test parameters are given in table A.6.8.1.3.1-2, and cell specific test parameters for T1 and T2 are listed in tables A.6.8.1.3.1-3 and A.6.8.1.3.1-4, respectively.

Table A.6.8.1.3.1-2: General test parameters for RSTD measurement with PRS aggregation reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1, Cell 4 | Reference cells are the cells in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell 1 is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6 | Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6 appear at the first, second, third, fourth and fifth places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 and (PCI of Cell 1 – PCI of Cell 4)mod6=0 and (PCI of Cell 1 – PCI of Cell 5)mod6=0 and (PCI of Cell 1 – PCI of Cell 6)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2, Cell 4, Cell 5 to Cell 1: 0 Cell 3, Cell 6 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2 to Cell 6: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 6 | Including the reference cells |
| PRS muting info |  |  | Cell 1, Cell 4: ‘10’ Cell 2, Cell 5: ‘01’ Cell 3, Cell 6: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1, Cell 4: 0 Cell 2, Cell 5: 0 Cell 3, Cell 6: 1 | Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6 |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.8.1.3.1-3: Cell-specific test parameters for RSTD measurement with PRS aggregation reporting delay during T1

| Parameter |  | Unit | Cell 1, Cell 4 | Cell 2, Cell 5 | Cell 3, Cell 6 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number # |  |  | 1, 2 | 1, 2 | 1, 2 |
| Positiong frequency layer |  |  | PFL 1, 2 | PFL 1, 2 | PFL 1, 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in clause A.3.2.1Note 1 |  |  | OP.1 | N/A | N/A |
| Note 3, 5 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB Note 5 |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4, 5 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4, 5 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Note 5: The values are defined separately for the cells from which the transmitted PRS resources are aggregated. |  |  |  |  |  |

Table A.6.8.1.3.1-4: Cell-specific test parameters for RSTD measurement with PRS aggregation reporting delay during T2

| Parameter |  | Unit | Cell 1, Cell 4 | Cell 2, Cell 5 | Cell 3, Cell 6 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number # |  |  | 1, 2 | 1, 2 | 1, 2 |
| Positiong frequency layer |  |  | PFL 1, 2 | PFL 1, 2 | PFL 1, 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in clause A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3, 4 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS Note 4 | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 3, 4 | Config 1 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 96.48MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88MHz | -61.44 | -61.44 | -61.44 |
| PRS Note 4 |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: The values are defined separately for the cells from which the transmitted PRS resources are aggregated. |  |  |  |  |  |

##### A.6.8.1.3.2	Test requirements

The RSTD measurement time with PRS aggregation in RRC_INACTIVE state fulfils the requirements specified in clause 5.6.2.6.

The UE shall perform and report the RSTD measurements by aggregating PRS resources from Cell 2 and Cell 5, Cell 3 and Cell 6 respectively with respect to the Cell 1 and Cell 4 from which the transmitted PRS resources are also aggregated, within the time duration specified in section 5.6.2.6 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events observed during repeated tests shall be at least 90%, where the reported RSTD measurement with PRS aggregation for each correct event shall be within the RSTD reporting range specified in clause 10.1.23A.3.

#### A.6.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for non-RedCap UE

##### A.6.8.1.4.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in clause 5.6.2.5 when the configured eDRX cycle is longer than 10.24s in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.8.1.4.1-1.

Table A.6.8.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2 and Cell 3. During T2 UE shall be in RRC_INACTIVE state and all three cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

NOTE: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.10, shall be provided to the UE during T1. The UE is configured to report positioning measurements every 20s via reportingInterval in nr-DL-TDOA-RequestLocationInformation such the value of reportingInterval  is set to "ri20". The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 is not limited to PTW.

The UE is configured with eDRX cycle of 40.96s.

The general test parameters are listed in table A.6.8.1.4.1-2, and cell specific test parameters are listed in table A.6.8.1.4.1-3 and table A.6.8.1.4.1-4.

Table A.6.8.1.4.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2) mod 6 = 0 and (PCI of Cell 1 – PCI of Cell 3) mod 6 = 0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  | s | 1.28 |  |
| CN and RAN eDRX configuration |  | s | eDRX length = 40.96 PTW length = 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to NR-MutingPattern defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.8.1.4.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in clause A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Noc Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS EsNoc |  | dB | -Infinity | -Infinity | -Infinity |
| SSB EsNoc |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.8.1.4.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| NocNote 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS EsNoc | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 96.48MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88MHz | -61.44 | -61.44 | -61.44 |
| PRS EsIot |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T2) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. |  |  |  |  |  |

##### A.6.8.1.4.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 5.6.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in clause 5.6.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

A test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

The rate of the correct events for each neighbour cell observed during the repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in the clause 10.1.23.3, i.e., between RSTD_000000000 and RSTD_126083073.

### A.6.8.2	PRS-RSRP measurements

#### A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE

##### A.6.8.2.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_INACTIVE meets the delay requirements specified in clause 5.6.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.8.2.1.1-1.

Table A.6.8.2.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

During T1 UE is in RRC_CONNECTED, the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.8.2.1.1-2, and cell specific test parameters are listed in table A.6.8.2.1.1-3.

Table A.6.8.2.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | 1.28 s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 6 |  |

Table A.6.8.2.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.2.1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE

##### A.6.8.2.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 5.6.3.5 in an environment with AWGN propagation conditions when single-sample measurements are requested by the LMF. This test is applicable to UEs that support supportedDL-PRS-ProcessingSamples-RRC-Inactive .

The supported test configurations are specified in table A.6. 8.2.2.1-1.

Table A.6. 8.2.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2. PRS resources from both cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-AoD assistance data and location information request.

The nr-DL-AoD-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to m1.

The general test parameters are listed in table A.6.8.2.2.1-2, and cell specific test parameters are listed in table A.6.8.2.2.1-3.

Table A.6.8.2.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle length | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 3 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.2.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration Note 5 |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 |  | 0 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -98 | -98 | -Infinity | -98 |
|  |  | 2 | -98 | -98 | -Infinity | -98 |
|  |  | 3 | -95 | -95 | -Infinity | -95 |
| Io Note 3 | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels apply only for symbols where PRS is transmitted. NOTE 4: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. NOTE 5: Void. |  |  |  |  |  |  |

##### A.6.8.2.2.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.2.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 with eDRX cycle > 10.24s

##### A.6.8.2.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_INACTIVE with eDRX meets the delay requirements specified in clause 5.6.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.8.2.3.1-1.

Table A.6.8.2.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

During T1 UE is in RRC_CONNECTED, the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [34] shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.8.2.3.1-2, and cell specific test parameters are listed in table A.6.8.2.3.1-3.

Table A.6.8.2.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 |  |
| eDRX cycle length (for both RAN and CN) | s | 1, 2, 3 | 40.96 |  |
| PTW window length | s | 1, 2, 3 | 1.28 |  |
| Reporting periodicity | s | 1, 2, 3 | 20 | reportingInterval for periodic reporting defined in TS 37.355 [34]. |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.8.2.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.2.3.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.3.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

A test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.8.3	UE Rx-Tx time difference measurements

#### A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.8.3.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement in RRC_INACTIVE state meets the requirements specified in clause 5.6.4 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.8.3.1.1-1.

Table A.6.8.3.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.8.3.1.1-2 and table A.6.8.3.1.1-3 respectively.

Table A.6.8.3.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle |  | 1, 2, 3 | 1.28 s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.8.3.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.8.3.1.2	Test requirements

The UE Rx-Tx time difference measurement time in RRC_INACTIVE state fulfils the requirements specified in clause 5.6.4.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA

##### A.6.8.3.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 5.6.4.5 in AWGN propagation condition in FR1 in standalone scenario when single-sample measurements are requested by the LMF. This test is applicable to UEs that support supportedDL-PRS-ProcessingSamples-RRC-Inactive.

The supported test configurations in listed in table A.6.8.3.2.1-1.

Table A.6.8.3.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2. PRS resources from both cells are transmitted within the initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The nr-Multi-RTT-RequestLocationInformation IE should indicate to the UE that single-sample measurements are requested, i.e. requestedDL-PRS-ProcessingSamples-r17 is set to m1.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in table A.6.8.3.2.1-2 and table A.6.8.3.2.1-3 respectively.

Table A.6.8.3.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle length | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 3 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.3.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration Note 5 |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 |  | 0 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -98 | -98 | -98 | -98 |
|  |  | 2 | -98 | -98 | -98 | -98 |
|  |  | 3 | -95 | -95 | -95 | -95 |
| Io Note 3 | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels apply only for symbols where PRS is transmitted. NOTE 4: OCNG shall be used such that all active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols, except for slots where PRS is transmitted by any of the cells. NOTE 5: Void. |  |  |  |  |  |  |

##### A.6.8.3.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 5.6.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.8.3.3	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA

##### A.6.8.3.3.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 5.6.4.5 for UE Rx-Tx measurements in RRC_INACTIVE with eDRX. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform UE Rx-Tx measurements on a single positioning frequency layer (PFL) in FR1.

The supported test configuration in listed in table A.6.8.3.3.1-1.

Table A.6.8.3.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB and PRS SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB and PRS SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB and PRS SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. Cell 1 and Cell 2 transmit PRS only during the second time interval of duration T2. Similarly, the UE is configured to transmit positioning SRS during only during the second time interval of duration T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12.1, shall be provided to the UE during T1. The last TTI of the last message shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The general test parameters and cell specific test parameters are as given in table A.6.8.3.3.1-2 and table A.6.8.3.3.1-3, respectively.

Table A.6.8.3.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle |  | 1, 2, 3 | 1.28s |  |
| eDRX cycle length (for both RAN and CN) | s | 1, 2, 3 | 40.96 |  |
| PTW window length | s | 1, 2, 3 | 1.28 |  |
| Reporting periodicity | s | 1, 2, 3 | 20 | reportingInterval for periodic reporting defined in TS 37.355 [34] |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.8.3.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.8.3.3.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 5.6.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE 1:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE 2:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

A test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.8.3.4	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA

##### A.6.8.3.4.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 5.6.4.6 for UE Rx-Tx measurements with PRS bandwidth aggregation. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform UE Rx-Tx measurements by aggregating two intra-band contiguous positioning frequency layers (PFLs) in FR1.

The supported test configurations in listed in table A.6.8.3.4.1-1.

Table A.6.8.3.4.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: Cell 1 (PCell) and Cell 2 (neighbor cell). Each cell is sssociated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData TS 37.355 [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, both cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and in RRC_INACTIVE state during T2. Cell 1 and Cell 2 transmit PRS only during the second time interval of duration T2. Similarly, the UE is configured to transmit positioning SRS during only during the second time interval of duration T2.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12, shall be provided to the UE during T1. The last TTI of the last message shall be provided to the UE at least T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle in RRC_INACTIVE.

The NR-Multi-RTT-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The NR-Multi-RTT-RequestLocationInformation message provided to the UE must request bandwidth aggregated measurements via jointMeasurementsReq and nr-DL-PRS-JointMeasurementRequestedPFL-List.

The general test parameters and cell specific test parameters are as given in table A.6.8.3.4.1-2 and table .6.8.3.4.1-3 respectively.

Table A.6.8.3.4.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2, all signals/channels except PRS in PFL2 are allocated within this channel. |
|  |  |  | 2 | For both Cell 1 and Cell 2, only PRS in PFL2 is allocated within this channel. |
| BWchannel | MHz | 1 | 20: NRB,c = 106 | For both RF channels |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells/TRPs |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.8.3.4.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns Note 1 |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1,4 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1, 4 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration (for both PFL1 and PFL2) |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 |  | 0 |  |
| PRS muting info (dl-PRS-MutingOption1 for both PFL1 and PFL2) |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/19.08 MHz | 1 | N/A | -65.05 | N/A | -65.05 |
|  | dBm/19.08 MHz | 2 |  | -65.05 |  | -65.05 |
|  | dBm/47.88 MHz | 3 |  | -61.06 |  | -61.06 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.8.3.4.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 5.6.4.6.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25A.2.

### A.6.8.4	PRS-RSRPP measurements

#### A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.8.4.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 5.6.5.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.8.4.1.1-1.

Table A.6.8.4.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2. During T2 UE shall be in RRC_INACTIVE state and all both cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.8.4.1.1-2, and cell specific test parameters are listed in table A.6.8.4.1.1-3.

Table A.6.8.4.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 | ON |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.4.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.4.1.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.5.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples

##### A.6.8.4.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 5.6.5.5 in an environment with AWGN propagation conditions for reduced number of samples. In this test UE that supports supportedDL-PRS-ProcessingSamples-RRC-Inactive is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in table A.6.8.4.2.1-1.

Table A.6.8.4.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2. During T2 UE shall be in RRC_INACTIVE state and both cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.8.4.2.1-2, and cell specific test parameters are listed in table A.6.8.4.2.1-3.

Table A.6.8.4.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NPRB,c = 106 |  |
|  |  | 2 | 20: NPRB,c = 106 |  |
|  |  | 3 | 50: NPRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 | ON |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.4.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS BW |  | 1,2,3 | 48 PRBs |  | 48PRBs |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 0 | -Infinity | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | 2.23 | -Infinity | -1.73 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 2 | -Infinity | -95.77 | -Infinity | -99.73 |
|  |  | 3 | -Infinity | -92.77 | -Infinity | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -100 | -Infinity | -104 |
|  |  | 2 | -88 | -100 | -Infinity | -104 |
|  |  | 3 | -85 | -97 | -Infinity | -101 |
| Io | dBm/19.08 MHz | 1 | N/A | -61.71 | N/A | -61.71 |
|  | dBm/19.08 MHz | 2 |  | -61.71 |  | -61.71 |
|  | dBm/47.88 MHz | 3 |  | -57.73 |  | -57.73 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.4.2.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.5.5, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.8.4.3	PRS-RSRPP reporting delay in RRC_INACTIVE with eDRX

##### A.6.8.4.3.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRPP measurement meets the delay requirements specified in clause 5.6.5.5 in an environment with a 2-tap channel propagation conditions in RRC_INACTIVE when configured with eDRX. The supported test configurations are specified in table A.6.8.4.3.1-1.

Table A.6.8.4.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2. During T2 UE shall be in RRC_INACTIVE state and all both cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [34], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first DRX cycle containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.8.4.3.1-2, and cell specific test parameters are listed in table A.6.8.4.3.1-3.

Table A.6.8.4.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX | s | 1, 2, 3 | 1.28 | ON |
| CN and RAN eDRX configuration |  | 1, 2, 3 | eDRX cycle = 40.96s PTW length = 1.28s |  |
| reportingInterval | s | 1, 2, 3 | 20 | PRS measurement reporting periodicity |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.8.4.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 [21] annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| Note 1: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.8.4.3.2	Test Requirements

The UE shall perform and report the PRS-RSRPP measurements for Cell 1 and Cell 2, within the time limit specified in clause 5.6.5.5 with Tavailable_PRS = 1.28s, starting from the beginning of time interval T2.

NOTE 1:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE 2:	The test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

A test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.8.5	RSCPD Measurements

#### A.6.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state

##### A.6.8.5.1.1	Test Purpose and Environment

The purpose of the test is to verify that the DL RSCPD reported with RSTD measurement meets the requirements specified in clause 5.6.7.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The test environment is the same as in clause A.6.8.1.1 with the following additional configuration in table A.6.8.5.1.1-1 and description.

In NR-DL-TDOA-RequestLocationInformation, the UE is configured to perform DL RSCPD measurement via dl-PRS-RSCPD-Request. The UE also is configured to perform both RSCPD and RSTD measurements within the time window indicated to UE via nr-DL-PRS-MeasurementTimeWindowsConfig.

The beginning of the time interval T2 shall be aligned with the first DRX cycle containing a DL PRS resource(s) to be measured within the configured time window.

Table A.6.8.5.1.1-1: Time window configuration

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Indicated time window configuration | Config 1,2,3 |  | MTW.1 | As specified in clause A.3.38 |

##### A.6.8.5.1.2	Test Requirements

The DL RSCPD reported with RSTD measurement time fulfils the requirements specified in clause 5.6.7.5.

The UE shall perform and report the DL RSCPD and DL RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in clause 5.6.7.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3 and the reported RSCPD measurement for each correct event shall be within the RSCPD reporting range specified in clause 10.1.43.3.

### A.6.8.6	RSCP Measurements

#### A.6.8.6.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.8.6.1.1	Test purpose and environment

The purpose of the test is to verify that the DL RSCP and UE Rx-Tx time difference measurements in RRC_INACTIVE state meet the requirements specified in clause 5.6.8.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured for both DL RSCP measurement and UE Rx-Tx time difference measurement.

The supported test configurations are listed in table A.6.8.6.1.1-1.

Table A.6.8.6.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE during T1. In NR-Multi-RTT-RequestLocationInformation, the UE is configured to perform DL RSCP measurement via nr-DL-PRS-RSCP-Request. The UE is configured to perform both DL RSCP and UE Rx-Tx time difference measurements within the time window indicated to UE via nr-DL-PRS-MeasurementTimeWindowsConfig but the time window periodicity is not configured. The last slot containing the two messages for the multi-RTTI assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_INACTIVE.

The beginning of the time interval T2 shall be aligned with the start of the configured time window containing the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The UE is configured to transmit positioning SRS during T2.

The general test parameters and cell specific test parameters are listed in table A.6.8.6.1.1-2 and table A.6.8.6.1.1-3.

Table A.6.8.6.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle |  | 1, 2, 3 | 1.28s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Time window configuration |  | 1, 2, 3 | MTW.2 | As specified in clause A.3.38 |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.8.6.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

##### A.6.8.6.1.2	Test requirements

The DL RSCP with UE Rx-Tx time difference measurement time in RRC_INACTIVE state fulfils the requirements specified in clause 5.6.8.

The UE shall perform and report the DL RSCP and UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified DL RSCP with UE Rx-Tx time difference measurement time specified in clause 5.6.8 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported DL RSCP measurement for each correct event shall be within the DL RSCP reporting range specified in clause 10.1.44 and the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.

## A.6.9	Measurement performance requirements in RRC_INACTIVE

### A.6.9.1	RSTD measurements

#### A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.9.1.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement in RRC_INACTIVE state meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.9.1.1.1-1.

Table A.6.9.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with DRX cycle of 1.28 s. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.5.

Table A.6.9.1.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.3 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -6 | -13 |  | -6 | -13 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |  | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -65.70 | -65.70 |  | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88 MHz | -61.72 | -61.72 |  | -61.72 | -61.72 |
|  | 1~3 | dB | -5.7 | -11.9 |  | -5.7 | -11.9 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. |  |  |  |  |  |  |  |

##### A.6.9.1.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state

##### A.6.9.1.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement in RRC_INACTIVE state meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions. In this test UE that supports supportedDL-PRS-ProcessingSamples-RRC-Inactive is configured by LMF to perform PRS measurement with reduced number of samples.

The supported test configurations are specified in table A.6.9.1.2.1-1.

Table A.6.9.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with DRX cycle of 1.28 s. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.5.

Table A.6.9.1.2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NPRB,c = 106 |  |  | 20: NPRB,c = 106 |  |
|  | 2 |  | 20: NPRB,c = 106 |  |  | 20: NPRB,c = 106 |  |
|  | 3 |  | 50: NPRB,c = 133 |  |  | 50: NPRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | - |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.3 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS BW | 1,2,3 |  | 52 PRBs for test 1 |  |  |  |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1~3 | dB | -3 | -6 |  | -3 | -6 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -99.44 | -101.65 |  | -99.44 | -101.65 |
|  | 3 |  | -96.44 | -98.65 |  | -96.44 | -98.65 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -99.44 | -101.65 |  | -99.44 | -101.65 |
|  | 3 |  | -96.44 | -98.65 |  | -96.44 | -98.65 |
| IoNote3 | 1,2 | dBm/ 19.08 MHz | -63.63 | -63.63 |  | -63.63 | -63.63 |
|  | 3 | dBm/ 47.88 MHz | -59.65 | -59.65 |  | -59.65 | -59.65 |
|  | 1~3 | dB | -1.44 | -3.65 |  | -1.44 | -3.65 |
| Propagation condition | 1~3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. |  |  |  |  |  |  |  |

##### A.6.9.1.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.9.1.3	RSTD measurement accuracy for PRS aggregation in FR1 in RRC_INACTIVE state

##### A.6.9.1.3.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement with PRS aggregation on two PFLs meets the accuracy requirements specified in clause 10.1.23A.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.9.1.3.1-1.

Table A.6.9.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on the same NR RF channel as Cell 1. Cell 3 and Cell 4 are neighbor cells in a different NR RF channel #2 in FR1. Cell 1 and Cell 3 are intra-band contiguous, and PRS resources from Cell 1 and Cell 3 are transmitted by the same Tx chain. Cell 2 and Cell 4 are intra-band contiguous, and PRS resources from Cell 2 and Cell 4 are transmitted by the same Tx chain.

The UE is configured with DRX cycle of 1.28 s.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.6.

In NR-TDOA-ProvideAssistanceData, there are two NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList. The first list indicates aggregation of resource sets from Cell 1 and Cell 3, and the second list indicates aggregation of resource sets from Cell 2 and Cell 4. In NR-TDOA-RequestLocationInformation, nr-DL-PRS-JointMeasurementRequestedPFL-List is included, and indicates aggregation of PFLs on NR RF channel #1 and NR RF channel #2.

Table A.6.9.1.3.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- | --- |
| PRS ARFCN | 1,2,3 |  | freq1 | freq1 | freq2 | freq2 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |  |  |
|  | 2 |  | 20: NRB,c = 106 |  |  |  |
|  | 3 |  | 50: NRB,c = 133 |  |  |  |
| Duplex mode | 1 |  | FDD |  |  |  |
|  | 2 |  | TDD |  |  |  |
|  | 3 |  | TDD |  |  |  |
| TDD configuration | 1 |  | N/A |  |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | - | - |
|  | 2 |  | SR.1.1 TDD | - | - |  |
|  | 3 |  | SR.2.1 FDD | - | - |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | - | - |
|  | 2 |  | CR.1.1 TDD | - | - | - |
|  | 3 |  | CR.2.1 FDD | - | - | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | - | - |
|  | 2 |  | CCR.1.1 TDD | - | - | - |
|  | 3 |  | CCR.2.1 TDD | - | - | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | - | - |
|  | 2 |  | TRS.1.1 TDD | - | - | - |
|  | 3 |  | TRS.1.2 TDD | - | - | - |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| Time offset with Cell 1 | 1 | s | - | 3 | 0 | 3 |
|  | 2,3 |  | - | 3 | 0 | 3 |
| PRS configuration | 1 |  | PRS.1.2 FR1 |  |  |  |
|  | 2 |  | PRS.1.2 FR1 |  |  |  |
|  | 3 |  | PRS.2.2 FR1 |  |  |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 | 0 | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 | 0 | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3 | dB | 0 | 0 | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  | -98 |  |
|  | 3 |  | -95 |  | -95 |  |
| PRS | 1,2,3 | dB | -6 | -13 | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -104 | -111 | -104 | -111 |
|  | 3 |  | -101 | -108 | -101 | -108 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.98 | -66.74 | -65.98 | -66.74 |
|  | 3 | dBm/ 47.88MHz | -62.00 | -62.76 | -62.00 | -62.76 |
| PRS | 1,2,3 | dB | -6 | -13 | -6 | -13 |
| Propagation condition | 1,2,3 | - | AWGN |  | AWGN |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

##### A.6.9.1.3.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23A.2.

### A.6.9.2	PRS-RSRP measurements

#### A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE

##### A.6.9.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy in RRC_INACTIVE is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.9.2.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.2.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.9.2.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.2.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.2.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| DRX Cycle |  |  | ms | 1280 |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  | -95 |  |  |
|  |  |  | dB | -2.41 | -12.12 | -2.41 |  | -12.12 |
|  |  |  | dB | -2 | -10 | -2 |  | -10 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -100 | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -64.57 |  | -64.57 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -60.59 |  | -60.59 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. NOTE 6: Void |  |  |  |  |  |  |  |  |

##### A.6.9.2.1.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state

##### A.6.9.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits provided that PRS is transmitted within the initial BWP of the UE. UE can support supportedDL-PRS-ProcessingSamples-RRC-Inactive, and the LMF indicates the UE to perform positioning measurements with reduced number of samples NsampleNsample via reducedDL-PRS-ProcessingSamples. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.9.2.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.2.2.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.9.2.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.2.2.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.2.2.2-2: PRS-RSRP test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 20: NPRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |
| BWP BW | Config 1 |  | 20: NPRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle |  | s | 1.28 |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| PRS configuration | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |
| PRS BW | Config 1 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 2 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 3 |  | 48 PRBs | 48 PRBs |  |  |
| PRS Resource slot offset (slot) | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |
| SSB configuration | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 | Config 1 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SMTC configuration | Config 1 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/15Khz | -98 |  | -98 |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | 0 | -6 | 0 | -6 |
|  |  | dB | 2.23 | -1.73 | 2.23 | -1.73 |
| PRS-RSRP Note3 | Config 1, 2 | dBm/SCS | -95.77 | -99.73 | -95.77 | -99.73 |
|  | Config 3 |  | -92.77 | -96.73 | -92.77 | -96.73 |
| IoNote3 | Config 1,2 | dBm/19.08 MHz | -61.71 |  | -61.71 |  |
|  | Config 3 | dBm/47.88 MHz | -57.73 |  | -57.73 |  |
| Propagation condition |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

##### A.6.9.2.2.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

### A.6.9.3	UE Rx-Tx time difference measurements

#### A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA

##### A.6.9.3.1.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario.

The supported test configurations in listed in table A.6.9.3.1.1.1-1.

Table A.6.9.3.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured to transmit SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.3.1.1.2 Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in table A.6.9.3.1.1.1-1.

Table A.6.9.3.1.1.1-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 | 1 | 1 |
| DRX | s | 1,2,3 | 1.28 |  | 1.28 |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | -12.12 | -2.41 | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | -10 | -2 | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -108 | -100 | -108 |
|  |  | 2 | -100 | -108 | -100 | -108 |
|  |  | 3 | -97 | -105 | -97 | -105 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/19.08 MHz | 2 | -64.57 | -64.57 | -64.57 | -64.57 |
|  | dBm/47.88 MHz | 3 | -60.59 | -60.59 | -60.59 | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  | AWGN |  |
| NOTE 1: Void. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.9.3.1.1.3	Test requirements

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

#### A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples

##### A.6.9.3.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in table A.6.9.3.2.1-1.

Table A.6.9.3.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test. The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation.

UE shall be configured to enter into RRC_INACTIVE state before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.3.2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in table A.6.9.3.2.2-1.

Table A.6.9.3.2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| DRX cycle |  | 1,2,3 | 1.28 s |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 52 PRBs | 52 PRBs |
|  |  | 2 | 52 PRBs | 52 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -6 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 2.23 | -1.73 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -99.73 |
|  |  | 2 | -95.77 | -99.73 |
|  |  | 3 | -92.77 | -96.73 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -61.71 |
|  | dBm/19.08 MHz | 2 | -61.71 | -61.71 |
|  | dBm/47.88 MHz | 3 | -57.73 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.9.3.2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE state fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

#### A.6.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA

##### A.6.9.3.3.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25A. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform UE Rx-Tx measurements by aggregating two intra-band contiguous positioning frequency layers (PFLs) in FR1.

The supported test configurations are listed in table A.6.9.3.3.1-1.

Table A.6.9.3.3.1-1: Supported test configurations

| PCell configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: Cell 1 (PCell) and Cell 2 (neighbor cell). Each cell is sssociated with a different TRP/DL PRS ID in the NR-DL-PRS-AssistanceData TS 37.355 [34]. Cell 1 transmissions other than DL PRS are allocated in RF channel #1. In addition, both cells/TRPs transmit DL PRS in two intra-band contiguous PFLs in RF channel #1 and RF channel #2. PFL1 is allocated within RF channel #1 and PFL2 is allocated within RF channel #2. Except for the frequency offset between them, both PFLs have identical PRS configuration.

The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12, shall be provided to the UE before the start of the test.

The NR-Multi-RTT-ProvideAssistanceData message provided to the UE must include NR-DL-PRS-AggregationInfo-r18 linking each PRS resource in PFL1 to the corresponding PRS resource in PFL2.

The NR-Multi-RTT-RequestLocationInformation message provided to the UE must request bandwidth aggregated measurements via jointMeasurementsReq and nr-DL-PRS-JointMeasurementRequestedPFL-List.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

The general test parameters and cell specific test parameters are as given in table A.6.9.3.3.1-2 and table A.6.9.3.3.1-3, respectively.

Table A.6.9.3.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2, all signals/channels except PRS in PFL2 are allocated within this channel. |
|  |  |  | 2 | For both Cell 1 and Cell 2, only PRS in PFL2 is allocated within this channel. |
| BWchannel | MHz | 1 | 20: NRB,c = 106 | For both RF channels |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX cycle | s | 1, 2, 3 | 1.28 |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells/TRPs |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |

Table A.6.9.3.3.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns Note 1 |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active BWP configuration |  | 1, 2, 3 | DLBWP.1.1 ULBWP.1.1 | N/A |
| PRS configuration (for both PFL1 and PFL2) |  | 1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 2 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  | 3 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slots | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -3 | -13 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -3 | -13 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -101 | -111 |
|  |  | 2 | -101 | -111 |
|  |  | 3 | -98 | -108 |
| Io Note 3 (on symbols where PRS is not allocated) | dBm/19.08 MHz | 1 | -65.05 | -65.05 |
|  | dBm/19.08 MHz | 2 | -65.05 | -65.05 |
|  | dBm/47.88 MHz | 3 | -61.06 | -61.06 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved on all OFDM symbols except those in which PRS is allocated. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters and they are provided for information only. They are not settable parameters themselves. |  |  |  |  |

##### A.6.9.3.3.2	Test requirements

The UE Rx-Tx time difference measurement fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25A for both Cell 1 and Cell 2.

### A.6.9.4	PRS-RSRPP measurements

#### A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE

##### A.6.9.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy in FR1 in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clauses 10.1.38.2.

##### A.6.9.4.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.4.1.2-1. In all test cases, Cell 1 is the PCell.

Table A.6.9.4.1.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.4.1.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 |  | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |  |  |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |  |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA | TRS.1.2 TDD | NA |  |
| DRX cycle length |  | Config 1,2,3 | ms | 1280 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 |  | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 |  | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - |  | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |
|  |  |  | dB | -2.41 | -12.12 |  | -2.41 |  | -12.12 |
|  |  |  | dB | -2 | -10 |  | -2 |  | -10 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -100 |  | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 |  | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -64.57 |  |  | -64.57 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -60.59 |  |  | -60.59 |  |  |
| Propagation condition |  |  |  | Two-tap channel Note 7 |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Void. NOTE 6: Void NOTE 7: The two-tap channel model is defined in 38.101-4 annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |  |  |  |

##### A.6.9.4.1.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.38.2.

#### A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE

##### A.6.9.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy with Nsample = 1 in FR1 in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clauses 10.1.38.2.The UE under test should support supportedDL-PRS-ProcessingSamples-RRC-Inactive, and the TE indicates the UE to perform positioning measurements with reduced number of samples. The PRS bandwidth is contained within the initial DL BWP and the power difference between the serving cell SS-RSRP and neighbour cell PRS-RSRP is within [6]dB, so that Nsample= 1 is assumed.

##### A.6.9.4.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.9.4.2.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.9.4.2.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.9.4.2.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.9.4.2.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1 | MHz | 20: NPRB,c = 106 |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |
| BWP BW |  | Config 1 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 2 |  | 20: NPRB,c = 106 |  |  |
|  |  | Config 3 |  | 50: NPRB,c = 133 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA |
| DRX Cycle length |  | Config 1,2,3 | ms | 1280 |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 |  | PRS.2.4 FR1 |
| PRS bandwidth |  | Config 1,2,3 |  | 52 PRBs |  | 52 PRBs |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 |
|  |  | Config 2,3 | s | - |  | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15 kHz | -98 |  |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |
|  | Config 3 |  |  | -95 |  |  |
|  |  |  | dB | 0 |  | -6 |
|  |  |  | dB | 2.23 |  | -1.73 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -95.77 |  | -99.73 |
|  | Config 3 |  |  | -92.77 |  | -96.73 |
| SS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -95.77 |  | -99.73 |
|  | Config 3 |  |  | -92.77 |  | -96.73 |
| IoNote3 | Config 1,2 |  | dBm/19.08 MHz | -61.71 |  |  |
|  | Config 3 |  | dBm/47.88 MHz | -57.73 |  |  |
| Propagation condition |  |  |  | Two-tap channel Note5 |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRS-RSRP, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The two-tap channel model is defined in 38.101-4 annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). |  |  |  |  |  |  |

##### A.6.9.4.2.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.38.2.

### A.6.9.5	RSCPD Measurements

#### A.6.9.5.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_INACTIVE

##### A.6.9.5.1.1	Test purpose and environment

The purpose of the test is to verify that the accuracy of RSCPD measurement reported with RSTD measurement accuracy in RRC_INACTIVE. This test will verify the requirements in clause 10.1.43.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are listed in table A.6.9. 5.1.1-1.

Table A.6.9. 5.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation with dl-PRS-RSCPD-Request from LMF via LPP as defined in TS 37.355 [34], clause 6.5.12, to enable UE to perform and report RSCPD in RRC_INACTIVE shall be provided to the UE before the start of the test. The UE is configured with DRX cycle of 1.28s.

##### A.6.9.5.1.2	Test parameters

The RSCPD with RSTD accuracy test parameters are given in table A.6.9.5.1.2-1.

Table A.6.9. 5.1.2-1: RSCPD with UE RSTD measurement accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1,2,3 |  | Freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |
|  | 2,3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3 | dB | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |
|  | 3 |  | -95 |  |
| PRS | 1,2,3 | dB | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |
| PRS | 1,2,3 | dB | -5.7 | -11.9 |
| SSB | 1,2,3 | dB | -5.7 |  |
| Propagation condition | 1,2,3 | - | AWGN |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |

##### A.6.9.5.1.3	Test requirements

The RSCPD reported together with RSTD fulfils RSCPD measurement accuracy requirements specified in clause 10.1.43.2 for Cell 2.

### A.6.9.6	RSCP Measurements

#### A.6.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA

##### A.6.9.6.1.1	Test purpose and environment

The purpose of the test is to verify that the accuracy of RSCP measurement reported with UE Rx-Tx time difference measurement in RRC_INACTIVE is within the specified limits. This test will verify the requirements in clause 10.1.44.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are listed in table A.6.9.6.1.1-1.

Table A.6.9.6.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData , NR-Multi-RTT-RequestLocationInformation with nr-DL-PRS-RSCP-Request from LMF via LPP and NR-Multi-RTT-MeasurementCapability as defined in TS 37.355 [34], clause 6.5.12, to enable UE to perform and report RSCP in RRC INACTIVE, shall be provided to the UE before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.6.1.2	Test parameters

The RSCP with UE Rx-Tx time difference accuracy test parameters are given in table A.6.9.6.1.2-1.

Table A.6.9.6.1.2-1: RSCP with UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| DRX | s | 1,2,3 | 1.28 |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 52 PRBs | 52 PRBs |
|  |  | 2 | 52 PRBs | 52 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -6 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 2.23 | -1.73 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -99.73 |
|  |  | 2 | -95.77 | -99.73 |
|  |  | 3 | -92.77 | -96.73 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -61.71 |
|  | dBm/19.08 MHz | 2 | -61.71 | -61.71 |
|  | dBm/47.88 MHz | 3 | -57.73 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void. |  |  |  |  |

##### A.6.9.6.1.3	Test requirements

The RSCP with UE Rx-Tx time difference measurement fulfils RSCP measurement accuracy specified in clause 10.1.44.2 for both Cell 1 and Cell 2.

## A.6.10	Measurement Procedure in RRC_IDLE

### A.6.10.1	RSTD Measurements

#### A.6.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE

##### A.6.10.1.1.1	Test purpose and environment

The purpose of the test is to verify the measurement requirements specified in clause 4.5.2.5 for RSTD measurements in RRC_IDLE without eDRX. The tests are conducted under AWGN propagation condition with the UE operating in FR1 stand-alone mode and configured to perform RSTD measurements on a single positioning frequency layer (PFL) in FR1.

The supported test configurations are listed in table A.6.10.1.1.1-1.

Table A.6.10.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are three cells in the test: Cell 1 (PCell and RSTD reference cell), Cell 2 (neighbor cell) and Cell 3 (neighbor cell). All cells are on the same RF channel in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. The UE shall be in RRC_CONNECTED state during T1 and released to RRC_IDLE state before the start of T2. All cells transmit PRS only during the second time interval of duration T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and NR-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.12, shall be provided to the UE during T1. The last TTI of the last message shall be provided to the UE at least T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the start of the first PRS resource instance received after the UE has transitioned to RRC_IDLE.

The general test parameters are listed in table A.6. 10.1.1.1-2 and cell specific test parameters are listed in table A.6.10.1.1.1-3 and table A.6.10.1.1.1-4.

Table A.6.10.1.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  | s | 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.10.1.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.10.1.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 96.48MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88MHz | -61.44 | -61.44 | -61.44 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T2) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.10.1.1.2	Test requirements

The RSTD measurement time shall fulfill the requirements specified in clause 4.5.2.5.

The UE shall perform and report the RSTD measurements for Cell 1, Cell 2 and Cell 3 within the specified measurement period duration starting from the beginning of time interval T2.

NOTE 1:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

NOTE 2:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3.

#### A.6.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s for non-RedCap UE

##### A.6.10.1.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement in RRC_IDLE state meets the requirements specified in clause 4.5.2.5 when eDRX cycle is longer than 10.24s in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in table A.6.10.1.2.1-1.

Table A.6.10.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall be in RRC_CONNECTED state and shall not have any timing information of Cell 2 and Cell 3. During T2 UE shall be in RRC_IDLE state and all three cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP.

NOTE: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34], clause 6.5.10, shall be provided to the UE during T1. The UE is configured to report positioning measurements every 20s by setting the value of reportingInterval to "ri20" in nr-DL-TDOA-RequestLocationInformation. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 is not limited to PTW.

The general test parameters are listed in table A.6.10.1.2.1-2, and cell specific test parameters are listed in table A.6.10.1.2.1-3 and table A.6.10.1.2.1-4.

Table A.6.10.1.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355 [34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2) mod 6 = 0 and (PCI of Cell 1 – PCI of Cell 3) mod 6 = 0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| CN and RAN eDRX configuration |  | s | eDRX length = 40.96 PTW length = 1.28 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance ta specified in TS 37.355 [34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to NR-MutingPattern defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 5 | The length of the time interval that follows immediately after time interval T1. |

Table A.6.10.1.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Noc Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS EsNoc |  | dB | -Infinity | -Infinity | -Infinity |
| SSB EsNoc |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 2 | dBm/ 19.08MHz | -56.54 | -56.54 | -56.54 |
|  | Config 3 | dBm/ 47.88MHz | -52.56 | -52.56 | -52.56 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.10.1.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2.

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1×2 Low | 1×2 Low | 1×2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| NocNote 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS EsNoc | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 19.08MHz | -65.43 | -65.43 | -65.43 |
|  | Config 2 | dBm/ 96.48MHz | -65.43 | -65.43 | -65.43 |
|  | Config 3 | dBm/ 47.88MHz | -61.44 | -61.44 | -61.44 |
| PRS EsIot |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T2) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned after the end of time period T2 to UEs that do not support SDT for measurement reporting. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. |  |  |  |  |  |

##### A.6.10.1.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in clause 4.5.2.5. The test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in clause 4.5.2.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during the repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in the clause 10.1.23.3, i.e., between RSTD_000000000 and RSTD_126083073.

#### A.6.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_IDLE state

##### A.6.10.1.3.1	Test purpose and environment

The purpose of the test is to verify that the RSTD measurement with PRS aggregation in RRC_IDLE state meets the requirements specified in clause 4.5.2.6 in AWGN propagation condition in FR1 in standalone scenario when two intra-band contiguous positioning frequency layers (PFLs) are configured.

The test environment and configurations reuse the test case for RRC_INACTIVE state defined in clause A.6.8.1.3 except that UE shall be in RRC_IDLE state and all 6 cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP during T2.

##### A.6.10.1.3.2	Test requirements

The RSTD measurement time with PRS aggregation in RRC_IDLE state fulfils the requirements specified in clause 4.5.2.6.

The UE shall perform and report the RSTD measurements by aggregating PRS resources from Cell 2 and Cell 5, Cell 3 and Cell 6 respectively with respect to the Cell 1 and Cell 4 from which the transmitted PRS resources are also aggregated, within the time duration specified in clause 4.5.2.6 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events observed during repeated tests shall be at least 90%, where the reported RSTD measurement with PRS aggregation for each correct event shall be within the RSTD reporting range specified in clause 10.1.23A.3.

### A.6.10.2		PRS-RSRP Measurements

#### A.6.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR1

##### A.6.10.2.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_IDLE state meets the delay requirements specified in clause 4.5.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.10.2.1.1-1.

Table A.6.10.2.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

During T1 UE is in RRC_CONNECTED, the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [34], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n. In the next DL slot after slot #n, UE is released into RRC_IDLE.

The beginning of the time interval T2 is the first PRS resource occasion occurring T after the slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in table A.6.10.2.1.1-2, and cell specific test parameters are listed in table A.6.10.2.1.1-3.

Table A.6.10.2.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | 1.28s |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 6 |  |

Table A.6.10.2.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -Infinity | -88 |
|  |  | 2 | -88 | -88 | -Infinity | -88 |
|  |  | 3 | -85 | -85 | -Infinity | -85 |
| Io | dBm/19.08 MHz | 1 | N/A | -64.57 | N/A | -64.57 |
|  | dBm/19.08 MHz | 2 |  | -64.57 |  | -64.57 |
|  | dBm/47.88 MHz | 3 |  | -60.59 |  | -60.59 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.10.2.1.2	Test Requirements

The PRS RSRP measurement time fulfils the requirements specified in clause 4.5.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 4.5.3.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

#### A.6.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s

##### A.6.10.2.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_IDLE with eDRX meets the delay requirements specified in clause 4.5.3.5 in an environment with AWGN propagation conditions.

The supported test configurations in table A.6.8.2.3.1-1 apply for this test.

The test procedure in clause A.6.8.2.3.1 apply for this test, except that during T2, UE is in RRC_IDLE state.

The general test parameters as specified in table A.6.8.2.3.1-2 apply for this test, except those specified in table A.6.10.2.2.1-1.

The cell specific test parameters as specified in table A.6.8.2.3.1-3 apply for this test.

Table A.6.10.2.2.1-1: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| eDRX cycle length (for CN eDRX) | s | 1, 2, 3 | 40.96 |  |

##### A.6.10.2.2.2	Test Requirements

The test requirements in clause A.6.8.2.3.2 apply for this test, except that the time limits are specified in clause 4.5.3.5.

### A.6.10.3	RSCPD Measurements

#### A.6.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state

##### A.6.10.3.1.1	Test Purpose and Environment

The purpose of the test is to verify that the DL RSCPD reported with RSTD measurement meets the requirements specified in clause 5.6.7.5 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The test environment is the same as in A.6.8.1.1 with the following additional configuration in table A.6. 10.3.1.1-1 and description.

In NR-DL-TDOA-RequestLocationInformation, the UE is configured to perform DL RSCPD measurement via dl-PRS-RSCPD-Request. The UE also is configured to perform both RSCPD and RSTD measurements within the time window indicated to UE via nr-DL-PRS-MeasurementTimeWindowsConfig.

The beginning of the time interval T2 shall be aligned with the first DRX cycle containing a DL PRS resource(s) to be measured within the configured time window.

Table A.6.10.3.1.1-1: Time window configuration

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Indicated time window configuration | Config 1,2,3 |  | MTW.1 | As specified in clause A.3.38 |

##### A.6.10.3.1.2	Test Requirements

The DL RSCPD reported with RSTD measurement time fulfils the requirements specified in clause 5.6.7.5.

The UE shall perform and report the DL RSCPD and DL RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in clause 5.6.7.5 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be higher than the time duration above because of the uncertainty in acquiring the first available PRACH occasion to transition to RRC_CONNECTED state to report the measurements.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in clause 10.1.23.3 and the reported RSCPD measurement for each correct event shall be within the RSCPD reporting range specified in clause 10.1.43.3.

## A.6.11	Measurement Performance Requirements in RRC_IDLE

### A.6.11.1	RSTD Measurements

#### A.6.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE

##### A.6.11.1.1.1	Test purpose and environment

The purpose of the test is to verify that the RSTD measurement in RRC_IDLE state without eDRX meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are listed in table A.6.11.1.1.1-1.

Table A.6.11.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with DRX cycle of 1.28s. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.5.

The RSTD accuracy test parameters are listed in table A.6.11.1.1.1-2.

Table A.6.11.1.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1,2,3 |  | freq1 | Freq1 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |  | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |  | FDD |  |
|  | 2 |  | TDD |  |  | TDD |  |
|  | 3 |  | TDD |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  | - | 3 |
|  | 2,3 |  | - | 3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.3 FR1 |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.3 FR1 |  |  | PRS.2.4 FR1 |  |
| PRS muting info | 1,2,3 |  | ‘10’ | ‘01’ |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  | -98 |  |
|  | 3 |  | -95 |  |  | -95 |  |
|  | 1,2,3 | dB | -6 | -13 |  | -6 | -13 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |  | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |  | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |  | -61.72 | -61.72 |
|  | 1,2,3 | dB | -5.7 | -11.9 |  | -5.7 | -11.9 |
| Propagation condition | 1,2,3 | - | AWGN |  |  | AWGN |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.11.1.1.2	Test requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_IDLE state with eDRX>10.24s for non-RedCap UE

##### A.6.11.1.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement performed by UE in RRC_IDLE state with eDRX > 10.24s meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in table A.6.11.1.2.1-1.

Table A.6.11.1.2.1-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The UE is configured with eDRX cycle of 40.96s. The UE is configured to report positioning measurements every 20s by setting the value of reportingInterval to "ri20" in nr-DL-TDOA-RequestLocationInformation. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 [34], shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 4.5.2.5.

Table A.6.11.1.2.1-2: RSTD accuracy test parameters.

| Parameter | Config | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  |
| PRS ARFCN | 1,2,3 |  | freq1 | Freq1 |  |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |  |
|  | 2 |  | 20: NRB,c = 106 |  |  |
|  | 3 |  | 50: NRB,c = 133 |  |  |
| Duplex mode | 1 |  | FDD |  |  |
|  | 2 |  | TDD |  |  |
|  | 3 |  | TDD |  |  |
| TDD configuration | 1 |  | N/A |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |  |
|  | 2 |  | SR.1.1 TDD |  |  |
|  | 3 |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |  |
|  | 2 |  | CR.1.1 TDD | - |  |
|  | 3 |  | CR.2.1 FDD | - |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |  |
|  | 2 |  | CCR.1.1 TDD | - |  |
|  | 3 |  | CCR.2.1 TDD | - |  |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - |
|  | 2 |  | TRS.1.1 TDD |  |  |
|  | 3 |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  |
|  | 2,3 |  | - | 3 |  |
| SMTC configuration | 1 |  | SMTC.2 |  |  |
|  | 2,3 |  | SMTC.1 |  |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  |  |
|  | 2 |  | PRS.1.3 FR1 |  |  |
|  | 3 |  | PRS.2.4 FR1 |  |  |
| PRS muting info | 1,2,3 |  | ‘10’ | ‘01’ |  |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  |
|  | 3 |  | -95 |  |  |
|  | 1,2,3 | dB | -6 | -13 |  |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |  |
|  | 3 |  | -100.7 | -106.9 |  |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |  |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |  |
|  | 1,2,3 | dB | -5.7 | -11.9 |  |
| Propagation condition | 1,2,3 | - | AWGN |  |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over sub-carriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

##### A.6.11.1.2.2	Test Requirements

The test is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval. The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_IDLE state

##### A.6.11.1.3.1	Test purpose and environment

The purpose of the test is to verify that the RSTD measurement results with PRS aggregation in RRC_IDLE state meets the requirements specified in clause 10.1.23A.2 in AWGN propagation condition in FR1 in standalone scenario when two intra-band contiguous positioning frequency layers (PFLs) are configured.

The test environment and configurations reuse the test case for RRC_INACTIVE state defined in clause A.6.9.1.3, except that UE shall be in RRC_IDLE state and all 4 cells transmit PRS resources within initial DL BWP of the UE and with the same numerology as the initial DL BWP during T2.

##### A.6.11.1.3.2	Test requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23A.2.

### A.6.11.2	PRS-RSRP measurements

#### A.6.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR1 in RRC_IDLE state

##### A.6.11.2.1.1	Test Purpose and Environment

The purpose of this test is to verify accuracy of PRS-RSRP measurement performed by UE in RRC_IDLE mode in FR1.

##### A.6.11.2.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.11.2.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.11.2.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.11.2.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.11.2.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 20: NRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NRB,c = 133 |  |  |  |  |
| BWP BW |  | Config 1 |  | 20: NRB,c = 106 |  |  |  |  |
|  |  | Config 2 |  | 20: NRB,c = 106 |  |  |  |  |
|  |  | Config 3 |  | 50: NRB,c = 133 |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |
| DRX Cycle |  |  | s | 1.28 |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |  |
|  |  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |  |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/15kHZ | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -98 |  | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |  |
|  | Config 3 |  |  | -95 |  | -95 |  |  |
| EsIot |  |  | dB | -2.41 | -12.12 | -2.41 |  | -12.12 |
| EsNoc |  |  | dB | -2 | -10 | -2 |  | -10 |
| PRS-RSRP Note3 | Config 1, 2 |  | dBm/SCS | -100 | -108 | -100 |  | -108 |
|  | Config 3 |  |  | -97 | -105 | -97 |  | -105 |
| IoNote3 | Config 1,2 |  | dBm/19.08MHz | -64.57 |  | -64.57 |  |  |
|  | Config 3 |  | dBm/47.88MHz | -60.59 |  | -60.59 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over sub-carriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.11.2.1.3	Test Requirements

In each test, the absolute PRS-RSRP measurement for each cell shall fulfil the absolute accuracy requirement in clause 10.1.24.2.1. The relative PRS-RSRP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause 10.1.24.2.2.

#### A.6.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s

##### A.6.11.2.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement in RRC_IDLE with eDRX meets the accuracy requirements specified in clauses 10.1.24.2.1 and 10.1.24.2.2 in an environment with AWGN propagation conditions.

##### A.6.11.2.2.2	Test parameters

The supported test configurations in table A.6.9.2.1.2-1 apply for this test.

The test procedure in clause A.6.9.2.1.2 apply for this test, except that UE is in RRC_IDLE state.

The test parameters as specified in table A.6.9.2.1.2-2 apply for this test, except those additionally specified in table A.6.11.2.2.2-1.

Table A.6.11.2.2.2-1: PRS-RSRP test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| eDRX cycle length (for CN eDRX) | s | 1, 2, 3 | 40.96 |  |
| PTW window length | s | 1, 2, 3 | 1.28 |  |

##### A.6.11.2.2.3	Test Requirements

The test requirements in clause A.6.9.2.1.3 apply for this test.

### A.6.11.3	RSCPD Measurements

#### A.6.11.3.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_IDLE

##### A.6.11.3.1.1	Test purpose and environment

The purpose of the test is to verify that the accuracy of RSCPD measurement reported with RSTD measurement in RRC_IDLE. This test will verify the requirements in clause 10.1.43.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are listed in table A.6.11.3.1.1-1.

Table A.6.11.3.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation with dl-PRS-RSCPD-Request from LMF via LPP as defined in TS 37.355 [34], clause 6.5.12, to enable UE to perform and report RSCPD in RRC_IDLE shall be provided to the UE before the start of the test. The UE is configured with DRX cycle of 1.28s.

##### A.6.11.3.1.2	Test parameters

The RSCPD with RSTD accuracy test parameters are given in table A.6.11.3.1.2.-1.

Table A.6.11.3.1.2-1: RSCPD with UE RSTD measurement accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1,2,3 |  | Freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1,2,3 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - | 3 |
|  | 2,3 |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |
|  | 2 |  | PRS.1.1 FR1 |  |
|  | 3 |  | PRS.2.1 FR1 |  |
| PRS Resource slot offset | 1, 2, 3 | slot | 0 | 4 |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |
| EPRE ratio of PSS to SSS | 1,2,3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3 | dB | 0 | 0 |
| Note2 | 1,2 | dBm/ SCS | -98 |  |
|  | 3 |  | -95 |  |
| PRS | 1,2,3 | dB | -6 | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -65.70 | -65.70 |
|  | 3 | dBm/ 47.88MHz | -61.72 | -61.72 |
| PRS | 1,2,3 | dB | -5.7 | -11.9 |
| SSB | 1,2,3 | dB | -5.7 |  |
| Propagation condition | 1,2,3 | - | AWGN |  |
| Antenna configuration | 1,2,3 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. |  |  |  |  |

##### A.6. 11.3.1.3	Test requirements

The RSCPD reported together with RSTD fulfils RSCPD measurement accuracy specified in clause 10.1.43.2 for Cell 2.