3GPP TS 36.133 V14.19.0 (2021-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	78

1	Scope	79

2	References	79

3	Definitions, symbols and abbreviations	80

3.1	Definitions	80

3.2	Symbols	82

3.3	Abbreviations	83

3.4	Test tolerances	85

3.5	Additional notation	85

3.5.1	Groups of bands	85

3.6	General	87

3.6.1	Applicability of requirements in this specification version	87

4	E-UTRAN RRC_IDLE state mobility	93

4.1	Cell Selection	93

4.2	Cell Re-selection	93

4.2.1	Introduction	93

4.2.2	Requirements	93

4.2.2.1	Measurement and evaluation of serving cell	94

4.2.2.2	Void	95

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	95

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	97

4.2.2.5	Measurements of inter-RAT cells	99

4.2.2.5.1	Measurements of UTRAN FDD cells	99

4.2.2.5.2	Measurements of UTRAN TDD cells	101

4.2.2.5.3	Measurements of GSM cells	102

4.2.2.5.4	Measurements of HRPD cells	103

4.2.2.5.5	Measurements of cdma2000 1X	104

4.2.2.6	Evaluation of cell re-selection criteria	105

4.2.2.7	Maximum interruption in paging reception	106

4.2.2.8	void	106

4.2.2.9	UE measurement capability	106

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	107

4.2.2.10	Reselection to CSG cells	107

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	107

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	108

4.2.2.11	Void	109

4.2.2.12	Void	109

4.2.2.13	Void	109

4.3	Minimization of Drive Tests (MDT)	109

4.3.1	Introduction	110

4.3.2	Measurements	110

4.3.2.1	Requirements	110

4.3.3	Relative Time Stamp Accuracy	110

4.3.3.1	Requirements	110

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	110

4.3.4.1	 Requirements	110

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	111

4.3.5.1	Requirements for timeSinceFailure	111

4.4	MBSFN Measurements	111

4.4.1	Introduction	111

4.4.2	MBSFN RSRP measurements	111

4.4.3	MBSFN RSRQ measurements	111

4.4.4	MCH BLER measurements	112

4.5	Proximity-based Services	112

4.5.1	Introduction	112

4.5.2	Requirements	112

4.5.2.1	Interruptions with ProSe Direct Discovery	112

4.5.2.2	Interruptions with ProSe Direct Communication	112

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	112

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	113

4.6	Cell Selection and Re-selection Requirements for UE category NB1	113

4.6.1	Cell Selection	113

4.6.2	Cell Re-selection	113

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	114

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	114

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	116

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	116

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	118

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	119

4.6.2.7	Maximum interruption in paging reception in normal coverage	120

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	120

4.6.2.8	UE measurement capability	121

4.7	Cell Selection and Re-selection Requirements for UE category M1	121

4.7.1	Cell Selection	121

4.7.2	Cell Re-selection	121

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	121

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	121

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	121

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	123

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	124

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	124

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	125

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	125

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	126

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	127

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	129

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	129

4.8	Idle State Positioning Measurement Requirements for UE category NB1	130

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	130

4.8.1.1	RSTD Measurement Reporting Delay	131

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	132

4.8.2.1	RSTD Measurement Reporting Delay	133

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	133

4.8.3.1	RSTD Measurement Reporting Delay	135

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	135

4.8.4.1	RSTD Measurement Reporting Delay	137

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	137

4.8.5.1	Measurement Reporting Delay	138

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	138

4.8.6.1	Measurement Reporting Delay	139

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	140

4.8.7.1	Measurement Reporting Delay	141

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	141

4.8.8.1	Measurement Reporting Delay	142

5	E-UTRAN RRC_CONNECTED state mobility	142

5.1	E-UTRAN Handover	143

5.1.1	Introduction	143

5.1.2	Requirements	143

5.1.2.1	E-UTRAN FDD – FDD	143

5.1.2.1.1	Handover delay	143

5.1.2.1.2	Interruption time	144

5.1.2.2	E-UTRAN FDD – TDD	145

5.1.2.2.1	(Void)	145

5.1.2.2.2	(Void)	145

5.1.2.3	E-UTRAN TDD – FDD	145

5.1.2.3.1	(Void)	145

5.1.2.3.2	(Void)	145

5.1.2.4	E-UTRAN TDD – TDD	145

5.1.2.4.1	Handover delay	145

5.1.2.4.2	Interruption time	146

5.1.2.5	E-UTRAN HD–FDD	147

5.1.2.5.1	Handover delay	147

5.1.2.5.2	Interruption time	147

5.2	Void	149

5.3	Handover to other RATs	149

5.3.1	E-UTRAN - UTRAN FDD Handover	149

5.3.1.1	Introduction	149

5.3.1.1.1	Handover delay	149

5.3.1.1.2	Interruption time	149

5.3.2	E-UTRAN - UTRAN TDD Handover	150

5.3.2.1	Introduction	150

5.3.2.2	Requirements	150

5.3.2.2.1	Handover delay	150

5.3.2.2.2	Interruption time	150

5.3.3	E-UTRAN - GSM Handover	151

5.3.3.1	Introduction	151

5.3.3.2	Requirements	151

5.3.3.2.1	Handover delay	151

5.3.3.2.2	Interruption time	151

5.4	Handover to Non-3GPP RATs	151

5.4.1	E-UTRAN – HRPD Handover	151

5.4.1.1	Introduction	151

5.4.1.1.1	Handover delay	152

5.4.1.1.2	Interruption time	152

5.4.2	E-UTRAN – cdma2000 1X Handover	152

5.4.2.1	Introduction	152

5.4.2.1.1	Handover delay	152

5.4.2.1.2	Interruption time	152

5.5	E-UTRAN Handover for Cat-M1 UEs	153

5.5.1	Introduction	153

5.5.2	Requirements in CEModeA	153

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	153

5.5.2.1.1	Handover delay	153

5.5.2.1.2	Interruption time	153

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	154

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	154

5.5.2.3.1	Void	154

5.5.2.3.2	Void	154

5.5.3	Requirements in CEModeB	154

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	154

5.5.3.1.1	Handover delay	154

5.5.3.1.2	Interruption time	155

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	155

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	155

5.6	Void	155

6	RRC Connection Mobility Control	155

6.1	RRC Re-establishment	155

6.1.1	Introduction	156

6.1.2	Requirements	156

6.1.2.1	UE Re-establishment delay requirement	156

6.2	Random Access	156

6.2.1	Introduction	156

6.2.2	Requirements	156

6.2.2.1	Contention based random access	157

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	157

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	157

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	157

6.2.2.1.4	Void	157

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	157

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	157

6.2.2.2	Non-Contention based random access	157

6.2.2.2.1	Correct behaviour when receiving Random Access Response	157

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	157

6.2.3	Requirements for Cat-M1 UEs	158

6.3	RRC Connection Release with Redirection	158

6.3.1	Introduction	158

6.3.2	Requirements	158

6.3.2.1	RRC connection release with redirection to UTRAN FDD	158

6.3.2.2	RRC connection release with redirection to GERAN	158

6.3.2.3	RRC connection release with redirection to UTRAN TDD	159

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	159

6.4.1	Introduction	159

6.4.2	Requirements	159

6.5	RRC Re-establishment for NB-IoT UEs	160

6.5.1	Introduction	160

6.5.2	Requirements	160

6.5.2.1	UE Re-establishment delay requirement in normal coverage	160

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	161

6.6	Random Access for UE category NB1	161

6.6.1	Introduction	161

6.6.2	Requirements	161

6.6.2.1	Correct behaviour when receiving Random Access Response reception	161

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	162

6.6.2.3	Correct behaviour when receiving a NACK on msg3	162

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	162

6.6.2.5	Correct behaviour when contention Resolution timer expires	162

6.6.2.6	MSG3-based channel quality report for UE Category NB1	162

6.6.3	Requirements for NPRACH configuration	162

6.7	RRC Re-establishment for Cat-M1 UEs	163

6.7.1	Introduction	163

6.7.2	Requirements	163

6.7.2.1	UE Re-establishment delay requirement for CEModeA	163

6.7.2.2	UE Re-establishment delay requirement for CEModeB	164

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	164

6.8.1	Introduction	164

6.8.2	Requirements	164

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	164

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	165

6.9.1	Introduction	165

6.9.2	Requirements	165

7	Timing and signalling characteristics	166

7.1	UE transmit timing	166

7.1.1	Introduction	166

7.1.2	Requirements	166

7.2	UE timer accuracy	167

7.2.1	Introduction	167

7.2.2	Requirements	167

7.3	Timing Advance	168

7.3.1	Introduction	168

7.3.2	Requirements	168

7.3.2.1	Timing Advance adjustment delay	168

7.3.2.2	Timing Advance adjustment accuracy	168

7.4	Cell phase synchronization accuracy (TDD)	168

7.4.1	Definition	168

7.4.2	Minimum requirements	168

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	169

7.5.1	Introduction	169

7.5.2	eNodeB Synchronization Requirements	169

7.5.2.1	Synchronized E-UTRAN	169

7.5.2.2	Non-Synchronized E-UTRAN	169

7.6	Radio Link Monitoring	169

7.6.1	Introduction	169

7.6.2	Requirements	171

7.6.2.1	Minimum requirement when no DRX is used	171

7.6.2.2	Minimum requirement when DRX is used	171

7.6.2.3	Minimum requirement at transitions	173

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	173

7.6.2.5	Minimum requirement under IDC Interference	173

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	173

7.7.1	Introduction	173

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	173

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	174

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	174

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	175

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	176

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	176

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	177

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	177

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	177

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	178

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.8	Interruptions with Carrier Aggregation	179

7.8.1	Introduction	179

7.8.2	Requirements	180

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	180

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	180

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	180

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	180

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	180

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	181

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	181

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	181

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	181

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	183

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	183

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	183

7.8.2.13	Interruptions at SRS carrier based switching	184

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	184

7.9.1	Introduction	184

7.9.2	Minimum Requirements for Interband Carrier Aggregation	184

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	185

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	185

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	185

7.10.1	Introduction	185

7.10.2	Requirements	185

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	186

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	186

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	186

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	187

7.11	Radio Link Monitoring for UE Category 0	187

7.11.1	Introduction	187

7.11.2	Requirements for FD-FDD and TDD	188

7.11.2.1	Minimum requirement when no DRX is used	188

7.11.2.2	Minimum requirement when DRX is used	189

7.11.2.3	Minimum requirement at transitions	189

7.11.3	Requirements for HD-FDD	190

7.11.3.1	Minimum requirement when no DRX is used	190

7.11.3.2	Minimum requirement when DRX is used	190

7.11.3.3	Minimum requirement at transitions	191

7.12	Interruptions with Dual Connectivity	191

7.12.1	Introduction	191

7.12.2	Requirements	191

7.12.2.1	Interruptions at PSCell addition/release	191

7.12.2.2	Interruptions at transitions between active and non-active during DRX	191

7.12.2.3	Interruptions at transitions from non-DRX to DRX	192

7.12.2.4	Interruptions at SCell addition/release	192

7.12.2.5	Interruptions at SCell activation/deactivation	192

7.12.2.6	Interruptions during measurements on SCC	193

7.12.2.7	Interruptions at SRS carrier based switching	193

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	194

7.13.1	Definition	194

7.13.2	Minimum requirements	194

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	194

7.14.1	Introduction	194

7.14.2	PSCell Addition Delay Requirement	194

7.14.3	PSCell Release Delay Requirement	195

7.15	Maximum Receive Timing Difference in Dual Connectivity	195

7.15.1	Introduction	195

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	195

7.16	Proximity-based Services	195

7.16.1	Introduction	195

7.16.2	Requirements	195

7.16.2.1	ProSe UE transmission timing	195

7.16.2.1.1	Serving cell or PCell as timing reference	196

7.16.2.1.2	SCell or non-serving cell as timing reference	196

7.16.3	Interruptions with ProSe	196

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	196

7.16.3.2	Interruptions at ProSe Direct Communication configuration	196

7.16.3.3	Interruptions during ProSe Direct Discovery	197

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	197

7.16.3.5	Interruptions during ProSe Direct Communication	197

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	198

7.16.4.1	Measurement and evaluation of selected cell	198

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	198

7.16.5	Selection / Reselection of ProSe relay UE	199

7.16.6	ProSe operation under deactivated SCell	199

7.17	Maximum Transmission Timing Difference in Dual Connectivity	199

7.17.1	Introduction	199

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	200

7.18.1	Introduction	200

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	200

7.19	Radio Link Monitoring for UE Category M1	200

7.19.1	Introduction	200

7.19.2	Requirements for FD-FDD and TDD CE mode A	200

7.19.2.1	Minimum requirement when no DRX is used	201

7.19.2.2	Minimum requirement when DRX is used	202

7.19.2.3	Minimum requirement at transitions	203

7.19.3	Requirements for HD-FDD with CE mode A	203

7.19.3.1	Minimum requirement when no DRX is used	204

7.19.3.2	Minimum requirement when DRX is used	204

7.19.3.3	Minimum requirement at transitions	205

7.19.4	Requirements for FD-FDD and TDD with CE mode B	205

7.19.4.1	Minimum requirement when no DRX is used	206

7.19.4.2	Minimum requirement when DRX is used	207

7.19.4.3	Minimum requirement at transitions	208

7.19.5	Requirements for HD-FDD with CE mode B	208

7.19.5.1	Minimum requirement when no DRX is used	209

7.19.5.2	Minimum requirement when DRX is used	209

7.19.5.3	Minimum requirement at transitions	210

7.20	UE transmit timing for NB-IoT	210

7.20.1	Introduction	210

7.20.2	Requirements	210

7.21	 UE timer accuracy for NB-IoT	211

7.21.1	Introduction	211

7.21.2	Requirements	211

7.22	Timing Advance for NB-IoT	211

7.22.1	Introduction	211

7.22.2	Requirements	211

7.22.2.1	Timing Advance adjustment delay	211

7.22.2.2	Timing Advance adjustment accuracy	212

7.23	Radio Link Monitoring for Category NB1 UE	212

7.23.1	Introduction	212

7.23.2	Requirements for HD-FDD Category NB1 UE	212

7.23.2.1	Minimum requirement when no DRX is used	212

7.23.2.2	Minimum requirement when DRX is used	213

7.23.2.3	Minimum requirement at transitions	213

7.24	UE transmit timing for Category M1	214

7.24.1	Introduction	214

7.24.2	Requirements	214

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	215

7.25.1	Definition	215

7.25.2	Minimum requirements	215

7.26	UE transmit timing for Category M2	215

7.26.1	Introduction	215

7.26.2	Requirements	215

7.27	UE timer accuracy for category M1	216

7.27.1	Introduction	216

7.27.2	Requirements	216

7.28	Timing Advance for Category M1	216

7.28.1	Introduction	216

7.28.2	Requirements	216

7.29	Interruptions requirements with FeMBMS	216

7.29.1	Introduction	216

7.29.2	Requirements	216

7.30	Numerology switching delay requirements with FeMBMS	216

7.30.1	Introduction	216

7.30.2	Requirements	216

8	UE Measurements Procedures in RRC_CONNECTED State	217

8.1	General Measurement Requirements	217

8.1.1	Introduction	217

8.1.2	Requirements	217

8.1.2.1	UE measurement capability	217

8.1.2.1.1	Monitoring of multiple layers using gaps	221

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	222

8.1.2.1.2	Network controlled small gap	223

8.1.2.2	E-UTRAN intra frequency measurements	225

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	225

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	229

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	233

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	234

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	235

8.1.2.3	E-UTRAN inter frequency measurements	235

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	236

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	241

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	248

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	248

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	248

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	249

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	251

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	252

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	253

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	259

8.1.2.4	Inter RAT measurements	259

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	259

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	264

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	264

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	268

8.1.2.4.5	E-UTRAN FDD – GSM measurements	268

8.1.2.4.6	E-UTRAN TDD – GSM measurements	273

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	273

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	275

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	275

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	275

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	276

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	276

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	276

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	276

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	278

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	278

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	279

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	279

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	281

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	281

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	281

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	283

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	285

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	286

8.1.2.6.5	(Void)	288

8.1.2.6.6	(Void)	288

8.1.2.6.7	(Void)	288

8.1.2.6.8	(Void)	288

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	288

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	289

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	290

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	292

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	294

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	295

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	297

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	299

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	301

8.1.2.7	E-UTRAN E-CID Measurements	302

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	302

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	304

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	306

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	306

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	309

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	312

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	315

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	319

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	319

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	320

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	320

8.2.1	Introduction	320

8.2.2	Requirements	321

8.3	Measurements for E-UTRA carrier aggregation	323

8.3.1	Introduction	323

8.3.2	Measurements of the primary component carrier	323

8.3.3	Measurements of a secondary component carrier	323

8.3.3.1	Measurements of a secondary component carrier with active SCell	323

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	324

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	324

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	325

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	326

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	326

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	327

8.4.1	Introduction	327

8.4.2	Measurements on the primary component carrier	327

8.4.3	Measurements on a secondary component carrier	328

8.4.4	Measurements on both primary component carrier and a secondary component carrier	328

8.4.5	Measurements on different secondary component carriers	329

8.5	Measurements for UE category 0	330

8.5.1	Introduction	330

8.5.2 	Requirements	331

8.5.2.1 	E-UTRAN intra frequency measurements	331

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	331

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	334

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	336

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	340

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	341

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	341

8.6	Discovery signal measurements	342

8.6.1	Introduction	342

8.6.2	Requirements for CRS based discovery signal measurements	342

8.6.2.1	E-UTRAN intra frequency measurements	342

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	342

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	345

8.6.2.2	E-UTRAN inter frequency measurements	347

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	348

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	350

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	353

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	353

8.6.3	Requirements for CSI-RS based discovery signal measurements	353

8.6.3.1	E-UTRAN intra frequency measurements	353

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	353

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	356

8.6.3.2	E-UTRAN inter frequency measurements	358

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	358

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	361

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	363

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	364

8.7	Discovery signal measurements for E-UTRA carrier aggregation	364

8.7.1	Introduction	364

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	364

8.7.2.1	Measurements of the primary component carrier	364

8.7.2.2	Measurements of a secondary component carrier	364

8.7.2.3	Measurements of a secondary component carrier with active SCell	364

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	364

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	365

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	366

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	367

8.7.3.1	Measurements of the primary component carrier	367

8.7.3.2	Measurements of a secondary component carrier	367

8.7.3.3	Measurements of a secondary component carrier with active SCell	368

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	368

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	368

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	369

8.8	Measurements for E-UTRA dual connectivity	371

8.8.1	Introduction	371

8.8.2	Intra-frequency measurements requirements on PCell	371

8.8.3	Intra-frequency measurements requirements on PSCell	371

8.8.4	Inter-frequency and inter-RAT measurement requirements	371

8.8.5	Intra-frequency measurements with autonomous gaps	371

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	371

8.8.5.2	ECGI reporting delay	373

8.8.6	Inter-frequency measurements with autonomous gaps	373

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	373

8.8.6.2	ECGI reporting delay	374

8.8.7	SSTD Measurements	374

8.8.7.1	Introduction	374

8.8.7.2	SSTD Measurement requirements	374

8.8.7.3	SSTD Measurement Reporting Delay	374

8.8.8	Intra-frequency measurements requirements on SCell	375

8.9	MBSFN Measurements	375

8.9.1	Introduction	375

8.9.2	MBSFN RSRP Measurements	375

8.9.3	MBSFN RSRQ Measurements	375

8.9.4	MCH BLER Measurements	375

8.10	Proximity-based Services	376

8.10.1	Introduction	376

8.10.2	Requirements	376

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	376

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	376

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	377

8.11.1	Introduction	377

8.11.2	CRS based discovery signal measurements	377

8.11.2.1	E-UTRAN intra-frequency measurements	377

8.11.2.1.1	Requirements	377

8.11.2.1.1.1	Requirements when no DRX is used	377

8.11.2.1.1.1.1	Measurement Reporting Requirements	379

8.11.2.1.1.2	Requirements when DRX is used	380

8.11.2.1.1.2.1	Measurement Reporting Requirements	382

8.11.2.2	E-UTRAN inter-frequency measurements	383

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	383

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	387

8.11.3	CSI-RS based discovery signal measurements	387

8.11.3.1	E-UTRAN intra-frequency measurements	387

8.11.3.1.1	Requirements	387

8.11.3.1.1.1	Requirements when no DRX is used	387

8.11.3.1.1.1.1	Measurement Reporting Requirements	389

8.11.3.1.1.2	Requirements when DRX is used	389

8.11.3.1.1.2.1	Measurement Reporting Requirements	390

8.11.3.2	E-UTRAN inter-frequency measurements	391

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	391

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	395

8.11.4	RSSI measurements	395

8.11.4.1	E-UTRAN intra-frequency measurements	395

8.11.4.2	E-UTRAN inter-frequency measurements	395

8.11.5	Channel occupancy measurements	396

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	396

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	396

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	396

8.12.1	Introduction	396

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	396

8.12.2.1	Introduction	396

8.12.2.2	Measurements of a secondary component carrier	396

8.12.2.3	Measurements of a secondary component carrier with active SCell	396

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	397

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	397

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	399

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	402

8.12.3.1	Introduction	402

8.12.3.2	Measurements of a secondary component carrier	402

8.12.3.3	Measurements of a secondary component carrier with active SCell	402

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	402

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	402

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	404

8.13	Measurements for UE Category M1	406

8.13.1	Introduction	406

8.13.2	Requirements for UE category M1 with CE mode A	406

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	407

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	407

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	411

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	413

8.13.2.2	Void	416

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	416

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	416

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	419

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	422

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	422

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	422

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	424

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	427

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	427

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	428

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	428

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	428

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	429

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	429

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	429

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	430

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	431

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	432

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	432

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	436

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	438

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	441

8.13.3	Requirements for UE category M1 with CE mode B	442

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	442

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	442

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	446

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	449

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	454

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	455

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	455

8.13.3.2	Void	456

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	456

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	456

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	459

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	461

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	462

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	463

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	463

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	464

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	464

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	464

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	468

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	470

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	475

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	476

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	476

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	478

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	480

8.14	Measurements for UE category NB1	481

8.14.1	Introduction	481

8.14.2	NB-IoT intra frequency measurements under normal coverage	481

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	481

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	481

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	481

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	481

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	481

8.15	Void	482

8.16	Measurements for UE Category M2	482

8.16.1	Introduction	482

8.16.2	Requirements for UE category M2 with CE mode A	482

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	482

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	482

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	483

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	483

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	484

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	484

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	484

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	484

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	487

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	489

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	490

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	490

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	492

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	494

8.16.3	Requirements for UE category M2 with CE mode B	495

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	495

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	495

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	497

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	500

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	500

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	500

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	502

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	505

8.17	Void	505

8.18	Measurements for non-BL/CE UE	505

8.18.1	Introduction	505

8.18.2	Requirements for non-BL/CE UE with CE Mode B	505

8.18.2.1	E-UTRAN intra frequency measurements	505

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	505

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	506

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	507

9	Measurements performance requirements for UE	507

9.1	E-UTRAN measurements	508

9.1.1	Introduction	508

9.1.2	Intra-frequency RSRP Accuracy Requirements	508

9.1.2.1	Absolute RSRP Accuracy	508

9.1.2.2	Relative Accuracy of RSRP	509

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	510

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	510

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	511

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	512

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	513

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	514

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	515

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	515

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	515

9.1.3	Inter-frequency RSRP Accuracy Requirements	516

9.1.3.1	Absolute RSRP Accuracy	516

9.1.3.2	Relative Accuracy of RSRP	517

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	517

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	518

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	519

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	519

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	519

9.1.4	RSRP Measurement Report Mapping	520

9.1.5	Intra-frequency RSRQ Accuracy Requirements	520

9.1.5.1	Absolute RSRQ Accuracy	520

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	521

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	522

9.1.5.4	Absolute WB-RSRQ Accuracy	523

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	524

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	525

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	525

9.1.6	Inter-frequency RSRQ Accuracy Requirements	525

9.1.6.1	Absolute RSRQ Accuracy	525

9.1.6.2	Relative Accuracy of RSRQ	526

9.1.6.3	Absolute WB-RSRQ Accuracy	527

9.1.6.4	Relative WB-RSRQ Accuracy	527

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	528

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	529

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	529

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	529

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	530

9.1.7	RSRQ Measurement Report Mapping	531

9.1.8	Power Headroom	531

9.1.8.1	Period	531

9.1.8.2	Reporting Delay	531

9.1.8.3	Void	532

9.1.8.4	Report Mapping	532

9.1.9	UE Rx – Tx time difference	532

9.1.9.1	Measurement Requirement	532

9.1.9.2	Measurement Report mapping	533

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	534

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	535

9.1.10	Reference Signal Time Difference (RSTD)	536

9.1.10.1	Intra-Frequency Accuracy Requirement	536

9.1.10.2	Inter-Frequency Accuracy Requirement	537

9.1.10.3	RSTD Measurement Report Mapping	539

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	539

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	540

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	541

9.1.11	Carrier aggregation measurement accuracy	542

9.1.11.1	Primary component carrier accuracy requirement	543

9.1.11.2	Secondary component carrier accuracy requirement	543

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	543

9.1.11.4	Secondary component carrier relative accuracy requirement	543

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	543

9.1.13	Measurement accuracy for UE category 0	544

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	544

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	544

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	545

9.1.14	Accuracy requirements for Discovery Signal Measurements	546

9.1.14.1	Introduction	546

9.1.14.2	RSRP measurements in discovery signal occasions	546

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	546

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	546

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	546

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	547

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	548

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	548

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	548

9.1.14.3.3	CSI-RSRP measurement report mapping	549

9.1.14.4	RSRQ measurements in discovery signal occasions	549

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	549

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	550

9.1.15.1.1	Primary component carrier accuracy requirement	550

9.1.15.1.2	Secondary component carrier accuracy requirement	550

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	550

9.1.15.1.4	Secondary component carrier relative accuracy requirement	550

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	550

9.1.15.2.1	Primary component carrier accuracy requirement	550

9.1.15.2.2	Secondary component carrier accuracy requirement	550

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	550

9.1.15.2.4	Secondary component carrier relative accuracy requirement	550

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	551

9.1.17	RS-SINR Measurements	551

9.1.17.1	Measurement Report Mapping	551

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	552

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	552

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	552

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	552

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	553

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	554

9.1.18.1	Introduction	554

9.1.18.2	RSRP measurements	554

9.1.18.2.1	RSRP measurement report mapping	554

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	554

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	555

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	555

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	556

9.1.18.3	RSRQ measurements	556

9.1.18.3.1	RSRQ measurement report mapping	556

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	556

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	557

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	557

9.1.18.4	CSI-RSRP measurements	558

9.1.18.4.1	CSI-RSRP measurement report mapping	558

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	558

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	558

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	559

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	559

9.1.18.5	RSSI measurements	560

9.1.18.5.1	RSSI measurement report mapping	560

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	560

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	561

9.1.18.6	Channel occupancy measurements	561

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	561

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	561

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	562

9.1.19.1	Introduction	562

9.1.19.2	Accuracy requirements for measurements on SCC	562

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	562

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	562

9.1.20	SFN and Subframe Time Difference (SSTD)	563

9.1.20.1	SSTD Accuracy Requirement	563

9.1.20.2	SSTD Measurement Report Mapping	563

9.1.21	Measurement accuracy for UE category M1	564

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	564

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	565

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	566

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	567

9.1.21.5	RSRP Measurement Report Mapping	568

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	568

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	569

9.1.21.8	RSRQ Measurement Report Mapping	570

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	570

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	571

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	572

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	573

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	574

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	575

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	576

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	577

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	578

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	579

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	580

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	581

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	583

9.1.22	Measurement accuracy for UE Category NB1	585

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	585

9.1.22.2	Void	585

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	585

9.1.22.4	Void	586

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	586

9.1.22.6	Void	586

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	586

9.1.22.8	Void	587

9.1.22.9	NRSRP Measurement Report Mapping	587

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	587

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	588

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	589

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	590

9.1.22.14	NRSRQ Measurement Report Mapping	591

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	592

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	592

9.1.23	Power Headroom for UE Category NB1	593

9.1.23.1	Period	593

9.1.23.2	Reporting Delay	593

9.1.23.3	Report Mapping for UE Category NB1	593

9.1.23.3.1	Void	594

9.1.23.3.2	Void	594

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	594

9.1.24	Void	594

9.1.25	Measurement accuracy for UE category M2	594

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	594

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	595

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	596

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	597

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	599

9.1.26	Measurement Accuracy for non-BL CE UE	600

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	601

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	601

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	602

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	603

9.1.26.5	RSRP Measurement Report Mapping	604

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	604

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	604

9.1.26.8	RSRQ Measurement Report Mapping	605

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	605

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	606

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	607

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	608

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	609

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	609

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	609

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	610

9.2	UTRAN FDD Measurements	611

9.2.1	UTRAN FDD CPICH RSCP	612

9.2.2	Void	612

9.2.3	UTRAN FDD CPICH Ec/No	612

9.3	UTRAN TDD Measurements	613

9.3.1	UTRAN TDD P-CCPCH RSCP	613

9.3.2	Void	613

9.3.3	Void	613

9.4	GSM Measurements	613

9.4.1	GSM carrier RSSI	613

9.5	CDMA2000 1x RTT Measurements	614

9.5.1	CDMA2000 1x RTT Pilot Strength	614

9.6	PCMAX,c	614

9.6.1	Report Mapping	614

9.6.2 	Estimation Period	614

9.6.3	Reporting Delay	614

9.7	IEEE802.11 Measurements	614

9.7.1	WLAN RSSI	615

9.7.2	WLAN RSSI Measurement Report Mapping	615

9.8	MBSFN Measurements	615

9.8.1	Introduction	615

9.8.2	MBSFN RSRP	615

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	615

9.8.2.2	MBSFN RSRP measurement report mapping	616

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	616

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	617

9.8.3	MBSFN RSRQ	617

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	617

9.8.3.2	MBSFN RSRQ measurement report mapping	617

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	618

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	618

9.8.4	MCH BLER	618

9.8.4.1	Measurement report mapping for MCH BLER	618

9.8.4.2	Measurement report mapping for MCH Block Number	619

9.9	ProSe Measurements	620

9.9.1	Introduction	620

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	620

9.9.2.1	Absolute S-RSRP Accuracy	620

9.9.2.2	Relative Accuracy of S-RSRP	621

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	622

9.9.3.1	Absolute SD-RSRP Accuracy	622

9.9.3.2	Relative Accuracy of SD-RSRP	622

9.10	V2X Measurements	623

9.10.1	Introduction	623

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	623

9.10.2.1	Absolute S-RSRP Accuracy	623

9.10.2.2	Relative Accuracy of S-RSRP	624

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	624

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	624

9.10.4	S-RSSI Measurement Accuracy Requirements	625

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	625

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	625

10	Measurements Performance Requirements for E-UTRAN	626

10.1	Received Interference Power	626

10.1.1	Absolute accuracy requirement	626

10.1.2	Relative accuracy requirement	626

10.1.3	Received Interference Power measurement report mapping	626

10.2	Angle of Arrival (AOA)	627

10.2.1	Range/mapping	627

10.3	Timing Advance (TADV)	627

10.3.1	Report mapping	627

11	ProSe Requirements in Any Cell Selection state	628

11.1	Introduction	628

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	628

11.2.1	Introduction	628

11.2.2	ProSe UE transmission timing	628

11.3	Initiation/Cease of SLSS Transmissions	628

11.3.1	Introduction	628

11.3.2	Requirements	628

11.4	Measurements for ProSe in Any Cell Selection State	629

11.4.1	Introduction	629

11.4.2	Requirements	629

11.4.2.1	E-UTRA FDD	629

11.4.2.2	E-UTRA TDD	629

11.5	Selection / Reselection of ProSe Synchronization Reference	630

11.5.1	Introduction	630

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	630

11.5.2.1	Introduction	630

11.5.2.2	Requirements	630

11.6	Void	630

11.7	Selection / Reselection of ProSe relay UE	630

11.7.1	Introduction	630

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	631

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	631

12.1	Introduction	631

12.2	Transmit Timing	631

12.2.1	GNSS as timing reference	631

12.3	Interruption	632

12.4	Reliability of GNSS signal	632

13	V2X Requirements	632

13.1	Introduction	632

13.2	UE Transmit Timing	632

13.2.1	Introduction	632

13.2.2	GNSS as synchronization reference source	632

13.2.3	Serving cell/PCell as synchronization reference source	633

13.2.4	SyncRef UE as synchronization reference source	633

13.3	Initiation/Cease of SLSS Transmissions	633

13.3.1	Introduction	633

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	633

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	634

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	634

13.4	Selection / Reselection of V2X Synchronization Reference Source	634

13.5	Autonomous Resource Selection/Reselection measurements	635

13.5.1	Introduction	635

13.5.2	PSSCH-RSRP measurements	635

13.5.3	S-RSSI measurements	636

13.6	Congestion Control measurements	636

13.7	Interruption	636

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	636

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	636

13.8	Reliability of GNSS signal	637

Annex A (normative): Test Cases	638

A.1	Purpose of annex	638

A.2	Requirement classification for statistical testing	638

A.2.1	Types of requirements in TS 36.133	638

A.2.1.1	Time and delay requirements on UE higher layer actions	638

A.2.1.2	Measurements of power levels, relative powers and time	638

A.2.1.3	Implementation requirements	639

A.2.1.4	Physical layer timing requirements	639

A.3	RRM test configurations	640

A.3.1	Reference Measurement Channels	640

A.3.1.1	PDSCH	640

A.3.1.1.1	FDD	640

A.3.1.1.2	TDD	641

A.3.1.1.3	FDD for UE category 0	643

A.3.1.1.4	HD-FDD for UE category 0	644

A.3.1.1.5	TDD for UE category 0	645

A.3.1.1.6	Frame Structure 3	646

A.3.1.2	PCFICH/PDCCH/PHICH	647

A.3.1.2.1	FDD	647

A.3.1.2.2	TDD	647

A.3.1.2.3	HD-FDD for UE category 0	648

A.3.1.2.4	FS 3	648

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	648

A.3.1.3.1	FDD in CEModeA	649

A.3.1.3.2	HD-FDD in CEModeA	649

A.3.1.3.3	TDD in CEModeA	650

A.3.1.3.4	FDD in CEModeB	650

A.3.1.3.5	HD-FDD in CEModeB	651

A.3.1.3.6	TDD in CEModeB	651

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	652

A.3.1.4.1	FDD in CEModeA	652

A.3.1.4.2	HD-FDD in CEModeA	653

A.3.1.4.3	TDD in CEModeA	654

A.3.1.4.4	FDD in CEModeB	655

A.3.1.4.5	 HD-FDD in CEModeB	656

A.3.1.4.6	TDD in CEModeB	656

A.3.1.5	NPDSCH Reference Channel for UE category NB1	657

A.3.1.5.1	HD-FDD in-band operation	657

A.3.1.5.2	Void	658

A.3.1.5.3	HD-FDD standalone operation	658

A.3.1.5.4	Void	659

A.3.1.5.5	HD-FDD guard band operation	659

A.3.1.5.6	Void	659

A.3.1.6	NPDCCH Reference Channel for UE category NB1	659

A.3.1.6.1	HD-FDD in-band operation	659

A.3.1.6.2	Void	660

A.3.1.6.3	HD-FDD standalone operation	660

A.3.1.6.4	Void	660

A.3.1.6.5	HD-FDD guard band operation	660

A.3.1.6.6	Void	661

A.3.2	OFDMA Channel Noise Generator (OCNG)	661

A.3.2.1	OCNG Patterns for FDD	661

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	662

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	662

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	663

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	664

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	665

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	665

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	666

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	666

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	667

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	667

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	668

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	669

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	670

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	670

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	671

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	671

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	672

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	673

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	673

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	674

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	674

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	675

A.3.2.2	OCNG Patterns for TDD	675

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	676

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	676

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	677

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	677

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	678

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	679

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	680

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	681

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	681

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	682

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	683

A.3.2.3	OCNG Patterns for Narrowband IoT	683

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	685

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	686

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	686

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	687

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	688

A.3.2.4	OCNG Patterns for V2X sidelink	689

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	690

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	690

A.3.3	Reference DRX Configurations	690

A.3.4	ABS Transmission Configurations	691

A.3.4.1	Non-MBSFN ABS Transmission Configurations	691

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	691

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	691

A.3.4.2	MBSFN ABS Transmission Configurations	692

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	692

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	693

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	693

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	693

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	694

A.3.6.1	Introduction	694

A.3.7	Test Cases with Different Channel Bandwidths	694

A.3.7.1	Introduction	694

A.3.7.2	Principle of testing	694

A.3.8	Antenna Configuration	694

A.3.8.1	Antenna connection for 4 Rx capable UEs	694

A.3.8.1.1 Introduction	694

A.3.8.1.2 Principle of testing	694

A.3.8.1.2.1 Single carrier tests	694

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	695

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	696

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	696

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	696

A.3.9.1	Introduction	696

A.3.9.2	Principle of testing	696

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	697

A.3.10.1	Introduction	697

A.3.10.2	Principle of testing	697

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	697

A.3.11.1	Introduction	697

A.3.11.2	Principle of Testing	697

A.3.12	Proximity-based Services	697

A.3.12.1	Introduction	697

A.3.12.2	Reference DRX configurations for ProSe tests	697

A.3.12.3	Test Cases with Different Channel Bandwidths	698

A.3.12.3.1	Introduction	698

A.3.12.3.2	Principle of testing	698

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	698

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	704

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	706

A.3.12.6.1	FDD	706

A.3.12.7	Reference measurement channels for ProSe Direct Communication	706

A.3.12.7.1	FDD	706

A.3.12.8	ProSe Receive Traffic Generator	707

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	707

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	707

A.3.13	Time Offset between Cells	708

A.3.13.1	Introduction	708

A.3.13.2	Definition	708

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	708

A.3.14.1	Introduction	708

A.3.14.2	Principle of testing	708

A.3.15	Dual connectivity test cases with different combination of duplex mode	708

A.3.15.1	Introduction	708

A.3.15.2	Principle of testing	708

A.3.16	Reference PRACH Configurations	709

A.3.17	Listen before talk model	709

A.3.17.1	Introduction	709

A.3.17.2	Definition	709

A.3.18	Reference NPRACH Configurations	710

A.3.19	Dual connectivity test cases with different bandwidth combinations	711

A.3.19.1	Introduction	711

A.3.19.2	Principle of testing	711

A.3.20	Category M1 UE Test Cases	711

A.3.20.1	Introduction	711

A.3.20.2	Principle of Cat-M1 UE Testing	711

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	712

A.3.21.1	Introduction	712

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	713

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	714

A.3.22	Category 1bis UE Test Cases	715

A.3.22.1	Introduction	715

A.3.22.2	Principle of Category 1bis UE Testing	715

A.3.23	Category NB2 UE Test Cases	718

A.3.23.1	Introduction	718

A.3.23.2	Principle of Category NB2 UE Testing	718

A.3.24	V2X sidelink communication	719

A.3.24.1	Introduction	719

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	720

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	723

A.3.25	Category M2 UE Test Cases	724

A.3.25.1	Introduction	724

A.3.25.2	Principle of Cat-M2 UE Testing	724

A.4	E-UTRAN RRC_IDLE state	726

A.4.2	Cell Re-Selection	726

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	726

A.4.2.1.1	Test Purpose and Environment	726

A.4.2.1.2	Test Requirements	727

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	728

A.4.2.2.1	Test Purpose and Environment	728

A.4.2.2.2	Test Requirements	729

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	730

A.4.2.3.1	Test Purpose and Environment	730

A.4.2.3.2	Test Requirements	731

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	732

A.4.2.4.1	Test Purpose and Environment	732

A.4.2.4.2	Test Requirements	733

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	734

A.4.2.5.1	Test Purpose and Environment	734

A.4.2.5.2	Test Requirements	735

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	736

A.4.2.6.1	Test Purpose and Environment	736

A.4.2.6.2	Test Requirements	737

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	738

A.4.2.7.1	Test Purpose and Environment	738

A.4.2.7.2	Test Requirements	739

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	740

A.4.2.8.1	Test Purpose and Environment	740

A.4.2.8.2	Test Requirements	741

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	742

A.4.2.9.1	Test Purpose and Environment	742

A.4.2.9.2	Test Requirements	742

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	742

A.4.2.10.1	Test Purpose and Environment	742

A.4.2.10.2	Test Requirements	746

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	746

A.4.2.11.1	Test Purpose and Environment	746

A.4.2.11.2	Test Requirements	750

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	750

A.4.2.12.1	Test Purpose and Environment	750

A.4.2.12.2	Test Requirements	752

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	753

A.4.2.13.1	Test Purpose and Environment	753

A.4.2.13.2	Test Requirements	754

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	755

A.4.2.14.1	Test Purpose and Environment	755

A.4.2.14.2	Test Requirements	756

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	757

A.4.2.15.1	Test Purpose and Environment	757

A.4.2.15.2	Test Requirements	758

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	759

A.4.2.16.1	Test Purpose and Environment	759

A.4.2.16.2	Test Requirements	760

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	761

A.4.2.17.1	Test Purpose and Environment	761

A.4.2.17.2	Test Requirements	762

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	763

A.4.2.18.1	Test Purpose and Environment	763

A.4.2.18.2	Test Requirements	765

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	766

A.4.2.19.1	Test Purpose and Environment	766

A.4.2.19.2	Test Requirements	768

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	769

A.4.2.20.1	Test Purpose and Environment	769

A.4.2.20.2	Test Requirements	770

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	771

A.4.2.21.1	Test Purpose and Environment	771

A.4.2.21.2	Test Requirements	772

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	773

A.4.2.22.1	Test Purpose and Environment	773

A.4.2.22.2	Test Requirements	774

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	775

A.4.2.23.1	Test Purpose and Environment	775

A.4.2.23.2	Test Requirements	776

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	777

A.4.2.24.1	Test Purpose and Environment	777

A.4.2.24.2	Test Requirements	780

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	781

A.4.2.25.1	Test Purpose and Environment	781

A.4.2.25.2	Test Requirements	782

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	783

A.4.2.26.1	Test Purpose and Environment	783

A.4.2.26.2	Test Requirements	784

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	785

A.4.2.27.1	Test Purpose and Environment	785

A.4.2.27.2	Test Requirements	786

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	787

A.4.2.28.1	Test Purpose and Environment	787

A.4.2.28.2	Test Requirements	788

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	789

A.4.2.29.1	Test Purpose and Environment	789

A.4.2.29.2	Test Requirements	790

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	791

A.4.2.30.1	Test Purpose and Environment	791

A.4.2.30.2	Test Requirements	792

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	793

A.4.2.31.1	Test Purpose and Environment	793

A.4.2.31.2	Test Requirements	794

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	795

A.4.2.32.1	Test Purpose and Environment	795

A.4.2.32.2	Test Requirements	796

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	797

A.4.2.33.1	Test Purpose and Environment	797

A.4.2.33.2	Test Requirements	798

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	799

A.4.2.34.1	Test Purpose and Environment	799

A.4.2.34.2	Test Requirements	800

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	801

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	801

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	801

A.4.3.1.1.1	Test Purpose and Environment	801

A.4.3.1.1.2	Test Requirements	803

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	803

A.4.3.1.2.1	Test Purpose and Environment	803

A.4.3.1.2.2	Test Requirements	806

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	806

A.4.3.1.3.1	Test Purpose and Environment	806

A.4.3.1.3.2	Test Requirements	809

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	809

A.4.3.1.4.1	Test Purpose and Environment	809

A.4.3.1.4.2	Test Requirements	810

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	810

A.4.3.1.5.1	Test Purpose and Environment	810

A.4.3.1.5.2	Test Requirements	813

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	814

A.4.3.2.1	Test Purpose and Environment	814

A.4.3.2.1.1	Void	814

A.4.3.2.1.2	1.28Mcps TDD option	814

A.4.3.2.1.3	Void	816

A.4.3.2.2	Test Requirements	816

A.4.3.2.2.1	1.28Mcps TDD option	816

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	816

A.4.3.2A.1	Test Purpose and Environment	816

A.4.3.2A.2	Test Requirements	820

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	820

A.4.3.3.1	Test Purpose and Environment	820

A.4.3.3.2	Test Requirements	823

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	823

A.4.3.3A.1	Test Purpose and Environment	823

A.4.3.3A.2	Test Requirements	827

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	828

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	828

A.4.3.4.1.1	Test Purpose and Environment	828

A.4.3.4.1.2	Test Requirements	830

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	830

A.4.3.4.2.1	Test Purpose and Environment	830

A.4.3.4.2.2	Test Requirements	832

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	832

A.4.3.4.3.1	Test Purpose and Environment	832

A.4.3.4.3.2	Test Requirements	835

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	835

A.4.3.4.4.1	Test Purpose and Environment	835

A.4.3.4.4.2	Test Requirements	839

A.4.4	E-UTRAN to GSM Cell Re-Selection	839

A.4.4.1	E-UTRAN FDD – GSM:	839

A.4.4.1.1	Test Purpose and Environment	839

A.4.4.1.2	Test Requirements	841

A.4.4.2	E-UTRAN TDD – GSM:	841

A.4.4.2.1	Test Purpose and Environment	841

A.4.4.2.2	Test Requirements	843

A.4.5	E-UTRAN to HRPD Cell Re-Selection	844

A.4.5.1	E-UTRAN FDD – HRPD	844

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	844

A.4.5.1.1.1	Test Purpose and Environment	844

A.4.5.1.1.2	Test Requirements	846

A.4.5.2	E-UTRAN TDD – HRPD	846

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	846

A.4.5.2.1.1	Test Purpose and Environment	846

A.4.5.2.1.2	Test Requirements	849

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	849

A.4.6.1	E-UTRAN FDD – cdma2000 1X	849

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	849

A.4.6.1.1.1	Test Purpose and Environment	849

A.4.6.1.1.2	Test Requirements	852

A.4.6.2	E-UTRAN TDD – cdma2000 1X	852

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	852

A.4.6.2.1.1	Test Purpose and Environment	852

A.4.6.2.1.2	Test Requirements	855

A.4.7	Idle State Positioning Measurement for UE category NB1	855

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	855

A.4.7.1.1	Test Purpose and Environment	855

A.4.7.1.2	Test Requirements	859

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	860

A.4.7.2.1	Test Purpose and Environment	860

A.4.7.4.2	Test Requirements	863

A.5	E-UTRAN RRC CONNECTED Mode Mobility	864

A.5.1	E-UTRAN Handover	864

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	864

A.5.1.1.1	Test Purpose and Environment	864

A.5.1.1.2	Test Requirements	866

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	866

A.5.1.2.1	Test Purpose and Environment	866

A.5.1.2.2	Test Requirements	868

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	868

A.5.1.3.1	Test Purpose and Environment	868

A.5.1.3.2	Test Requirements	870

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	870

A.5.1.4.1	Test Purpose and Environment	870

A.5.1.4.2	Test Requirements	872

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	872

A.5.1.5.1	Test Purpose and Environment	872

A.5.1.5.2	Test Requirements	874

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	874

A.5.1.6.1	Test Purpose and Environment	874

A.5.1.6.2	Test Requirements	876

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	876

A.5.1.7.1	Test Purpose and Environment	876

A.5.1.7.2	Test Requirements	879

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	879

A.5.1.8.1	Test Purpose and Environment	879

A.5.1.8.2	Test Requirements	882

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	882

A.5.1.9.1	Test Purpose and Environment	882

A.5.1.9.2	Test Requirements	883

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	883

A.5.1.10.1	Test Purpose and Environment	883

A.5.1.10.2	Test Requirements	885

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	885

A.5.1.11.1	Test Purpose and Environment	885

A.5.1.11.2	Test Requirements	887

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	887

A.5.1.12.1	Test Purpose and Environment	887

A.5.1.12.2	Test Requirements	889

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	889

A.5.1.13.1	Test Purpose and Environment	889

A.5.1.13.2	Test Requirements	891

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	892

A.5.1.14.1	Test Purpose and Environment	892

A.5.1.14.2	Test Requirements	893

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	894

A.5.1.15.1	Test Purpose and Environment	894

A.5.1.15.2	Test Requirements	895

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	896

A.5.1.16.1	Test Purpose and Environment	896

A.5.1.16.2	Test Requirements	897

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	898

A.5.1.17.1	Test Purpose and Environment	898

A.5.1.17.2	Test Requirements	899

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	900

A.5.1.18.1	Test Purpose and Environment	900

A.5.1.18.2	Test Requirements	901

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	902

A.5.1.19.1	Test Purpose and Environment	902

A.5.1.19.2	Test Requirements	903

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	904

A.5.1.20.1	Test Purpose and Environment	904

A.5.1.20.2	Test Requirements	905

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	906

A.5.1.21.1	Test Purpose and Environment	906

A.5.1.21.2	Test Requirements	907

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	907

A.5.1.22.1	Test Purpose and Environment	907

A.5.1.22.2	Test Requirements	909

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	909

A.5.1.23.1	Test Purpose and Environment	909

A.5.1.23.2	Test Requirements	911

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	911

A.5.1.24.1	Test Purpose and Environment	911

A.5.1.24.2	Test Requirements	913

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	913

A.5.1.25.1	Test Purpose and Environment	913

A.5.1.25.2	Test Requirements	915

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	915

A.5.1.26.1	Test Purpose and Environment	915

A.5.1.26.2	Test Requirements	917

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	917

A.5.1.27.1	Test Purpose and Environment	917

A.5.1.27.2	Test Requirements	919

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	920

A.5.1.28.1	Test Purpose and Environment	920

A.5.1.28.2	Test Requirements	921

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	922

A.5.1.29.1	Test Purpose and Environment	922

A.5.1.29.2	Test Requirements	923

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	924

A.5.1.30.1	Test Purpose and Environment	924

A.5.1.30.2	Test Requirements	925

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	926

A.5.1.31.1	Test Purpose and Environment	926

A.5.1.31.2	Test Requirements	927

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	928

A.5.1.32.1	Test Purpose and Environment	928

A.5.1.32.2	Test Requirements	929

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	930

A.5.1.33.1	Test Purpose and Environment	930

A.5.1.13.2	Test Requirements	931

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	932

A.5.1.34.1	Test Purpose and Environment	932

A.5.1.34.2	Test Requirements	933

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	934

A.5.1.35.1	Test Purpose and Environment	934

A.5.1.35.2	Test Requirements	935

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	936

A.5.1.36.1	Test Purpose and Environment	936

A.5.1.36.2	Test Requirements	937

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	938

A.5.1.37.1	Test Purpose and Environment	938

A.5.1.37.2	Test Requirements	939

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	940

A.5.1.38.1	Test Purpose and Environment	940

A.5.1.38.2	Test Requirements	941

A.5.2	E-UTRAN Handover to other RATs	942

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	942

A.5.2.1.1	Test Purpose and Environment	942

A.5.2.1.2	Test Requirements	943

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	944

A.5.2.2.1	Test Purpose and Environment	944

A.5.2.2.2	Test Requirements	947

A.5.2.3 E-UTRAN FDD- GSM Handover	947

A.5.2.3.1	Test Purpose and Environment	947

A.5.2.3.2	Test Requirements	948

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	949

A.5.2.4.1	Test Purpose and Environment	949

A.5.2.4.1.1	Void	949

A.5.2.4.1.2	1.28 Mcps TDD option	949

A.5.2.4.1.3	Void	951

A.5.2.4.2	Test Requirements	951

A.5.2.4.2.1	Void	951

A.5.2.4.2.2	1.28 Mcps TDD option	951

A.5.2.4.2.3	Void	951

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	951

A.5.2.5.1	Test Purpose and Environment	951

A.5.2.5.1.3	Void	954

A.5.2.5.2	Test Requirements	954

A.5.2.5.2.1	Void	954

A.5.2.5.2.2	1.28 Mcps TDD option	954

A.5.2.5.2.3	Void	954

A.5.2.6	E-UTRAN TDD - GSM Handover	954

A.5.2.6.1	Test Purpose and Environment	954

A.5.2.6.2	Test Requirements	956

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	957

A.5.2.7.1	Test Purpose and Environment	957

A.5.2.7.2	Test Requirements	959

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	959

A.5.2.8.1	Test Purpose and Environment	959

A.5.2.8.2	Test Requirements	960

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	961

A.5.2.9.1	Test Purpose and Environment	961

A.5.2.9.2	Test Requirements	962

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	963

A.5.2.10.1	Test Purpose and Environment	963

A.5.2.10.2	Test Requirements	965

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	965

A.5.2.10A.1	Test Purpose and Environment	965

A.5.2.10A.2	Test Requirements	968

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	968

A.5.2.10B.1	Test Purpose and Environment	968

A.5.2.10B.2	Test Requirements	971

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	971

A.5.2.11.1	Test Purpose and Environment	971

A.5.2.11.2	Test Requirements	971

A.5.3	E-UTRAN Handover to Non-3GPP RATs	972

A.5.3.1	E-UTRAN FDD – HRPD Handover	972

A.5.3.1.1	Test Purpose and Environment	972

A.5.3.1.2	Test Requirements	974

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	974

A.5.3.2.1	Test Purpose and Environment	974

A.5.3.2.2	Test Requirements	977

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	977

A.5.3.3.1	Test Purpose and Environment	977

A.5.3.3.2	Test Requirements	979

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	980

A.5.3.4.1	Test Purpose and Environment	980

A.5.3.4.2	Test Requirements	981

A.5.3.5	E-UTRAN TDD – HRPD Handover	982

A.5.3.5.1	Test Purpose and Environment	982

A.5.3.5.2	Test Requirements	984

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	985

A.5.3.6.1	Test Purpose and Environment	985

A.5.3.6.2	Test Requirements	988

A.6	RRC Connection Control	988

A.6.1	RRC Re-establishment	988

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	988

A.6.1.1.1	Test Purpose and Environment	988

A.6.1.1.2	Test Requirements	989

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	990

A.6.1.2.1	Test Purpose and Environment	990

A.6.1.2.2	Test Requirements	991

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	992

A.6.1.3.1	Test Purpose and Environment	992

A.6.1.3.2	Test Requirements	993

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	994

A.6.1.4.1	Test Purpose and Environment	994

A.6.1.4.2	Test Requirements	995

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	996

A.6.1.5.1	Test Purpose and Environment	996

A.6.1.5.2	Test Requirements	996

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	996

A.6.1.6.1	Test Purpose and Environment	996

A.6.1.6.2	Test Requirements	998

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	999

A.6.1.7.1	Test Purpose and Environment	999

A.6.1.7.2	Test Requirements	1000

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1001

A.6.1.8.1	Test Purpose and Environment	1001

A.6.1.8.2	Test Requirements	1002

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1003

A.6.1.9.1	Test Purpose and Environment	1003

A.6.1.9.2	Test Requirements	1004

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1005

A.6.1.10.1	Test Purpose and Environment	1005

A.6.1.10.2	Test Requirements	1006

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1007

A.6.1.11.1	Test Purpose and Environment	1007

A.6.1.11.2	Test Requirements	1008

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1009

A.6.1.12.1	Test Purpose and Environment	1009

A.6.1.12.2	Test Requirements	1010

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1011

A.6.1.13.1	Test Purpose and Environment	1011

A.6.1.13.2	Test Requirements	1012

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1013

A.6.1.14.1	Test Purpose and Environment	1013

A.6.1.14.2	Test Requirements	1014

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1015

A.6.1.15.1	Test Purpose and Environment	1015

A.6.1.15.2	Test Requirements	1017

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1018

A.6.1.16.1	Test Purpose and Environment	1018

A.6.1.16.2	Test Requirements	1020

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1021

A.6.1.17.1	Test Purpose and Environment	1021

A.6.1.17.2	Test Requirements	1022

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1023

A.6.1.18.1	Test Purpose and Environment	1023

A.6.1.18.2	Test Requirements	1024

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1025

A.6.1.19.1	Test Purpose and Environment	1025

A.6.1.19.2	Test Requirements	1026

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1027

A.6.1.20.1	Test Purpose and Environment	1027

A.6.1.20.2	Test Requirements	1028

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1029

A.6.1.21.1	Test Purpose and Environment	1029

A.6.1.21.2	Test Requirements	1030

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1031

A.6.1.22.1	Test Purpose and Environment	1031

A.6.1.22.2	Test Requirements	1032

A.6.2	Random Access	1033

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1033

A.6.2.1.1	Test Purpose and Environment	1033

A.6.2.1.2	Test Requirements	1035

A.6.2.1.2.1	Random Access Response Reception	1035

A.6.2.1.2.2	No Random Access Response Reception	1035

A.6.2.1.2.3	Receiving a NACK on msg3	1035

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1035

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1036

A.6.2.1.2.6	Contention Resolution Timer expiry	1036

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1036

A.6.2.2.1	Test Purpose and Environment	1036

A.6.2.2.2	Test Requirements	1037

A.6.2.2.2.1	Random Access Response Reception	1038

A.6.2.2.2.2	No Random Access Response Reception	1038

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1038

A.6.2.3.1	Test Purpose and Environment	1038

A.6.2.3.2	Test Requirements	1040

A.6.2.3.2.1	Random Access Response Reception	1040

A.6.2.3.2.2	No Random Access Response reception	1040

A.6.2.3.2.3	Receiving a NACK on msg3	1040

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1041

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1041

A.6.2.3.2.6	Contention Resolution Timer expiry	1041

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1041

A.6.2.4.1	Test Purpose and Environment	1041

A.6.2.4.2	Test Requirements	1043

A.6.2.4.2.1	Random Access Response Reception	1043

A.6.2.4.2.2	No Random Access Response Reception	1043

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1043

A.6.2.5.1	Test Purpose and Environment	1043

A.6.2.5.2	Test Requirements	1044

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1044

A.6.2.6.1	Test Purpose and Environment	1044

A.6.2.6.2	Test Requirements	1044

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1045

A.6.2.7.1	Test Purpose and Environment	1045

A.6.2.7.2	Test Requirements	1047

A.6.2.7.2.1	Random Access Response Reception	1047

A.6.2.7.2.2	No Random Access Response Reception	1047

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1047

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1048

A.6.2.8.1	Test Purpose and Environment	1048

A.6.2.8.2	Test Requirements	1050

A.6.2.8.2.1	Random Access Response Reception	1050

A.6.2.8.2.2	No Random Access Response Reception	1050

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1050

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1051

A.6.2.9.1	Test Purpose and Environment	1051

A.6.2.9.2	Test Requirements	1053

A.6.2.9.2.1	Random Access Response Reception	1053

A.6.2.9.2.2	No Random Access Response Reception	1054

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1054

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1055

A.6.2.10.1	Test Purpose and Environment	1055

A.6.2.10.2	Test Requirements	1057

A.6.2.10.2.1	Random Access Response Reception	1057

A.6.2.10.2.2	No Random Access Response Reception	1057

A.6.2.10.2.3	Receiving a NACK on msg3	1058

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1058

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1058

A.6.2.10.2.6	Contention Resolution Timer expiry	1058

A.6.2.10.2.7	PRACH Resource Selection	1058

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1058

A.6.2.11.1	Test Purpose and Environment	1058

A.6.2.11.2	Test Requirements	1060

A.6.2.11.2.1	Random Access Response Reception	1060

A.6.2.11.2.2	No Random Access Response Reception	1061

A.6.2.11.2.3	Receiving a NACK on msg3	1061

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1061

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1061

A.6.2.11.2.6	Contention Resolution Timer expiry	1061

A.6.2.11.2.7	PRACH Resource Selection	1062

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1062

A.6.2.12.1	Test Purpose and Environment	1062

A.6.2.12.2	Test Requirements	1064

A.6.2.12.2.1	Random Access Response Reception	1064

A.6.2.12.2.2	No Random Access Response Reception	1065

A.6.2.12.2.3	Receiving a NACK on msg3	1065

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1065

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1065

A.6.2.12.2.6	Contention Resolution Timer expiry	1065

A.6.2.12.2.7	PRACH Resource Selection	1066

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1066

A.6.2.13.1	Test Purpose and Environment	1066

A.6.2.13.2	Test Requirements	1068

A.6.2.13.2.1	Random Access Response Reception	1068

A.6.2.13.2.2	No Random Access Response Reception	1068

A.6.2.13.2.3	Receiving a NACK on msg3	1069

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1069

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1069

A.6.2.13.2.6	Contention Resolution Timer expiry	1069

A.6.2.13.2.7	PRACH Resource Selection	1069

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1069

A.6.2.14.1	Test Purpose and Environment	1069

A.6.2.14.2	Test Requirements	1071

A.6.2.14.2.1	Random Access Response Reception	1071

A.6.2.14.2.2	No Random Access Response Reception	1072

A.6.2.14.2.3	Receiving a NACK on msg3	1072

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1072

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1072

A.6.2.14.2.6	Contention Resolution Timer expiry	1072

A.6.2.14.2.7	PRACH Resource Selection	1073

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1073

A.6.2.15.1	Test Purpose and Environment	1073

A.6.2.15.2	Test Requirements	1075

A.6.2.15.2.1	Random Access Response Reception	1075

A.6.2.15.2.2	No Random Access Response Reception	1076

A.6.2.15.2.3	Receiving a NACK on msg3	1076

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1076

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1076

A.6.2.15.2.6	Contention Resolution Timer expiry	1076

A.6.2.15.2.7	PRACH Resource Selection	1077

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1077

A.6.2.16.1	Test Purpose and Environment	1077

A.6.2.16.2	Test Requirements	1080

A.6.2.16.2.1	Random Access Response Reception	1080

A.6.2.16.2.2	No Random Access Response Reception	1081

A.6.2.16.2.3	Receiving a NACK on msg3	1081

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1081

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1081

A.6.2.16.2.6	Contention Resolution Timer expiry	1081

A.6.2.16.2.7	NPRACH Resource Selection	1081

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1082

A.6.2.17.1	Test Purpose and Environment	1082

A.6.2.17.2	Test Requirements	1085

A.6.2.17.2.1	Random Access Response Reception	1085

A.6.2.17.2.2	No Random Access Response Reception	1086

A.6.2.17.2.3	Receiving a NACK on msg3	1086

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1086

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1086

A.6.2.17.2.6	Contention Resolution Timer expiry	1086

A.6.2.17.2.7	NPRACH Resource Selection	1086

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1087

A.6.2.18.1	Test Purpose and Environment	1087

A.6.2.18.2	Test Requirements	1090

A.6.2.18.2.1	Random Access Response Reception	1090

A.6.2.18.2.2	No Random Access Response Reception	1091

A.6.2.18.2.3	Receiving a NACK on msg3	1091

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1091

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1091

A.6.2.18.2.6	Contention Resolution Timer expiry	1091

A.6.2.18.2.7	NPRACH Resource Selection	1091

A.6.3	RRC Connection Release with Redirection	1092

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1092

A.6.3.1.1	Test Purpose and Environment	1092

A.6.3.1.2	Test Requirements	1094

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1094

A.6.3.2.1	Test Purpose and Environment	1094

A.6.3.2.2	Test Requirements	1096

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1096

A.6.3.3.1	Test Purpose and Environment	1096

A.6.3.3.2	Test Requirements	1097

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1098

A.6.3.4.1	Test Purpose and Environment	1098

A.6.3.4.2	Test Requirements	1099

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1100

A.6.3.5.1	Test Purpose and Environment	1100

A.6.3.5.2	Test Requirements	1102

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1102

A.6.3.6.1	Test Purpose and Environment	1102

A. 6.3.6.2	Test Requirements	1104

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1104

A.6.3.7.1	Test Purpose and Environment	1104

A.6.3.7.2	Test Requirements	1107

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1107

A.6.3.8.1	Test Purpose and Environment	1107

A.6.3.8.2	Test Requirements	1110

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1110

A.6.3.9.1	Test Purpose and Environment	1110

A.6.3.9.2	Test Requirements	1112

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1112

A.6.3.10.1	Test Purpose and Environment	1112

A.6.3.10.2	Test Requirements	1114

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1114

A.6.3.11.1	Test Purpose and Environment	1114

A.6.3.11.2	Test Requirements	1116

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1116

A.6.3.12.1	Test Purpose and Environment	1116

A.6.3.12.2	Test Requirements	1119

A.7	Timing and Signalling Characteristics	1120

A.7.1	UE Transmit Timing	1120

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1120

A.7.1.1.1	Test Purpose and Environment	1120

A.7.1.1.2	Test Requirements	1121

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1122

A.7.1.2.1	Test Purpose and Environment	1122

A.7.1.2.2	Test Requirements	1124

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1125

A.7.1.3.1	Test Purpose and Environment	1125

A.7.1.3.2	Test Requirements	1127

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1128

A.7.1.4.1	Test Purpose and Environment	1128

A.7.1.4.2	Test Requirements	1130

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1131

A.7.1.4A.1	Test Purpose and Environment	1131

A.7.1.4A.2	Test Requirements	1131

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1131

A.7.1.5.1	Test Purpose and Environment	1131

A.7.1.5.2	Test Requirements	1132

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1132

A.7.1.6.1	Test Purpose and Environment	1132

A.7.1.6.2	Test Requirements	1134

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1135

A.7.1.7.1	Test Purpose and Environment	1135

A.7.1.7.2	Test Requirements	1136

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1137

A.7.1.7A.1	Test Purpose and Environment	1137

A.7.1.7A.2	Test Requirements	1137

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1137

A.7.1.7B.1	Test Purpose and Environment	1137

A.7.1.7B.2	Test Requirements	1137

A.7.1.8	Void	1138

A.7.1.8.1	Void	1138

A.7.1.8.2	Void	1138

A.7.1.9	Void	1138

A.7.1.9.1	Void	1138

A.7.1.9.2	Void	1138

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1138

A.7.1.10.1	Test Purpose and Environment	1138

A.7.1.10.2	Test Requirements	1140

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1140

A.7.1.11.1	Test Purpose and Environment	1140

A.7.1.11.2	Test Requirements	1142

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1142

A.7.1.12.1	Test Purpose and Environment	1142

A.7.1.12.2	Test Requirements	1144

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1144

A.7.1.13.1	Test Purpose and Environment	1144

A.7.1.13.2	Test Requirements	1146

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1147

A.7.1.14.1	Test Purpose and Environment	1147

A.7.1.14.2	Test Requirements	1148

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1148

A.7.1.15.1	Test Purpose and Environment	1148

A.7.1.15.2	Test Requirements	1149

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1150

A.7.1.16.1	Test Purpose and Environment	1150

A.7.1.16.2	Test Requirements	1151

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1152

A.7.1.17.1	Test Purpose and Environment	1152

A.7.1.17.2	Test Requirements	1154

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1155

A.7.1.18.1	Test Purpose and Environment	1155

A.7.1.18.2	Test Requirements	1157

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1158

A.7.1.19.1	Test Purpose and Environment	1158

A.7.1.19.2	Test Requirements	1159

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1159

A.7.1.20.1	Test Purpose and Environment	1159

A.7.1.20.2	Test Requirements	1161

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1161

A.7.1.21.1	Test Purpose and Environment	1161

A.7.1.21.2	Test Requirements	1163

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1163

A.7.1.22.1	Test Purpose and Environment	1163

A.7.1.22.2	Test Requirements	1165

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1165

A.7.1.23.1	Test Purpose and Environment	1165

A.7.1.23.2	Test Requirements	1167

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1167

A.7.1.24.1	Test Purpose and Environment	1167

A.7.1.24.2	Test Requirements	1168

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1169

A.7.1.25.1	Test Purpose and Environment	1169

A.7.1.25.2	Test Requirements	1170

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1171

A.7.1.26.1	Test Purpose and Environment	1171

A.7.1.26.2	Test Requirements	1172

A.7.2	UE Timing Advance	1173

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1173

A.7.2.1.1	Test Purpose and Environment	1173

A.7.2.1.2	Test Requirements	1175

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1175

A.7.2.2.1	Test Purpose and Environment	1175

A.7.2.2.2	Test Requirements	1177

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1177

A.7.2.3.1	Test Purpose and Environment	1177

A.7.2.3.2	Test Requirements	1178

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1178

A.7.2.4.1	Test Purpose and Environment	1178

A.7.2.4.2	Test Requirements	1180

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1180

A.7.2.5.1	Test Purpose and Environment	1180

A.7.2.5.2	Test Requirements	1182

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1182

A.7.2.5A.1	Test Purpose and Environment	1182

A.7.2.5A.2	Test Requirements	1183

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1183

A.7.2.5B.1	Test Purpose and Environment	1183

A.7.2.5B.2	Test Requirements	1183

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1183

A.7.2.6.1	Test Purpose and Environment	1183

A.7.2.6.2	Test Requirements	1186

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1186

A.7.2.7.1	Test Purpose and Environment	1186

A.7.2.7.2	Test Requirements	1188

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1188

A.7.2.8.1	Test Purpose and Environment	1188

A.7.2.8.2	Test Requirements	1190

A.7.2.9.2	Test Requirements	1191

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1192

A.7.2.10.1	Test Purpose and Environment	1192

A.7.2.10.2	Test Requirements	1193

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1193

A.7.2.11.1	Test Purpose and Environment	1193

A.7.2.11.2	Test Requirements	1195

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1195

A.7.2.12.1	Test Purpose and Environment	1195

A.7.2.12.2	Test Requirements	1197

A.7.3	Radio Link Monitoring	1197

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1198

A.7.3.1.1	Test Purpose and Environment	1198

A.7.3.1.2	Test Requirements	1202

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1202

A.7.3.2.1	Test Purpose and Environment	1202

A.7.3.2.2	Test Requirements	1206

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1206

A.7.3.3.1	Test Purpose and Environment	1206

A.7.3.3.2	Test Requirements	1210

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1210

A.7.3.4.1	Test Purpose and Environment	1210

A.7.3.4.2	Test Requirements	1214

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1214

A.7.3.5.1	Test Purpose and Environment	1214

A.7.3.5.2	Test Requirements	1217

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1218

A.7.3.6.1	Test Purpose and Environment	1218

A.7.3.6.2	Test Requirements	1221

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1221

A.7.3.7.1	Test Purpose and Environment	1221

A.7.3.7.2	Test Requirements	1224

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1224

A.7.3.8.1	Test Purpose and Environment	1224

A.7.3.8.2	Test Requirements	1227

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1227

A.7.3.9.2	Test Requirements	1230

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1230

A.7.3.10.1	Test Purpose and Environment	1230

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1234

A.7.3.11.1	Test Purpose and Environment	1234

A.7.3.11.2	Test Requirements	1239

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1239

A.7.3.12.1	Test Purpose and Environment	1239

A.7.3.12.2	Test Requirements	1244

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1244

A.7.3.13.1	Test Purpose and Environment	1244

A.7.3.13.2	Test Requirements	1247

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1247

A.7.3.14.1	Test Purpose and Environment	1247

A.7.3.14.2	Test Requirements	1250

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1250

A.7.3.15.1	Test Purpose and Environment	1250

A.7.3.15.2	Test Requirements	1254

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1254

A.7.3.16.1	Test Purpose and Environment	1254

A.7.3.16.2	Test Requirements	1259

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1259

A.7.3.17.1	Test Purpose and Environment	1259

A.7.3.17.2	Test Requirements	1263

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1263

A.7.3.18.1	Test Purpose and Environment	1263

A.7.3.18.2	Test Requirements	1267

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1267

A.7.3.19.1	Test Purpose and Environment	1267

A.7.3.19.2	Test Requirements	1272

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1272

A.7.3.20.1	Test Purpose and Environment	1272

A.7.3.20.2	Test Requirements	1277

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1277

A.7.3.21.1	Test Purpose and Environment	1277

A.7.3.21.2	Test Requirements	1282

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1282

A.7.3.22.1	Test Purpose and Environment	1282

A.7.3.22.2	Test Requirements	1287

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1287

A.7.3.23.1	Test Purpose and Environment	1287

A.7.3.23.2	Test Requirements	1288

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1288

A.7.3.24.1	Test Purpose and Environment	1288

A.7.3.24.2	Test Requirements	1289

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1289

A.7.3.25.1	Test Purpose and Environment	1289

A.7.3.25.2	Test Requirements	1290

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1290

A.7.3.26.1	Test Purpose and Environment	1290

A.7.3.26.2	Test Requirements	1293

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1293

A.7.3.27.1	Test Purpose and Environment	1293

A.7.3.27.2	Test Requirements	1296

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1296

A.7.3.28.1	Test Purpose and Environment	1296

A.7.3.28.2	Test Requirements	1299

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1299

A.7.3.29.1	Test Purpose and Environment	1299

A.7.3.29.2	Test Requirements	1302

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1302

A.7.3.30.1	Test Purpose and Environment	1302

A.7.3.30.2	Test Requirements	1305

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1305

A.7.3.31.1	Test Purpose and Environment	1305

A.7.3.31.2	Test Requirements	1308

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1308

A.7.3.32.1	Test Purpose and Environment	1308

A.7.3.32.2	Test Requirements	1311

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1311

A.7.3.33.1	Test Purpose and Environment	1311

A.7.3.33.2	Test Requirements	1314

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1314

A.7.3.34.1	Test Purpose and Environment	1314

A.7.3.34.2	Test Requirements	1317

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1317

A.7.3.35.1	Test Purpose and Environment	1317

A.7.3.35.2	Test Requirements	1320

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1320

A.7.3.36.1	Test Purpose and Environment	1320

A.7.3.36.2	Test Requirements	1323

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1323

A.7.3.37.1	Test Purpose and Environment	1323

A.7.3.37.2	Test Requirements	1326

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1326

A.7.3.38.1	Test Purpose and Environment	1326

A.7.3.38.2	Test Requirements	1329

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1329

A.7.3.39.1	Test Purpose and Environment	1329

A.7.3.39.2	Test Requirements	1332

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1333

A.7.3.40.1	Test Purpose and Environment	1333

A.7.3.40.2	Test Requirements	1335

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1336

A.7.3.41.1	Test Purpose and Environment	1336

A.7.3.41.2	Test Requirements	1339

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1339

A.7.3.42.1	Test Purpose and Environment	1339

A.7.3.42.2	Test Requirements	1342

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1342

A.7.3.43.1	Test Purpose and Environment	1342

A.7.3.43.2	Test Requirements	1345

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1345

A.7.3.44.1	Test Purpose and Environment	1345

A.7.3.44.2	Test Requirements	1348

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1349

A.7.3.45.1	Test Purpose and Environment	1349

A.7.3.45.2	Test Requirements	1351

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1352

A.7.3.46.1	Test Purpose and Environment	1352

A.7.3.46.2	Test Requirements	1355

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1355

A.7.3.47.1	Test Purpose and Environment	1355

A.7.3.47.2	Test Requirements	1358

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1358

A.7.3.48.1	Test Purpose and Environment	1358

A.7.3.48.2	Test Requirements	1361

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1361

A.7.3.49.1	Test Purpose and Environment	1361

A.7.3.49.2	Test Requirements	1364

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1364

A.7.3.50.1	Test Purpose and Environment	1364

A.7.3.50.2	Test Requirements	1367

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1367

A.7.3.51.1	Test Purpose and Environment	1367

A.7.3.51.2	Test Requirements	1370

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1370

A.7.3.52.1	Test Purpose and Environment	1370

A.7.3.52.2	Test Requirements	1373

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1373

A.7.3.53.1	Test Purpose and Environment	1373

A.7.3.53.2	Test Requirements	1376

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1376

A.7.3.54.1	Test Purpose and Environment	1376

A.7.3.54.2	Test Requirements	1379

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1379

A.7.3.55.1	Test Purpose and Environment	1379

A.7.3.55.2	Test Requirements	1382

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1382

A.7.3.56.1	Test Purpose and Environment	1382

A.7.3.56.2	Test Requirements	1385

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1385

A.7.3.57.1	Test Purpose and Environment	1385

A.7.3.57.2	Test Requirements	1388

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1388

A.7.3.58.1	Test Purpose and Environment	1388

A.7.3.58.2	Test Requirements	1391

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1391

A.7.3.59.1	Test Purpose and Environment	1391

A.7.3.59.2	Test Requirements	1394

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1394

A.7.3.60.1	Test Purpose and Environment	1394

A.7.3.60.2	Test Requirements	1398

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1398

A.7.3.61.1	Test Purpose and Environment	1398

A.7.3.61.2	Test Requirements	1402

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1402

A.7.3.62.1	Test Purpose and Environment	1402

A.7.3.62.2	Test Requirements	1406

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1406

A.7.3.63.1	Test Purpose and Environment	1406

A.7.3.63.2	Test Requirements	1410

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1410

A.7.3.64.1	Test Purpose and Environment	1410

A.7.3.64.2	Test Requirements	1413

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1414

A.7.3.65.1	Test Purpose and Environment	1414

A.7.3.65.2	Test Requirements	1417

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1418

A.7.3.66.1	Test Purpose and Environment	1418

A.7.3.66.2	Test Requirements	1420

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1421

A.7.3.67.1	Test Purpose and Environment	1421

A.7.3.67.2	Test Requirements	1424

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1425

A.7.3.68.1	Test Purpose and Environment	1425

A.7.3.68.2	Test Requirements	1427

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1427

A.7.3.69.1	Test Purpose and Environment	1427

A.7.3.69.2	Test Requirements	1429

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1429

A.7.3.70.1	Test Purpose and Environment	1429

A.7.3.70.2	Test Requirements	1431

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1431

A.7.3.71.1	Test Purpose and Environment	1431

A.7.3.71.2	Test Requirements	1434

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1434

A.7.3.72.1	Test Purpose and Environment	1434

A.7.3.72.2	Test Requirements	1436

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1436

A.7.3.73.1	Test Purpose and Environment	1436

A.7.3.73.2	Test Requirements	1438

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1438

A.7.3.74.1	Test Purpose and Environment	1438

A.7.3.74.2	Test Requirements	1441

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1441

A.7.3.75.1	Test Purpose and Environment	1441

A.7.3.75.2	Test Requirements	1443

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1444

A.7.3.76.1	Test Purpose and Environment	1444

A.7.3.76.2	Test Requirements	1447

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1447

A.7.3.77.1	Test Purpose and Environment	1447

A.7.3.77.2	Test Requirements	1450

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1450

A.7.3.78.1	Test Purpose and Environment	1450

A.7.3.78.2	Test Requirements	1453

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1453

A.7.3.79.1	Test Purpose and Environment	1453

A.7.3.79.2	Test Requirements	1456

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1456

A.7.3.80.1	Test Purpose and Environment	1456

A.7.3.80.2	Test Requirements	1459

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1459

A.7.3.81.1	Test Purpose and Environment	1459

A.7.3.81.2	Test Requirements	1462

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1462

A.7.3.82.1	Test Purpose and Environment	1462

A.7.3.82.2	Test Requirements	1465

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1465

A.7.3.83.1	Test Purpose and Environment	1465

A.7.3.83.2	Test Requirements	1466

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1467

A.7.3.84.1	Test Purpose and Environment	1467

A.7.3.84.2	Test Requirements	1469

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1469

A.7.3.85.1	Test Purpose and Environment	1469

A.7.3.85.2	Test Requirements	1470

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1471

A.7.3.86.1	Test Purpose and Environment	1471

A.7.3.86.2	Test Requirements	1473

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1473

A.7.3.87.1	Test Purpose and Environment	1473

A.7.3.87.2	Test Requirements	1475

A.7.4	Interruption for Dual Connectivity	1476

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1476

A.7.4.1.1	Test Purpose and Environment	1476

A.7.4.1.2	Test Requirements	1478

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1478

A.7.4.2.1	Test Purpose and Environment	1478

A.7.4.2.2	Test Requirements	1480

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1480

A.7.4.3.1	Test Purpose and Environment	1480

A.7.4.3.2	Test Requirements	1482

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1482

A.7.4.4.1	Test Purpose and Environment	1482

A.7.4.4.2	Test Requirements	1485

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1485

A.7.4.5.1	Test Purpose and Environment	1485

A.7.4.5.2	Test Requirements	1487

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1487

A.7.4.6.1	Test Purpose and Environment	1487

A.7.4.6.2	Test Requirements	1489

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1489

A.7.4.7.1	Test Purpose and Environment	1489

A.7.4.7.2	Test Requirements	1493

A.7.5	Proximity-based Services	1493

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1493

A.7.5.1.1	Test Purpose and Environment	1493

A.7.5.1.2	Test Requirements	1494

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1495

A.7.5.2.1	Test Purpose and Environment	1495

A.7.5.1.2	Test Requirements	1495

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1496

A.7.5.3.1	Test Purpose and Environment	1496

A.7.5.3.2	Test Requirements	1497

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1498

A.7.5.4.1	Test Purpose and Environment	1498

A.7.5.4.2	Test Requirements	1499

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1500

A.7.5.5.1	Test Purpose and Environment	1500

A.7.5.5.2	Test Requirements	1502

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1503

A.7.5.6.1	Test Purpose and Environment	1503

A.7.5.6.2	Test Requirements	1504

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1505

A.7.5.7.1	Test Purpose and Environment	1505

A.7.5.7.2	Test Requirements	1507

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1507

A.7.5.8.1	Test Purpose and Environment	1507

A.7.5.8.2	Test Requirements	1509

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1510

A.7.5.9.1	Test Purpose and Environment	1510

A.7.5.9.2	Test Requirements	1512

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1513

A.7.5.10.1	Test Purpose and Environment	1513

A.7.5.10.2	Test Requirements	1515

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1516

A.7.5.11.1	Test Purpose and Environment	1516

A.7.5.11.2	Test Requirements	1518

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1518

A.7.5.12.1	Test Purpose and Environment	1518

A.7.5.12.2	Test Requirements	1522

A.7.6	Interruption for carrier aggregation	1523

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1523

A.7.6.1.1	Test Purpose and Environment	1523

A.7.6.1.2	Test Requirements	1526

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1526

A.7.6.2.1	Test Purpose and Environment	1526

A.7.6.2.2	Test Requirements	1529

A.8	UE Measurements Procedures	1529

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1529

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1529

A.8.1.1.1	Test Purpose and Environment	1529

A.8.1.1.2	Test Requirements	1531

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1532

A.8.1.2.1	Test Purpose and Environment	1532

A.8.1.2.2	Test Requirements	1533

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1534

A.8.1.3.1	Test Purpose and Environment	1534

A.8.1.3.2	Test Requirements	1536

A.8.1.4	Void	1536

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1536

A.8.1.5.1	Test Purpose and Environment	1536

A.8.1.5.2	Test Requirements	1538

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1538

A.8.1.6.1	Test Purpose and Environment	1538

A.8.1.6.2	Test Requirements	1540

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1540

A.8.1.7.1	Test Purpose and Environment	1540

A.8.1.7.2	Test Requirements	1542

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1543

A.8.1.8.1	Test Purpose and Environment	1543

A.8.1.8.2	Test Requirements	1545

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1546

A.8.1.9.1	Test Purpose and Environment	1546

A.8.1.9.2	Test Requirements	1546

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1546

A.8.1.10.1	Test Purpose and Environment	1546

A.8.1.10.2	Test Requirements	1547

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1547

A.8.1.11.1	Test Purpose and Environment	1547

A.8.1.11.2	Test Requirements	1549

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1550

A.8.1.12.1	Test Purpose and Environment	1550

A.8.1.12.2	Test Requirements	1551

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1552

A.8.1.13.1	Test Purpose and Environment	1552

A.8.1.13.2	Test Requirements	1554

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1554

A.8.1.14.1	Test Purpose and Environment	1554

A.8.1.14.2	Test Requirements	1556

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1557

A.8.1.15.1	Test Purpose and Environment	1557

A.8.1.15.2	Test Requirements	1558

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1559

A.8.1.16.1	Test Purpose and Environment	1559

A.8.1.16.2	Test Requirements	1561

A.8.1.17	Void	1561

A.8.1.18	Void	1561

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1561

A.8.1.19.1	Test Purpose and Environment	1561

A.8.1.19.2	Test Requirements	1563

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1564

A.8.1.20.1	Test Purpose and Environment	1564

A.8.1.20.2	Test Requirements	1566

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1566

A.8.1.21.1	Test Purpose and Environment	1566

A.8.1.21.2	Test Requirements	1568

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1569

A.8.1.22.1	Test Purpose and Environment	1569

A.8.1.22.2	Test Requirements	1571

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1571

A.8.1.23.1	Test Purpose and Environment	1571

A.8.1.23.2	Test Requirements	1572

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1573

A.8.1.24.1	Test Purpose and Environment	1573

A.8.1.24.2	Test Requirements	1575

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1575

A.8.1.25.1	Test Purpose and Environment	1575

A.8.1.25.2	Test Requirements	1577

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1577

A.8.1.26.1	Test Purpose and Environment	1577

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1580

A.8.1.27.1	Test Purpose and Environment	1580

A.8.1.27.2	Test Requirements	1582

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1582

A.8.1.28.1	Test Purpose and Environment	1582

A.8.1.28.2	Test Requirements	1584

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1584

A.8.1.29.1	Test Purpose and Environment	1584

A.8.1.29.2	Test Requirements	1586

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1587

A.8.1.30.1	Test Purpose and Environment	1587

A.8.1.30.2	Test Requirements	1589

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1589

A.8.1.31.1	Test Purpose and Environment	1589

A.8.1.31.2	Test Requirements	1591

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1592

A.8.1.32.1	Test Purpose and Environment	1592

A.8.1.32.2	Test Requirements	1593

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1594

A.8.1.33.1	Test Purpose and Environment	1594

A.8.1.33.2	Test Requirements	1595

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1596

A.8.1.34.1	Test Purpose and Environment	1596

A.8.1.34.2	Test Requirements	1597

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1598

A.8.1.35.1	Test Purpose and Environment	1598

A.8.1.35.2	Test Requirements	1599

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1600

A.8.1.36.1	Test Purpose and Environment	1600

A.8.1.36.2	Test Requirements	1601

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1602

A.8.1.37.1	Test Purpose and Environment	1602

A.8.1.37.2	Test Requirements	1604

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1604

A.8.1.38.1	Test Purpose and Environment	1604

A.8.1.38.2	Test Requirements	1605

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1606

A.8.1.39.1	Test Purpose and Environment	1606

A.8.1.39.2	Test Requirements	1608

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1608

A.8.1.40.1	Test Purpose and Environment	1608

A.8.1.40.2	Test Requirements	1610

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1610

A.8.1.41.1	Test Purpose and Environment	1610

A.8.1.41.2	Test Requirement	1612

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1612

A.8.1.42.1	Test Purpose and Environment	1612

A.8.1.42.2	Test Requirement	1613

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1614

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1614

A.8.2.1.1	Test Purpose and Environment	1614

A.8.2.1.2	Test Requirements	1615

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1616

A.8.2.2.1	Test Purpose and Environment	1616

A.8.2.2.2	Test Requirements	1618

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1618

A.8.2.3.1	Test Purpose and Environment	1618

A.8.2.3.2	Test Requirements	1620

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1621

A.8.2.4.1	Test Purpose and Environment	1621

A.8.2.4.2	Test Requirements	1623

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1623

A.8.2.5.1	Test Purpose and Environment	1623

A.8.2.5.2	Test Requirements	1625

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1626

A.8.2.6.1	Test Purpose and Environment	1626

A.8.2.6.2	Test Requirements	1629

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1630

A.8.2.7.1	Test Purpose and Environment	1630

A.8.2.7.2	Test Requirements	1631

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1632

A.8.2.8.1	Test Purpose and Environment	1632

A.8.2.8.2	Test Requirements	1634

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1634

A.8.2.9.1	Test Purpose and Environment	1634

A.8.2.9.2	Test Requirements	1636

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1637

A.8.2.10.1	Test Purpose and Environment	1637

A.8.2.10.2	Test Requirements	1639

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1639

A.8.2.11.1	Test Purpose and Environment	1639

A.8.2.11.2	Test Requirements	1641

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1641

A.8.2.12.1	Test Purpose and Environment	1641

A.8.2.12.2	Test Requirements	1643

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1644

A.8.2.13.1	Test Purpose and Environment	1644

A.8.2.13.2	Test Requirements	1646

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1646

A.8.2.14.1	Test Purpose and Environment	1646

A.8.2.14.2	Test Requirement	1648

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1649

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1649

A.8.3.1.1	Test Purpose and Environment	1649

A.8.3.1.2	Test Requirements	1650

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1651

A.8.3.2.1	Test Purpose and Environment	1651

A.8.3.2.2	Test Requirements	1653

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1653

A.8.3.3.1	Test Purpose and Environment	1653

A.8.3.3.2	Test Requirements	1656

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1656

A.8.3.4.1	Test Purpose and Environment	1656

A.8.3.4.2	Test Requirements	1658

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1658

A.8.3.5.2	Test Requirements	1660

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1660

A.8.3.6.1	Test Purpose and Environment	1660

A.8.3.6.2	Test Requirements	1662

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1663

A.8.3.7.1	Test Purpose and Environment	1663

A.8.3.7.2	Test Requirements	1665

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1666

A.8.3.8.1	Test Purpose and Environment	1666

A.8.3.8.2	Test Requirements	1669

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1669

A.8.3.9.1	Test Purpose and Environment	1669

A.8.3.9.2	Test Requirements	1673

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1674

A.8.3.10.1	Test Purpose and Environment	1674

A.8.3.10.2	Test Requirements	1675

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1676

A.8.3.11.1	Test Purpose and Environment	1676

A.8.3.11.2	Test Requirement	1677

A.8.3.12	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1678

A.8.3.12.1	Test Purpose and Environment	1678

A.8.3.12.2	Test Requirement	1679

A.8.3.13	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1680

A.8.3.13.1	Test Purpose and Environment	1680

A.8.3.13.2	Test Requirement	1681

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1682

A.8.3.14.1	Test Purpose and Environment	1682

A.8.3.14.2	Test Requirement	1683

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1684

A.8.3.15.1	Test Purpose and Environment	1684

A.8.3.15.2	Test Requirement	1685

A.8.3.16	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1686

A.8.3.16.1	Test Purpose and Environment	1686

A.8.3.16.2	Test Requirement	1688

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1688

A.8.3.17.1	Test Purpose and Environment	1688

A.8.3.17.2	Test Requirement	1691

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1691

A.8.3.18.1	Test Purpose and Environment	1691

A.8.3.18.2	Test Requirement	1694

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1694

A.8.3.19.1	Test Purpose and Environment	1694

A.8.3.19.2	Test Requirement	1697

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1697

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1697

A.8.4.1.1	Test Purpose and Environment	1697

A.8.4.1.2	Test Requirements	1699

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1700

A.8.4.2.1	Test Purpose and Environment	1700

A.8.4.2.2	Test Requirements	1702

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1702

A.8.4.3.1	Test Purpose and Environment	1702

A.8.4.3.2	Test Requirements	1704

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1705

A.8.4.4.1	Test Purpose and Environment	1705

A.8.4.4.2	Test Requirements	1707

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1708

A.8.4.5.1	Test Purpose and Environment	1708

A.8.4.5.2	Test Requirements	1710

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1710

A.8.4.6.1	Test Purpose and Environment	1710

A.8.4.6.2	Test Requirements	1711

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1711

A.8.4.7.1	Test Purpose and Environment	1711

A.8.4.7.2	Test Requirements	1715

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1715

A.8.4.8.1	Test Purpose and Environment	1715

A.8.4.8.2	Test Requirements	1720

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1720

A.8.4.9.1	Test Purpose and Environment	1720

A.8.4.9.2	Test Requirements	1725

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1725

A.8.4.10.1	Test Purpose and Environment	1725

A.8.4.10.2	Test Requirements	1727

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1727

A.8.4.11.1	Test Purpose and Environment	1727

A.8.4.11.2	Test Requirement	1729

A.8.4.12	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1730

A.8.4.12.1	Test Purpose and Environment	1730

A.8.4.12.2	Test Requirement	1731

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1732

A.8.4.13.1	Test Purpose and Environment	1732

A.8.4.13.2	Test Requirement	1733

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1734

A.8.4.14.1	Test Purpose and Environment	1734

A.8.4.14.2	Test Requirement	1736

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1736

A.8.4.15.1	Test Purpose and Environment	1736

A.8.4.15.2	Test Requirement	1739

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1739

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1739

A.8.5.1.1	Test Purpose and Environment	1739

A.8.5.1.2	Test Requirements	1741

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1741

A.8.5.2.1	Test Purpose and Environment	1741

A.8.5.2.2	Test Requirements	1743

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1743

A.8.5.3.1	Test Purpose and Environment	1743

A.8.5.3.2	Test Requirements	1746

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1746

A.8.5.4.1	Test Purpose and Environment	1746

A.8.5.4.2	Test Requirements	1749

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1749

A.8.5.5.1	Test Purpose and Environment	1749

A.8.5.5.2	Test Requirements	1751

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1751

A.8.5.6.1	Test Purpose and Environment	1751

A.8.5.6.2	Test Requirements	1752

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1753

A.8.5.7.1	Test Purpose and Environment	1753

A.8.5.7.2	Test Requirements	1753

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1753

A.8.5.8.1	Test Purpose and Environment	1753

A.8.5.8.2	Test Requirements	1756

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1756

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1756

A.8.6.1.1	Test Purpose and Environment	1756

A.8.6.1.2	Test Requirements	1758

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1759

A.8.6.2.1	Test Purpose and Environment	1759

A.8.6.2.2	Test Requirements	1761

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1761

A.8.6.3.1	Test Purpose and Environment	1761

A.8.6.3.2	Test Requirements	1764

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1764

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1764

A.8.7.1.1	Test Purpose and Environment	1764

A.8.7.1.1.1	Void	1764

A.8.7.1.1.2	1.28 Mcps TDD option	1764

A.8.7.1.1.3	Void	1766

A.8.7.1.2	Test Requirements	1766

A.8.7.1.2.1	Void	1766

A.8.7.1.2.2	1.28 Mcps TDD option	1766

A.8.7.1.2.3	Void	1766

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1766

A.8.7.2.1	Test Purpose and Environment	1766

A.8.7.2.2	Test Requirements	1769

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1770

A.8.7.3.1	Test Purpose and Environment	1770

A.8.7.3.2	Test Parameters	1770

A.8.7.3.3	Test Requirements	1771

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1772

A.8.7.4.1	Test Purpose and Environment	1772

A.8.7.4.2	Test Requirements	1774

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1774

A.8.7.5.1	Test Purpose and Environment	1774

A.8.7.5.2	Test Requirements	1776

A.8.7A	TBD	1776

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1776

A.8.7A.1.1	Test Purpose and Environment	1776

A.8.7A.1.2	Test Requirements	1779

A.8.8	E-UTRAN FDD – GSM Measurements	1779

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1779

A.8.8.1.1	Test Purpose and Environment	1779

A.8.8.1.2	Test Requirements	1781

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1781

A.8.8.2.1	Test Purpose and Environment	1781

A.8.8.2.2	Test Requirements	1783

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1784

A.8.8.3.1	Test Purpose and Environment	1784

A.8.8.3.2	Test Requirements	1785

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1786

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1786

A.8.9.1.1	Test Purpose and Environment	1786

A.8.9.1.2	Test Requirements	1787

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1788

A.8.9.2.1	Test Purpose and Environment	1788

A.8.9.2.2	Test Requirements	1790

A.8.10	E-UTRAN TDD – GSM Measurements	1790

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1790

A.8.10.1.1	Test Purpose and Environment	1790

A.8.10.1.2	Test Requirements	1791

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1792

A.8.10.2.1	Test Purpose and Environment	1792

A.8.10.2.2	Test Requirements	1794

A.8.11	Monitoring of Multiple Layers	1794

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1794

A.8.11.1.1	Test Purpose and Environment	1794

A.8.11.1.2	Test Requirements	1796

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1797

A.8.11.2.1	Test Purpose and Environment	1797

A.8.11.2.2	Test Requirements	1798

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1799

A.8.11.3.1	Test Purpose and Environment	1799

A.8.11.3.2	Test Requirements	1801

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1801

A.8.11.4.1	Test Purpose and Environment	1801

A.8.11.4.2	Test Requirements	1804

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1804

A.8.11.5.1	Test Purpose and Environment	1804

A.8.11.5.2	Test Requirements	1806

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1807

A.8.11.6.1	Test Purpose and Environment	1807

A.8.11.6.2	Test Requirements	1809

A.8.12	RSTD Intra-frequency Measurements	1810

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1810

A.8.12.1.1	Test Purpose and Environment	1810

A.8.12.1.2	Test Requirements	1814

A.8.12.1.2A	Test Requirements for UE Category 1bis	1814

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1814

A.8.12.2.1	Test Purpose and Environment	1814

A.8.12.2.2	Test Requirements	1819

A.8.12.2.2A	Test Requirements for UE Category 1bis	1819

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1819

A.8.12.3.1	Test Purpose and Environment	1819

A.8.12.3.2	Test Requirements	1825

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1825

A.8.12.4.1	Test Purpose and Environment	1825

A.8.12.4.2	Test Requirements	1830

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1830

A.8.12.5.1	Test Purpose and Environment	1830

A.8.12.5.2	Test Requirements	1835

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1835

A.8.12.6.1	Test Purpose and Environment	1835

A.8.12.6.2	Test Requirements	1840

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1840

A.8.12.7.1	Test Purpose and Environment	1840

A.8.12.7.2	Test Requirements	1845

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1845

A.8.12.8.1	Test Purpose and Environment	1845

A.8.12.8.2	Test Requirements	1850

A.8.13	RSTD Inter-frequency Measurements	1850

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1850

A.8.13.1.1	Test Purpose and Environment	1850

A.8.13.1.2	Test Requirements	1855

A.8.13.1.2A	Test Requirements for UE Category 1bis	1855

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1855

A.8.13.2.1	Test Purpose and Environment	1855

A.8.13.2.2	Test Requirements	1861

A.8.13.2.2A	Test Requirements for UE Category 1bis	1861

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1861

A.8.13.3.1	Test Purpose and Environment	1861

A.8.13.3.2	Test Requirements	1868

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1868

A.8.13.4.1	Test Purpose and Environment	1868

A.8.13.4.2	Test Requirements	1874

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1874

A.8.13.5.1	Test Purpose and Environment	1874

A.8.13.5.2	Test Requirements	1880

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1880

A.8.13.6.1	Test Purpose and Environment	1880

A.8.13.6.2	Test Requirements	1886

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1886

A.8.13.7.1	Test Purpose and Environment	1886

A.8.13.7.2	Test Requirements	1892

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1892

A.8.13.8.1	Test Purpose and Environment	1892

A.8.13.8.2	Test Requirements	1898

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1898

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1898

A.8.14.1.1	Test Purpose and Environment	1898

A.8.14.1.2	Test Requirements	1900

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1900

A.8.14.2.1	Test Purpose and Environment	1900

A.8.14.2.2	Test Requirements	1903

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1903

A.8.14.3.1	Test Purpose and Environment	1903

A.8.14.3.2	Test Requirements	1905

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1906

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1906

A.8.15.1.1	Test Purpose and Environment	1906

A.8.15.1.2	Test Requirements	1907

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1907

A.8.15.2.1	Test Purpose and Environment	1907

A.8.15.2.2	Test Requirements	1910

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1910

A.8.15.3.1	Test Purpose and Environment	1910

A.8.15.3.2	Test Requirements	1912

A.8.16	E-UTRAN Carrier Aggregation Measurements	1913

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1913

A.8.16.1.1	Test Purpose and Environment	1913

A.8.16.1.2	Test Requirements	1915

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1915

A.8.16.2.1	Test Purpose and Environment	1915

A.8.16.2.2	Test Requirements	1917

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1917

A.8.16.3.1	Test Purpose and Environment	1917

A.8.16.3.2	Test Requirements	1919

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1920

A.8.16.3A.1	Test Purpose and Environment	1920

A.8.16.3A.2	Test Requirements	1921

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1922

A.8.16.4.1	Test Purpose and Environment	1922

A.8.16.4.2	Test Requirements	1923

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1924

A.8.16.4A.1	Test Purpose and Environment	1924

A.8.16.4A.2	Test Requirements	1926

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1927

A.8.16.5.1	Test Purpose and Environment	1927

A.8.16.5.2	Test Requirements	1927

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1927

A.8.16.6.1	Test Purpose and Environment	1927

A.8.16.6.2	Test Requirements	1928

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1928

A.8.16.7.1	 Test Purpose and Environment	1928

A.8.16.7.2	 Test Requirements	1929

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1929

A.8.16.8.1	 Test Purpose and Environment	1929

A.8.16.8.2	 Test Requirements	1930

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1930

A.8.16.9.1	Test Purpose and Environment	1930

A.8.16.9.2	Test Requirements	1931

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1931

A.8.16.10.1	Test Purpose and Environment	1931

A.8.16.10.2	Test Requirements	1932

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1932

A.8.16.11.1	 Test Purpose and Environment	1932

A.8.16.11.2	 Test Requirements	1933

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1933

A.8.16.12.1	 Test Purpose and Environment	1933

A.8.16.12.2	 Test Requirements	1933

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1933

A.8.16.13.1	Test Purpose and Environment	1933

A.8.16.13.2	Test Requirements	1934

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1934

A.8.16.14.1	Test Purpose and Environment	1934

A.8.16.14.2	Test Requirements	1935

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1935

A.8.16.15.1	 Test Purpose and Environment	1935

A.8.16.7.2	 Test Requirements	1935

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1935

A.8.16.16.1	 Test Purpose and Environment	1935

A.8.16.16.2	 Test Requirements	1936

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1936

A.8.16.17.1	Test Purpose and Environment	1936

A.8.16.17.2	Test Requirements	1938

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1939

A.8.16.17A.1	Test Purpose and Environment	1939

A.8.16.17A.2	Test Requirements	1939

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1939

A.8.16.17B.1	Test Purpose and Environment	1939

A.8.16.17B.2	Test Requirements	1940

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1940

A.8.16.17C.1	Test Purpose and Environment	1940

A.8.16.17C.2	Test Requirements	1940

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1940

A.8.16.18.1	Test Purpose and Environment	1940

A.8.16.18.2	Test Requirements	1942

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1943

A.8.16.18A.1	Test Purpose and Environment	1943

A.8.16.18A.2	Test Requirements	1943

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1943

A.8.16.18B.1	Test Purpose and Environment	1943

A.8.16.18B.2	Test Requirements	1944

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1944

A.8.16.18C.1	Test Purpose and Environment	1944

A.8.16.18C.2	Test Requirements	1944

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1944

A.8.16.18D.1	Test Purpose and Environment	1944

A.8.16.18D.2	Test Requirements	1945

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1945

A.8.16.19.1	Test Purpose and Environment	1945

A.8.16.19.2	Test Requirements	1947

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1947

A.8.16.19A.1	Test Purpose and Environment	1947

A.8.16.19A.2	Test Requirements	1948

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1948

A.8.16.19B.1	Test Purpose and Environment	1948

A.8.16.19B.2	Test Requirements	1948

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1948

A.8.16.19C.1	Test Purpose and Environment	1948

A.8.16.19C.2	Test Requirements	1949

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1949

A.8.16.20.1	Test Purpose and Environment	1949

A.8.16.20.2	Test Requirements	1951

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1952

A.8.16.20A.1	Test Purpose and Environment	1952

A.8.16.20A.2	Test Requirements	1952

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1952

A.8.16.20B.1	Test Purpose and Environment	1952

A.8.16.20B.2	Test Requirements	1953

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1953

A.8.16.20C.1	Test Purpose and Environment	1953

A.8.16.20C.2	Test Requirements	1953

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1953

A.8.16.20D.1	Test Purpose and Environment	1953

A.8.16.20D.2	Test Requirements	1954

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1954

A.8.16.21.1	Test Purpose and Environment	1954

A.8.16.21.2	Test Requirements	1956

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1956

A.8.16.22.1	Test Purpose and Environment	1956

A.8.16.22.2	Test Requirements	1958

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1958

A.8.16.23.1	Test Purpose and Environment	1958

A.8.16.23.2	Test Requirements	1961

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1961

A.8.16.24.1	Test Purpose and Environment	1961

A.8.16.24.2	Test Requirements	1964

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1964

A.8.16.25.1	Test Purpose and Environment	1964

A.8.16.25.2	Test Requirements	1967

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1967

A.8.16.26.1	Test Purpose and Environment	1967

A.8.16.26.2	Test Requirements	1970

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1970

A.8.16.27.1	Test Purpose and Environment	1970

A.8.16.27.2	Test Requirements	1975

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1975

A.8.16.28.1	Test Purpose and Environment	1975

A.8.16.28.2	Test Requirements	1980

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1980

A.8.16.29.1	Test Purpose and Environment	1980

A.8.16.29.2	Test Requirements	1985

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1985

A.8.16.30.1	Test Purpose and Environment	1985

A.8.16.30.2	Test Requirements	1990

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1990

A.8.16.31.1	Test Purpose and Environment	1990

A.8.16.31.2	Test Requirements	1995

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1995

A.8.16.32.1	Test Purpose and Environment	1995

A.8.16.32.2	Test Requirements	2000

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2000

A.8.16.33.1	Test Purpose and Environment	2000

A.8.16.33.2	Test Requirements	2005

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2005

A.8.16.34.1	Test Purpose and Environment	2005

A.8.16.34.2	Test Requirements	2010

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2010

A.8.16.35.1	Test Purpose and Environment	2010

A.8.16.35.2	Test Requirements	2012

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2013

A.8.16.36.1	Test Purpose and Environment	2013

A.8.16.36.2	Test Requirements	2015

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2016

A.8.16.37.1	Test Purpose and Environment	2016

A.8.16.37.2	Test Requirements	2018

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2019

A.8.16.38.1	Test Purpose and Environment	2019

A.8.16.38.2	Test Requirements	2021

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2022

A.8.16.39.1	Test Purpose and Environment	2022

A.8.16.39.2	Test Requirements	2025

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2025

A.8.16.40.1	Test Purpose and Environment	2025

A.8.16.40.2	Test Requirements	2028

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2028

A.8.16.41.1	Test Purpose and Environment	2028

A.8.16.41.2	Test Requirements	2030

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2031

A.8.16.42.1	Test Purpose and Environment	2031

A.8.16.42.2	Test Requirements	2033

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2034

A.8.16.43.1	Test Purpose and Environment	2034

A.8.16.43.2	Test Requirements	2037

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2037

A.8.16.44.1	Test Purpose and Environment	2037

A.8.16.44.2	Test Requirements	2040

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2040

A.8.16.45.1	Test Purpose and Environment	2040

A.8.16.45.2	Test Requirements	2043

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2043

A.8.16.46.1	Test Purpose and Environment	2043

A.8.16.46.2	Test Requirements	2045

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2046

A.8.16.47.1	Test Purpose and Environment	2046

A.8.16.47.2	Test Requirements	2048

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2049

A.8.16.48.1	Test Purpose and Environment	2049

A.8.16.48.2	Test Requirements	2051

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2052

A.8.16.49.1	Test Purpose and Environment	2052

A.8.16.49.2	Test Requirements	2055

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2055

A.8.16.50.1	Test Purpose and Environment	2055

A.8.16.50.2	Test Requirements	2057

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2058

A.8.16.51.1	Test Purpose and Environment	2058

A.8.16.51.2	Test Requirements	2063

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2063

A.8.16.52.1	Test Purpose and Environment	2063

A.8.16.52.2	Test Requirements	2068

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2068

A.8.16.53.1	Test Purpose and Environment	2068

A.8.16.53.2	Test Requirements	2072

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2072

A.8.16.54.1	Test Purpose and Environment	2072

A.8.16.54.2	Test Requirements	2076

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2076

A.8.16.55.1	Test Purpose and Environment	2076

A.8.16.55.2	Test Requirements	2083

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2083

A.8.16.56.1	Test Purpose and Environment	2083

A.8.16.56.2	Test Requirements	2089

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2089

A.8.16.57.1	Test Purpose and Environment	2089

A.8.16.57.2	Test Requirements	2091

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2092

A.8.16.58.1	Test Purpose and Environment	2092

A.8.16.58.2	Test Requirements	2094

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2095

A.8.16.59.1	Test Purpose and Environment	2095

A.8.16.59.2	Test Requirements	2099

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2100

A.8.16.60.1	Test Purpose and Environment	2100

A.8.16.60.2	Test Requirements	2103

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2104

A.8.16.61.1	Test Purpose and Environment	2104

A.8.16.61.2	Test Requirements	2107

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2107

A.8.16.62.1	Test Purpose and Environment	2107

A.8.16.62.2	Test Requirements	2109

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2110

A.8.16.63.1	Test Purpose and Environment	2110

A.8.16.63.2	Test Requirements	2113

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2114

A.8.16.64.1	Test Purpose and Environment	2114

A.8.16.64.2	Test Requirements	2118

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2118

A.8.16.65.1	Test Purpose and Environment	2118

A.8.16.65.2	Test Requirements	2123

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2123

A.8.16.66.1	Test Purpose and Environment	2123

A.8.16.66.2	Test Requirements	2128

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2128

A.8.16.67.1	Test Purpose and Environment	2128

A.8.16.67.2	Test Requirements	2133

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2133

A.8.16.68.1	Test Purpose and Environment	2133

A.8.16.68.2	Test Requirements	2138

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2138

A.8.16.69.1	Test Purpose and Environment	2138

A.8.16.69.2	Test Requirements	2142

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2142

A.8.16.70.1	Test Purpose and Environment	2142

A.8.16.70.2	Test Requirements	2146

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2146

A.8.16.71.1	Test Purpose and Environment	2146

A.8.16.71.2	Test Requirements	2151

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2151

A.8.16.72.1	Test Purpose and Environment	2151

A.8.16.72.2	Test Requirements	2156

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2156

A.8.16.73.1	Test Purpose and Environment	2156

A.8.16.73.2	Test Requirements	2163

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2163

A.8.16.74.1	Test Purpose and Environment	2163

A.8.16.74.2	Test Requirements	2170

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2170

A.8.16.75.1	Test Purpose and Environment	2170

A.8.16.75.2	Test Requirements	2176

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2176

A.8.16.76.1	Test Purpose and Environment	2176

A.8.16.76.2	Test Requirements	2182

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2182

A.8.16.77.1	Test Purpose and Environment	2182

A.8.16.77.2	Test Requirements	2186

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2186

A.8.16.78.1	Test Purpose and Environment	2186

A.8.16.78.2	Test Requirements	2188

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2189

A.8.16.79.1	Test Purpose and Environment	2189

A.8.16.79.2	Test Requirements	2196

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2196

A.8.16.80.1	Test Purpose and Environment	2196

A.8.16.80.2	Test Requirements	2203

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2203

A.8.16.81.1	Test Purpose and Environment	2203

A.8.16.81.2	Test Requirements	2209

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2209

A.8.16.82.1	Test Purpose and Environment	2209

A.8.16.82.2	Test Requirements	2215

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2215

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2215

A.8.17.1.1	Test Purpose and Environment	2215

A.8.17.1.2	Test Requirements	2221

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2221

A.8.17.2.1	Test Purpose and Environment	2221

A.8.17.2.2	Test Requirements	2228

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2228

A.8.17.3.1	Test Purpose and Environment	2228

A.8.17.3.2	Test Requirements	2229

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2229

A.8.17.4.1	Test Purpose and Environment	2229

A.8.17.4.2	Test Requirements	2230

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2230

A.8.17.5.1	Test Purpose and Environment	2230

A.8.17.5.2	Test Requirements	2231

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2231

A.8.17.6.1	Test Purpose and Environment	2231

A.8.17.6.2	Test Requirements	2232

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2233

A.8.17.7.1	Test Purpose and Environment	2233

A.8.17.7.2	Test Requirements	2233

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2234

A.8.17.8.1	Test Purpose and Environment	2234

A.8.17.8.2	Test Requirements	2234

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2235

A.8.17.9.1	Test Purpose and Environment	2235

A.8.17.9.2	Test Requirements	2236

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2236

A.8.17.10.1	Test Purpose and Environment	2236

A.8.17.10.2	Test Requirements	2243

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2244

A.8.17.11.1	Test Purpose and Environment	2244

A.8.17.11.2	Test Requirements	2250

A.8.18	E-UTRAN TDD – HRPD Measurements	2251

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2251

A.8.18.1.1	Test Purpose and Environment	2251

A.8.18.1.2	Test Requirements	2253

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2253

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2253

A.8.19.1.1	Test Purpose and Environment	2253

A.8.19.1.2	Test Requirements	2254

A.8.20	Inter-frequency/RAT Measurements in CA mode	2255

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2255

A.8.20.1.1	Test Purpose and Environment	2255

A.8.20.1.2	Test Requirements	2256

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2257

A.8.20.2.1	Test Purpose and Environment	2257

A.8.20.2.2	Test Requirements	2258

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2259

A.8.20.2A.1	Test Purpose and Environment	2259

A.8.20.2A.2	Test Requirements	2259

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2259

A.8.20.2B.1	Test Purpose and Environment	2259

A.8.20.2B.2	Test Requirements	2262

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2262

A.8.20.3.1	Test Purpose and Environment	2262

A.8.20.3.2	Test Requirements	2264

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2264

A.8.20.4.1	Test Purpose and Environment	2264

A.8.20.4.1.1	1.28 Mcps TDD option	2264

A.8.20.4.2	Test Requirements	2266

A.8.20.4.2.1	1.28 Mcps TDD option	2266

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2266

A.8.20.4A.1	Test Purpose and Environment	2266

A.8.20.4A.1.1	1.28 Mcps TDD option	2266

A.8.20.4A.2	Test Requirements	2266

A.8.20.4A.2.1	1.28 Mcps TDD option	2266

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2267

A.8.20.4B.1	Test Purpose and Environment	2267

A.8.20.4B.1.1	1.28 Mcps TDD option	2267

A.8.20.4B.2	Test Requirements	2269

A.8.20.4B.2.1	1.28 Mcps TDD option	2269

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2269

A.8.21.1	Test Purpose and Environment	2269

A.8.21.2 Test Requirements	2273

A.8.22	E-UTRAN Discovery Signal Measurements	2273

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2273

A.8.22.1.1	Test Purpose and Environment	2273

A.8.22.1.2	Test Requirements	2276

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2276

A.8.22.2.1	Test Purpose and Environment	2276

A.8.22.2.2	Test Requirements	2279

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2279

A.8.22.3.1	Test Purpose and Environment	2279

A.8.22.3.2	Test Requirements	2282

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2282

A.8.22.4.1	Test Purpose and Environment	2282

A.8.22.4.2	Test Requirements	2285

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2285

A.8.22.5.1	Test Purpose and Environment	2285

A.8.22.5.2	Test Requirements	2289

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2289

A.8.22.6.1	Test Purpose and Environment	2289

A.8.22.6.2	Test Requirements	2293

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2293

A.8.22.7.1	Test Purpose and Environment	2293

A.8.22.7.2	Test Requirements	2297

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2297

A.8.22.8.1	Test Purpose and Environment	2297

A.8.22.8.2	Test Requirements	2301

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2301

A.8.22.9.1	Test Purpose and Environment	2301

A.8.22.9.2	Test Requirements	2303

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2304

A.8.22.10.1	Test Purpose and Environment	2304

A.8.22.10.2	Test Requirements	2306

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2307

A.8.22.11.1	Test Purpose and Environment	2307

A.8.22.11.2	Test Requirements	2310

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2310

A.8.22.12.1	Test Purpose and Environment	2310

A.8.22.12.2	Test Requirements	2314

A.8.23	E-UTRAN Dual Connectivity Measurements	2314

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2314

A.8.23.1.1	Test Purpose and Environment	2314

A.8.23.1.2	Test Requirements	2317

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2317

A.8.23.2.1	Test Purpose and Environment	2317

A.8.23.2.2	Test Requirements	2320

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2320

A.8.23.3.1	Test Purpose and Environment	2320

A.8.23.3.2	Test Requirements	2323

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2323

A.8.23.4.1	Test Purpose and Environment	2323

A.8.23.4.2	Test Requirements	2326

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2326

A.8.23.5.1	Test Purpose and Environment	2326

A.8.23.5.2	Test Requirements	2329

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2329

A.8.23.6.1	Test Purpose and Environment	2329

A.8.23.6.2	Test Requirements	2332

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2332

A.8.23.7.1	Test Purpose and Environment	2332

A.8.23.7.2	Test Requirements	2334

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2335

A.8.23.8.1	Test Purpose and Environment	2335

A.8.23.8.2	Test Requirements	2337

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2338

A.8.23.9.1	Test Purpose and Environment	2338

A.8.23.9.2	Test Requirements	2341

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2341

A.8.23.10.1	Test Purpose and Environment	2341

A.8.23.10.2	Test Requirements	2344

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2344

A.8.23.11.1	Test Purpose and Environment	2344

A.8.23.11.2	Test Requirements	2347

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2347

A.8.23.12.1	Test Purpose and Environment	2347

A.8.23.12.2	Test Requirements	2350

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2350

A.8.23.13.1	Test Purpose and Environment	2350

A.8.23.13.2	Test Requirements	2353

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2353

A.8.23.14.1	Test Purpose and Environment	2353

A.8.23.14.2	Test Requirements	2356

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2356

A.8.23.15.1	Test Purpose and Environment	2356

A.8.23.15.2	Test Requirements	2359

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2359

A.8.23.16.1	Test Purpose and Environment	2359

A.8.23.16.2	Test Requirements	2360

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2361

A.8.23.17.1	Test Purpose and Environment	2361

A.8.23.17.2	Test Requirements	2363

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2363

A.8.23.18.1	Test Purpose and Environment	2363

A.8.23.18.2	Test Requirements	2364

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2365

A.8.23.19.1	Test Purpose and Environment	2365

A.8.23.19.2	Test Requirements	2366

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2367

A.8.23.20.1	Test Purpose and Environment	2367

A.8.23.20.2	Test Requirements	2368

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2369

A.8.23.21.1	Test Purpose and Environment	2369

A.8.23.21.2	Test Requirements	2370

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2371

A.8.23.22.1	Test Purpose and Environment	2371

A.8.23.22.2	Test Requirements	2373

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2373

A.8.23.23.1	Test Purpose and Environment	2373

A.8.23.23.2	Test Requirements	2375

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2376

A.8.23.24.1	Test Purpose and Environment	2376

A.8.23.24.2	Test Requirements	2378

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2379

A.8.23.25.1	Test Purpose and Environment	2379

A.8.23.25.2	Test Requirements	2381

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2382

A.8.23.26.1	Test Purpose and Environment	2382

A.8.23.26.2	Test Requirements	2384

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2385

A.8.23.27.1	Test Purpose and Environment	2385

A.8.23.27.2	Test Requirements	2388

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2388

A.8.23.28.1	Test Purpose and Environment	2388

A.8.23.28.2	Test Requirements	2392

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2392

A.8.23.29.1	Test Purpose and Environment	2392

A.8.23.29.2	Test Requirements	2396

A.8.24	Proximity-based Services	2396

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2396

A.8.24.1.1	Test Purpose and Environment	2396

A.8.24.1.2	Test Requirements	2397

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2398

A.8.24.2.1	Test Purpose and Environment	2398

A.8.24.2.2	Test Requirements	2399

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2399

A.8.24.3.1	Test Purpose and Environment	2400

A.8.24.3.2	Test Requirements	2401

A.8.25	E-UTRAN-WLAN Measurements	2402

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2402

A.8.25.1.1	Test Purpose and Environment	2402

A.8.25.1.2	Test Requirements	2404

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2404

A.8.25.2.1	Test Purpose and Environment	2404

A.8.25.2.2	Test Requirements	2406

A.8.26 Frame Structure 3 (FS3)	2406

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2406

A.8.26.1.1	Test Purpose and Environment	2406

A.8.26.1.2	Test Requirements	2408

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2409

A.8.26.2.1	Test Purpose and Environment	2409

A.8.26.2.2	Test Requirements	2411

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2412

A.8.26.3.1	Test Purpose and Environment	2412

A.8.26.3.2	Test Requirements	2415

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2415

A.8.26.3A.1	Test Purpose and Environment	2415

A.8.26.3A.2	Test Requirements	2419

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2419

A.8.26.4.1	Test Purpose and Environment	2419

A.8.26.4.2	Test Requirements	2423

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2423

A.8.26.4A.1	Test Purpose and Environment	2423

A.8.26.4A.2	Test Requirements	2427

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2427

A.8.26.5.1	Test Purpose and Environment	2427

A.8.26.5.2	Test Requirements	2431

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2431

A.8.26.5A.1	Test Purpose and Environment	2431

A.8.26.5A.2	Test Requirements	2435

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2435

A.8.26.6.1	Test Purpose and Environment	2435

A.8.26.6.2	Test Requirements	2439

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2439

A.8.26.6A.1	Test Purpose and Environment	2439

A.8.26.6A.2	Test Requirements	2443

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2443

A.8.26.7.1	Test Purpose and Environment	2443

A.8.26.7.2	Test Requirements	2446

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2446

A.8.26.8.1	Test Purpose and Environment	2446

A.8.26.8.2	Test Requirements	2450

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2450

A.8.26.9.1	Test Purpose and Environment	2450

A.8.26.9.2	Test Requirements	2453

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2453

A.8.26.10.1	Test Purpose and Environment	2453

A.8.26.10.2	Test Requirements	2456

A.9	Measurement Performance Requirements	2457

A.9.1	RSRP	2457

A.9.1.1	FDD Intra frequency case	2457

A.9.1.1.1	Test Purpose and Environment	2457

A.9.1.1.2	Test parameters	2457

A.9.1.1.3	Test Requirements	2459

A.9.1.2	TDD Intra frequency case	2459

A.9.1.2.1	Test Purpose and Environment	2459

A.9.1.2.2	Test parameters	2459

A.9.1.2.3	Test Requirements	2460

A.9.1.3	FDD—FDD Inter frequency case	2461

A.9.1.3.1	Test Purpose and Environment	2461

A.9.1.3.2	Test parameters	2461

A.9.1.3.3	Test Requirements	2463

A.9.1.4	TDD—TDD Inter frequency case	2463

A.9.1.4.1	Test Purpose and Environment	2463

A.9.1.4.2	Test parameters	2463

A.9.1.4.3	Test Requirements	2465

A.9.1.5	FDD—TDD Inter frequency case	2466

A.9.1.5.1	Test Purpose and Environment	2466

A.9.1.5.2	Test parameters	2466

A.9.1.5.3	Test Requirements	2467

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2468

A.9.1.6.1	Test Purpose and Environment	2468

A.9.1.6.2	Test parameters	2468

A.9.1.6.3	Test Requirements	2470

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2470

A.9.1.7.1	Test Purpose and Environment	2470

A.9.1.7.2	Test parameters	2470

A.9.1.7.3	Test Requirements	2472

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2472

A.9.1.8.1	Test Purpose and Environment	2472

A.9.1.8.2	Test parameters	2472

A.9.1.8.3	Test Requirements	2475

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2475

A.9.1.9.1	Test Purpose and Environment	2475

A.9.1.9.2	Test parameters	2475

A.9.1.9.3	Test Requirements	2478

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2478

A.9.1.10.1	Test Purpose and Environment	2478

A.9.1.10.2	Test parameters	2478

A.9.1.10.3	Test Requirements	2480

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2480

A.9.1.11.1	Test Purpose and Environment	2480

A.9.1.11.2	Test parameters	2480

A.9.1.11.3	Test Requirements	2483

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2483

A.9.1.12.1	Test Purpose and Environment	2483

A.9.1.12.2	Test parameters	2483

A.9.1.12.3	Test Requirements	2484

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2484

A.9.1.13.1	Test Purpose and Environment	2484

A.9.1.13.2	Test parameters	2484

A.9.1.13.3	Test Requirements	2485

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2485

A.9.1.14.1	Test Purpose and Environment	2485

A.9.1.14.2	Test parameters	2485

A.9.1.14.3	Test Requirements	2488

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2488

A.9.1.15.1	Test Purpose and Environment	2488

A.9.1.15.2	Test parameters	2488

A.9.1.15.3	Test Requirements	2490

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2491

A.9.1.16.1	Test Purpose and Environment	2491

A.9.1.16.2	Test parameters	2491

A.9.1.16.3	Test Requirements	2492

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2492

A.9.1.17.1	Test Purpose and Environment	2492

A.9.1.17.2	Test parameters	2493

A.9.1.17.3	Test Requirements	2494

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2494

A.9.1.18.1	Test Purpose and Environment	2494

A.9.1.18.2	Test parameters	2494

A.9.1.18.3	Test Requirements	2495

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2495

A.9.1.19.1	Test Purpose and Environment	2496

A.9.1.19.2	Test parameters	2496

A.9.1.19.3	Test Requirements	2496

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2496

A.9.1.20.1	Test Purpose and Environment	2496

A.9.1.20.2	Test parameters	2496

A.9.1.20.3	Test Requirements	2497

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2498

A.9.1.21.1	Test Purpose and Environment	2498

A.9.1.21.2	Test parameters	2498

A.9.1.21.3	Test Requirements	2498

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2498

A.9.1.22.1	Test Purpose and Environment	2498

A.9.1.22.2	Test parameters	2499

A.9.1.22.3	Test Requirements	2502

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2502

A.9.1.23.1	Test Purpose and Environment	2502

A.9.1.23.2	Test parameters	2502

A.9.1.23.3	Test Requirements	2504

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2505

A.9.1.24.1	Test Purpose and Environment	2505

A.9.1.24.2	Test parameters	2505

A.9.1.24.3	Test Requirements	2505

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2506

A.9.1.25.1	Test Purpose and Environment	2506

A.9.1.25.2	Test parameters	2506

A.9.1.25.3	Test Requirements	2508

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2508

A.9.1.26.1	Test Purpose and Environment	2508

A.9.1.26.2	Test parameters	2508

A.9.1.26.3	Test Requirements	2510

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2510

A.9.1.27.1	Test Purpose and Environment	2510

A.9.1.27.2	Test parameters	2510

A.9.1.27.3	Test Requirements	2512

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2512

A.9.1.28.1	Test Purpose and Environment	2512

A.9.1.28.2	Test parameters	2512

A.9.1.28.3	Test Requirements	2514

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2514

A.9.1.29.1	Test Purpose and Environment	2514

A.9.1.29.2	Test parameters	2514

A.9.1.29.3	Test Requirements	2516

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2516

A.9.1.30.1	Test Purpose and Environment	2516

A.9.1.30.2	Test parameters	2516

A.9.1.30.3	Test Requirements	2518

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2518

A.9.1.31.1	Test Purpose and Environment	2518

A.9.1.31.2	Test parameters	2518

A.9.1.31.3	Test Requirements	2520

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2520

A.9.1.32.1	Test Purpose and Environment	2520

A.9.1.32.2	Test parameters	2521

A.9.1.32.3	Test Requirements	2523

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2523

A.9.1.33.1	Test Purpose and Environment	2523

A.9.1.33.2	Test parameters	2523

A.9.1.33.3	Test Requirements	2525

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2525

A.9.1.34.1	Test Purpose and Environment	2525

A.9.1.34.2	Test parameters	2525

A.9.1.34.3	Test Requirements	2527

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2527

A.9.1.35.1	Test Purpose and Environment	2527

A.9.1.35.2	Test parameters	2527

A.9.1.35.3	Test Requirements	2529

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2530

A.9.1.36.1	Test Purpose and Environment	2530

A.9.1.36.2	Test parameters	2530

A.9.1.36.3	Test Requirements	2532

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2532

A.9.1.37.1	Test Purpose and Environment	2532

A.9.1.37.2	Test parameters	2532

A.9.1.37.3	Test Requirements	2534

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2535

A.9.1.38.1	Test Purpose and Environment	2535

A.9.1.38.2	Test parameters	2535

A.9.1.38.3	Test Requirements	2537

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2538

A.9.1.39.1	Test Purpose and Environment	2538

A.9.1.39.2	Test parameters	2538

A.9.1.39.3	Test Requirements	2542

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2542

A.9.1.40.1	Test Purpose and Environment	2542

A.9.1.40.2	Test parameters	2542

A.9.1.40.3	Test Requirements	2547

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2547

A.9.1.41.1	Test Purpose and Environment	2547

A.9.1.41.2	Test parameters	2547

A.9.1.41.3	Test Requirements	2549

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2549

A.9.1.42.1	Test Purpose and Environment	2549

A.9.1.42.2	Test parameters	2549

A.9.1.42.3	Test Requirements	2551

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2551

A.9.1.43.1	Test Purpose and Environment	2551

A.9.1.43.2	Test parameters	2551

A.9.1.43.3	Test Requirements	2552

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2553

A.9.1.44.1	Test Purpose and Environment	2553

A.9.1.44.2	Test parameters	2553

A.9.1.44.3	Test Requirements	2557

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2557

A.9.1.45.1	Test Purpose and Environment	2557

A.9.1.45.2	Test parameters	2557

A.9.1.45.3	Test Requirements	2561

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2561

A.9.1.46.1	Test Purpose and Environment	2561

A.9.1.46.2	Test parameters	2561

A.9.1.46.3	Test Requirements	2567

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2567

A.9.1.47.1	Test Purpose and Environment	2567

A.9.1.47.2	Test parameters	2568

A.9.1.47.3	Test Requirements	2574

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2574

A.9.1.48.1	Test Purpose and Environment	2574

A.9.1.48.2	Test parameters	2575

A.9.1.48.3	Test Requirements	2582

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2583

A.9.1.49.1	Test Purpose and Environment	2583

A.9.1.49.2	Test parameters	2583

A.9.1.49.3	Test Requirements	2590

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2591

A.9.1.50.1	Test Purpose and Environment	2591

A.9.1.50.2	Test parameters	2591

A.9.1.50.3	Test Requirements	2596

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2596

A.9.1.51.1	Test Purpose and Environment	2596

A.9.1.51.2	Test parameters	2596

A.9.1.51.3	Test Requirements	2600

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2601

A.9.1.52.1	Test Purpose and Environment	2601

A.9.1.52.2	Test parameters	2601

A.9.1.52.3	Test Requirements	2603

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2603

A.9.1.52A.1	Test Purpose and Environment	2603

A.9.1.52A.2	Test parameters	2603

A.9.1.52A.3	Test Requirements	2604

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2604

A.9.1.53.1	Test Purpose and Environment	2604

A.9.1.53.2	Test parameters	2605

A.9.1.53.3	Test Requirements	2607

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2607

A.9.1.53A.1	Test Purpose and Environment	2607

A.9.1.53A.2	Test parameters	2607

A.9.1.53A.3	Test Requirements	2608

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2609

A.9.1.54.1	Test Purpose and Environment	2609

A.9.1.54.2	Test parameters	2609

A.9.1.54.3	Test Requirements	2611

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2611

A.9.1.55.1	Test Purpose and Environment	2611

A.9.1.55.2	Test parameters	2611

A.9.1.55.3	Test Requirements	2614

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2614

A.9.1.56.1	Test Purpose and Environment	2614

A.9.1.56.2	Test parameters	2614

A.9.1.56.3	Test Requirements	2616

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2617

A.9.1.57.1	Test Purpose and Environment	2617

A.9.1.57.2	Test parameters	2617

A.9.1.57.3	Test Requirements	2619

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2619

A.9.1.57A.1	Test Purpose and Environment	2619

A.9.1.57A.2	Test parameters	2619

A.9.1.57A.3	Test Requirements	2620

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2620

A.9.1.58.1	Test Purpose and Environment	2620

A.9.1.58.2	Test parameters	2621

A.9.1.58.3	Test Requirements	2623

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2623

A.9.1.58A.1	Test Purpose and Environment	2623

A.9.1.58A.2	Test parameters	2623

A.9.1.58A.3	Test Requirements	2624

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2624

A.9.1.59.1	Test Purpose and Environment	2624

A.9.1.59.2	Test parameters	2625

A.9.1.59.3	Test Requirements	2627

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2627

A.9.1.60.1	Test Purpose and Environment	2627

A.9.1.60.2	Test parameters	2627

A.9.1.60.3	Test Requirements	2629

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2630

A.9.1.61.1	Test Purpose and Environment	2630

A.9.1.61.2	Test parameters	2630

A.9.1.61.3	Test Requirements	2632

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2632

A.9.1.62.1	Test Purpose and Environment	2632

A.9.1.62.2	Test parameters	2632

A.9.1.62.3	Test Requirements	2634

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2634

A.9.1.63.1	Test Purpose and Environment	2634

A.9.1.63.2	Test parameters	2634

A.9.1.63.3	Test Requirements	2636

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2636

A.9.1.64.1	Test Purpose and Environment	2636

A.9.1.64.2	Test parameters	2636

A.9.1.64.3	Test Requirements	2637

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2638

A.9.1.65.1	Test Purpose and Environment	2638

A.9.1.65.2	Test parameters	2638

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2642

A.9.1.66.1	Test Purpose and Environment	2642

A.9.1.66.2	Test parameters	2642

A.9.1.66.3	Test Requirements	2646

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2646

A.9.1.67.1	Test Purpose and Environment	2646

A.9.1.67.2	Test parameters	2646

A.9.1.67.3	Test Requirements	2647

A.9.2	RSRQ	2648

A.9.2.1	FDD Intra frequency case	2648

A.9.2.1.1	Test Purpose and Environment	2648

A.9.2.1.2	Test parameters	2648

A.9.2.1.3	Test Requirements	2650

A.9.2.2	TDD Intra frequency case	2650

A.9.2.2.1	Test Purpose and Environment	2650

A.9.2.2.2	Test parameters	2650

A.9.2.2.3	Test Requirements	2652

A.9.2.3	FDD—FDD Inter frequency case	2652

A.9.2.3.1	Test Purpose and Environment	2652

A.9.2.3.2	Test parameters	2652

A.9.2.3.3	Test Requirements	2654

A.9.2.4	TDD—TDD Inter frequency case	2654

A.9.2.4.1	Test Purpose and Environment	2654

A.9.2.4.2	Test parameters	2654

A.9.2.4.3	Test Requirements	2657

A.9.2.4A	FDD—TDD Inter frequency case	2657

A.9.2.4A.1	Test Purpose and Environment	2657

A.9.2.4A.2	Test parameters	2657

A.9.2.4A.3	Test Requirements	2659

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2660

A.9.2.5.1	Test Purpose and Environment	2660

A.9.2.5.2	Test parameters	2660

A.9.2.5.3	Test Requirements	2662

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2662

A.9.2.6.1	Test Purpose and Environment	2662

A.9.2.6.2	Test parameters	2662

A.9.2.6.3	Test Requirements	2664

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2664

A.9.2.7.1	Test Purpose and Environment	2664

A.9.2.7.2	Test parameters	2664

A.9.2.7.3	Test Requirements	2667

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2667

A.9.2.8.1	Test Purpose and Environment	2667

A.9.2.8.2	Test parameters	2667

A.9.2.8.3	Test Requirements	2670

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2670

A.9.2.9.1	Test Purpose and Environment	2670

A.9.2.9.2	Test parameters	2670

A.9.2.9.3	Test Requirements	2673

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2673

A.9.2.10.1	Test Purpose and Environment	2673

A.9.2.10.2	Test parameters	2673

A.9.2.10.3	Test Requirements	2676

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2676

A.9.2.11.1	Test Purpose and Environment	2676

A.9.2.11.2	Test parameters	2676

A.9.2.11.3	Test Requirements	2677

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2677

A.9.2.12.1	Test Purpose and Environment	2677

A.9.2.12.2	Test parameters	2677

A.9.2.12.3	Test Requirements	2678

A.9.2.13	Void	2679

A.9.2.13.1	Void	2679

A.9.2.13.2	Void	2679

A.9.2.13.3	Void	2679

A.9.2.14	Void	2679

A.9.2.14.1	Void	2679

A.9.2.14.2	Void	2679

A.9.2.14.3	Void	2679

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2679

A.9.2.15.1	Test Purpose and Environment	2679

A.9.2.15.2	Test parameters	2679

A.9.2.15.3	Test Requirements	2682

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2682

A.9.2.16.1	Test Purpose and Environment	2682

A.9.2.16.2	Test parameters	2682

A.9.2.16.3	Test Requirements	2685

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2685

A.9.2.17.1	Test Purpose and Environment	2685

A.9.2.17.2	Test parameters	2685

A.9.2.17.3	Test Requirements	2687

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2687

A.9.2.18.1	Test Purpose and Environment	2687

A.9.2.18.2	Test parameters	2687

A.9.2.18.3	Test Requirements	2689

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2689

A.9.2.19.1	Test Purpose and Environment	2689

A.9.2.19.2	Test parameters	2689

A.9.2.19.3	Test Requirements	2691

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2691

A.9.2.20.1	Test Purpose and Environment	2691

A.9.2.20.2	Test parameters	2691

A.9.2.20.3	Test Requirements	2693

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2693

A.9.2.21.1	Test Purpose and Environment	2693

A.9.2.21.2	Test parameters	2693

A.9.2.21.3	Test Requirements	2695

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2695

A.9.2.22.1	Test Purpose and Environment	2695

A.9.2.22.2	Test parameters	2695

A.9.2.22.3	Test Requirements	2696

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2696

A.9.2.23.1	Test Purpose and Environment	2696

A.9.2.23.2	Test parameters	2696

A.9.2.23.3	Test Requirements	2698

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2698

A.9.2.24.1	Test Purpose and Environment	2698

A.9.2.24.2	Test parameters	2698

A.9.2.24.3	Test Requirements	2698

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2699

A.9.2.25.1	Test Purpose and Environment	2699

A.9.2.25.2	Test parameters	2699

A.9.2.25.3	Test Requirements	2702

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2702

A.9.2.26.1	Test Purpose and Environment	2702

A.9.2.26.2	Test parameters	2702

A.9.2.26.3	Test Requirements	2705

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2705

A.9.2.27.1	Test Purpose and Environment	2705

A.9.2.27.2	Test parameters	2705

A.9.2.27.3	Test Requirements	2706

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2706

A.9.2.28.1	Test Purpose and Environment	2706

A.9.2.28.2	Test parameters	2706

A.9.2.28.3	Test Requirements	2708

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2708

A.9.2.29.1	Test Purpose and Environment	2708

A.9.2.29.2	Test parameters	2708

A.9.2.29.3	Test Requirements	2710

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2710

A.9.2.30.1	Test Purpose and Environment	2710

A.9.2.30.2	Test parameters	2710

A.9.2.30.3	Test Requirements	2712

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2712

A.9.2.31.1	Test Purpose and Environment	2712

A.9.2.31.2	Test parameters	2712

A.9.2.31.3	Test Requirements	2714

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2714

A.9.2.32.1	Test Purpose and Environment	2714

A.9.2.32.2	Test parameters	2714

A.9.2.32.3	Test Requirements	2716

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2716

A.9.2.33.1	Test Purpose and Environment	2716

A.9.2.33.2	Test parameters	2716

A.9.2.33.3	Test Requirements	2718

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2718

A.9.2.34.1	Test Purpose and Environment	2718

A.9.2.34.2	Test parameters	2718

A.9.2.34.3	Test Requirements	2720

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2720

A.9.2.35.1	Test Purpose and Environment	2720

A.9.2.35.2	Test parameters	2720

A.9.2.35.3	Test Requirements	2722

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2722

A.9.2.36.1	Test Purpose and Environment	2722

A.9.2.36.2	Test parameters	2722

A.9.2.36.3	Test Requirements	2723

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2724

A.9.2.37.1	Test Purpose and Environment	2724

A.9.2.37.2	Test parameters	2724

A.9.2.37.3	Test Requirements	2725

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2726

A.9.2.38.1	Test Purpose and Environment	2726

A.9.2.38.2	Test parameters	2726

A.9.2.38.3	Test Requirements	2729

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.39.1	Test Purpose and Environment	2729

A.9.2.39.2	Test parameters	2729

A.9.2.39.3	Test Requirements	2732

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2732

A.9.2.40.1	Test Purpose and Environment	2732

A.9.2.40.2	Test parameters	2732

A.9.2.40.3	Test Requirements	2734

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2734

A.9.2.41.1	Test Purpose and Environment	2734

A.9.2.41.2	Test parameters	2735

A.9.2.41.3	Test Requirements	2737

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2737

A.9.2.42.1	Test Purpose and Environment	2737

A.9.2.42.2	Test parameters	2737

A.9.2.42.3	Test Requirements	2739

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2739

A.9.2.43.1	Test Purpose and Environment	2739

A.9.2.43.2	Test parameters	2739

A.9.2.43.3	Test Requirements	2741

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2741

A.9.2.44.1	Test Purpose and Environment	2741

A.9.2.44.2	Test parameters	2741

A.9.2.44.3	Test Requirements	2743

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2743

A.9.2.45.1	Test Purpose and Environment	2743

A.9.2.45.2	Test parameters	2743

A.9.2.45.3	Test Requirements	2746

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2746

A.9.2.46.1	Test Purpose and Environment	2747

A.9.2.46.2	Test parameters	2747

A.9.2.46.3	Test Requirements	2750

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2750

A.9.2.47.1	Test Purpose and Environment	2751

A.9.2.47.2	Test parameters	2751

A.9.2.47.3	Test Requirements	2754

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2754

A.9.2.48.1	Test Purpose and Environment	2754

A.9.2.48.2	Test parameters	2754

A.9.2.48.3	Test Requirements	2757

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2757

A.9.2.49.1	Test Purpose and Environment	2757

A.9.2.49.2	Test parameters	2757

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2766

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2766

A.9.2.51.1	Test Purpose and Environment	2766

A.9.2.51.2	Test parameters	2767

A.9.2.51.3	Test Requirements	2770

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2770

A.9.2.52.1	Test Purpose and Environment	2770

A.9.2.52.2	Test parameters	2770

A.9.2.52.3	Test Requirements	2772

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2773

A.9.2.53.1	Test Purpose and Environment	2773

A.9.2.53.2	Test parameters	2773

A.9.2.53.3	Test Requirements	2777

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2778

A.9.2.54.1	Test Purpose and Environment	2778

A.9.2.54.2	Test parameters	2778

A.9.2.54.3	Test Requirements	2782

A.9.3	UTRAN FDD CPICH RSCP	2782

A.9.3.1	E-UTRAN FDD	2782

A.9.3.1.1	Test Purpose and Environment	2782

A.9.3.1.2	Parameters	2782

A.9.3.1.3	Test Requirements	2785

A.9.3.2 E-UTRAN TDD	2785

A.9.3.2.1	Test Purpose and Environment	2785

A.9.3.2.2	Parameters	2785

A.9.3.2.3	Test Requirements	2788

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2788

A.9.3.3.1	Test Purpose and Environment	2788

A.9.3.3.2	Parameters	2788

A.9.3.3.3	Test Requirements	2789

A.9.4	UTRAN FDD CPICH Ec/No	2790

A.9.4.1	E-UTRAN FDD	2790

A.9.4.1.1	Test Purpose and Environment	2790

A.9.4.1.2	Parameters	2790

A.9.4.1.3	Test Requirements	2792

A.9.4.2	E-UTRAN TDD	2793

A.9.4.2.1	Test Purpose and Environment	2793

A.9.4.2.2	Parameters	2793

A.9.4.2.3	Test Requirements	2795

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2796

A.9.4.3.1	Test Purpose and Environment	2796

A.9.4.3.2	Parameters	2796

A.9.4.3.3	Test Requirements	2797

A.9.5	UTRAN TDD measurement	2797

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2797

A.9.5.1.1	Test Purpose and Environment	2797

A.9.5.1.2	Test parameters	2797

A.9.5.1.3	Test Requirements	2799

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2800

A.9.5.2.1	Test Purpose and Environment	2800

A.9.5.2.2	Test parameters	2800

A.9.5.2.3	Test Requirements	2801

A.9.6	GSM Carrier RSSI	2802

A.9.6.1	E-UTRAN FDD	2802

A.9.6.1.1	Test Purpose and Environment	2802

A.9.6.1.2	Test Requirements	2803

A.9.6.2	E-UTRAN TDD	2803

A.9.6.2.1	Test Purpose and Environment	2803

A.9.6.2.2	Test Requirements	2806

A.9.7	UE Rx – Tx Time Difference	2806

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2806

A.9.7.1.1	Test Purpose and Environment	2806

A.9.7.1.2	Test parameters	2806

A.9.7.1.3	Test Requirements	2808

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2808

A.9.7.2.1	Test Purpose and Environment	2808

A.9.7.2.2	Test parameters	2808

A.9.7.2.3	Test Requirements	2810

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2810

A.9.7.3.1	Test Purpose and Environment	2810

A.9.7.3.2	Test parameters	2810

A.9.7.3.3	Test Requirements	2813

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2813

A.9.7.4.1	Test Purpose and Environment	2813

A.9.7.4.2	Test Parameters	2813

A.9.7.4.3	Test Requirements	2816

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2816

A.9.7.5.1	Test Purpose and Environment	2816

A.9.7.5.2	Test parameters	2816

A.9.7.5.3	Test Requirements	2819

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2819

A.9.7.6.1	Test Purpose and Environment	2819

A.9.7.6.2	Test Parameters	2819

A.9.7.6.3	Test Requirements	2822

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2822

A.9.7.7.1	Test Purpose and Environment	2822

A.9.7.7.2	Test parameters	2822

A.9.7.7.3	Test Requirements	2823

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2824

A.9.7.8.1	Test Purpose and Environment	2824

A.9.7.8.2	Test parameters	2824

A.9.7.8.3	Test Requirements	2825

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2825

A.9.7.9.1	Test Purpose and Environment	2825

A.9.7.9.2	Test parameters	2825

A.9.7.9.3	Test Requirements	2826

A.9.8	RSTD	2827

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2827

A.9.8.1.1	Test Purpose and Environment	2827

A.9.8.1.2	Test Requirements	2829

A.9.8.1.2A	Test Requirements for UE Category 1bis	2829

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2830

A.9.8.2.1	Test Purpose and Environment	2830

A.9.8.2.2	Test Requirements	2832

A.9.8.2.2A	Test Requirements for UE Category 1bis	2832

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2833

A.9.8.3.1	Test Purpose and Environment	2833

A.9.8.3.2	Test Requirements	2835

A.9.8.3.2A	Test Requirements for UE Category 1bis	2835

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2836

A.9.8.4.1	Test Purpose and Environment	2836

A.9.8.4.2	Test Requirements	2838

A.9.8.4.2A	Test Requirements for UE Category 1bis	2838

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2839

A.9.8.5.1	Test Purpose and Environment	2839

A.9.8.5.2	Test Requirements	2841

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2841

A.9.8.6.1	Test Purpose and Environment	2841

A.9.8.6.2	Test Requirements	2844

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2844

A.9.8.7.1	Test Purpose and Environment	2844

A.9.8.7.2	Test Requirements	2845

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2845

A.9.8.8.1	Test Purpose and Environment	2845

A.9.8.8.2	Test Requirements	2846

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2846

A.9.8.9.1	Test Purpose and Environment	2846

A.9.8.9.2	Test Requirements	2847

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2847

A.9.8.10.1	Test Purpose and Environment	2847

A.9.8.10.2	Test Requirements	2847

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2847

A.9.8.11.1	Test Purpose and Environment	2847

A.9.8.11.2	Test Requirements	2848

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2848

A.9.8.12.1	Test Purpose and Environment	2848

A.9.8.12.2	Test Requirements	2849

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2849

A.9.8.13.1	Test Purpose and Environment	2849

A.9.8.13.2	Test Requirements	2850

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2850

A.9.8.14.1	Test Purpose and Environment	2850

A.9.8.14.2	Test Requirements	2855

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2855

A.9.8.15.1	Test Purpose and Environment	2855

A.9.8.15.2	Test Requirements	2859

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2859

A.9.8.16.1	Test Purpose and Environment	2859

A.9.8.16.2	Test Requirements	2862

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2862

A.9.8.17.1	Test Purpose and Environment	2862

A.9.8.17.2	Test Requirements	2865

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2865

A.9.8.18.1	Test Purpose and Environment	2865

A.9.8.18.2	Test Requirements	2868

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2868

A.9.8.19.1	Test Purpose and Environment	2868

A.9.8.19.2	Test Requirements	2871

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2871

A.9.8.20.1	Test Purpose and Environment	2871

A.9.8.20.2	Test Requirements	2873

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2873

A.9.8.21.1	Test Purpose and Environment	2873

A.9.8.21.2	Test Requirements	2876

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2876

A.9.8.22.1	Test Purpose and Environment	2876

A.9.8.22.2	Test Requirements	2879

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2880

A.9.8.23.1	Test Purpose and Environment	2880

A.9.8.23.2	Test Requirements	2882

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2882

A.9.8.24.1	Test Purpose and Environment	2882

A.9.8.24.2	Test Requirements	2885

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2886

A.9.8.25.1	Test Purpose and Environment	2886

A.9.8.25.2	Test Requirements	2888

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2889

A.9.8.26.1	Test Purpose and Environment	2889

A.9.8.26.2	Test Requirements	2891

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2891

A.9.8.27.1	Test Purpose and Environment	2891

A.9.8.27.2	Test Requirements	2894

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2894

A.9.8.28.1	Test Purpose and Environment	2894

A.9.8.28.2	Test Requirements	2897

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2897

A.9.8.29.1	Test Purpose and Environment	2897

A.9.8.29.2	Test Requirements	2900

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2901

A.9.8.30.1	Test Purpose and Environment	2901

A.9.8.30.2	Test Requirements	2903

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2904

A.9.8.31.1	Test Purpose and Environment	2904

A.9.8.31.2	Test Requirements	2907

A.9.9	RSRP and RSRQ on the serving cell	2907

A.9.9.1	FDD Intra frequency serving cell case	2907

A.9.9.1.1	Test Purpose and Environment	2907

A.9.9.1.2	Test parameters	2907

A.9.9.1.3	Test Requirements	2909

A.9.9.2	TDD Intra frequency serving cell case	2909

A.9.9.2.1	Test Purpose and Environment	2909

A.9.9.2.2	Test parameters	2909

A.9.9.2.3	Test Requirements	2911

A.9.10	SSTD	2911

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2911

A.9.10.1.1	Test Purpose and Environment	2911

A.9.10.1.2	Test parameters	2911

A.9.10.1.3	Test Requirements	2913

A.9.10.2	Void	2913

A.9.10.3	Void	2913

A.9.10.4	Void	2913

A.9.11	RSSI	2913

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2913

A.9.11.1.1	Test Purpose and Environment	2913

A.9.11.1.2	Test parameters	2913

A.9.11.1.3	Test Requirements	2915

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2915

A.9.11.2.1	Test Purpose and Environment	2915

A.9.11.2.2	Test parameters	2915

A.9.12	Channel occupancy	2917

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2917

A.9.12.1.1	Test Purpose and Environment	2917

A.9.12.1.2	Test parameters	2917

A.9.12.1.3	Test Requirements	2919

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2919

A.9.12.2.1	Test Purpose and Environment	2919

A.9.12.2.2	Test parameters	2919

A.9.12.2.3	Test Requirements	2921

A.9.13	RS-SINR	2921

A.9.13.1	FDD Intra-Frequency Case	2921

A.9.13.1.1	Test Purpose and Environment	2921

A.9.13.1.2	Test parameters	2921

A.9.13.1.3	Test Requirements	2923

A.9.13.2	TDD Intra-Frequency Case	2923

A.9.13.2.1	Test Purpose and Environment	2923

A.9.13.2.2	Test parameters	2923

A.9.13.2.3	Test Requirements	2926

A.9.13.3	FDD—FDD Inter frequency case	2926

A.9.13.3.1	Test Purpose and Environment	2926

A.9.13.3.2	Test parameters	2926

A.9.13.3.3	Test Requirements	2929

A.9.13.4	TDD—TDD Inter frequency case	2929

A.9.13.4.1	Test Purpose and Environment	2929

A.9.13.4.2	Test parameters	2929

A.9.13.4.3	Test Requirements	2933

A.9.13.5	FDD—TDD Inter frequency case	2934

A.9.13.5.1	Test Purpose and Environment	2934

A.9.13.5.2	Test parameters	2934

A.9.13.5.3	Test Requirements	2938

A.9.13.6	TDD—FDD Inter frequency case	2938

A.9.13.6.1	Test Purpose and Environment	2938

A.9.13.6.2	Test parameters	2938

A.9.13.6.3	Test Requirements	2941

A.9.14	Channel quality reporting accuracy	2941

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2941

A.9.14.1.1	Test Purpose and Environment	2941

A.9.14.1.2	Test parameters	2941

A.9.14.1.3	Test Requirements	2942

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2942

A.9.14.2.1	Test Purpose and Environment	2942

A.9.14.2.2	Test parameters	2943

A.9.14.2.3	Test Requirements	2943

A.10	Proximity-based Services in Any Cell Selection State	2944

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2944

A.10.1.1	Test Purpose and Environment	2944

A.10.1.2	Test Requirements	2945

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2945

A.10.2.1	Test Purpose and Environment	2945

A.10.2.2	Test Requirements	2946

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2947

A.10.3.1	Test Purpose and Environment	2947

A.10.3.2	Test Requirements	2948

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2949

A.10.4.1	Test Purpose and Environment	2949

A.10.4.2	Test Requirements	2951

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2951

A.11.1	V2V UE Transmission Timing Accuracy Test	2951

A.11.1.1	Test Purpose and Environment	2951

A.11.1.2	Test requirements	2952

A.11.2	Interruptions due to V2V sidelink communication	2952

A.11.2.1	Test Purpose and Environment	2952

A.11.2.2	Test Requirements	2954

A.12	2954

A.12.1	V2X UE Transmission Timing Accuracy Test	2954

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2954

A.12.1.1.1	Test Purpose and Environment	2954

A.12.1.1.2	Test requirements	2955

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	2956

A.12.1.2.1	Test Purpose and Environment	2956

A.12.1.2.2	Test Requirements	2956

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2957

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	2957

A.12.2.1.1	Test Purpose and Environment	2957

A.12.2.1.2	Test Requirements	2958

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	2959

A.12.2.2.1	Test Purpose and Environment	2959

A.12.2.2.2	Test Requirements	2960

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	2960

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	2960

A.12.3.1.1	Test Purpose and Environment	2960

A.12.3.1.2	Test Requirements	2962

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	2963

A.12.3.2.1	Test Purpose and Environment	2963

A.12.3.1.2	Test Requirements	2965

A.12.4	Congestion Control Measurement Test for V2X UE	2965

A.12.4.1	Test Purpose and Environment	2965

A.12.4.2	Test Requirements	2967

A.12.5	Interruptions due to V2X Sidelink Communication	2967

A.12.5.1	Test Purpose and Environment	2967

A.12.5.2	Test Requirements	2969

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	2969

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	2969

A.12.6.1.1	Test Purpose and Environment	2969

A.12.6.1.2	Test Requirements	2971

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	2971

A.12.6.2.1	Test Purpose and Environment	2971

A.12.6.1.2	Test Requirements	2973

Annex B (normative): Conditions for RRM requirements applicability for operating bands	2974

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2974

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2974

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2974

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	2974

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2976

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2976

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	2977

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	2977

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	2977

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	2978

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	2980

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2980

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2980

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2981

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2981

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2981

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2982

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2982

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2982

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2983

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2983

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2984

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2984

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2984

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2985

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2985

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2985

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2986

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2986

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2986

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2986

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2987

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2988

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	2988

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	2989

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	2989

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	2991

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	2991

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	2991

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	2991

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	2992

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	2992

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	2994

B.3	Conditions for measurements performance requirements for UE	2994

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2994

B.3.2	Void	2994

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2994

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2994

B.3.5	Conditions for UE Rx – Tx time difference	2994

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2995

B.3.7	Conditions for inter-frequency RSTD measurements	2995

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2995

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2995

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2995

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2996

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2996

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2996

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2996

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2996

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2996

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2997

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2997

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2997

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2998

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2998

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2998

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2998

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2998

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2998

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2998

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2998

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2998

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2999

B.3.21.1	Conditions for RSRP measurements	2999

B.3.21.2	Conditions for RSRQ measurements	2999

B.3.21.3	Conditions for CSI-RSRP measurements	2999

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2999

B.3.22.1	Conditions for RSRP measurements	2999

B.3.22.2	Void	3000

B.3.22.3	Conditions for CSI-RSRP measurements	3000

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3000

B.3.23.1	Conditions for RSRP measurements	3000

B.3.23.2	Conditions for RSRQ measurements	3000

B.3.23.3	Conditions for CSI-RSRP measurements	3000

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3001

B.3.24.1	Conditions for RSRP measurements	3001

B.3.24.2	Conditions for RSRQ measurements	3001

B.3.24.3	Conditions for CSI-RSRP measurements	3001

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3001

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3001

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3001

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3002

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3002

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3002

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3002

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3002

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3003

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3003

B.4	RRM Requirements Exceptions	3003

B.4.1	General	3003

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3003

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3003

B.4.3.1	Inter-band carrier aggregation	3003

B.4.3.2	Intra-band non-contiguous carrier aggregation	3003

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3004

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3004

B.5.1	Conditions for S-RSRP Accuracy Requirements	3004

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3004

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3005

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3005

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3006

B.6	Conditions for V2X	3006

B.6.1	Test parameters for GNSS signals	3006

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3006

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3006

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3007

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3007

Annex C (informative): Change history:	3008