3GPP TSG-RAN WG4 Meeting #100-e	R4-2111729

Electronic Meeting, 16th – 27th August 2021

Source:	Qualcomm

Title:	MSD for CA_n12-n77, CA_n14-n77, and CA_n30-n77

Agenda item:	8.35.2

Document for:	Approval

# 1. Introduction

MSD analysis for CA_n12-n77, CA_n14-n77, and CA_n30-n77.

# 2. Discussion

We derive MSD for PC2 band combinations based on agreed MSD for PC3 and measured IMD increases from PC3 to PC2 for the major planned FE modules used in the band combinations.

For CA_n12-n77, CA_n14-n77, we expect RIMD due to 4 x -1 IMD5 to dominate over FIMD components hence the 10dB difference of IMD levels between antenna ports. The measured ΔIM5 increase in n77 and n12/n14 PA module from 20+20 case to 23+23 case is 9dB, hence the increase in the interference levels as shown in the table 1 below.

Table 2-1: MSD analysis for n12/n14-n77

For CA_n30-n77, we expect FIMD components to dominate due to -2 x 2 IMD4, so we have equal PRX and DRX interference levels. The measured ΔIM4 increase in n77 and n30 PA module from 20+20 case to 23+23 case is 10.3dB, hence the increase in the interference levels as shown in the table 2 below.

Table 2-2: MSD analysis for n30-n77

Table 2-3: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n12-n77 | n12 | 705.5 | 5 | 20 | 760.5 | 11.7 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD |
| CA_n14-n77 | n14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |

For harmonic mixing, the n77 power has increased by 3dB, which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 2-4: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 NR CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n12zz | 34 | 31 | 29.2 |  |  |  |  |  |  |  |  |
| n77 | n14zz | 34 | 31 | 29.2 |  |  |  |  |  |  |  |  |
| NOTE zz: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal: Use MSD values as shown in Table 2-3 and 2-4.

References