3GPP TSG-RAN WG4 Meeting #100-e	R4-2111730

Electronic Meeting, 16th – 27th August 2021

Source:	Qualcomm

Title:	MSD for DC_12_n77, DC_14_n77, and DC_30_n77

Agenda item:	8.36.2

Document for:	Approval

# 1. Introduction

MSD analysis for DC_12_n77, DC_14_n77, and DC_30_n77.

# 2. Discussion

We derive MSD for PC2 band combinations based on agreed MSD for PC3 and measured IMD increases from PC3 to PC2 for the major planned FE modules used in the band combinations.

For DC_12_n77, DC_14_n77, we expect RIMD due to 4 x -1 IMD5 to dominate over FIMD components hence the 10dB difference of IMD levels between antenna ports. The measured ΔIM5 increase in n77 and B12/B14 PA module from 20+20 case to 23+23 case is 9dB, hence the increase in the interference levels as shown in the table 1 below.

Table 2-1: MSD analysis for 12/14_n77

For DC_30_n77, we expect FIMD components to dominate due to -2 x 2 IMD4, so we have equal PRX and DRX interference levels. The measured ΔIM4 increase in n77 and B30 PA module from 20+20 case to 23+23 case is 10.3dB, hence the increase in the interference levels as shown in the table 2 below.

Table 2-2: MSD analysis for 30_n77

Table 2-3: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n77A | 12 | 705.5 | 5 | 20 | 760.5 | 11.7 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |

For harmonic mixing, the n77 power has increased by 3dB, which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 2-4: MSD exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 122 | 34 | 31 | 29.2 |  |  |  |  |  |  |  |  |
| n77 | 142 | 34 | 31 | 29.2 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal: Use MSD values as shown in Table 2-3 and 2-4.

References