3GPP TSG-RAN WG4 Meeting #100-e	R4-2111821

Electronic Meeting, 16th – 27th August 2021

Source:	AT&T

Title:	TP for TR 38.841 Addition of CA_n14-n77

Agenda item:			8.35.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.841 to add CA_n14-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n14-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n77A | CA_n14A-n77A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n14A-n77(2A) | CA_n14A-n77A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2	Maximum output power

Table 5.X.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n14A-n77A | CA power class | Carrier n12 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n14A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also, when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.X.3	REFSENS requirements

According to the PC3 CA_n14A-n77A study captured in TR 38.717-02-01, the following co-existence issues were identified:

- 5th order harmonic of band n14 UL may fall into own Rx frequencies of band n77.

- band n77 UL might affect band n14 DL via 5th order harmonic mixing

- 5th order IMD products may fall into own Rx frequencies of band n14.

Therefore, additional MSD values for IMD5 and for 5th order harmonic mixing need to be defined for PC2 CA_n14A-n77A.

For the 5th order harmonic of band n14 UL, the MSD values due to 5th order harmonic product of the corresponding PC3 CA case can be applied since the maximum output power of band n14 is unchanged for case A and case B.

#### 5.X.3.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 CA_n14A-n77A.

The additional MSD due to intermodulation for PC2 CA_n14A-n77A are defined in Table 5.X.3.1-1.

Table 5.X.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n14-n77 | n14 | 795.5 | 5 | 15 | 765.5 | 11.7 | FDD | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | TDD | N/A |

#### 5.X.3.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for Case B are defined in Table 5.X.3.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.X.3.2-1: Reference sensitivity exceptions due to harmonic mixing for PC2 CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n14zz | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE ZZ: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n14A-n77A are same as the Case A defined in Table 5.X.3.1-1.

### 5.X.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n14A-n77A requirements are applied for PC2 CA_n14A-n77A.

---End of changes---

# References