3GPP TSG-RAN4 Meeting #100-e	R4-2111858

# Electronic meeting,  16th Aug – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to update RMC and SCell SSB burst position for A.6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The interruption can be only observed as DTX caused by PDSCH decode failure or PUSCH/PUCCH transmission failure. Thus any of PDSCH or PUSCH need be scheduled on concerning slots for interuption measurement. As discussed at R4-2108850, RMC for TDD15KHz and SSB burst posiion of SCell will be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ・Defined new RMC of SR.1.2 TDD for SCS15kHz TDD which schedules PDSCH even at partical-downlink slots. ・Changed SSB burst position on SCell to at even SFN(SSB.5 FR1 or SSB.6 FR1) ・SMTC for monitoring SCell SSB was updated to cover the new SSB burst position(SMTC.4) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption cannot be evaluated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1.2, A.6.5.2.1 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change>>

A.3.1.1.2	TDD

Table A.3.1.1.2-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD | SR.1.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | Defined in test case |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |  |  |  |  |  |
| Radio frame not containing SSB | slots | 4 | 6 |  |  |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 |  |  |  |  |  |
| Number of control symbols |  | 2 | 2 |  |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 | 1608 |  |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 |  |  |  |  |  |
| For special slots | bits | N/A | 1128 |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 | 5184 |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 |  |  |  |  |  |
| For special slots Note 6 | bits | - | 3744 |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change>>

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1

##### A.6.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE missed ACK/NACK rate does not exceed the limits at NR PSCell interruptions during the measurement on the deactivated NR SCC. This test will verify the missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2. Supported test configurations are shown in table A.6.5.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.6.5.2.1.1-2 and A 6.5.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell, Cell2 is an NR deactivated SCell. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, PCell is continuously scheduled in DL.

Table A.6.5.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.2.1.1-2: General test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.6.5.2.1.1-3: NR cell specific test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | FDD |
|  | Config 2,5 |  | TDD | TDD |
|  | Confiq 3 |  | TDD | FDD |
|  | Confiq 4 |  | FDD | TDD |
| TDD configuration | Config 1 |  | Not Applicable | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  | Confiq 4 |  | Not Applicable | TDDConf.1.1 |
|  | Confiq 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 | 10 MHz: NRB,c = 52 |
|  | Config 5 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.0.1 | DLBWP.0.1 |
|  | Config 5 |  | DLBWP.0.1 | DLBWP.0.1 |
| Dedicated DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.1.1 | DLBWP.1.1 |
|  | Config 5 |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.0.1 | ULBWP.0.1 |
|  | Config 5 |  | ULBWP.0.1 | ULBWP.0.1 |
| Dedicated UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.1.1 | ULBWP.1.1 |
|  | Config 5 |  | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | Config 2 |  | SR.1.2 TDD | SR.1.2 TDD |
|  | Config 3 |  | SR.1.2 TDD | SR.1.1 FDD |
|  | Confiq 4 |  | SR.1.1 FDD | SR.1.2 TDD |
|  | Confiq 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| CSI-RS for tracking | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.1 TDD | TRS.1.1 FDD |
|  | Confiq 4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  | Confiq 5 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  | Confiq 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Confiq 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.4 |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 | SSB.5 FR1 |
|  | Config 5 |  | SSB.2 FR1 | SSB.6 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 | -87 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 | -104 |
|  | Config 5 |  | -101 | -101 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

A.6.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-1.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-2.

Table A.6.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.6.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

<<End of change>>