3GPP TSG-RAN WG4 Meeting # 100-e	R4-2111923

Electronic Meeting, August 16-27, 2021

Title: 	Discussion on NR repeater other requirements for FR2

Source: 	CATT

Agenda item:	9.5.3.3

Document for:	Discussion

- Introduction

This contribution provides our further views on the some FR2 other requirements including EVM and out of band gain.

- Discussion

  - EVM

The EVM requirement analysis is similar to our FR1 analysis [2], the difference is that if several levels are defined, 3.5% should be excluded because we think that scenario is difficult to be supported for mmWave repeater deployment.

Observation 1: Defining only one EVM limit is clearer. If several EVM limits are defined, they can’t be correlated to the modulation schemes. FR2 EVM should exclude 3.5% case.

  - Out of band gain

For the FR2 out of band gain, our understanding is that some common assumption may be needed. For example, what’s the maximum in band gain for FR2. The followings are two existing mmWave repeater performance specifications.

Example (1)			Example (2)

Figure 1: Two examples of mmWave repeater specification

So if 95 dB or up to > 100 dB should be supposed for the out of band gain analysis. And for the scenarios being analysed, what is the distance or the coupling loss assumption for the donor and other BS? When these assumptions are fixed, further analysis can be conducted. Or some inputs from the mmWave vendors can also contribute to this analysis, for example the in band gain and out of band gain performance.

Observation 2: The FR2 repeater in band gain and the assumptions of analysis scenario for out of band gain requirement should be decided to further analyze this requirement.

- Summary

This contribution provides our further consideration on the FR1 other requirements. We have the following observations and proposal.

Observation 1: Defining only one EVM limit is clearer. If several EVM limits are defined, they can’t be correlated to the modulation schemes. FR2 EVM should exclude 3.5% case.

Observation 2: The FR2 repeater in band gain and the assumptions of analysis scenario for out of band gain requirement should be decided to further analyze this requirement.

Reference

[1] R4-2108632, WF on other RF radiated requirements, CATT

[2] R4-2111922, Discussion on NR repeater other requirements for FR1, CATT