3GPP TSG-RAN WG4 Meeting # 100-e		R4-2111951

Electronic Meeting, August 16-27, 2021

Agenda item:	9.8.2.1.1

Source: 	CATT

Title: 	Discussion on remaining issues for intra-frequency measurement for NR FR1 HST RRM enhancement

Document for:	Discussion

Introduction

In RAN4#99-e meeting, a way forward on RRM requirements for NR FR1 HST was approved in [1].

In this contribution, we provide the further consideration of several open issues for intra-frequency measurement and give our proposals.

Discussion

In way forward [1], there are two open issues which are listed as below:

- NSCC_SSB for CSSFoutside_gap,i - Option 1(Nokia, Xiaomi, CMCC, vivo, QC, Ericsson, Apple): NSCC_SSB clarification/correction should cover non-HST as well as HST, i.e. unified NSCC_SSB design for both HST and non-HST - Option 2 (CMCC, MTK): for high speed train scenario, it is proposed to clarify that only SCell(s) measured without MG are counted in NSCC_SSB for the calculation of CSSFoutside_gap,i - Option 3 (HW): keep NSCC_SSB as it is in the spec - Kp for the requirements on deactivated SCell - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

In previous meeting, it was agreed on the interpretation of NSCC_SSB. Both SCell(s) measured without MG and SCell(s) measured with MG are counted in NSCC_SSB when calculating CSSFoutside_gap,i. In our view, it is not necessary to separate different CSSF definition for non-HST and HST.

Proposal 1: NSCC_SSB clarification/correction should cover non-HST as well as HST, i.e. unified NSCC_SSB design for both HST and non-HST.

In RAN4#99-e meeting, it is agreed for Rel-15 as below:

Issue 1-1-2: Kp factor for measurement on deactivated SCC - Agreements: - Kp shall also apply for measurement requirements on SCC with deactivated SCell, where Kp = 1/(1- (SMTC period /MGRP)) - Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra

Proposal 2: The Rel-15/Rel-16 requirements correction shall also apply for R17 HST.

Conclusion

In this contribution, we provide the further consideration of several open issues for intra-frequency measurement and present our proposals as below:

Proposal 1: NSCC_SSB clarification/correction should cover non-HST as well as HST, i.e. unified NSCC_SSB design for both HST and non-HST.

Proposal 2: The Rel-15/Rel-16 requirements correction shall also apply for R17 HST.

Reference

[1] R4-2108341, WF on RRM for FR1 HST, CMCC