3GPP TSG-RAN WG4 Meeting # 100-e		R4-2111954

Electronic Meeting, August 16-27, 2021

Agenda item:	9.9.4.2

Source: 	CATT

Title: 	Discussion on number of RX beams for HST RRM in FR2

Document for:	Discussion

Introduction

In RAN4#99-e meeting, a way forward on RRM requirements for NR FR2 HST was approved in [1].

In this contribution, we provide the further consideration of number of RX beams for HST RRM in FR2 and give our proposals.

Discussion

In way forward [1], it is listed as below:

- FFS the feasible number of RX beams from RRM perspective - deployment scenario study is used as reference

In way forward [2], for Scenario-A,

- Comparison btw. uni- and bi-directional RRH deployments for Scenario-A: - From signal strength and beam coverage perspective: - Bi-directional deployment will not provide significant throughput improvement comparing to uni-directional deployment based on deployment scenario analysis. - Only need to consider uni-directional deployment for Scenario-A - Bi-directional deployment can be considered if the feasibility issue of uni-directional deployment is identified.

For uni-directional, the directions of DL and UL are the same which along with rail track. One RX beam is enough to achieve good performance.

Proposal 1: For Scenario-A, uni-directional deployment, the RX beam number can be 1.

- Number of Beam for bi-directional RRH deployment, Scenario-A - If bi-directional deployment is confirmed to be used for Scenario-A: - 1 beam per RRH panel, two panels in opposite directions - 1 beam per UE panel (i.e., 2 beam per UE), already agreed in RAN4#98-Bis-e

For bi-directional, the beam directions are also along with rail track. One RX beam is enough to achieve good performance for one direction.

Proposal 2: For Scenario-A, bi-directional deployment, the RX beam number can be 1 RX beam per panel, 2 in total.

For Scenario-B, it is agreed that both uni-directional and bi-directional deployment can be deployed. From RRM perspective, 1 RX beam per panel is enough.

Proposal 3: For Scenario-B, uni-directional deployment, the RX beam number can be 1 RX beam per panel.

For bi-directional deployment, in Scenario-B, Dmin = 150m. The RRH is located 150m away from railway which is much larger than Scenario-A. The beam directions are not along with rail track always. RX beam can be larger than 1 beam per panel to achieve better coverage. E.g. [2] RX beams per panel can be used.

Proposal 4: For Scenario-B, bi-directional deployment, the RX beam number can be larger than 1 RX beam per panel. E.g. [2] RX beams per panel can be used.

- Network assistance for reducing RX  beam number

- FFS: to be discussed depending on the outcomes of the deployment scenarios

In Scenario-A and Scenario-B, the different Dmin leads to different RX beams in two scenarios. In many aspects of RRM requirement, the RX beam number will impact the scaling factor directly. It is necessary that network indicates such information.

Proposal 5: In Scenario-A and Scenario-B, the RX beam number is different. It is necessary that network indicates such information.

Conclusion

In this contribution, we provide the further consideration of number of RX beams for HST RRM in FR2 and present our proposals as below:

Proposal 1: For Scenario-A, uni-directional deployment, the RX beam number can be 1.

Proposal 2: For Scenario-A, bi-directional deployment, the RX beam number can be 1 RX beam per panel, 2 in total.

Proposal 3: For Scenario-B, uni-directional deployment, the RX beam number can be 1 RX beam per panel.

Proposal 4: For Scenario-B, bi-directional deployment, the RX beam number can be larger than 1 RX beam per panel. E.g. [2] RX beams per panel can be used.

Proposal 5: In Scenario-A and Scenario-B, the RX beam number is different. It is necessary that network indicates such information.

Reference

[1] R4-2108342, WF on FR2 HST RRM requirements, Nokia, Nokia Shanghai Bell

[2] R4-2108660, WF on FR2 HST Deployment Scenario Analysis