3GPP TSG-RAN WG4 Meeting #100-e                                                            R4-2111972

Electronic Meeting, August 16 - 27, 2021

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	9.16.5.1

Document for:	Approval

- Introduction

In RAN4#99-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved.

In this contribution, we provide our views on remaining issues in WF.

- Discussion

Fractionl bandwidth (FBW)

As per the WF [1], the agreements and the remaining issues concerning fractional bandwidth are shown as below:

- Re-use fractional bandwidth concept from current FR2. - Further study whether 6% is correct breakpoint for fractional bandwidth.

From TR 38.803, Annex C, the element directivity can be expressed as:

(Equation 1)

Where,  is the antenna aperture in m2, generally, it is designed based on centre frequency of antenna, and  is the wave length in m.

The directivity difference, , in dB due to large supported bandwidth can be expressed as the difference between the directivity at fh  and the directivity at fc as:

(Equation 2)

Generally, the high frequency can be expressed as:

(Equation 3)

Where,  is fractional bandwidth (FBW).

So, according to equation 3, the equation 2 can be further expressed as:

(Equation 4)

Figure 2-1 relationship between directivity difference and fractional bandwidth(FBW)

Using equation 4, the relationship between directivity difference and fractional bandwidth (FBW) is shown in Figure 2-1. From figure 2-1, the directivity difference becomes bigger as fractional bandwidth becomes bigger. The directivity difference for FR2 operating bands and possible operating band in 52.6-71GHz is shown in table 2-1.

Table 2-1 directivity difference for FR2 operating band and possible operating band in 52.6-71GHz

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Fractional bandwidth | directivity difference(dB) |
| --- | --- | --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD | 10.7% | +/- 0.45 |
| n258 | 24250 MHz – 27500 MHz | TDD | 12.6% | +/- 0.53 |
| n259 | 39500 MHz – 43500 MHz | TDD | 9.6% | +/- 0.41 |
| n260 | 37000 MHz – 40000 MHz | TDD | 7.8% | +/- 0.33 |
| n261 | 27500 MHz – 28350 MHz | TDD | 3% | +/- 0.13 |
| Licensed band | [66000 MHz – 71000MHz] | TDD | 7.3% | +/- 0.31 |
| Un-licensed band | [57000 MHz – 71000MHz] | TDD | 21.9% | +/- 0.9 |

Table 2-2 directivity difference for higher band and lower band of  [57000 MHz – 71000MHz] in 52.6-71GHz

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Fractional bandwidth | directivity difference(dB) |
| --- | --- | --- | --- | --- |
| Un-licensed band | [57000 MHz – 71000MHz] | TDD | 21.9% | +/- 0.9 |
| Lower band | [57000 MHz – 64000MHz] | TDD | 11.6% | +/- 0.48 |
| Higher band | [64000 MHz – 71000MHz] | TDD | 10.4% | +/- 0.45 |

According to existing 6% breakpoint for fractional bandwidth, the un-licensed band ([57000 MHz – 71000MHz]) need to be separated to lower band ([57000MHz-64000MHz]) and higher band ([64000MHz-71000MHz]) to declare two EIRP. The directivity difference for lower band( [57000 MHz – 64000MHz]) and higher band ([64000MHz-71000MHz]) is +/- 0.48 and +/- 0.45 respectively, which are less than +/-0.5dB.  From perspective of antenna product, +/-0.5dB array accuracy is feasible. So FR2 6% breakpoint for fractional bandwidth can be reused for 52.6-71GHz.

Proposal 1: Re-use FR2 6% breakpoint for fractional bandwidth for 52.6-71GHz.

EIRP accuracy and TRP accuracy

As per the WF [1], the agreements and the remaining issues concerning EIRP accuracy and TRP accuracy are shown as below:

- Re-use FR2 EIRP accuracy as baseline. - Proponents of new EIPR accuracy limits (other than the current FR2 ones) should provide technical justifications in next RAN4 meeting.

The factors [2] which influence EIRP and TRP accuracy are but not limited to the following:

1) Transceiver unit accuracy;

2) Array accuracy, including sub-array gain accuracy, mismatch errors and insertion loss variations;

3) Steer accuracy, including phase shift accuracy among transceiver units, and phase shifter accuracy.

Referring to TR 37.842 and TR 37.843, EIRP accuracy and TRP accuracy can be expressed as equation 1 and equation 2 respectively.

[dB]                                                                                                       (Equation 1)

[dB]                                                                                                             (Equation 2)

Table 2-3 Expected value for transceiver unit accuracy, Array accuracy, and Steer accuracy for 52.6-71GHz

| Array accuracy | Transceiver unit accuracy | Steer accuracy |
| --- | --- | --- |
| 0.5dB | 2.958dB | 1.6dB |

I it can he acy s             bl

From above directivity difference for un-licensed band [64000 MHz – 71000MHz], the 0.5dB array accuracy is feasible. If FR2 EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) are reused for 52.6-71GHz, which can be divided into the transceiver unit accuracy , Array accuracy, and Steer accuracy using equation 1 and equation 2, the expected values for transceiver unit accuracy, Array accuracy, and Steer accuracy for 52.6-71GHz are shown in table 2-3. From perspective of AAU product, the above expected values are feasible. So the existing FR2 EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) for FR2 do not make implementation to restrictive, and they can be reused for 52.6-71GHz.

Proposal 2: Re-use existing FR2 EIRP accuracy (±3.4dB) and FR2 TRP accuracy (±3dB) for 52.6-71GHz.

OBUE

As per the WF [1], the open issues concerning OBUE are shown as below:

- For licensed operation, further consider how to adjust OBUE for 52.6 – 71 GHz taking to account the larger channel bandwidths.

The NR operating bands for FR2 and their FDL,high – FDL,low are shown in 4th column in table 5.2-2. Maximum  FDL,high – FDL,low  is 4000MHz. The ΔfOBUE is only defined for Operating band characteristics for FR2 is FDL,high – FDL,low ≤ 4000 MHz in the Table 9.7.1-1 of TS 38.104.

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | FDL,high – FDL,low |
| --- | --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD | 3000 MHz |
| n258 | 24250 MHz – 27500 MHz | TDD | 3250 MHz |
| n259 | 39500 MHz – 43500 MHz | TDD | 4000 MHz |
| n260 | 37000 MHz – 40000 MHz | TDD | 3000 MHz |
| n261 | 27500 MHz – 28350 MHz | TDD | 850 MHz |

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 4000 MHz | 1500 |

The possible licensed band in 52.6-71GHz is [66000 MHz – 71000MHz], its Operating band characteristics FDL,high – FDL,low is 5000MHz, which is greater than 4000MHz. So there is need to consider the ΔfOBUE for 4000 MHz ≤ FDL,high – FDL,low  ≤ 5000 MHz for licensed operating band. ΔfOBUE can be further discussed.

Proposal 3: Add maximum offset ΔfOBUE for 4000 MHz ≤ FDL,high – FDL,low  ≤ 5000MHz in 52.6-71GHz  for licensed band.

OBUE mask for 52.6 – 71GHz need to be adjusted according to ACLR. If ACLR is reduced by x dB relative to 26dBc for 37 – 52.6 GHz, OBUE limit need to be increased by x dB relative to 37 – 52.6 GHz. The OBUE limits applicable in the frequency range 52.6 GHz-71 GHz are shown in the following table.

Table 9.7.4.3.2-3: OBUE limits applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – (33-x) dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – (41-x) dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Proposal 4: If ACLR is reduced by x dB relative to 26dBc for 37 – 52.6 GHz, OBUE limit need to be increased by x dB relative to 37 – 52.6 GHz.

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz for licensed operation. The following proposals and observations are concluded as follows:

Proposal 1: Re-use FR2 6% breakpoint for fractional bandwidth for 52.6-71GHz.

Proposal 2: Re-use existing FR2 EIRP accuracy (±3.4dB) and FR2 TRP accuracy (±3dB) for 52.6-71GHz.

Proposal 3: Add maximum offset ΔfOBUE for 4000 MHz ≤ FDL,high – FDL,low  ≤ 5000MHz in 52.6-71GHz  for licensed band.

Proposal 4: If ACLR is reduced by x dB relative to 26dBc for 37 – 52.6 GHz, OBUE limit need to be increased by x dB relative to 37 – 52.6 GHz.

- References

- R4-2108638，WF on BS TX RF requirements for 52.6 – 71 GHz，Nokia, RAN4#99-e

- R4-1714118,WF on TRP and EIRP accuracy requirements for NR BS type 2-O,Nokia, Nokia Shanghai Bell, Huawei, ZTE, Ericsson, NEC, RAN4#85

- TYA06U060S, www.tycc.cn

- 3GPP TR 38.803 V14.2.0

- YD/T 2867-2015, multiple bands base station passive antennas specifications for mobile communication, CCSA, 2017

- R4-2105410，WF on [137] NR_ext_to_71GHz_Part1，WF on [137] NR_ext_to_71GHz_Part1，Intel Corporation (Moderator), RAN4#98-bis-e

- R4-2107997，WF on [145] NR_ext_to_71GHz Part1，Intel Corporation (Moderator), RAN4#99-e

- RP-211583, Summary of email discussions on Extending NR operation to 71GHz, Moderator (Qualcomm), RAN#92e