3GPP TSG-RAN4 Meeting #100e	R4-2111987

# Electronic meeting, Aug. 16 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on ECID measurement requirements and AoA/ZoA report mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The chapter instructions for ECID measurement and accuracy requirements are incomplete. - The measured quantity value in AoA/ZoA report mapping table is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the indication of ECID measurement and accuracy requirements. - Correct the measured quantity value in AoA/ZoA report mapping table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The ECID measurement and accuracy requirements are not clear. - The AoA/ZoA report mapping table is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.5, 13.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.5	NR E-CID measurements

9.9.5.1	Introduction

The requirements in clause 9.9.5 shall apply provided the UE has received nr-ECID-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more of the following measurements for NR E-CID positioning [22]: SS-RSRP, SS-RSRQ, CSI-RSRP, and CSI-RSRQ.

9.9.5.2	Measurement Requirements

9.9.5.2.1	Intra-frequency Measurement Requirements

The intra-frequency NR E-CID measurements shall meet the requirements in clause 9.2 and clause 9.10.2, except the measurement reporting requirements. The NR E-CID measurement reporting requirements are defined in clause 9.9.5.2.3.

The reported intra-frequency NR E-CID measurements shall also meet:

-	for FR1 SS-RSRP, the accuracy requirements in clauses 10.1.2.1,

-	for FR1 SS-RSRQ, the accuracy requirements in clauses 10.1.7.1,

-	for FR1 CSI-RSRP, the accuracy requirements in clause 10.1.2.3,

-	for FR1 CSI-RSRQ, the accuracy requirements in clause 10.1.7.2,

-	for FR2 SS-RSRP, the accuracy requirements in clauses 10.1.3.1,

-	for FR2 SS-RSRQ, the accuracy requirements in clauses 10.1.8.1,

-	for FR2 CSI-RSRP, the accuracy requirements in clause 10.1.3.3,

-	for FR2 CSI-RSRQ, the accuracy requirements in clause 10.1.8.2.

9.9.5.2.2	Inter-frequency Measurement Requirements

The inter-frequency NR E-CID measurements shall meet the requirements in clause 9.3 and 9.10.4, except the measurement reporting requirements. The NR E-CID measurement reporting requirements are defined in clause 9.9.5.2.3.

The reported inter-frequency NR E-CID measurements shall also meet:

-	for FR1 SS-RSRP, the accuracy requirements in clauses 10.1.4.1,

-	for FR1 SS-RSRQ, the accuracy requirements in clauses 10.1.9.1,

-	for FR1 CSI-RSRP, the accuracy requirements 10.1.4.3,

-	for FR1 CSI-RSRQ, the accuracy requirements 10.1.9.2,

-	for FR2 SS-RSRP, the accuracy requirements in clauses 10.1.5.1,

-	for FR2 SS-RSRQ, the accuracy requirements in clauses 10.1.10.1,

-	for FR2 CSI-RSRP, the accuracy requirements 10.1.5.3,

-	for FR2 CSI-RSRQ, the accuracy requirements 10.1.10.2.

9.9.5.2.3	Measurement Reporting Delay

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported NR E-CID measurement values contained in periodically triggered measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.6 for SS-RSRP and CSI-RSRP, and clause 10.1.11 for SS-RSRQ and CSI-RSRQ.

The UE shall not send any measurement reports as long as no corresponding reporting criteria specified in clause 9.1.4 are fulfilled.

<End of Change 1>

<Start of Change 2>

13.4	AoA/ZoA

13.4.1	Report mapping

The reporting range of UL Angle of Arrival, as defined in Clause 5.2.4 of TS 38.215 [4], is defined from -180 degree to +180 degree for azimuth angle (AoA). The reporting resolution is 0.1 degree.

The reporting range of UL Angle of Arrival, as defined in Clause 5.2.4 of TS 38.215 [4], is defined from 0 degree to +180 degree for vertical angle (ZoA). The reporting resolution is 0.1 degree.

The mapping of AoA measured quantity is defined in Table 13.4.1-1. The mapping of ZoA measured quantity is defined in Table 13.4.1-2.

Table 13.4.1-1: AoA measurement report mapping

| Reported value | Measured quantity value (AoA) | Unit |
| --- | --- | --- |
| AoA_0 | -180 ≤ AoA < -179.9 | degree |
| AoA_1 | -179.9 ≤ AoA < -179.8 | degree |
| AoA_2 | -179.8 ≤ AoA < -179.7 | degree |
| … | … | … |
| AoA_1798 | -0.2 ≤ AoA ≤ -0.1 | degree |
| AoA_1799 | -0.1 ≤ AoA < 0 | degree |
| AoA_1800 | 0 ≤ AoA < 0.1 | degree |
| AoA_1801 | 0.1 ≤ AoA < 0.2 | degree |
| AoA_1802 | 0.2 ≤ AoA < 0.3 | degree |
| … | … | … |
| AoA_3598 | 179.8 ≤ AoA < 179.9 | degree |
| AoA_3599 | 179.9 ≤ AoA < 180 | degree |

Table 13.4.1-2: ZoA measurement report mapping

| Reported value | Measured quantity value (ZoA) | Unit |
| --- | --- | --- |
| ZoA_0 | 0 ≤ ZoA < 0.1 | degree |
| ZoA _1 | 0.1 ≤ ZoA < 0.2 | degree |
| ZoA _2 | 0.2 ≤ ZoA < 0.3 | degree |
| … | … | … |
| ZoA _1798 | 179.8 ≤ ZoA < 179.9 | degree |
| ZoA _1799 | 179.9 ≤ ZoA < 180 | degree |

<End of Change 2>