3GPP TSG-RAN WG4 Meeting #100-e	R4-2112018

# Electronic Meeting, 16th to 27th Aug, 2021

Agenda Item:	8.6.1

Source: 	MediaTek Inc.

Title: 	Further discussion on MSD due to UL IMD for DC_8A-20A_n28A

Document for:	Approval

- 1. Introduction

DC_8-20_n28 was requested in RANP#90 and continue discussion in RAN4#99-e meeting. Based on the approved WF [1], companies are encouraged to provide analysis/discussion for the combination DC_8A-20A_n28A

- 2. Discussion

For the FE architecture, we start from the WF[1] and modify a little bit for band allocation as below diagram:

Figure 1 The RF Front End Architecture

And the parameters in our analysis in table 1.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| PA OIP3, FWD and REV | 37 | dBm |
| PA gain | 25 | dB |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| B20 filter rejection at B8 RX range | 40 | dB |
| n28 filter rejection at B8 RX range | 30 | dB |
| PA RXBN noise | -125 | dBm/Hz |
| Thermal noise at n66 RX ANT port | -163.5 | dBm/Hz |
| Transceiver effective phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| FWD IMD3 spur level at B20 PA output port | -69 | -69 | dBm |
| Reversed IMD3 spur level at B20 PA output port | -20 | -20 | dBm |
| FWD IMD3 spur level at n28 PA output port | -134 | -134 | dBm |
| Reversed IMD3 spur level at n28 PA output port | -38 | -38 | dBm |
| B20 PA spur at B8 LNA input | -74 | -74 | dBm/5MHz |
| n28 PA spur at B8 LNA input | -82 | -82 | dBm/5MHz |
| Thermal noise at RX ant port | -163.5 | -163.5 | dBm/Hz |
| Total noise level refers to receiver LNA port | -73.3 | -73.3 | dBm |
| Combined |  |  |  |
| Combined B66 REFSENS (5MHz BW) | -73.3 |  | dBm |
| MSD (5MHz BW) | 23.7 |  | dB |

Table 2 Link analysis for B8 5MHz REFSENS calculation

Proposal 1: For the DC_8A-20A_n28A MSD relaxation, we propose as below:

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-20A_n28A | 8 | 897 | 5 | 25 | 942 | 23.7 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |

Table 3 : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

Conclusion

Proposal 1: For the DC_8A-20A_n28A MSD relaxation, we propose as below:

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-20A_n28A | 8 | 897 | 5 | 25 | 942 | 23.7 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |

Table 3 : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

Reference:

- R4-2107804	WF on architecture and device type for DC_8A-20A_n28A, RAN4#99-e