3GPP TSG-RAN WG4 Meeting #100-e              	R4-2112046

Electrical Meeting, 16th Aug-27th Aug, 2021

Agenda item:	9.9.5.3.3

Source:	Samsung

Title:	View on PRACH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on PRACH requirement for Rel-17 FR2 HST were further discussed. The related agreement was captured in the WF [1] as

- Frequency offset - 19444Hz with 350km/h at 30GHz carrier frequency - Test Preamble Configuration for Ncs - Ncs = 0 as baseline - Timing offset configuration - Option 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e, 0.8us - Option 2: Update the timing offset configuration based on the largest expected cell radius, i.e, derived from scenario B - Note - Scenario A (Ds =700m, Dmin =10m), cell radius=700m - Scenario B (Ds=700m, Dmin =150m), cell radius =716m - Timing error tolerance - 0.07 us for AWGN, as a default value for 120 KHz SCS

In this contribution, the view on remaining issue is provided, meanwhile, the initial simulation results are provided to finalize the test parameters for PRACH requirement.

# 2	Discussion

Timing offset configuration

Regarding the timing offset, there are two principles need to be considered. One is the largest time offset is no larger than the CP length of PRACH as

12NcsLRA *2048*κ*2-μ*Tc+Δτ+τdelay<NCPRA*κ*2-μ*Tc

Another is the largest time offset with propagation delay cannot be large than the length of detection window (based on v and Ncs) as

12NcsLRA *2048*κ*2-μ*Tc+Δτ+τdelay<NcsLRA *2048*κ*2-μ*Tc

Based on this two principles, timing offset should be fulfilled with the following condition as

Δτ<minNCPRA*Tc-12NcsLRA *2048*κ*2-μ*Tc,12∙NcsLRA *2048*κ*2-μ*Tc-τdelay

During Rel-15 NR PRACH demodulation discussion, RAN4 has specified the requirement with different preamble formats with A1, A2. A3, B4, C0 and C2. In order to simply the test setup, a unified timing offset scheme for all the preamble formats was defined, where format A1 with 120 kHz SCS has the smallest timing offset (due to the short CP length), as  0.88us.

For FR2 HST WI, there are two deployment scenarios under discussion. Based on parameters (Ds, Dmin) for these two scenarios, the related maximum timing offset can be calculated as following

Table 1: Maximum timing offset for difference scenarios

| Scenario | Cell radius (m) | Maximum timing offset (us) |
| --- | --- | --- |
| Scenario A (Ds =700m, Dmin =10m) | 700 | 2.33 |
| Scenario B (Ds=700m, Dmin=150m) | 716 | 2.39 |

Firstly, whether to define the requirements for these two scenarios are still under discussion.

From PRACH detection algorithm perspective, there is no different receiver behavior foreseen with different value of timing offset. The purpose is to verify PRACH receiver with large frequency offset impact.

Therefore, we prefer to reuse the rel-15 FR2 timing offset configuration for PRACH.

Proposal 1:  Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e, 0.8us.

# 3	Simulation Results

In this contribution, the initial simulation result was provided. The simulation assumption can be referred as following table

Table 1: Simulation assumption for PRACH

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Timing offset | 0.8us, 2.39us |

Table 2: Initial simulation results for PRACH with format C2 (to be updated)

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | Timing offset | SNR [dB] Ideal |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 19444Hz | 0.8us |  |
| 1 | 2 | AWGN | 19444Hz | 2.39us |  |

# 4	Conclusion

In this contribution, the view on the remaining issue for PRACH requirement

Proposal 1:  Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e, 0.8us.

# References

R4-2108637, WF on FR2 HST demodulation, Samsung