3GPP TSG-RAN WG4 Meeting #100-e	R4-2112054

# Electronic Meeting, 16 - 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1: Addition of the missing note for DC_n77-n79 and the maximum output power for some DC conigurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softbank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Non-simultaneous Rx/Tx operation for DC_n77-n79 is not clarified. Some UL DC configurations are missing in Table 6.2B.1.3-1. (They have already been defined in Table 5.5B.1-1. ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The note for clarifying non-simultaneous Rx/Tx operation for DC_n77-n79 is added to Table 5.5B.1-1. The following UL DC configurations are added to Table 6.2B.1.3-1. - DC_n2A-n66A - DC_n3A-n28A - DC_n3A-n78A - DC_n3A-n79A - DC_n5A-n66A - DC_n28A-n78A - DC_n28A-n79A - DC_n77A-n79A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC_n77-n79, DC_n2-n66, DC_n3-n28, DC_n3-n78, DC_n3-n79, DC_n5-n66, DC_n28-n78, DC_n28-n79, DC_n77-n79 are not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B (Table 5.5B.1-1), 6.2B.1 (Table 6.2B.1.3-1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected parts omitted]

5.5B	Configurations for DC

For an NR DC configuration specified in 5.5B.1-1, the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3,i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity.

Table 5.5B.1-1: Inter-band NR DC configurations  (two bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n2A-n5A | DC_n2A-n5A |
| DC_n2A-n48A | DC_n2A-n48A |
| DC_n2A-n48B | DC_n2A-n48A |
| DC_n2A-n48C | DC_n2A-n48A |
| DC_n2A-n48(2A) | DC_n2A-n48A |
| DC_n2A-n48(A-C) | DC_n2A-n48A |
| DC_n2A-n66A | DC_n2A-n66A |
| DC_n2A-n66B | DC_n2A-n66A |
| DC_n2A-n77A | DC_n2A-n77A |
| DC_n2A-n77(2A) | DC_n2A-n77A |
| DC_n2(2A)-n77A | DC_n2A-n77A |
| DC_n2(2A)-n77C | DC_n2A-n77A |
| DC_n2A-n77C | DC_n2A-n77A |
| DC_n3A-n28A | DC_n3A-n28A |
| DC_n3A-n41A | DC_n3A-n41A |
| DC_n3A-n77A | DC_n3A-n77A |
| DC_n3A-n77(2A) | DC_n3A-n77A |
| DC_n3A-n78A | DC_n3A-n78A |
| DC_n3A-n79A | DC_n3A-n79A |
| DC_n5A-n48A | DC_n5A-n48A |
| DC_n5A-n48(2A) | DC_n5A-n48A |
| DC_n5A-n48B | DC_n5A-n48A |
| DC_n5A-n48C | DC_n5A-n48A |
| DC_n5A-n66A | DC_n5A-n66A |
| DC_n5A-n66(2A) | DC_n5A-n66A |
| DC_n5A-n77A | DC_n5A-n77A |
| DC_n5A-n77(2A) | DC_n5A-n77A |
| DC_n5A-n77C | DC_n5A-n77A |
| DC_n5(2A)-n77A | DC_n5A-n77A |
| DC_n5(2A)-n77C | DC_n5A-n77A |
| DC_n28A-n41A | DC_n28A-n41A |
| DC_n28A-n77A | DC_n28A-n77A |
| DC_n28A-n78A | DC_n28A-n78A |
| DC_n28A-n77(2A) | DC_n28A-n77A |
| DC_n28A-n79A | DC_n28A-n79A |
| DC_n41A-n77A | DC_n41A-n77A |
| DC_n41A-n78A | DC_n41A-n78A |
| DC_n46A-n48A | DC_n46A-n48A |
| DC_n46A-n48B | DC_n46A-n48A |
| DC_n46A-n48C | DC_n46A-n48A |
| DC_n46B-n48A | DC_n46A-n48A |
| DC_n46B-n48B | DC_n46A-n48A |
| DC_n46B-n48C | DC_n46A-n48A |
| DC_n46C-n48A | DC_n46A-n48A |
| DC_n46C-n48B | DC_n46A-n48A |
| DC_n46C-n48C | DC_n46A-n48A |
| DC_n46D-n48A | DC_n46A-n48A |
| DC_n46D-n48B | DC_n46A-n48A |
| DC_n46D-n48C | DC_n46A-n48A |
| DC_n46E-n48A | DC_n46A-n48A |
| DC_n48A-n66A | DC_n48A-n66A |
| DC_n48B-n66A | DC_n48A-n66A |
| DC_n48(2A)-n66A | DC_n48A-n66A |
| DC_n48C-n66A | DC_n48A-n66A |
| DC_n48(A-C)-n66A) | DC_n48A-n66A |
| DC_n66A-n77A | DC_n66A-n77A |
| DC_n66A-n77(2A) | DC_n66A-n77A |
| DC_n66A-n77C | DC_n66A-n77A |
| DC_n66(2A)-n77(2A) | DC_n66A-n77A |
| DC_n66(2A)-n77C | DC_n66A-n77A |
| DC_n66B-n77A | DC_n66A-n77A |
| DC_n66B-n77C | DC_n66A-n77A |
| DC_n77A-n79A1 | DC_n77A-n79A |
| DC_n77(2A)-n79A1 | DC_n77A-n79A |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR DC configuration is part of a higher order configuration. |  |

Table 5.5B.1-2: Inter-band NR DC configurations  (three bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n77A | DC_n3A-n28A DC_n3A-n77A DC_n28A-n77A |
| DC_n3A-n28A-n77(2A) | DC_n3A-n28A DC_n3A-n77A DC_n28A-n77A |

[Unaffected parts omitted]

6.2B.1	UE maximum output power for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the transmitter power requirements in clause 6.2 apply per band.

For inter-band NR-DC with one uplink assigned per band, the UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, the maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2B.1.3-1.

Table 6.2B.1.3-1 UE Power Class for inter-band NR-DC

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n2A-n5A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n2A-n48A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n2A-n66A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n2A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n28A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n41A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n78A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n79A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n5A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n41A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n28A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n41A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n41A-n78A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n46A-n48A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n66A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n77A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high within each CG, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

[Unaffected parts omitted]