3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112109

# Electronic Meeting, August 16-27, 2021

Agenda item:	9.19.3.2

Source:	Apple Inc.

Title:	Discussion on RRM requirements for L1/L2 Centric Mobility and Unified TCI

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#99-e the work plan for RRM requirements for FeMIMO was discussed and way forward [1] was approved.

RAN4 Prioritized objectives - 1-a Unified TCI for DL and UL - 1-b: L1/L2 centric inter-cell mobility - 1-c: Beam indication signaling medium

In this contribution we present our views on scope and impact of RRM requirements related to L1/L2 centric mobility and unified TCI framework.

# 2. Discussion

## Unified TCI for DL and UL

RAN1 is discussing enhancements for unified TCI for UL and DL. The following agreements were made in RAN1#104-e:

Based on the agreements in RAN1, we need to discuss and define requirements for unified TCI framework.

QCL Definition

One of the enhancements in RAN1 for unified TCI framework in Rel-17 is to introduce UL TCI. In R15/16, TCI state/ QCL was only associated with DL signals, but in Rel-17 they can also be indicated for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC. With this enhancement, the QCL definition in  Applicability of requirements in TS 38.133 § 3.6.7 should be also be updated. Currently, the QCL definition only applies to DL signal – PDSCH and PDCCH. The definition needs to be extended to PUCSH and PUCCH in Rel-17.

Observation #1: Currently the definition of QCL is only applicable to PDSCH and PDCCH.

We propose to update the definition of QCL to be applicable to PUCCH and PUSCH.

Proposal #1: Update the definition of QCL to be applicable to PUCCH and PUSCH in Applicability of requirements for QCL.

Based on RAN1 agreements, the supported source RS for UL TX spatial filter are,

- SRS

- SSB

- CSI-RS for tracking

- CSI-RS for Beam Management

While SRS and SSB can only be directly associated with target UL signal, CSI-RS for tracking and beam management can be part of a QCL chain. We propose to include SRS as resource in TCI chain definition.

Proposal #2: Update TCI chain definition to include SRS.

Based on the above proposals, we recommend updating the applicability of QCL definition as below.

TCI State Switching Requirements

Based on the enhancement for unified TCI framework, RAN4 needs to define requirements for TCI state switch for these cases. Firstly, we need to discuss TCI state switching for UL in case of separate TCI state for UL. This shall be applicable for cases where separate UL TCI is specified.

In R15/16 we have requirements for DL TCI state switch and UL spatial relation info switch. In Rel-17, as part of unified TCI framework, we have joint UL/DL TCI. RAN4 should discuss and define TCI state switching delay for joint TCI for DL and UL.

Proposal #3: For the unified TCI framework, RAN4 discusses and defines requirements for (1) TCI state switching delay for UL for separate TCI for UL, (2) TCI state switching delay for DL and UL for joint TCI.

## L1/L2 Centric Mobility

In RAN#92e, there was agreement to limit the scope of inter-cell beam management to intra-DU, intra-frequency cases. There was also clarification that UE can only transmit and receive from a single cell.

## L1-RSRP Measurements

To enable L1/L2 centric mobility, the UE needs to be configured with measurements for non-serving cell. RAN1 has the following agreements:

Agreements in RAN1#104e:

Agreements in RAN1#105-e:

Based on the above agreements in RAN1 and RAN#92e, the measurements on non-serving cell are –

- SSB based

- Limited to measurements on same frequency layer/ intra-frequency measurements

- L1-RSRP based measurements

It is still FSS if CSI-RS based measurements are supported and if the measurements are limited to SMTC.

Observation #2: Based on current agreements in RAN1 and RAN Plenary, neighbor cell measurements to support L1/L2 centric mobility are SSB based intra-frequency L1-RSRP measurements.

Firstly, we would like to discuss SSB based L1-RSRP measurements for neighbor cell. RAN4 needs to specify:

- Measurement period

- Measurement restriction

- Scheduling restriction

Proposal #4: RAN4 should define (1) Measurement period, (2) Measurement restriction and (3) Scheduling restriction for SSB based L1-RSRP measurements on neighbor cell.

Measurement period

The measurement period for neighbor cell L1-RSRP measurements could be based on the existing serving cell L1-RSRP measurements for FR1 and FR2.

Proposal #5: Define SSB based L1-RSRP measurement period for neighbor cell similar to existing serving cell requirements.

The measurement period on neighbor cell for FR1 and FR2 could be defined as:

Measurement period TL1-RSRP_Neighbor_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Neighbor_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_neighbor) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_neighbor)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_neighbor is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise K = 1.5. |  |

Measurement period TL1-RSRP_Neighbor_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Neighbor_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_neighbor) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_Neighbor)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_neighbor is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Where,

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

The definition of P shall depend on whether the non-serving cell L1-RSRP measurements shall be limited to SMTC or not.

Observation #3: Definition of ‘P’ in measurement period shall depend on whether L1-RSRP measurements for non-serving cell are limited to SMTC.

For example, in FR1, if neighbor cell measurement is not limited to SMTC, not overlapping with SSB of serving cell

-	P=11-TSSB_NeighborMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

If non-serving cell measurement is limited to SMTC, we need to consider resource sharing between L3 measurements of other neighbor cells. Whereas, if it is not limited to SMTC, resource sharing between serving and non-serving cell for L1-RSRP measurements. RAN4 should discuss definition of ‘P’ once RAN1 concludes on whether L1-RSRP measurements should be limited to SMTC or not.

Proposal #6: RAN4 further discussed definition of P for L1-RSRP measurement on neighbor cell once RAN1 has agreement on whether it should be limited to SMTC or not.

Measurement restriction

We need to define measurement restriction on SSB based L1-RSRP measurements, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements as follows:

For FR1, when the SSB from neighbor cell for L1-RSRP measurement is in the same OFDM symbol as CSI-RS from serving cell for RLM, BFD, CBD or L1-RSRP measurement,

- If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

- If SSB and CSI-RS have different SCS,

  - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

  - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR1, when the SSB from neighbor cell for L1-RSRP measurement is in the same OFDM symbol as SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure SSB from both serving and neighbor cell without any restriction.

For FR2, when the SSB from neighbor cell for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS or SSB from serving cell for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, there is no measurement restriction allowed when the network configures mixed numerology between SSB from neighbor cell for L1-RSRP measurement on one FR2 band and CSI-RS or SSB from serving cell for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, provided that UE is capable of independent beam management on this FR2 band pair.

Proposal #7: Define Measurement restriction on SSB based L1-RSRP measurements for non-serving cell, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements.

Scheduling Restrictions

For SSB based L1-RSRP measurements on non-serving cell, the following scheduling availability need to be defined:

- Scheduling availability of UE performing L1-RSRP measurement on with a same subcarrier spacing as PDSCH/PDCCH on FR1

- Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

- Scheduling availability of UE performing L1-RSRP measurement on FR2

- Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

Proposal #8: Define scheduling availability for UE performing L1-RSRP measurement on neighbor cell.

## TCI State Switching Delay

To enable L1/L2 based inter-cell mobility, the UE shall be indicated a new beam from non-serving cell via TCI state switch command. The new beam indication could be MAC CE or DCI based. RAN4 should define requirements for TCI state switch for neighbor cell.

Proposal #9: RAN4 needs to define requirements for TCI state switch for neighbor cell to support L1/L2 centric inter-cell mobility.

For TCI state switch on serving cell, we have known condition for TCI state. The switching delay requirements are based on the type of switch (MAC-CE, DCI or RRC based), whether the TCI state is known or unknown, whether the target TCI state is in the active TCI state list monitored by the UE. Similarly for TCI state switch on neighbor cell, we need to define known TCI state. The definition of known TCI state can be based on the existing definition for TCI state switch on serving cell.

Proposal #10: Define known TCI state for non-serving cell based on the definition for serving cell.

The new beam indication could be sent on MAC CE or DCI. Hence, MAC-CE and DCI based TCI state switch for non-serving cell should be defined.

Proposal #11: Define MAC CE and DCI based TCI state switch for non-serving cell.

In R15, we defined TCI state switching delay for serving cell. For TCI state switch on serving cell, the components of delay were:

- MAC-CE Decoding/ RRC processing time

- Time acquisition delay if target TCI was not in active TCI list monitored by UE

- Time for RX beam acquisition if target TCI state is unknown.

The switching delay for TCI state switch for non-serving cell needs to consider the following:

- MAC-CE decoding or DCI processing time

- Time acquisition delay if target TCI was not in active TCI list monitored by UE

- Time for RX beam acquisition if target TCI state is unknown.

- Time for Active BWP switch

The new component of delay for TCI state switch for non-serving cell that needs to be considered is BWP switch. The SCS and BW of the non-serving cell could be different from the serving cell, and hence may require a BWP switch to receive from the non-serving cell.

Proposal #12: Components of delay for TCI state switching for non-serving cell are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (4) Time for Active BWP switch.

If TCI state change involves a BWP switch, it might cause interruption to other serving CCs. Hence, we might need to also consider interruption requirements for TCI state switch to non-serving cell.

Proposal #13: Discuss and define interruption requirements for TCI state switch to non-serving cell.

It is agreed to use unified TCI framework for inter-cell mobility, so any new requirements for unified TCI state switch should also consider TCI state switch to non-serving cell.

Proposal #14: Consider TCI state switch to non-serving cell in new requirements for unified TCI framework.

# 3. Conclusion

In this paper, we provide our views on scope and impact of RRM requirements related to L1/L2 centric mobility and unified TCI framework Our observations and proposals are captured below:

Unified TCI for DL and UL

Observation #1: Currently the definition of QCL is only applicable to PDSCH and PDCCH.

Proposal #1: Update the definition of QCL to be applicable to PUCCH and PUSCH in Applicability of requirements for QCL.

Proposal #2: Update TCI chain definition to include SRS.

Proposal #3: For the unified TCI framework, RAN4 discusses and defines requirements for (1) TCI state switching delay for UL for separate TCI for UL, (2) TCI state switching delay for DL and UL for joint TCI.

L1/L2 Centric Mobility

L1-RSRP Measurements for non-serving cell

Observation #2: Based on current agreements in RAN1 and RAN Plenary, neighbor cell measurements to support L1/L2 centric mobility are SSB based intra-frequency L1-RSRP measurements.

Proposal #4: RAN4 should define (1) Measurement period, (2) Measurement restriction and (3) Scheduling restriction for SSB based L1-RSRP measurements on neighbor cell.

Proposal #5: Define SSB based L1-RSRP measurement period for neighbor cell similar to existing serving cell requirements.

Observation #3: Definition of ‘P’ in measurement period shall depend on whether L1-RSRP measurements for non-serving cell are limited to SMTC.

Proposal #6: RAN4 further discussed definition of P for L1-RSRP measurement on neighbor cell once RAN1 has agreement on whether it should be limited to SMTC or not.

Proposal #7: Define Measurement restriction on SSB based L1-RSRP measurements for non-serving cell, if the SSB from non-serving cell is on the same OFDM symbol as SSB or CSI-RS from serving cell for other L1 measurements.

Proposal #8: Define scheduling availability for UE performing L1-RSRP measurement on neighbor cell.

TCI State Switch for non-serving cell

Proposal #9: RAN4 needs to define requirements for TCI state switch for neighbor cell to support L1/L2 centric inter-cell mobility.

Proposal #10: Define known TCI state for non-serving cell based on the definition for serving cell.

Proposal #11: Define MAC CE and DCI based TCI state switch for non-serving cell.

Proposal #12: Components of delay for TCI state switching for non-serving cell are (1) MAC-CE decoding or DCI processing time, (2) Time acquisition delay if target TCI was not in active TCI list monitored by UE, (3) Time for RX beam acquisition if target TCI state is unknown, (4) Time for Active BWP switch.

Proposal #13: Discuss and define interruption requirements for TCI state switch to non-serving cell.

Proposal #14: Consider TCI state switch to non-serving cell in new requirements for unified TCI framework.

# Reference

- R4-2108358, “Work plan for Rel-17 FeMIMO RRM”, Samsung.

- RP-211586, “Revised WID: Further enhancements on MIMO for NR”, Samsung