3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112110

# Electronic Meeting, August 16-27, 2021

Agenda item:	10.7.2

Source:	Apple Inc.

Title:	Simulation results for Physical Layer Throughput

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#99-e the test methodology and test parameters for study on 5G NR UE Application Layer Data Throughput performance requirements was discussed and way forward [1] was agreed.

Simulation Results Alignment - Number of slots containing PDSCH grant: Skip scheduling PDSCH grant on special slots and slots containing SSB, TRS, CSI-RS for CSI reporting - Number of allocated full DL slots within 20ms: 14 for FR1 FDD, 22 for FR1 TDD and 58 for FR2 TDD. - Processing Delay: FR1 FDD: 6ms; FR1 TDD: 5.5ms; FR2 TDD: 1.375ms - Median CQI calculation: Based on CQI values belonging to median RI for considered SNR point - Reported metrics (for alignment): Median CQI, RI, BLER with link adaptation for each SNR point Simulation Assumptions - OLLA not enabled for BS/TE as baseline

The simulation assumptions were agreed, and companies were encouraged to provide simulation results. In this contribution we also present simulation results for link adaptation physical layer throughput.

# 2. Simulation Results

The following simulation assumptions were agreed for link adaptation physical layer throughput:

- Baseline test cases – RI reporting tests from Rel-15:

  - FR1 2 RX: Table 6.4.2.1-1 (Test 1) and Table 6.4.2.2-1 (Test 1)

  - FR1 4 RX: Table 6.4.3.1-1 (Test 1) and Table 6.4.3.2-1 (Test 1)

  - FR2: Table 8.4.2.2-1 (Test 1)

- Application of reported CQI/PMI/RI

  - TE schedules the PDSCH transport block, rank, and precoding slot by slot, according to the reported CQI/PMI/RI.

- Test Metric

  - Absolute Physical Layer Throughput.

- SNR Points for FR1

  - 0:2:20 dB

- SNR Points for FR2

  - 0:2:16 dB.

- CSI Reporting

  - Aperiodic for both FR1 and FR2.

- PMI Codebook

  - CodebookSubsetRestriction is set so that all the possible Type-I single panel PMI matrices are allowed.

- Channel Model for FR1

  - TDLA30-5

- Channel Model for FR2

  - TDLA30-35

Based on the simulation assumptions above, we provide absolute throughput, median CQI and median RI for FR1-2RX, FR1-4RX, FR2-2RX below for alignment.

FR1 – 2 RX

Figure 1: FR1- 2x2

Table 1: Results for FR1-2RX

| SNR (dB) | FDD |  |  |  | TDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | Med CQI | Med RI | TP (Mbps) | BLER | Med CQI | Med RI |
| 2 | 5.076 | 0.117 | 4 | 1 | 14.943 | 0.069 | 3 | 1 |
| 4 | 6.951 | 0.109 | 5 | 1 | 20.423 | 0.080 | 4 | 1 |
| 6 | 8.917 | 0.107 | 6 | 1 | 26.301 | 0.086 | 5 | 1 |
| 8 | 11.003 | 0.085 | 6 | 1 | 33.522 | 0.054 | 6 | 1 |
| 10 | 13.380 | 0.091 | 7 | 1 | 41.083 | 0.052 | 7 | 1 |
| 12 | 15.918 | 0.106 | 7 | 1 | 48.726 | 0.076 | 7 | 1 |
| 14 | 18.510 | 0.114 | 7 | 2 | 56.203 | 0.107 | 6 | 2 |
| 16 | 21.615 | 0.114 | 7 | 2 | 65.272 | 0.117 | 7 | 2 |
| 18 | 24.357 | 0.138 | 8 | 2 | 74.578 | 0.123 | 8 | 2 |
| 20 | 26.541 | 0.185 | 9 | 2 | 81.981 | 0.165 | 8 | 2 |

FR1 – 4 RX

Figure 2: FR1- 2x4

Table 2: Results for FR1-4RX

| SNR (dB) | FDD |  |  |  | TDD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | Med CQI | Med RI | TP (Mbps) | Med CQI | BLER | Med RI |
| 0 | 3.910 | 0.480 | 4 | 1 | 16.356 | 0.229 | 3 | 1 |
| 2 | 6.649 | 0.233 | 3 | 2 | 23.589 | 0.121 | 3 | 2 |
| 4 | 7.067 | 0.381 | 4 | 2 | 27.681 | 0.144 | 3 | 2 |
| 6 | 10.056 | 0.356 | 5 | 2 | 39.178 | 0.169 | 4 | 2 |
| 8 | 14.101 | 0.278 | 6 | 2 | 52.158 | 0.122 | 5 | 2 |
| 10 | 19.795 | 0.149 | 6 | 2 | 66.267 | 0.078 | 6 | 2 |
| 12 | 23.448 | 0.152 | 8 | 2 | 78.426 | 0.071 | 7 | 2 |
| 14 | 26.336 | 0.193 | 8 | 2 | 93.292 | 0.067 | 8 | 2 |
| 16 | 30.944 | 0.184 | 9 | 2 | 107.737 | 0.071 | 9 | 2 |
| 18 | 36.427 | 0.155 | 10 | 2 | 115.809 | 0.129 | 10 | 2 |
| 20 | 39.527 | 0.181 | 11 | 2 | 122.800 | 0.179 | 11 | 2 |

FR2 – 2 RX

Figure 3: FR2- 2x2

Table 3: Results for FR2-2RX

| SNR (dB) |  | TDD |  |  |
| --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | Med CQI | Med RI |
| 2 | 23.033 | 0.123 | 6 | 1 |
| 4 | 31.720 | 0.072 | 7 | 1 |
| 6 | 40.571 | 0.086 | 8 | 1 |
| 8 | 49.996 | 0.098 | 9 | 1 |
| 10 | 60.193 | 0.096 | 10 | 1 |
| 12 | 72.778 | 0.098 | 8 | 2 |
| 14 | 86.550 | 0.102 | 9 | 2 |
| 16 | 97.187 | 0.133 | 9 | 2 |
| 18 | 99.646 | 0.226 | 10 | 2 |
| 20 | 110.644 | 0.266 | 11 | 2 |

In RAN4#99e, options for results alignment criteria were discussed.

- Alignment Results Criteria

  - Option 1: Absolute throughput span within X% of average throughput across companies at a given SNR.

    - Decide X based on simulation results. Possible values of X = [5]% or [10]%.

  - Option 2: SNR G±Gspan can be reached for the T% of maximum throughput

    - Maximum throughput is derived with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE.

    - Decide Gspan based on simulation results. Candidate option is Gspan = [2.5] dB.

Based on the results alignment criteria the requirements definition is to be decided.

  - Set the physical layer throughput requirements:

    - Option 1: By multiplying the averaged throughput by Y (%), e.g., Y=95% or 90%.

    - Option 2: Use methodology from PDSCH demodulation requirements with fixed RMC (i.e. average of impairments results + X dB margin).

    - Depends on outcome for alignment result criteria on Slide 4. If Option 1 is agreed, use Option 1. If Option 2 is agreed, use Option 2.

In RAN4 typically option 2 is used for requirements definition to allow for impairment margin. Defining requirements based on averaged TP multiplied by Y% may not guarantee sufficient margin in all cases. Hence, we propose to use Option 2 for results alignment criteria and throughput requirements definition.

Proposal #1: To align results, criteria used is: SNR G±Gspan can be reached for the T% of maximum throughput.

Proposal #2: Define physical layer TP requirements based on methodology for PDSCH demodulation requirements.

# 3. Conclusion

In this paper, we provide simulation results our views on requirements definition for link adaptation physical layer throughput requirements. Our proposals are captured below:

Proposal #1: To align results, criteria used is: SNR G±Gspan can be reached for the T% of maximum throughput.

Proposal #2: Define physical layer TP requirements based on methodology for PDSCH demodulation requirements.

# Reference

- R4-2108102, “WF on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm.