3GPP TSG-RAN4 Meeting #100-e	R4-2112127

# Online, 16 – 27 Aug, 2021

Agenda item:	9.13.5.3

Source:	Apple

Title:	Discussion on mobility for NR NTN

Document for:	Discussion

# Introduction

In last RAN4 meeting, the mobility requirement for NR NTN has been discussed and the agreements were captured in the WF[1], duplicated as below.

Topic #2: Mobility-related measurement requirements - Sub-topic 2-1: General Mobility - Issue 2-1-1: Inter-cell mobility requirements - The issue relies on L1/L3 mobility and beam management mechanism firstly defined by RAN1 and RAN2. RAN4 can develop corresponding requirements based on RAN1/2 procedures. - Issue 2-1-2: Number of measurement cells - RAN4 to discuss/determine measurement capacity needed for intra-satellite and inter-satellite first and then the number of measurement cells for intra-satellite and inter-satellite. FFS on the specific number of cells. - Issue 2-1-3: Location/timer-based measurement relaxation - Defer discussion until RAN2 has progressed on the issue - Sub-topic 2-2: Conditional Handover - All CHO-related issues are depending on ongoing RAN2 discussion. Defer discussion in RAN4 until RAN2 has concluded on the issue. - Sub-topic 2-3: Cell selection and reselection - Issue 2-3-1: Cell reselection delay requirements - FFS, more conclusions from RAN2 necessary. - Issue 2-3-2: Cell reselection margin - FFS, more conclusions from RAN2 necessary. - Issue 2-3-3: RRM measurement requirements for cell reselection - FFS, more conclusions from RAN2 necessary. - Issue 2-3-4: Impact of timing/location accuracy on cell reselection performance - FFS, more conclusions from RAN2 necessary.

Even though the CHO requirement could be deferred until RAN2 has more conclusions, we still think it’s worthwhile to discuss some conditions to apply such CHO requirement in RAN4. In this contribution, we discuss the assumption of K_offset and common TA in CHO requirement.

# RAN4 assumption for NTN CHO

In legacy TN CHO requirement, we have the delay requirement in TS38.133 as below,

SI (e.g., SIB reading) is not required in CHO procedure, and RO information would be configured by serving cell for the handover. We think the same assumption shall be applied for CHO in NTN as well, otherwise the CHO delay would be much longer than the legacy TN case.

Proposal 1: same as legacy TN system, UE is not required to read SI during the NTN CHO.

In RAN1, K_offset and common TA were introduced in the system information for UE to access to new NTN cell, and those parameters are essential for cell accessing and communication.

The K_offset was defined in RAN1 as:

Agreement: • Confirm the following working assumption: K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH. The K_offset value signaled in system information is always used for - The transmission timing of RAR / fallbackRAR grant scheduled PUSCH - The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI - The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI - The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI FFS: how to treat additional transmission timings related to fallback DCI formats FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

Even though the ending point of CHO requirement is the msg1 transmission timing, the UE still needs to know the K_offset before msg1 transmission for msg3/msgB to complete a handover procedure in field. Since we don’t think this SI reading shall be conducted during the CHO procedure, it makes sense to let source serving cell provide such K_offset info of target serving cell to UE for NTN CHO.

The common TA was defined in RAN1 as:

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The common TA would also be applied to the UE msg1/msgA transmission to the target cell in CHO case based on the above definition. However, we don’t think UE shall be required to acquire common TA in SI of target cell during the CHO procedure, and therefore we also propose to let source serving cell provide such common TA info of target serving cell to UE for NTN CHO.

Proposal 2: In NTN CHO requirement, RAN4 assumes that source serving cell provides the ‘K_offset' and ‘common TA’ of target cell to the UE.

# Conclusion

In this contribution, we discuss timing requirements for NR NTN.

Proposal 1: same as legacy TN system, UE is not required to read SI during the NTN CHO.

Proposal 2: In NTN CHO requirement, RAN4 assumes that source serving cell provides the ‘K_offset' and ‘common TA’ of target cell to the UE.

# References

[1] R4-2108033, WF on general, GNSS & measurement-related NR NTN RRM requirements, Fraunhofer HHI, RAN4 #99-e