3GPP TSG-RAN WG4 Meeting # 100-e      		R4-2112209

Electronic Meeting, August 16-27, 2021

Agenda Item:	9.12.2.1.2

Source: 	CMCC

Title: 	Discussion on CSI reporting related issues for inter-cell interference MMSE-IRC

Document for:	Discussion

- Introduction

In RAN4#99-e meeting, we discussed the CQI reporting requirements for inter-cell interference MMSE-IRC receiver, a way forward was agreed in [1]. In this contribution, we further discuss the general issues and interference model for CSI reporting.

- Discussion

  - General issues

Issue 1-1: Whether to define CQI reporting requirements with inter-cell interference scenario

Agreement in RAN4#99-e meeting:

- Option 1: Yes

- Option 2: Need further discussion

- Option 3: No

By introducing the CQI reporting requirement, we can check the MMSE-IRC receiver’s gain on improving the MCS.

Proposal 1: Define the CQI reporting requirements with inter-cell interference scenario.

  - Interference Model

Issue 2-1: CSI-IM on target cell

Agreement in RAN4#99-e meeting:

- Option 1: Overlaps with PDSCH from interference

- Option 2: Overlaps with NZP CSI-RS from interference

With MMSE-IRC receiver, the UE estimation behavior under Option 1 and Option2 is the same. For Homogenous scenario, we slightly prefer Option 2 which is aligned with practical network configuration.

Proposal 2: For Homogenous scenario, CSI-IM on target cell overlaps with NZP CSI-RS from interference.

Issue 2-2: NZP CSI-RS on target cell

Agreement in RAN4#99-e meeting:

- Option 1: Overlaps with PDSCH from interference

- Option 2: Overlaps with NZP CSI-RS from interference

- Option 3: Interference free

We are fine with Option 2 and Option 3. For Homogenous scenario, NZP CSI-RS colliding with NZP CSI-RS interference or ZP CSI-RS are typical network configurations.

Proposal 3: For Homogenous scenario, NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference or interference free.

- Conclusion

In this contribution, we discuss the general issues and interference model of CSI related requirements for inter-cell interference suppression with MMSE-IRC receiver, and provide our proposals. The proposals are:

Proposal 1: Define the CQI reporting requirements with inter-cell interference scenario.

Proposal 2: For Homogenous scenario, CSI-IM on target cell overlaps with NZP CSI-RS from interference.

Proposal 3: For Homogenous scenario, NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference or interference free.

- Reference

[1] R4- 2108665, WF on CQI reporting requirements for inter-cell interference MMSE-IRC, Ericsson.

[2] R4-2106116, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[3] RP-202000, New WID: Further enhancement on NR demodulation performance, China telecom.