3GPP TSG-RAN WG4 Meeting # 100-e      		R4-2112213

Electronic Meeting, August. 16-27, 2021

Agenda Item:	9.12.3.1

Source: 	CMCC

Title: 	Simulation results for PUSCH 256QAM performance

Document for:	Discussion

- Introduction

In RAN4#99-e meeting, we discussed the test parameters of BS demodulation for FR1 256QAM. Some companies provided partial simulation results for alignment. A way forward was agreed in [1]. In this contribution, we provide our simulation results for PUSCH 256QAM performance requirement.

- Simulation Results

Before meeting, companies had the consensus that focus on 10MHz/15kHz and 40MHz/30kHz for this meeting. Our related simulation results are summarized in table 1.

Table 1. Simulation results for 10MHz/15kHz and 40MHz/30kHz

|  |  | MCS | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- | --- | --- |
| 15kHz | 10MHz | MCS 20 | 17.0 | 12.95 | 9.55 |
|  |  | MCS 21 | 17.7 | 13.7 | 10.35 |
|  |  | MCS 22 | 18.2 | 14.55 | 11.02 |
|  |  | MCS 24 | 20.4 | 16.5 | 13.2 |
| 30kHz | 40MHz | MCS 20 | 17.2 | 13.15 | 9.7 |
|  |  | MCS 21 | 17.6 | 13.4 | 10.1 |
|  |  | MCS 22 | 18.6 | 14.7 | 11.2 |
|  |  | MCS 24 | 20.75 | 16.75 | 13.22 |

Observation 1: The simulation results of MCS 24 is lower than 21dB.

In addition, we simulated part of results of other CBWs, we list as follows:

Table 2. Simulation results for other CBWs

|  |  | MCS | 2Rx | 4Rx |
| --- | --- | --- | --- | --- |
| 15kHz | 20MHz | MCS 22 | -- | 14.7 |
|  |  | MCS 24 | 20.55 | 16.65 |
| 30kHz | 10MHz | MCS 22 | -- | 14.6 |
|  |  | MCS 24 | -- | 16.40 |
|  | 100MHz | MCS 22 | -- | 15.75 |
|  |  | MCS 24 | -- | 17.95 |

Observation 2: For 30kHz MCS 24, the SNR gap between 10MHz and 100MHz is about 1.5dB.

- Conclusion

In this contribution, we provide our simulation results for 256QAM demodulation requirements. The observations are:

Observation 1: The simulation results of MCS 24 is lower than 21dB.

Observation 2: For 30kHz MCS 24, the SNR gap between 10MHz and 100MHz is about 1.5dB.

- Reference

[1] R4-2108667, Way forward on FR1 PUSCH 256QAM performance requirements, Huawei.

[2] R4-2106120, Way forward for PUSCH 256QAM performance requirements, Huawei.

[3] RP-202000, New WID: Further enhancement on NR demodulation performance, China telecom.

- Annex

Table 3. Simulation assumptions for PUSCH 256QAM performance requirement

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups with data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | [20, 21, 22, 24] |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz, [20MHz]; 30kHz: 10MHz, [20MHz], 40MHz, [100MHz] |
| SRS | Not configured |
| PTRS | Not configured |
| Phase noise modelling | No explicit PN modelling |
| Tx EVM | Not consider |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |

Figure 1. Simulation results for 30kHz 40MHz and 15kHz 20MHz