3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2112284

# Electronic Meeting, 16-27 May 2021

Source: Qualcomm Incorporated

Title: 	 MPR specifications for intra-band con-current V2X operation

Agenda item:	9.15.5.4

Document for:	Approval

# Introduction

In RAN4#98-e [1] there was a WF agreed on MPR&AMPR simulations assumptions for NR V2X intra-band con-current operation.  This paper discusses our understanding of the current specifications and suggests how best to derive these MPR/AMPR specifications.

# Discussion

As mentioned in [1] it was agreed that both separate and shared RF architectures will be considered for Uu and SL when deriving MPR & AMPR specifications for NR V2X intra-band con-current operation. For the case of separate Uu and SL RF architectures, a study of [2] reveals that the MPR development for cellular intra-band EN-DC, outlined in section 6.2B.2, can be reused for V2X intra-band con-current operation. Pertinent sections of 6.2B.2 from 38.101-3 have been reproduced below:

************************Section 6.2B.2 from 38.101-3 *******************

#### 6.2B.2.1	Intra-band contiguous EN-DC

##### 6.2B.2.1.1	General

When the UE is configured for intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

MPRENDC is defined in Clause 6.2B.2.1.2

- 6.2B.2.1.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in clause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	15	;	 0 ≤ B < 0.5

10	;	 0.5 ≤ B < 1.0

8	; 	1.0 ≤ B < 2.0

6	; 	2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

- 6.2B.2.2	Intra-band non-contiguous EN-DC

- 6.2B.2.2.1	General

When the UE is configured for intra-band non-contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

MPRENDC is defined in Clause 6.2B.2.2.2

##### 6.2B.2.2.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply.  The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	18	; 	0 ≤ B < 1.0

17	; 	1.0 ≤ B < 2.0

16	; 	2.0 ≤ B < 5.0

15	; 	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

******************************************************************

It is our understanding that the current intra-band EN-DC MPR development outlined in 38.101-3 can be reused without modification for V2X intra-band concurrent operation where separate RF chains are used for Uu and SL. We feel that the above MPR development should be adopted without modification for V2X because modifications may lead to more stringent requirements for intra-band con-current V2X compared to cellular intra-band EN-DC. It is reasonable to assume that the same RF circuitry will be used for both cellular EN-DC and NR V2X intra-band con-current operation. Therefore, the MPR specifications for V2X intra-band con-current operation should not be more stringent than what is currently in [2] for cellular EN-DC. Therefore, it is proposed that the MPRs for cellular EN-DC be adopted for intra-band con-current V2X operation where separate RF chains are used for Uu and SL.

Proposal 1: MPR development outlined in section 6.2B.2 of 38.101-3 [2] for cellular EN-DC shall be reused without modification for V2X intra-band con-current operation where separate RF chains are used for Uu and SL

Proposal 2: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for intra-band con-current V2X operation where separate RF chains are used for Uu and SL.

# Conclusion

In this paper the following observations are made regarding the development of MPRs for intra-band con-current V2X operation where separate RF chains are used for Uu and SL:

Proposal 1: MPR development outlined in section 6.2B.2 of 38.101-3 [2] for cellular EN-DC shall be reused without modification for V2X intra-band con-current operation where separate RF chains are used for Uu and SL

Proposal 2: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for intra-band con-current V2X operation where separate RF chains are used for Uu and SL.

# Reference

- R4-2103246, WF on MPR&AMPR simulations assumptions for NR V2X intra-band con-current operation, RAN4#98-e, Jan-Feb 2021

- 38.101-3, V17.2.0, User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios