3GPP TSG-RAN4 Meeting #100-e	R4-2112315

Online, , 16th Aug 2021 - 27th Aug 2021

Source: 	Ligado Networks

Title: 					CAT-M1/M2 Simulation Results for LTE band 24

Agenda Item: 		11.7.2

Document for:	Approval

# Introduction

This contribution presents results of A-MPR simulations for LTE Band 24 CAT-M1 UE as well results of sub-PRB A-MPR simulation for B24 CAT M1/M2 UEs.

# 2. Simulation Scenarios and Assumptions

A-MPR assumptions used and simulation scenarios for CAT-M1/M2 devices used were as agreed to in R4-2016932 [1] and R4-2016896 [2] and are listed below.

- 0 dBi UE antenna gain

- 4 dB post-PA losses

- PA calibrated at 1 dB MPR for -30 dBc EUTRA ACLR with 20 MHz DFT-s-OFDM 100 RB0 waveform

- Image/carrier leakage = -25 dBc for LTE

- -60 dBc CIM3 and -70 dBc CIM5

- Filter attenuation to be used:

  - Tx attenuation in 1541 – 1612 MHz = 41.0 dB

  - Tx attenuation in 1612 – 1617 MHz = dB linear interpolation from 41.0 dB at 1612 MHz to 12.5 dB at 1617 MHz

  - Tx attenuation in 1617 – 1622 MHz = dB linear interpolation from 12.5 dB at 1617 MHz to 0 dB at 1622 MHz

  - Tx attenuation in 1622 – 1627.5 MHz = 0 dB

- Scenarios:

  - 5 MHz carrier from 1627.5 – 1632.5 MHz

  - 5 MHz carrier from 1632.5 – 1637.5 MHz

  - 5 MHz carrier from 1646.5 – 1651.5 MHz

  - 5 MHz carrier from 1651.5 – 1656.5 MHz

  - 10 MHz carrier from 1627.5 – 1637.5 MHz

  - 10 MHz carrier from 1646.5 – 1656.5 MHz

For SubPRB A-MPR simulations, approach detailed in R4-1802545 [1] was used.

# 3. A-MPR Results for B24 CAT-M1 PRB

Simulations results for A-MPR for CAT-M1 PRB are shown in Figure 1 and 2 below.

Figure 1: A-MPR results for 5 MHz CAT-M1 PRB

Simulation results showed that there is no A-MPR required for NBindex = 1, 2, and 3 of lower 5 MHz (1627.5 - 1632.5 MHz) channel and for NBindex = 0, 1, 2, and 3 of upper 5 MHz (1632.5 – 1637.5 MHz) channel.

Figure 2: A-MPR results for 10 MHz CAT-M1 PRB

Simulation results showed that there is no A-MPR required for NBindex = 1, 2, 3, 4, 5, 6, and 7 of 10 MHz (1627.5 - 1637.5 MHz) channel.

The A-MPR for NS_56, based on the above simulation results for CAT-M1 PRB are summarized in Table 1 below.

| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | 1630.0, 1630.3 | (NBindex, Rbstart) | (0,0) | (0,0) | (0,0) | (0,1) | (0,1) | (0,1) |
|  |  | LCRB [RBs] | 1 | >1 | >3 | >1 | >2 | >3 |
|  |  | A-MPR [dB] | 1 | 4 | 5 | 1 | 3 | 5 |
|  | 1635 | No A-MPR needed |  |  |  |  |  |  |
|  | 1649 |  |  |  |  |  |  |  |
|  | 1654 |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | (NBindex, Rbstart) | (0,0) | (0,1) | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] | >3 | >4 | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] | 2 | 1 | N/A | N/A | N/A | N/A |
|  | 1651.5 | No A-MPR needed |  |  |  |  |  |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks |  |  |  |  |  |  |  |  |
| NOTE 2: LCRB is the length of a contiguous resource block allocation |  |  |  |  |  |  |  |  |
| NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis |  |  |  |  |  |  |  |  |
| NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |  |
| NOTE 5: For CAT-M1 device, the NB index is the starting index allocated from DCI[6], the RBstart is indexed within the NB allocated to cat-M1 device. |  |  |  |  |  |  |  |  |

Table 1: A-MPR for CAT-M1

Proposal 1: Incorporate Table 1 as A-MPR for CAT-M1 for “NS_56” in TS 36.101

# 4. A-MPR Results for B24 CAT-M1 Sub-PRB

Simulations results for A-MPR for CAT-M1 sub-PRB are shown in Figure 1 and 2 below.

Figure 3: Sub-PRB A-MPR for NBindex = 0 of lower 5 MHz channel

Simulation results show that there is no A-MPR required for NBindex = 1, 2, and 3 of lower 5 MHz (1627.5 - 1632.5 MHz) channel and for NBindex = 0, 1, 2, and 3 of upper 5 MHz (1632.5 – 1637.5 MHz) channel.

Figure 4: Sub-PRB A-MPR for NBindex=0 of lower 10 MHz channel

Simulation results show that there is no A-MPR required for NBindex = 1, 2, 3, 4, 5, 6, and 7 of 10 MHz (1627.5 - 1637.5 MHz) channel.

The A-MPR for NS_56, based on the above simulation results for CAT-M1 sub-PRB are summarized in Table 2 below.

| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 MHz | 1630.0, 1630.3 | (NBindex, SCstart) | (0, =9) | (0, =18) | (0, =6) |
|  |  | Lcsc | 2,3 | 2,3 | 6 |
|  |  | A-MPR [dB] | ≤5 | ≤4.5 | ≤2.5 |
|  | 1635 | No A-MPR needed |  |  |  |
|  | 1649 |  |  |  |  |
|  | 1654 |  |  |  |  |
| 10 MHz | 1632.5 | (NBindex, SCstart) | (0, =6) | N/A | N/A |
|  |  | Lcsc | 6 | N/A | N/A |
|  |  | A-MPR [dB] | ≤2.5 | N/A | N/A |
|  | 1651.5 | No A-MPR needed |  |  |  |
| Note 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |  |
| Note 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |

Table 2: sub-PRB A-MPR for CAT-M1

Proposal 2: Incorporate Table 2 as sub-PRB A-MPR for CAT-M1 for “NS_56” in TS 36.101.

# 5. A-MPR Results and Proposals for CAT-M2 Sub-PRB

Simulations results for sub-PRB A-MPR for CAT-M2 are shown in  Figure 5 and Figure 6 below.

Figure 5: Sub-PRB A-MPR for WBindex = 0 of Lower 5 MHz Channel

Simulation results show that there is no A-MPR required for WBindex = 0 of uppler 5 MHz (1632.5 - 1637.5 MHz) channel.

Figure 6: Sub-PRB A-MPR for WBindex = 0 of 10MHz Channel

Simulation results show that there is no A-MPR required for WBindex = 1 of 10MHz (1627.5 - 1637.5 MHz) channel.

The A-MPR for NS_56, based on the above simulation results for CAT-M2 sub-PRB are summarized in Table 3 below.

| Channel BW | Carrier centre freq. (FC) (MHz) | Parameter | Region |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | 1630.0, 1630.3 | (WBindex, SCstart) | (0, <=3) | (0, <=27) | (0, =66) | (0, =93) | (0, =0) | (0, <= 18) | (0, =24) | (0, <=42) | (0, =66) |
|  |  | Lcsc | 2,3 | 2,3 | 2,3 | 2,3 | 6 | 6 | 6 | 6 | 6 |
|  |  | A-MPR [dB] | ≤4.5 | ≤4 | ≤5 | ≤4 | ≤4.5 | ≤3.5 | ≤2.5 | ≤1.5 | ≤1.5 |
|  | 1635 | No A-MPR needed |  |  |  |  |  |  |  |  |  |
|  | 1649 |  |  |  |  |  |  |  |  |  |  |
|  | 1654 |  |  |  |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | (WBindex, SCstart) | (0, <=12) | (0, <=12) | (0, =48) | (0, <=6) | (0, =12) | (0, <=30) | N/A | N/A | N/A |
|  |  | Lcsc | 2 | 3 | 3 | 6 | 6 | 6 | N/A | N/A | N/A |
|  |  | A-MPR [dB] | ≤4 | ≤2 | ≤2 | ≤3 | ≤1 | ≤1.5 | N/A | N/A | N/A |
|  | 1651.5 | No A-MPR needed |  |  |  |  |  |  |  |  |  |
| Note 1: WBindex is the wideband index that is defined in [4]. |  |  |  |  |  |  |  |  |  |  |  |
| Note 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |  |  |  |  |  |

Table 3: sub-PRB A-MPR for CAT M2

Proposal 3: Incorporate Table 3 as sub-PRB A-MPR for CAT-M2 for “NS_56” in TS 36.101.

# 6. Proposals

Based on the A-MPR simulations for CAT-M1/M2 devices for LTE Band 24, it proposed to:

Proposal 1: Incorporate Table 1 as A-MPR for CAT-M1 PRB for “NS_56” in TS 36.101.

Proposal 2: Incorporate Table 2 as A-MPR for CAT-M1 Sub-PRB for “NS_56” in TS 36.101.

Proposal 3: Incorporate Table 3 as A-MPR for CAT-M2 Sub-PRB for “NS_56” in TS 36.101.

# 7. References

[1] R4-2016932, “A-MPR simulation assumptions for B24 CAT-M1/M2 device”, Ligado Networks

[2] R4-2016896, “WF on work items LTE_B24_mod and NR_band_n24”, Ligado Networks & Skyworks Solutions Inc.

[3] R4-1802545, “subPRB feature impact on MPR and A-MPR of CAT-M1 device”, Ericsson.