3GPP TSG-RAN WG4 Meeting #100e	R4-2112327

Electronic Meeting, 16th - 27th Aug, 2021

Source:	ZTE Corporation

Title:	Further discussion on immediate wider channel bandwidth

Agenda Item:	10.2.2

Document for:	Discussion

# Introduction

In RAN4#99e, some agreements were made on a GTW session regarding the use of immediate wider channel bandwidth [3]:

- For widerCHBW approach, to support irregular CBW for DL only the following discussions are needed - Need further discussion on support of asymmetric bandwidths - Need further discussion on support of TX-RX separation

Following this agreement, we continue to further discuss these two aspects,

# Discussion

According to the latest WID [2], the channel bandwidths requested are shown below:

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11, 12 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

Except n29 is an SDL band, the rest is FDD bands, and we can focus on the support of asymmetric bandwidth operation for FDD bands.

Observation 1: the irregular channel bandwidths requested up to now are FDD bands except only one SDL band.

As agreed, the method of using the immediate wider channel bandwidth (i.e., WiderCBW) is intended for DL only, not for UL, thus the support of asymmetric channel bandwidths is a must if applying WiderCBW in the DL direction, and at the UL direction, ordinary channel bandwidth which is immediate smaller channel bandwidth (i.e., SmallerCBW) can maximize utilization of the irregular UL bandwidth, therefore, the asymmetric channel bandwidth consisting of SmallerCBW at UL and WiderCBW at DL may enable the operation of using the immediate wider channel bandwidth.

Proposal 1: The asymmetric channel bandwidth consisting of SmallerCBW at UL and WiderCBW at DL may enable the method of using the immediate wider channel bandwidth.

Fig. 1, Asymmetric channel bandwidth operation for WiderCBW method

Under the current framework of supporting asymmetric channel bandwidth operation, as illustrated in Fig. 1, the deviation to the Tx-Rx carrier centre frequency due to the asymmetric operation can be calculated as:

∆FWiderCBW=WiderCBW-SmallerCBW2

Proposal 2: Apply the deviation to the Tx-Rx carrier centre frequency for the WiderCBW method as 
∆FWiderCBW=WiderCBW-SmallerCBW2

# Conclusion

In this paper, we have the following observation and proposals for using the immediate wider channel bandwidth for irregular bandwidths:

Observation 1: the irregular channel bandwidths requested up to now are FDD bands except only one SDL band.

Proposal 1: The asymmetric channel bandwidth consisting of SmallerCBW at UL and WiderCBW at DL may enable the method of using the immediate wider channel bandwidth.

Proposal 2: Apply the deviation to the Tx-Rx carrier centre frequency for the WiderCBW method as 
∆FWiderCBW=WiderCBW-SmallerCBW2

# Reference

RP-211545 SR: Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, Ericsson

RP-211468 WID: Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, Ericsson

R4-2107960, Email discussion summary for [99-e][150] FS_NR_eff_BW_util, Moderator(Ericsson)