3GPP TSG-RAN WG4 Meeting #100-e		R4-2112409

Electronic meeting, 16th – 27th Augest, 2021

Source:	Ericsson

Title:	Simulation results for the PUSCH demodulation performance requirements for 256QAM

Agenda item:	9.12.3.1

Document for:	Information

# Introduction

In this contribution, based on the simulation assumptions in [1], simulation results for the PUSCH demodulation performance requirements for 256QAM will be shown. The detailed simulation assumptions are listed in table 1.

Table 1	Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Bandwidth |  | 15kHz SCS: 5, 10 MHz 30kHz SCS: 10, 40 MHz |
| CP |  | Normal |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | {0, 2, 3, 1} |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | 1-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | PUSCH to DM-RS EPRE Ratio | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| TD-RA | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| FD-RA | RB assignment | Full applicable test bandwidth |
|  | FH | Disabled |
| CBG-based PUSCH transmission |  | Disabled |
| MCS |  | MCS#20, 21, 22, 24 |
| LBRM |  | Disabled |
| SRS |  | Not configured |
| Number of Tx antennas |  | 1 |
| Number of Rx antennas |  | 2, 4, 8 |
| Phase noise |  | No modeling |
| Tx EVM |  | No modeling |
| Channel model |  | TDLA30-10 Low |
| Timing offset, Frequency offset |  | 0, 0 |
| Test metric |  | 70% of max. throughput |

# Simulation results

Simulation results are summarized in table 2. Detailed performance curves can be found in the appendix.

Table 2	Simulation results

|  |  | 2Rx (1Tx) |  |  |  | 4Rx (1Tx) |  |  |  | 8Rx (1Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MCS#20 | MCS#21 | MCS#22 | MCS#24 | MCS#20 | MCS#21 | MCS#22 | MCS#24 | MCS#20 | MCS#21 | MCS#22 | MCS#24 |
| 15 kHz | 5 MHz | 16.7 | 17.6 | 18.4 | 20.5 | 13.1 | 14 | 14.8 | 16.8 | 9.8 | 10.6 | 11.4 | 13.5 |
|  | 10 MHz | 17.1 | 17.9 | 18.6 | 20.7 | 13.4 | 14.2 | 14.9 | 16.9 | 10 | 10.8 | 11.6 | 13.5 |
| 30 kHz | 10 MHz | 17 | 17.4 | 18.8 | 20.5 | 13.3 | 13.7 | 15 | 16.8 | 10.1 | 10.5 | 11.7 | 13.4 |
|  | 40 MHz | 17.7 | 18.1 | 19.1 | 21 | 13.7 | 14.1 | 15.1 | 17 | 10.3 | 10.7 | 11.8 | 13.7 |

# References

[1]	R4-2108667	WF on FR1 PUSCH 256QAM performance requirements	RAN4#99-e

# Appendix