3GPP TSG RAN WG4 Meeting #100-e	R4-2112558

Electronic Meeting, August 16-27, 2021

Agenda item:			9.23.2.3

Source:	vivo

Title:	Discussion on reference point for Te requirements

Document for:	Discussion

- Introduction

In the RAN4#98bis-e meeting, a reply RS [1] on UE transmit timing error to RAN1 was approved. However, there were different understandings of downlink reference timing in Te requirements. RAN4 agreed to further discuss the reference point definition.

In the RAN4#99-e meeting, there were further discussions on reference point for Te requirements. The related issues were captured in WF [2]. There were also CRs to change legacy R15/R16 Te requirements. No agreements were reached.

In this contribution, we provide our views on the reference downlink timing for Te requirements.

- Discussion

2.1 Te requirements

The legacy Te requirements related to reference downlink timing in TS 38.133 are as follows.

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.. The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

In the legacy Te requirements, the referenece downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. The requirements itself are clear enough and it is the correct definition based on how the requirements were defined.

In RAN4#85 meeting, the principle for defining initial UE transmit timing error requirements was agreed.

- Initial UE transmit timing error (Te):

- Principle:

  - For Te values, the downlink bandwidth of SSB and/or TRS and uplink SCS should be taken into account.

  - The different RF margins apply to different frequency ranges.

    - The margin decrease with the increasing SCS

    - The lower boundary of Te for each bandwidth/SCS should be expected

The principle imply Te comes from timing error of UE detected path due to resolution of SSB signals and RF margin at least. The components of Te are mainly downlink timing detection error due to resolution of reference signals which is SSB in defining Te requirements, RF margin for both DL and UL, and maybe UL jitter. Downlink timing detection error of SSB signals as proposed in [3] was used to derive Te requirements. All the other components are considered as margin. The error due to downlink timing detection error and margins are summarized in Table 1.

Table 1. Te requirements

| FR | SCS of SSB (KHz) | SSB BW (MHz) | SCS of UL (KHz) | Te (64*Tc) | Te,DL_BW (Ts) | Te, margin (Ts) | Te (ns) | Te,DL_BW (ns) | Te, margin (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 3.6 | 15 | 12*64*Tc | 4.267 | 7.733 | 391 | 139 | 252 |
|  |  |  | 30 | 10*64*Tc |  | 5.733 | 326 | 139 | 187 |
|  |  |  | 60 | 10*64*Tc |  | 5.733 | 326 | 139 | 187 |
|  | 30 | 7.2 | 15 | 8*64*Tc | 2.133 | 5.867 | 260 | 69 | 191 |
|  |  |  | 30 | 8*64*Tc |  | 5.867 | 260 | 69 | 191 |
|  |  |  | 60 | 7*64*Tc |  | 4.867 | 228 | 69 | 158 |
| 2 | 120 | 28.8 | 60 | 3.5*64*Tc | 0.533 | 2.967 | 114 | 17 | 97 |
|  |  |  | 120 | 3.5*64*Tc |  | 2.967 | 114 | 17 | 97 |
|  | 240 | 57.6 | 60 | 3*64*Tc | 0.266 | 2.733 | 98 | 9 | 89 |
|  |  |  | 120 | 3*64*Tc |  | 2.733 | 98 | 9 | 89 |

Since downlink timing detection error is derived based on resolution of downlink SSB signals, it has to be based on UE first detected path.

Observation 1: The Te requirements are derived based on UE first detected path.

In the reply LS [1], it was concluded RAN4 has the common understanding that downlink frame timing detection error is already included in UE transmit timing error (i.e. Te defined in section 7.1.2 in TS 38.133).

This may not be fully correct. For AWGN channel, the downlink timing detection error, including downlink path detection error and RF margin for DL RF chain are included in Te requirements. But for multipath fading channel the path detection error was not included.

In [4], similar analysis was made as follows.

while the UE performance in AWGN channel may be close to these values, the timing detection error caused by multipath channel is not taken into account, even though the error by multipath likely dominates.

The relationship of downlink timing detection error and Te can be illustrated in Fig 1.

It is noted that Te,DL in Fig 1 includes error of Te,DL-BW in Table 1 and RF margin for downlink RF chain and other downlink relevant timing errors.

Te,margin is Te minus Te,DL.

TUE, Rx, error is the entire downlink frame timing detection error, including all timing errors of downlink timing compared to reference downlink timing. It is supposed to be based on first path.

TUE,path-detection is the timing error between first path and UE first detected path.

Fig 1. Downlink timing detection error model

In summary, following further observations can be made.

Observation 2: Multipath detection error is not taken into consideration for downlink timing detection error in Te.

2.2 Reference point

There was proposal to change reference point for downlink timing. One is to change the reference path from UE first detected path to ‘detectable path’ or ‘first path’. The other is to clarify the reference point is UE antenna. We think it would be if it is further clarified that the corresponding downlink frame from the reference cell is received at the UE antenna. However, Te requirements are neither derived from ‘detectable path’ nor ‘first path’.

In Appendix, simulation results for multi-path detection under different channel models as specified in TS 38.101-4, which are TDL-A 30ns, TDL-B 100ns and TDL-C 300ns for FR1 and TDL-A 30ns and TDL-C 60ns for FR2, are provided. The simulations are performed under both -3dB and 3dB SNR, and 15kHz SCS for FR1 and 240kHz SCS for FR2.

For convenience, the results for TDL-B 100ns and 15kHz SCS are copied as below. Other results can be found in Appendix.

Fig 2. Downlink timing detection error in multi-path fading channel TDL-B 100ns, 15kHz SCS

For the ‘detectable path’, it cannot be used as reference downlink timing because it is totally ambiguous with detailed conditions. In Fig 2 it can be seen that most of the paths of TDL-B 100ns channel are detectable. However, RRM requirements are generally based on 90-th percentile of success rate. The multi-path detection error could be 45ns with 90-th percentile of success rate for TDL-B 100ns channel. This was not considered in Te requirements.

The multipath detection error is summarized in Table 2.

Table 2. Multipath detection error with -3 dB SNR

| TDL-A 30ns | TDL-B 100ns | TDL-C 300ns | TDL-A 30ns | TDL-C 60ns |
| --- | --- | --- | --- | --- |
| 15ns | 45ns | 190ns | 10ns | 40ns |

It can be seen obviously it is not possible to include the multipath detection error for TDL-C 300ns channel into Te requirements.

If ‘detectable path’ is based on 90-th percentile of multipath detection error, then the requirements don’t include most of the multipath detection error. Furthermore, the reference timing is ambiguous to UE. What UE can be used as reference downlink timing is the UE detected path. The detectable path would be changing from time to time and is unknown to UE.

Observation 3: ‘Detectable path cannot be used as reference downlink reference timing due to it is unknown to UE.

For the ‘first path’, which may be interpreted as ‘truly arrived’, it is not appropriate to define Te requirements. The ‘first path’ is unknow to UE and the uplink timing should anyway based on UE detected path. If ‘first path’ is used then Te requirements should be re-evaluated. At least the multipath detection error in Table 2 should be added to Te additionally.

Observation 4: ‘First path’ is also not appropriate to define Te requirements as it is unknown to UE and re-evaluations are needed if it is to be used as reference.

The ‘first detected path’ can be used by UE to adjust uplink timing accordingly. The Te requirements are crystal clear.

Changing legacy Te requirements has big impact on legacy UE. It is not expected to change any R15/R16 Te requirements.

In summary following proposals are present.

  - Proposal 1: No change to legacy Te requirements in terms of reference path.

  - Proposal 2. The reference point can be further clarified in Te requirements that the downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame from the reference cell is received at UE antenna.

2.3 Power delay compensation

Since Te requirements don’t include the entire downlink timing detection error, there may be impact on TA error budget calculation. One possible way to define enhance Te requirements for IIoT PDC. We think it would be feasible as PDC capable UE should outperform legacy UE in terms of reducing downlink timing error. The Te requirements may either based on first detected path or first path depending on how multipath detection error is counted in TA error budget.

  - Proposal 3: Enhance Te requirements for IIoT PDC can be considered.

- Summary

In this contribution we further provide our views on the reference downlink timing for Te requirements. Based on analysis following observations and proposals are present.

  - Observation 1: The Te requirements are derived based on UE first detected path.

Observation 2: Multipath detection error is not taken into consideration for downlink timing detection error in Te.

Observation 3: ‘Detectable path cannot be used as reference downlink reference timing due to it is unknown to UE.

Observation 4: ‘First path’ is also not appropriate to define Te requirements as it is unknown to UE and re-evaluations are needed if it is to be used as reference.

  - Proposal 1: No change to legacy Te requirements in terms of reference path.

  - Proposal 2. The reference point can be further clarified in Te requirements that the downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame from the reference cell is received at UE antenna.

  - Proposal 3: Enhance Te requirements for IIoT PDC can be considered.

- Appendix

The simulation results for multi-path detection under different channel models as specified in TS 38.101-4, which are TDL-A 30ns, TDL-B 100ns and TDL-C 300ns for FR1 and TDL-A 30ns and TDL-C 60ns for FR2, are provided respectively. The simulations are performed under -3dB and 3dB SNR.

Table B.2.1.1-2 TDLA30 (DS = 30 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Fig 3. Downlink timing detection error in multi-path fading channel TDL-A 30ns, 15kHz SCS

Table B.2.1.1-3 TDLB100 (DS = 100ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 10 | -2.2 | Rayleigh |
| 3 | 20 | -0.6 | Rayleigh |
| 4 | 30 | -0.6 | Rayleigh |
| 5 | 35 | -0.3 | Rayleigh |
| 6 | 45 | -1.2 | Rayleigh |
| 7 | 55 | -5.9 | Rayleigh |
| 8 | 120 | -2.2 | Rayleigh |
| 9 | 170 | -0.8 | Rayleigh |
| 10 | 245 | -6.3 | Rayleigh |
| 11 | 330 | -7.5 | Rayleigh |
| 12 | 480 | -7.1 | Rayleigh |

Fig 4. Downlink timing detection error in multi-path fading channel TDL-B 100ns, 15kHz SCS

Table B.2.1.1-4 TDLC300 (DS = 300 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Rayleigh |
| 2 | 65 | 0 | Rayleigh |
| 3 | 70 | -7.7 | Rayleigh |
| 4 | 190 | -2.5 | Rayleigh |
| 5 | 195 | -2.4 | Rayleigh |
| 6 | 200 | -9.9 | Rayleigh |
| 7 | 240 | -8.0 | Rayleigh |
| 8 | 325 | -6.6 | Rayleigh |
| 9 | 520 | -7.1 | Rayleigh |
| 10 | 1045 | -13.0 | Rayleigh |
| 11 | 1510 | -14.2 | Rayleigh |
| 12 | 2595 | -16.0 | Rayleigh |

Fig 5. Downlink timing detection error in multi-path fading channel TDL-C 300ns, 15kHz SCS

Table B.2.1.2-2 TDLA30 (DS = 30 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Fig 6. Downlink timing detection error in multi-path fading channel TDL-A 30ns, 240kHz SCS

Table B.2.1.2-3 TDLC60 (DS = 60 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -7.8 | Rayleigh |
| 2 | 15 | -0.3 | Rayleigh |
| 3 | 40 | 0 | Rayleigh |
| 4 | 50 | -8.9 | Rayleigh |
| 5 | 55 | -14.5 | Rayleigh |
| 6 | 75 | -8.5 | Rayleigh |
| 7 | 80 | -10.2 | Rayleigh |
| 8 | 130 | -12.1 | Rayleigh |
| 9 | 210 | -13.9 | Rayleigh |
| 10 | 300 | -15.2 | Rayleigh |
| 11 | 360 | -16.9 | Rayleigh |
| 12 | 520 | -19.4 | Rayleigh |

Fig 7. Downlink timing detection error in multi-path fading channel TDL-C 60ns, 240kHz SCS

AWGN

Fig 8. Downlink timing detection error in AWGN channel, 15kHz SCS

- References

- R4-2105850, Reply LS on UE transmit timing error, Huawei

- R4-2108368	WF on RRM for NR IIoT and URLLC, Nokia, Nokia Shanghai Bell

- R4-1713717 Initial transmit timing error in NR, Qualcomm

- R4-2110415	Propagation Delay Compensation Enhancements for Time Synchronization, Ericsson