3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2112684

# Electronic Meeting, 16th – 27th August 2021

Source:		Verizon, Samsung

Title:			TP for TR 37.827 for DC_2_n2-n77

Agenda item:		8.38.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include DC_2A_n2-n77 according to the request in [1].

Reference

[1] RP-211172, Revised WID: Power Class 2 for EN-DC with x LTE band + y NR TDD band

# Text Proposal

<Start of Text Proposal>

## 5.x	DC_2A_n2A-n77A

### 5.x.1	Transmitter Characteristics

#### 5.x.1.1	Maximum Output Power

Table 5.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.x.1.2	Configurations for EN-DC

Table 5.x.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n2A-n77A | DC_2A_n77A |

#### 5.x.1.3	Co-existence study

According to the PC2 coexistence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_2A_n77A, IMD2, IMD4 and IMD5 products fall into the band n2 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.x.2	Receiver Characteristics

#### 5.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.x.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.x.2.1.1-1 for this dual connectivity configuration.

Table 5.x.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_2A_n2A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 32.0 | FDD | IMD2 |
|  |  |  |  |  |  | 34.75 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 20.0 | FDD | IMD44 |
|  |  |  |  |  |  | 22.75 |  |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

#### 5.x.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.x.2.1.1-1.

<End of Text Proposal>