3GPP TSG-RAN WG4 Meeting #100-e		R4-2112721

Electronic meeting, 16th – 27th Aug, 2021

Agenda item:	8.7.3

Source:	ZTE Corporation

Title:	Optimization to NR FR2 configurations for intra-band non-contiguous CA

Document for:	Approval

1	Introduction

In TS 38.101-2 [2], the configurations for intra-band non-contiguous CA are listed with the items of DL CA configurations, UL CA configurations, Sub-blocks, (BWChannel,block) (MHz) and BCS. There are two tables for intra-band non-contiguous CA in clause 5.5A.2, one is with single CA bandwidth class defined in Table 5.5A.2-1 and the other is with multiple CA bandwidth classes defined in Table 5.5A.2-2.

In Rel-17, for Table 5.5A.2-1 it consists of 14 sub-blocks while for Table 5.5A.2-2 it consists of 12 sub-blocks in the configurations. Too many columns of sub-block in the table cause the width of table to be too wide and even out-of page. However, if we have a further look at the configuration table, we will find that the sub-block column in the table contains too little information, and its content can be deduced almost directly from the CA configuration.

In this paper, we’d like to share our views on possible optimization for intra-band non-contiguous CA.

- Discussion

In clause 5.5A.2, there is a note which states that sub-blocks belonging to a CA configuration can be in any order. In other words certain CA configuration acronym includes all sub-block arrangements which have exactly the same sub-block set. As an example, CA_n260(2G-3O) denotes CA_n260(2O-2G-O), CA_n260(G-3O-G) etc, but these are not listed in tables separately. In this sense, the configuration table does not explicitly list all the possible arrangement of sub-blocks.

There are two possible ways to optimize the configuration table. We slightly prefer to Option 2 since the sub-blocks can be deduced directly from the CA configuration and the sub-block arrangement is also not fully covered by Option1. Considering that not all the possible arrangement of sub-blocks can be explicitly shown in Option 1, there is no additional information in Option 1 than Option 2.

(Option 1) One column for all sub-blocks

| NR CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | Sub-blocks | (BWChannel,block) (MHz) | BCS |
| CA_n260(2G-3O) | - | CA_n260(2G) & CA_n260(3O) | 1000 | 0 |

(Option 2) No column for sub-blocks

| NR CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n260(2G-3O) | - | 1000 | 0 |

Observation 1:	 Too many columns of sub-block are redundant in the configuration table for intra-band non-contiguous CA which contain no additional information.

Proposal 1:	 To optimize the configuration table for intra-band non-contiguous CA in FR2, Option 2 of no column for sub-blocks is suggested. Take CA_n260(2G-3O) as an example, the suggested optimization for the configuration can be as follows.

(Option 2) No column for sub-blocks

| NR CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n260(2G-3O) | - | 1000 | 0 |

3	Conclusions

In this paper we discussed the possible optimization for intra-band non-contiguous CA in FR2. Based on the discussion, we have the following observation and proposal.

Observation 1:	 Too many columns of sub-block are redundant in the configuration table for intra-band non-contiguous CA which contain no additional information.

Proposal 1:	 To optimize the configuration table for intra-band non-contiguous CA in FR2, Option 2 of no column for sub-blocks is suggested. Take CA_n260(2G-3O) as an example, the suggested optimization for the configuration can be as follows.

(Option 2) No column for sub-blocks

| NR CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n260(2G-3O) | - | 1000 | 0 |

4	References

TS 38.101-2, NR User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone.