3GPP TSG-RAN WG4 Meeting #100-e	R4-2112759

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, T-Mobile USA

Title:	TP for TR 37.717-21-11to include DC_2_(n)66

Agenda Item:	8.15.2

Document for:	Approval

# Introduction

This contribution provides a text proposal to introduce DC_2_(n)66 in 37.717-21-11.

# Text Proposal

# ---Start of changes---

## 5.X	DC_2_(n)66

### 5.X.1	Operating bands for EN-DC

Table 5.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-(n)66 | CA_2-66 | n66 |

### 5.X.2	Configuration for DC

Table 5.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-(n)66AA | DC_2A_n66A | CA_2A-66A | n66A |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.X.3	Co-existence studies

Based on co-existence studies of CA_n2-n66, the followings are concluded: .

-	IMD3 generated by UL n66 might affect DL band 2.

-     IMD5 generated by UL band 2 might affect DL n66.

### 5.X.4		∆TIB and ∆RIB values

For DC_2_(n)66 the ΔTIB,c and ΔRIB,c values are derived from CA_n2-n66 combination.

Table 5.x.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.x.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.X.5	REFSENS requirements

For DC_2_(n)66, REFSENS are reused from the CA_n2-n66 and are given in the table below.

Table 5.3.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-(n)66AA | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |

---End of changes---

# Reference

- RP-211371, 37.717-21-11, Rel-17 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), V0.5.0