TSG-RAN Working Group 4 (Radio) meeting #100-e	  R4-2112828

Electronic meeting, 16 – 27 August 2021

Source:	Ericsson

Title:	ULFPTx and the TxD capability

Agenda item:	9.7.3.2

Document for:	Approval

# Background

Regarding the relation of the TxD with full power capability the way forward [1] states:

Clarify the relationship between transparent TxD capability signalling and full Tx power capability

- Proposals

  - Option 1: No dependency

  - Option 2: A UE can support only one of the two capabilities

      - Agreements:

  - FFS

In this contribution we consider the relation between full-power UL-MIMO and the TxD capability and the fallback behaviour of full-power modes that must virtualize antennas to meet the requirement of the advertised power class.

# Relation to TxD

First we reiterate that

Proposal 1: RAN4 is to confirm which multi-antenna features have UE behavior and performance that is unaffected by TxD capability.

A UE with two half power Tx chains supporting full power Mode 1 must virtualize the antennas to meet the advertised NR power class for the band (the ue-PowerClass field) when scheduled with DCI 0_0 (while recognizing that a UE could have ful-power TX chains and support Mode 1). Moreover, a UE using Mode 1 to attain full power according to the power class for PUSCH transmission must also use virtualization to meet the power class for PUCCH transmissions. Hence a UE supporting Mode 1 is likely to indicate TxD capability in addition.

A UE supporting full power Mode 2 can use virtualization to attain full power, but this shall not be required for UEs supporting Mode 2 with full-power TPMI. This UE can meet the power class for PUCCH transmissions without TxD.

The behaviour of full power UL-MIMO is not modified when the UE is indicating TxD capability in addition if

--  MPR is unaffected

-- SRS transmissions with usage ‘antennaSwitching’ is according to the full-power mode supported (i.e. antenna switching with a full-power PA with Mode 2 even if support of TxD capability is indicated)

For TxD, the UE should not virtualize SRS transmissions, that is, it should use one of its half-power PAs, see [2]. This implies a 3 dB lower power for SRS transmissions as compared to the Mode 2 case for the purpose of DL CSI acquisition. The antenna switching behaviour should follow that for the full-power UL-MIMO mode when configured (Mode 2 in this case).

The UEs shall be able to virtualize antennas with a full-power mode without support of the TxD capability.

Proposal 2: Clarify in the RAN2 specification that the ul-FullPowerTransmission capability is not conditioned on indication of txDiversity-16 for any full-power mode.

# Single-antenna fallback for FP Mode 1 and Mode 2

Before discussing the specification of the fall-back behaviour with single-antenna port transmissions it may be useful to discuss the expected behaviour for different modes. We assume a UE indicating PC2 in its NR band capability (the ue-PowerClass).

- ‘non-coherent’ (not FP), 1 SRS resource set with nrofSRS-Ports = 2 for all resources with use ‘codebook’

DCI 0_0 and DCI 0_1 with single antenna-port: 26 dBm per layer, no scaling, split over one antenna port with non-zero PUSCH, total power 26 dBm

The UE can use virtualisation (TxD) to attain full power (only 26 dBm per TX antenna)

-- There is no one-port SRS (only for Mode 2), so the network has to guess what the effective channel for one port looks like

- FP Mode 1, 1 SRS resource set with nrofSRS-Ports = 2 for all resources with use ‘codebook’

DCI 0_0 and DCI 0_1 the single antenna-port: 26 dBm per layer, no scaling, split over one antenna port with non-zero PUSCH, total power 26 dBm

The UE must use virtualization to attain full power (only 26 dBm per TX antenna)

-- There is no one-port SRS (only for Mode 2), so the network has to guess what effective channel for one port looks like

c. FP mode 2, 1 SRS resource set with two SRS resources one with nrofSRS-Ports = 1 and another with nrofSRS-Ports = 2 with use ‘codebook’

DCI 0_0 and DCI 0_1 the single antenna-port: 26 dBm per layer, no scaling, split over one antenna port with non-zero PUSCH, total power 26 dBm

The UE can use virtualisation to attain full power for Mode-2, virtualization is only supported for SRI indicating one SRS port

The UE can use virtualisation to attain full power with DCI 0_0 or DCI 0_1 on the single antenna-port, not needed if the UE supports a full-power TPMI ([1 0] or [0 1])

The modification of current specifications needed would be as follows:

< start of changes >

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1-1 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling unless otherwise stated. For UEs supporting ULFPTx Mode-1 or ULFPTx Mode-2 without full-power TPMI, the requirement in clause [6.2.1 for TxD] for the power class as indicated by the ue-PowerClass when the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 and ul-FullPowerTransmission is provided.

< end of changes >

UEs capable of Mode-2 with full-power TPMI (implemented with a full-power PA) would meet the requirements in 6.2.1. Mode-full power (Mode-0) is covered by the general fall-back requirement; the requirements in clause 6.2.1 are met at the antenna connectors.

It is remarked that the UE shall comply with the advertised power class for all transmissions.

# Proposal

We propose that

Proposal 1: RAN4 is to confirm which multi-antenna features have UE behavior and performance that is unaffected by TxD capability.

Proposal 2: Clarify in the RAN2 specification that the ul-FullPowerTransmission capability is not conditioned on indication of txDiversity-16 for any full-power mode.

Proposal 3: for single-antenna fallback for full-power modes the following exception for full-power modes: “For UEs supporting ULFPTx Mode-1 or ULFPTx Mode-2 without full-power TPMI, the requirement in clause [6.2.1 for TxD] for the power class as indicated by the ue-PowerClass when the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 and ul-FullPowerTransmission is provided.”

# References

1.    R4-2107740, “Way Forward on NR TxD & Power Class”, vivo

2.    R4-2112827, “SRS antenna switching with antenna virtualization”, Ericsson