3GPP TSG-RAN WG4 Meeting # 100-e                                                       R4-2112914

Electronic Meeting, Aug. 16- 27, 2021

Source:	ZTE Corporation

Title:	Templates on BCS4/5

Agenda Item:	8.28.2.2

Document for:	Approval

- 1	Introduction

- In last meeting, some agreements other than MSD for BCS4 were captured in the WF[1], in which:

- Proposal 1: N/A. (Agreement from Tuesday GTW is documented on slide 3) - Proposal 2: BCS4 is optional for a given combination, allocated as requested - Proposal 3: Option 3 which covers inter-band and intra-band - Proposal 4: Continue discussions for how to represent BCS4 for intra-band NR-CA at RAN4#100.

- According to the above agreements, it can be seen that there are still some open issues, such as signalling, template for intra-band NR CA.

- For the signalling issue, a LS to RAN2 [3] was approved to ask RAN2 to design the signalling by considering RAN4’s information/alternatives.

- To our understanding, except for the MSD issues, the open issue for BCS4 is mainly on the intra-band application. Meanwhile, we think the approved templates for BCS4 need to be updated slightly to include BCS5. Therefore, we give some further discussion on the above open issues in this paper,.

- 2	Discussion

## 2.1 Intra-band NR CA

How to apply BCS4 to intra-band NR CA was triggered in [2], however, no extensive discussions in last meeting. In our understanding, the situation is similar for SUL and inter-band NR CA, since the supported channel bandwidth are defined for each band associated with the corresponding BCS. However, for intra-band NR CA, different with inter-band NR CA, the supported bandwidths are specified for each CC which is restricted by CA bandwidth class. Therefore, it may not feasible to support all possible channel bandwidths for intra-band NR CA.

We take CA_n41C as an example:

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n41C | CA_n41C | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 190 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 40 | 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 80, 90 | 60, 80, 90, 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 190 | 2 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 30, 40 | 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 80, 90 | 60, 80, 90, 100 |  |  |  |  |  |

For class C, the range for the aggregated channel bandwidth is 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max. which means some channel bandwidths combination, such as 10MHz+10MHz, 10MHz+80MHz, etc,  cannot be supported by CA_n41C since it is out of the aggregated channel bandwidth range. Meanwhile, we can see the Maximum aggregated bandwidth for CA_n41C is 180MHz or 190MHz, which is less than  2 x BWChannel,max  (=200MHz), this may because 200MHz cannot be achieved considering the operator actual spectrum holding, that’s why 100MHz + 100MHz bandwidth combination is excluded for CA_n41C.

However, for some other band configurations, the upper value of Maximum aggregated bandwidth is equal to 2 x BWChannel,max..

Therefore, the sum of the supported channel bandwidth of each carrier is restricted not only by the aggregated channel bandwidth range, but also by the Maximum aggregated bandwidth, although Maximum aggregated bandwidth shall be less than or equal to the upper limit of the Aggregated channel bandwidth for the corresponding NR CA bandwidth class.

Table 1. A new single row for BCS4 or BCS5 for intra-band contiguous NR CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_nXC | CA_nXC | 50, 60, 80,100 | 60, 80, 100 |  |  |  |  | 200 | 0 |
|  |  | See nX channel bandwidths for each carrier in Table 5.3.5-1, where sum of the supported channel bandwidth of each carrier are compliance to [lower value, upper value] x |  |  |  |  |  | upper value | 4 or 5 |
| Note x: Lower value is the lower limit of the Aggregated channel bandwidth for the corresponding NR CA bandwidth class in table 5.3A.5-1. Upper value shall be less than or equal to the upper limit of the Aggregated channel bandwidth for the corresponding NR CA bandwidth class in table 5.3A.5-1. |  |  |  |  |  |  |  |  |  |

Proposal 1. For intra-band contiguous NR CA, a new single row for BCS4 or BCS5 are shown in tables 1.

## 2.2 Inter-band NR CA

The templates for BCS4 were agreed in last meeting. Due to the same functional BCS5 was introduced, and considering different release independent for BCS4 and BCS5, so it is proposed to update the agreed templates slightly to include BCS5, as shown in table 2.

Table 2. Updating for BCS5 for inter-band contiguous NR CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_nXA-nYA | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | See nX and nY channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 or 5 |
| CA_nXA-nYC | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | See nX channel bandwidths in Table 5.3.5-1 and nYC channel bandwidths in Table 5.5A.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 or 5 |

Proposal 2. For inter-band contiguous NR CA, updating for BCS5 are shown in tables 2.

- 3 Conclusion

- In this contribution, we provide some discussion on the templates for BCS4/5, especially for the intra-band contiguous NR CA. Also, to include the BCS5, it is necessary to update the existing BCS4 templates. The proposals are summarized below.

Proposal 1. For intra-band contiguous NR CA, a new single row for BCS4 or BCS5 are shown in tables 1.

Proposal 2. For inter-band contiguous NR CA, updating for BCS5 are shown in tables 2.

- 4	Reference

[1] R4-2107821, Way forward for general questions for BCS4 (excluding MSD), T-Mobile USA

[2] R4-2110432, Discussion on BCS4, ZTE Corporation

[3] R4-2110432, LS on NR CA capability for BCS5, Xiaomi