3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112961

# Electronic Meeting, 16 – 27 August, 2021

Source:	LG Electronics

Title:	Analysis MPR for inner region and EVM based on reverse IMD for PC1.5

Agenda item:	8.30.2.1

Document for:	Approval

1. Introduction

In RAN4 #99-e meeting, the WF [1] was approved to provide additional data in order to improve PC1.5 MPR, in next meeting.

In this contribution, we provide our EVM Re-measurement results and propose new Inner MPR values for PC1.5.

2. Measurement assumptions for MPR

- Antenna isolation of 10 dB

- Post PA loss of 4 dB

- Two 26 dBm Tx chains (NR)

- Equal Power on both transmit chains

- Various channel and allocation BWs, with focus on “worst case” allocations

- RB size, allocation position, waveform, and modulation should be the same between two transmitters

- Results for both CP-OFDM and DFT-S-OFDM are welcome, with the priority being CP-OFDM because it is expected to be worst case

- Determine back-off required to meet OOBE, ACLR and EVM specifications

- The purpose of the test is to check whether the inner RB MPR can be improved.

The following measurement assumptions for MPR have been captured in [3].

3. Measurement results

Our EVM Re-measurement results for PC1.5 are provided in this section. We focused on evaluating the inner RB MPR.

Table 1. PC1.5 EVM measurement results of DFT-s-OFDM by using 3GPP MPR (Table 6.2.2-2)

| Modulation |  | Increased EVM due to RIMD3 [%] |
| --- | --- | --- |
|  |  | Inner RB allocations (135RB@67) |
| DFT-s-OFDM | Pi/2 BPSK [30%] | + 0.75 |
|  | QPSK [17.5%] | + 1.23 |
|  | 16 QAM [12.5%] | + 0.5 |
|  | 64 QAM [8%] | + 0.97 |
|  | 256 QAM [3.5%] | + 0.62 |

Table 2. PC1.5 EVM measurement results of CP-OFDM by using 3GPP MPR (Table 6.2.2-2)

| Modulation |  | Increased EVM due to RIMD3 [%] |
| --- | --- | --- |
|  |  | Inner RB allocations (137RB@68) |
| CP-OFDM | QPSK [17.5%] | + 1.91 |
|  | 16 QAM [12.5%] | + 1.64 |
|  | 64 QAM [8%] | + 1.87 |
|  | 256 QAM [3.5%] | + 0.72 |

Table 3. PC1.5 EVM measurement results of DFT-s-OFDM by using 3GPP MPR (Table 6.2.2-2) with additional MPR relaxation values 1dB.

| Modulation |  | Increased EVM due to RIMD3 [%] |
| --- | --- | --- |
|  |  | Inner RB allocations (135RB@67) |
| DFT-s-OFDM | Pi/2 BPSK [30%] | + 0.07 |
|  | QPSK [17.5%] | + 0.48 |
|  | 16 QAM [12.5%] | +0.05 |
|  | 64 QAM [8%] | +0.19 |
|  | 256 QAM [3.5%] | + 0.24 |

Table 4. PC1.5 EVM measurement results of CP-OFDM by using 3GPP MPR (Table 6.2.2-2) with additional MPR relaxation values 1dB.

| Modulation |  | Increased EVM due to RIMD3 [%] |
| --- | --- | --- |
|  |  | Inner RB allocations (137RB@68) |
| CP-OFDM | QPSK [17.5%] | +0.57 |
|  | 16 QAM [12.5%] | + 0.32 |
|  | 64 QAM [8%] | +0.41 |
|  | 256 QAM [3.5%] | + 0.38 |

Observation 1: Our EVM measurements for PC1.5 can be found in Table 1, 2, 3, and 4.

Observation 2: Based on our EVM measurement results for PC1.5, 1dB additional relaxation is needed due to EVM.

|  | Total MPR relaxation |
| --- | --- |
| Inner RB allocations | 1dB Exception (256QAM modulation) : 1.5dB for 256QAM (it was already aligned with agreed MPR Table in WF) |

Table5. Proposed MPR for PC1.5

| Modulation |  | MPR (dB) |
| --- | --- | --- |
|  |  | Inner RB allocations |
| DFT-s- OFDM | Pi/2 BPSK | ≤ 1 |
|  | QPSK | ≤ 1 |
|  | 16 QAM | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |
|  | 256 QAM | ≤ 6 |
| CP-OFDM | QPSK | ≤ 2.5 |
|  | 16 QAM | ≤ 3 |
|  | 64 QAM | ≤ 4.5 |
|  | 256 QAM | ≤ 8 |

Proposal 1: Based on the observation 1 and 2, it is proposed to take Table 5 as MPR for PC1.5 UL-MIMO.

4. Conclusion

Observation 1: Our EVM measurements for PC1.5 can be found in Table 1, 2, 3, and 4.

Observation 2: Based on our EVM measurement results for PC1.5, 1dB additional relaxation is needed due to EVM.

Proposal 1: Based on the observation 1 and 2, it is proposed to take Table 5 as MPR for PC1.5 UL-MIMO.

Reference

[1] R4-2108116, “WF on MPR for PC1.5 smartphones and FWA, Qualcomm”

[2] R4-2105492, “Way Forward on PC1.5 MPR, T-Mobile USA”.

[3] R4-2005190, “Way Forward on MPR for PC1.5 UL MIMO and Tx div, T-Mobile USA”