# 3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113017

Electronic Meeting, August 16th - 27th, 2021

Source: 	vivo

Title: 	Discussion on impact of MPE requirements

Agenda Item:	9.19.2.2

Document for:	Discussion

Introduction

The RAN4 impact of further enhancements on MIMO has been discussed in RAN4#99-e, and a WF [1] has been agreed in which copied below:

- No transmission requirements will be specified for multi-panel UE in Rel-17 FeMIMO WI

- RAN4 will further study if any impact to the reception requirements for multi-panel UE in Rel-17 FeMIMO WI considering RAN1 status

- RAN4 will further study the UE RF requirements impact for MPE mitigation. Companies are encouraged to provide the initial analysis for UE RF impact for MPE in the next RAN4 meeting considering the latest RAN1 progress considering RAN1 status

- No UE RF impact for CSI enhancement and link recovery for FR2 serving cells in Rel-17 FeMIMO WI

In this contribution, the initial analysis of UE RF impact of RAN1 MPE mitigation is provided.

Discussion

- 2.1 RAN4 background

In Rel-16, the MPE enhancement was studied in UE RF enhancement for FR2. The reporting of P-MPR is finally agreed, and other schemes such dynamic duty cycle was not.

It should be noted that though the schemes and conclusions were made in RAN4, the main specification work such as the reporting scheme of P-MPR was actually in RAN2 scope, and RAN4 send the conclusions via LS [2][3]. The basic RAN4 specification changes are only P-MPR reporting mapping in 38.133 [4] and clarification note of P-MPR reporting in Pcmax related clause [5] as following:

NOTE 3:	MPE P-MPR Reporting, as defined in TS 38.306 [14], is an optional UE capability to report P-MPRf,c when the reporting conditions configured by gNB are met. This UE capability is applicable to all FR2 power classes.

In Rel-17, there is currently no specific work for further enhancement related to MPE scheme.

Observation 1: The MPE related enhancement discussed and concluded in RAN4 are focused on P-MPR reporting which is mainly defined in RAN2 specs with a few minor changes in RAN4 spec.

- 2.2 Current RAN1 status

In the feMIMO WI in RAN1, there is a scope for the multi-beam operation enhancement in which MPE mitigation is being discussed:

- Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

According to RAN1 Chairman’s notes, following agreements on MPE were made in RAN1#104b-e and no conclusion were made in RAN1#105-e:

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options:

- Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version

  - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported.

  - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting

  - FFS: how to determine the virtual PHR or the modified version.

- Opt 1D. {Rel.16 P-MPR based (beam/panel-level)}

  - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting

- Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)

  - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel

  - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR]

  - FFS: Whether/how to enhance existing beam reporting format to support Option 2A

  - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting

  - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated

  - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added

FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting

FFS: If differential report is supported when multiple UL beams are reported in the same report

The Opt 1A and opt 1D are mainly based on extension of P-MPR from a per-UE to per-beam or per-panel operation, the virtual PHR or related extension is also a possible supplementary information. Option 2A is trying to utilizing some current measurements and also some UL/DL symmetric effects to facilitate MPE mitigation. However, no conclusion has been made yet.

Observation 2: RAN1 discussion is still on-going and the main scheme involves extension P-MPR to per-beam or per-panel level.

- 2.3 RAN4 Impact analysis

As illustrated, RAN1 MPE mitigation discussion is still on-going and the main scheme involves extension P-MPR to per-beam or per-panel level.

Currently, RAN4 does not have any per-beam or per-panel level maximum power related definition. It means that even if P-MPR is extended to per-beam/per-panel level, it is still not clear how to use it. For example:

In 38.101-2, The P-MPR is used for the restriction of Pumax as following:

PPowerclass + PIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

In 38.213, there are implicit per-beam operation in the power control equation, however, the upper cap Pcmax is still defined by RAN4 and no per-beam / per-panel operation is involved.

Observation 3: Currently RAN4 does not have any per-beam or per-panel maximum power related definition, and this may impact the use of per-beam/per-panel P-MPR definition if agreed by RAN1.

One of the most straight forward thinking is to keep using per-beam/panel P-MPR for the per-UE limitation of maximum power, but this may still needs further discussion and also clarification of RAN1 if agreed.

Proposal 1: Discuss in RAN4 whether per-beam or per-panel P-MPR can be utilized in RAN4. Further conclusion and input from RAN1 is also needed.

For the P-MPR mapping, it seems that reusing Rel-16 conclusion should be ok, even if there are extensions to per-beam/panel.

For the option 2A based on SSBRI(s)/CRI(s)/L1-RSRP, no direct RAN4 impact is observed.

For Virtual PHR, RAN4 also does not have specific specification except PHR mapping. It should be also have no impact to RAN4.

Proposal 2: The possible modification to virtual PHR, and also scheme based on SSBRI(s)/CRI(s)/L1-RSRP does not have RAN4 impact.

Proposal 3: P-MPR mapping can be re-used even if extended to per-beam or per-panel.

Conclusion

In this paper, the initial analysis of UE RF impact of RAN1 MPE mitigation were provided.

Observation 1: The MPE related enhancement discussed and concluded in RAN4 are focused on P-MPR reporting which is mainly defined in RAN2 specs with a few minor changes in RAN4 spec.

Observation 2: RAN1 discussion is still on-going and the main scheme involves extension P-MPR to per-beam or per-panel level.

Observation 3: Currently RAN4 does not have any per-beam or per-panel maximum power related definition, and this may impact the use of per-beam/per-panel P-MPR definition if agreed by RAN1.

Proposal 1: Discuss in RAN4 whether per-beam or per-panel P-MPR can be utilized in RAN4. Further conclusion and input from RAN1 is also needed.

Proposal 2: The possible modification to virtual PHR, and also scheme based on SSBRI(s)/CRI(s)/L1-RSRP does not have RAN4 impact.

Proposal 3: P-MPR mapping can be re-used even if extended to per-beam or per-panel.

- References

[1] R4-2107882, WF on impact to RF requirements for FeMIMO, Samsung, RAN4#99-e

[2] R4-2005670, [Draft] LS on MPE enhancements, RAN4, RAN4#94-bis-e

[3] R4-2011733, LS on MPE enhancements, RAN4, RAN4#96-e

[4] R4-2011736, Introduction of the P-MPR 2 bits report mapping in 38.133, Interdigital, RAN4#96-e

[5] R4-2011735, Introduction of MPE related P-MPR operation in sub-clause 6.2.4, Interdigital, RAN4#96-e