3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113066

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	Regulatory requirements for full unlicensed band 5925-7125MHz

Agenda Item:	8.2.2

Document for:	Approval

Introduction

At RAN#92 Meeting, a new WI on introduction of operation in full unlicensed band 5925-7125MHz was approved in [1]. The objectives for the WI are,

-	Analyse regulatory requirements for countries/regions that have adopted license-exempt operation in the full 5925-7125MHz frequency range:

NOTE 1: The scope of the WI is Region 2 (Canada, Brazil, Peru, Chile) and Region 3 (South Korea) countries, regulatory parameters for which can be found from TR 37.890;

NOTE 2: Should more countries finalise the regulatory framework and/or a particular country changes the existing regulation for the full 5925-7125MHz frequency range, it can be considered within the timeline of this WI subject for further RAN discussions.

-	Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed.

-	Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed.

-	Define or update (if needed) system parameters such as channel bandwidths and channel arrangements.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the UE.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the BS.

In this contribution, we provide some analysis on the regulatory requirements.

Discussion

In Rel-16 for NR unlicensed operation, a new band n96 was defined according to the FCC Report and Order FCC 20-51. Standard-power operation with the use of an Automated Frequency Coordination (AFC) and low-power indoor operations are allowed per FCC 20-51. Rel-16 defines power class 5 with max output power 20 dBm together with NS_53 and NS_54 to cover the FCC regulatory requirements. The scope of the WI covers Canada, Brazil, Peru, Chile in Region 2 and South Korea in Region 3. The regulatory requirements for client devices are compared with FCC 20-51 as below.

Table 2-1: Additional requirements

| Country/NS | Permissible operation | Frequency range (MHz) | Maximum mean EIRP for in-band emissions(dBm) | Maximum mean power density (dBm/MHz) | Maximum mean density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| NS_53 | LPI | 5925 – 7125 | 24 | -1 | -27 dBm/MHz (f ≤ 5925, f ≥ 7125) |
| NS_54 | SP | 5925 – 6425 | 30 | 17 | -27 dBm/MHz (f ≤ 5925, f ≥ 7125) |
|  |  | 6525 – 6875 |  |  |  |
| Canada | SP | 5925-6875 | 30 | 17 | None |
| Canada | LPI | 5925 – 7125 | 30 | 5 | None |
| Canada | VLP | 5925 – 7125 | 14 | -8 | None |
| Brazil | LPI | 5925 – 7125 | 24 | -1 | -27 dBm/MHz |
| Brazil | VLP | 5925 – 7125 | 17 | -5 | -27 dBm/MHz |
| Peru | LPI | 5925 – 7125 | 24 | -1 | None |
| Chile | LPI | 5925 – 7125 | 24 | -1 | None |
| South Korea | LPI | 5925 – 7125 | 24dBm | 2dBm | -27 dBm/MHz |
| South Korea | VLP | 5925 – 7125 | 14dBm | 1dBm | -34 dBm/MHz |

Some observations are listed as below,

For Canada SP operation, it should be further confirmed if A-MPR is not needed since there is no out of band emission requirements and 17 dBm/MHz PSD will not be exceeded for PC 5 UE.

For Canada LPI operation, new NS may be needed to address in-band 5 dBm/MHz PSD limit for PC5 UE

For Canada VLP operation, new power class and new NS may be needed to address in-band 5 dBm/MHz PSD limit.

For Brazil LPI operation, NS_53 may be reused.

For Brazil VLP operation, new power class and new NS may be needed to address in-band -5 dBm/MHz PSD limit and out of band -27 dBm/MHz limit.

For Peru and Chile LPI operation, new NS may be needed to address in-band -1 dBm/MHz PSD limit for PC5 UE

For South Korea LPI operation, new NS may be needed to address in-band 2 dBm/MHz PSD limit and out of band limits for PC5 UE.

For South Korea VLP operation, new power class and new NS may be needed to address in-band 1 dBm/MHz PSD limit and out of band limits.

Conclusions

In the contribution, we provide some analysis on the regulatory requirements.

Proposals:

For Canada LPI operation, new NS may be needed to address in-band 5 dBm/MHz PSD limit for PC5 UE

For Canada VLP operation, new power class and new NS may be needed to address in-band 5 dBm/MHz PSD limit.

For Brazil LPI operation, NS_53 may be reused.

For Brazil VLP operation, new power class and new NS may be needed to address in-band -5 dBm/MHz PSD limit and out of band -27 dBm/MHz limit.

For Peru and Chile LPI operation, new NS may be needed to address in-band -1 dBm/MHz PSD limit for PC5 UE

For South Korea LPI operation, new NS may be needed to address in-band 2 dBm/MHz PSD limit and out of band limits for PC5 UE.

For South Korea VLP operation, new power class and new NS may be needed to address in-band 1 dBm/MHz PSD limit and out of band limits.

References

RP-211602, New WID for Introduction of operation in full unlicensed band 5925-7125MHz, Apple Inc.