3GPP TSG-RAN WG4 Meeting #100-e								          R4-2113067

Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	100 MHz channel bandwidth for NR-U

Agenda Item:	8.26.2.2

Document for:	Approval

Introduction

In this contribution, we discuss the requirements for 100MHz CBW in NR-U.

Discussion

Channel raster

In RAN4 Rel-16 discusion, we have agreed that 60MHz CBW might not have co-existence issue with WIFI for type A multi LBT sub-band access. Similarly, if using type A multi LBT sub-band channel access, there is also no coexistence issue for 100MHz CBW. The following channel raster for NRU with 100MHz bandwidth is proposed for n46 in Table 1.

Table 1 channel raster for 100MHz CBW in NRU

| Channel frequency (MHz) | ARFCN | Reference frequency (MHz) |
| --- | --- | --- |
| 5200 | 746668 | 5200.02 |
| 5300 | 753332 | 5299.98 |
| 5520 | 768000 | 5520 |
| 5540 | 769332 | 5539.98 |
| 5560 | 770668 | 5560.02 |
| 5580 | 772000 | 5580 |
| 5620 | 774668 | 5620.02 |
| 5640 | 776000 | 5640 |
| 5660 | 777332 | 5659.98 |
| 5680 | 778668 | 5680.02 |
| 5785 | 785668 | 5785.02 |
| 5865 | 791000 | 5865 |

Figure 1 channel raster for n46

Proposal 1: Channel raster for 100MHz CBW in NRU as listed in Table 1 is proposed for n46.

MPR

As discussed in [1], the MPR can be defined bandwidth agnostic, i.e. existing MPR definition can be applied for 100 MHz channel bandwidth.

Table 2 MPR evaluation in [1]

| Full allocation (15kHz) |  | 20M | 40M | 60M | 80M | 100M | Max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | CP-OFDM | 3.2 | 3.2 |  |  |  | 3.2 |
|  | DFT-S | 1.2 | 1.6 |  |  |  | 1.6 |
| Full allocation (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3 | 3.2 | 3.2 | 3.2 | 3.3 | 3.3 |
|  | DFT-S | 1.2 | 1.2 | 1.6 | 1.3 | 1.3 | 1.6 |
| Full allocation (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.2 | 3.3 |  |  |  | 3.3 |
|  | DFT-S | 1.8 | 2.1 |  |  |  | 2.1 |
| Full allocation (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3 | 3.2 | 3.3 | 3.3 | 3.3 | 3.3 |
|  | DFT-S | 1.7 | 1.8 | 2.1 | 2 | 2 | 2.1 |
| Interlace 0 (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3.4 | 3.4 |  |  |  | 3.4 |
|  | DFT-S | 2.3 | 2.4 |  |  |  | 2.4 |
| Interlace 0 (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3.5 | 3.3 | 3.2 | 3.2 | 3.2 | 3.5 |
|  | DFT-S | 2.3 | 2.3 | 2.3 | 2.3 | 2 | 2.3 |
| Interlace 0 (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.4 | 3.3 |  |  |  | 3.4 |
|  | DFT-S | 2.7 | 2.7 |  |  |  | 2.7 |
| Interlace 0 (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.3 | 3.3 | 3.3 | 3.2 | 3.2 | 3.3 |
|  | DFT-S | 2.7 | 2.7 | 2.6 | 2.4 | 2.4 | 2.7 |

Proposal 2: existing MPR values can be applied for 100 MHz channel bandwidth

Emission mask

The general mask in Figure.1 which also applies for 100MHz CBW for both BS and UE.

Figure.1 general spectrum emission mask in NRU

In the case of one or two non-transmitted 20 MHz channels between transmitted channels, existing emission mask applies for BS,

- for single non-transmitted channel in the middle, the emission mask of the puncture center is limited at -23dBr

- for two non-transmitted channels in the middle, the emission mask in the middle is floored at -25dBr

For 3 non-transmitted channels in the middle for 100MHz CBW, it is proposed to align ETSI BRAN mask.

- for 3 non-transmitted channels in the middle, the emission mask in the middle is floored at -25dBr

Proposal 3: The BS spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single non-transmitted channel in the middle, the emission mask of the puncture center is limited at -23dBr

- for multiple non-transmitted channels in the middle, the emission mask in the middle is floored at -25dBr

Conclusions

In this contribution, we discussed the RF requirements for NR-U 100 MHz CBW, and the following proposal was made:

Proposal 1: Channel raster for 100MHz CBW in NRU as listed in Table 1 is proposed for n46.

Proposal 2: existing MPR values can be applied for 100 MHz channel bandwidth

Proposal 3: The BS spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single non-transmitted channel in the middle, the emission mask of the puncture center is limited at -23dBr

- for multiple non-transmitted channels in the middle, the emission mask in the middle is floored at -25dBr

References

R4-201049,  Discussion on NR-U UE ACLR and MPR evaluation, Huawei, HiSilicon