3GPP TSG-RAN WG4 Meeting # 100-e	                                  R4-2113152

Electronic Meeting, August 16-27, 2021

Agenda Item:	9.11.2.3

Source:	Intel Corporation

Title:	Discussion on NCSG in NR

Document for:		Discussion

# Introduction

In the last meeting, the objective 3 of NR measurement gap enhancement WI [1] below has been discussed and a WF for the multiple concurrent measurement gap in Rel17 was also agreed [2].

In this contribution the following open issues will be further discussed.

- NCSG using scenarios

- NCSG patten

- How to configure NCSG pattern

- RRM requirements impacts

# Scenarios and Use Cases

In [2], the following agreements on NCSG using scenarios was achieved.

- In principle, NCSG can be used for intra-frequency measurements with MG, inter-frequency measurements with MG, inter-RAT measurements. - FFS on whether NW should configure the legacy MG rather than NCSG even UE can support both of them.

As agreed in the last meeting, for some general measurements (e.g. gap-based intra-f measurement and inter-f measurement) both the legacy and NCSG can be used for the capable UEs. That is which types of MGs configured to UE is completely up to the network.

Observation 1: For UE which can support NCSG which types of MG (legacy MG or NCSG) shall be configured is up to NW implementation

Additionally, as we also agreed the explicit configuration for NCSG used, which one was configured by NW can be completed independent. Therefore, we can conclude that:

Proposal 1: From RAN4 RRM requirement perspective, there is not any restrictions on the types of MG configured.

# NCSG Pattern

As we agreed in the last meeting, the NCSG pattern design principle can be summarized as:

- Define NCSG patterns for subset of the legacy MG patterns in [TS38.133 v16.5.0]. - FFS on which subset of legacy MG patterns - FFS on RAN4 needs to define separate NCSG patterns for sync and async scenarios - Option 1 (Ericsson, Nokia, Intel, Qualcomm): Yes - Different NCSG patterns for synchronous and asynchronous operations in FR1 - Same NCSG patterns for synchronous and asynchronous operations in FR2. - Option 2 (ZTE, OPPO, Huawei,MTK,vivo, CATT): No - No need to separate NCSG patterns needed for synchronous and asynchronous operations. - Option 3 (CMCC, MTK, vivo, Apple): same NCSG patterns for synchronous and asynchronous operations, provided that the NCSG pattern only comprise the RF retuning time and ML. Interruption is not captured in VIL(RRT) and specified separately.

With these agreements, the more detailed design on NCSG pattern can be provided below.

# Separated NSCG pattern for sync and async scenario

In our view, for async case even the RTT time is same as that of part of sync case the left effective measurement window may be longer than that of sync. That is it is more efficient to distinct these two at least in FR1 the difference of the effective measurement window is large enough.

Observation 2: Different NCSG patterns are needed for synchronous and asynchronous operations in FR1

# General NCSG design principles

In the last meeting, whether the legacy gap patterns in Rel16 can be used for NCSG was extensively discussed. The main purpose of such reusage is to avoid the unnecessary specification works. Therefore, the following options will be FFS.

- Define NCSG patterns for subset of the legacy MG patterns in [TS38.133 v16.5.0]. - FFS on which subset of legacy MG patterns

However, since the parameters for NCSG pattern include “VIL, ML, VIRP” instead of “MGL, MGRP” of legacy MG, how the legacy pattern can be reused shall be clarified. For an example, it is clear that the MGRP of legacy MG shall be reused as VIRP of NCSG. But for the measurement length (ML) there were still diverse views below.

    - Option1-1: ML=MGL-VIL1-VIL2

    - Option1-2: ML=legacy MG window length

And regarding to agreements in R4#98b-e below, the MGL of legacy MG is reused as ML+VIL1+VIL2 of NCSG

- ML: If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2

Proposal 2: For the NCSG pattern which reused from the legacy MG pattern, the dependency of the MG parameters can be:

- VIRP = MRGP

- ML+VIL1+VIL2=MGL

The main concern for NCSG pattern is which legacy gap patterns can be reused. This is also up to the main using scenario of NCSG. However, in our view, regarding to the real using scenarios, some of patterns can be feasible indeed. At least #24, 25 couldn’t be used. The other patterns may be not feasible. For an example, the gap patterns which have the quite shorter MGRP/VIRP (e.g. 20ms), in case of SCell measurement with “measCycleSCell” which shall be larger than 160 slots [4], it seems no necessary to defined the same NCSG. Also for the gap pattern with shorter MGL(e.g. #10, #11), as MGL= VIL1+VIL2+ML if VIL1 and VIL2 is 1ms, the measurement window will be only 1ms. Whether is such gap useful for NCSG shall be also checked.

Observation 3: Some of legacy MG patterns is infeasible to be reused for NCSG because of too short measurement window length left excluded VIL.

Therefore, we can safely propose that:

Proposal 3:  Reuse part of the legacy MG patterns in [2] only as the new NCSG patterns in NR.

# Visible Interruption Length (VIL)

Secondly, regarding introduction of NCSG, another critical issue is the feasible NCSG patterns definition including Visible Interruption Length (VIL) , Measurement Length (ML), and Visible Interruption Repetition Period (VIRP) as defined in LTE [3, TS36.133].

- FFS on VIL: - Option 1a (Qualcomm, Intel, Ericsson, vivo, ZTE, OPPO): VIL should be explicitly defined based on the number of interrupted durations in absolute time - Option 1b (Apple, CATT, Qualcomm, ZTE): VIL should be explicitly defined based on the number of interrupted duration in slot - Option 2( MTK, Huawei, CMCC, Nokia): based on absolute RF retuning time (tentatively denoted as “RRT”).

In principle, how to decide VIL depends on the feasible RF retuning time which could introduce the unexpected interruption to the serving carriers/cells. As in NR Rel15, the assumption on the duration of RF retuning in LTE/NR PCell/PSCell can be less than 500us and 250us for FR1 and FR2 respectively. In our view, in Rel17 the same assumption can be reused.

In our views, Option 1a, 1b are more reasonable as the interruption need consider more other parameters beside RF returning time (e.g. MGTA). But regarding to VIL defined with slot need the specific NCSG pattern definition depending on the numerology, we prefer Option 1a.

In addition to the interruption in case of both synchronous and asynchronous DC are different in MR DC, the VIL in NR NCSG for the different FR can be defined as:

|  | VIL in Sync (ms) | VIL in Async(ms) |
| --- | --- | --- |
| FR1 | 1 | 2 |
| FR2 | 0.75 |  |

# Measurement Length (ML)

In the last meeting, the following agreement was achieved.

- FFS on ML of NCSG (or the total length of NCSG). - Option 1: the total length of NCSG (“ML + VIL1+VIL2”) is same as MGL of the legacy gap - Option 2: the total length of NCSG (“ML + VIL1+VIL2”) is larger than MGL of the legacy gap and the effective measurement window of NCSG (which is equal to ML) is same as “legacy MGL – 2 *RRT)”.

Technically speaking, if following Option 2. The total gap length of NCSG can be larger than that of legacy of MG. At a same time the periodicity of them are same. Thus the overhead of NCSG is a little bit higher than that of legacy MG, which is not preferable by the network from overhead perspective.

Also, in R4#98b-e the following agreements was achieved. Since in the last meeting, the concenus on the reuseing of legacy MG for NCSG. Therefore, we can see the Option 1 is quite straightforward way to define the NSCG with the legacy MG>

- ML: If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2

On the other hand, if the ML + VIL1+VIL2 is equal to the legacy MGL, some restriction on the legacy MG patterns shall be considered. For example, if the effective MG window is less than the possible SMTC.

That is measurement length is highly dependent on measurement object configuration. For an example, for SSB measurement gap, ML is up to SMTC configuration. Hence ML defined in NR legacy gap patterns [2, TS38.133] can be feasible.

# Visible Interruption Repetition Period (VIRP)

VIRP can rely on the SCell measurement cycle configure for SCell. And regarding to such NCSG will be applicable for all SCell carriers, if there are multiple deactivated SCells to be measured, more than one NCSG are needed. That is the VIRP can be less than measCycleSCell. Meanwhile, it is better to align these SMTC window for multiple SCells.

In conclusion, given the principles of NCSG pattern above, the feasible NCSG patterns can be listed in the table below.

Table 9.1.2-1: Gap Pattern Configurations [2,TS 38.133]

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

On the other hand, from the technical perspective, some of these legacy patterns are infeasible, which are also highlighted in the table above. For an example, the gap patterns which have the quite shorter MGRP/VIRP (e.g. 20ms), in case of SCell measurement with “measCycleSCell” which shall be larger than 160 slots [4], it seems no necessary to defined the same NCSG. And for gap#24, 25 which is used for PRS measurement, can be not in the candidate NCSG patterns.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected …… measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL -- Need R ]], [[ smtc3list-r16 SSB-MTC3List-r16 OPTIONAL, -- Need R rmtc-Config-r16 SetupRelease {RMTC-Config-r16} OPTIONAL, -- Need M t312-r16 SetupRelease { T312-r16 } OPTIONAL -- Need M ]] }

In summary, considering the analysis on VIL, ML and VIRP above, the several new NCSG patterns based on the legacy gap pattern (e.g. MGRP) are defined in the table below.

Table 2: Gap Pattern Configurations for NCSG

| Gap Pattern Id | VIL1 (MGL, ms) | ML | VIL2 | Measurement Gap Repetition Period (VIRP, ms) |
| --- | --- | --- | --- | --- |
| ncsg0 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 40 |
| ncsg 1 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 80 |
| ncsg 2 | 1, sync | 3 - 2*1, sync | 1, sync | 40 |
| ncsg 3 | 1, sync | 3 - 2*1, sync | 1, sync | 80 |
| ncsg 5 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 160 |
| ncsg 7 | 1, sync | 4 - 2*1, sync | 1, sync | 40 |
| ncsg 8 | 1, sync | 4 - 2*1, sync | 1, sync | 80 |
| ncsg 9 | 1, sync | 4 - 2*1, sync | 1, sync | 160 |
| ncsg 11 | 1, syn | 3 - 2*1, sync | 1, sync | 160 |
| ncsg 13 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 40 |
| ncsg 14 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 80 |
| ncsg 15 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 160 |
| ncsg 17 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 40 |
| ncsg 18 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 80 |
| ncsg 19 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 160 |

# RRM Requirements Impacts

If NCSG is used for the measurement for UE who can perform with multiple RF chains, the RRM requirements with NCSG shall be investigated. For an example, the interruption requirements shall be updated if NCSG is used. (e.g. interruption for SCell activation in 8.2.1.2.5.1 of TS38.133[2] and 7.32.2.6 in TS36.133)

In the last meeting, there is some preliminary discussion on these requirements.

- FFS on Interruption requirements - Option 1: The interruption requirements in TS38.133 and TS36.133 shall be revisited - Option 1a. for UE supporting per-FR gap, VIL is allowed only on the serving cell in the same FR wherein there is NCSG operation. Otherwise, VIL is allowed on all serving cells. - Option 2: Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements - Option 3: the interruption is proposed as following SCS; Synchronous; Asynchronous. interruption length before measurement; interruption length after measurement; interruption length before measurement; interruption length after measurement. 15KHz SCS; 1 slot; 1 slot; 2 slots; 2 slots. 30KHz SCS; 2 slots; 2 slots; 3 slots; 3 slots. 60KHz SCS; 3 slots; 3 slots; 4 slots; 4 slots. 120KHz SCS; 6 slots; 6 slots; 7 slots; 7 slots. - Option 4: RAN4 to further discuss the condition, capability and impacts to measurement requirements for UE to use NCSG to control interruptions due to measurement on deactivated SCC or Scell in dormancy

In our views, the options above are not contradict with each other, which are targeted to the different aspects of RRM requirements in TS38.133. For an example, Option 1 & 2 are for the requirements defined in 8.2. of TS38.133 [] but Option 3 for the interruption slot in 9.1.2 of TS38.133.

Observation 4: the interruption requirements defined in TS38.133 impacted due to NCSG shall be clarified.

From the interruption requirements due to measurement gap operation defined in 8.2. of TS38.133, it is common understanding that the new interruption due to NCSG shall be defined. And we can also to agree to identify the potential requirements impact in high level firstly. The detailed requirements can be FFS after the gap pattern design is settle down.

Proposal 4: The interruption requirements during measurements on SCC defined in TS38.133 and TS36.133 shall be revisited because of NCSG is used.

On the other hand, whether the VIL is identical with the interrupted slots of serving cells [Table 9.1.2-4 and 9.1.2.-5] in TS38.133 can be FFS up to the basic NCSG pattern design finalized.

Proposal 5: the interrupted slots of serving cells (Table 9.1.2-4 and 9.1.2.-5 in TS38.133) can be FFS up to the basic NCSG pattern design finalized.

# Signaling

As stated in [1], UE measurement via the NCSG required RAN2 to design the necessary signaling support. Regarding to in Rel16 “Needforgap” singaling was introduced [4]. In our view, this signaling can indicate the gap for each band/carrier. The similar signaling of needForGap can be applicable for NR NCSG also.

Proposal 6: The “NeeForGap” signaling structure can be reused for NR NCSG as a start point.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI are discussed. The proposals can be drawn as:

Observation 1: For UE which can support NCSG which types of MG (legacy MG or NCSG) shall be configured is up to NW implementation

Proposal 1: From RAN4 RRM requirement perspective, there is no any restrictions on the types of MG configured.

Observation 2: Different NCSG patterns are needed for synchronous and asynchronous operations in FR1

Proposal 2: For the NCSG pattern which reused from the legacy MG pattern, the dependency of the MG parameters can be:

- VIRP = MRGP

- ML+VIL1+VIL2=MGL

Observation 3: Some of legacy MG patterns is infeasible to be reused for NCSG because of too short measurement window length left excluded VIL.

Proposal 3:  Reuse part of the legacy MG patterns in [2] only as the new NCSG patterns in NR.

Observation 4: the interruption requirements defined in TS38.133 impacted due to NCSG shall be clarified.

Proposal 4: The interruption requirements during measurements on SCC defined in TS38.133 and TS36.133 shall be revisited because of NCSG is used.

Proposal 5: the interrupted slots of serving cells (Table 9.1.2-4 and 9.1.2.-5 in TS38.133) can be FFS up to the basic NCSG pattern design finalized.

Proposal 6: The “NeeForGap” signaling structure can be reused for NR NCSG as a start point.

# References

- RP-202658 Revised WID on NR and MR-DC measurement gap enhancements

- TS38.133 v16.8.0

- TS36.133 v16.5.0

- TS38.331 v16.2.0