3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113250

# Electronic Meeting, Aug. 16-27, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U discussion and simulation results

Agenda item: 	6.1.1.7.4

Document for: 	Discussion

Introduction

This paper presents updated simulation results for operation on unlicensed bands [1].

Simulation assumptions

Table 1 Simulation assumptions for PUCCH format 3 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols and UCI information bits | 4 bits and 4 symbols |
| Channel bandwidth | 20MHz |
| SCS | 15kHz 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | n0 |
| Length of OCC | n1 |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |
| Bit pattern | [1 1 1 1] including HARQ-ACK information only |

Table 2 CG-UCI simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | TDLA30-10 |
| Bandwidth |  | 20 MHz |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| MCS |  | MCS16, MCS 20 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain | PUSCH mapping type | A,B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 1 interlace |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
|  | Number GC-UCI information bit payload | 18 |
|  | Scaling | 1 |
| UCI | betaOffsetGC-UCI | 8 |
|  | UCI partition for frequency hopping | Disabled |

Simulation results

Table 3 Simulation results for interlaced PUCCH format 3

| Format | Propagation conditions | SCS | UCI bits | OCC length | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 3 | TDLA30-10 | 15 kHz | 4 | 1 | 1% ACK miss | -8.2 | -5.7 |
|  |  |  |  |  | 1% UCI BLER | -9.9 | -7.4 |
|  |  | 30 kHz | 4 | 1 | 1% ACK miss | -7.1 | -4.6 |
|  |  |  |  |  | 1% UCI BLER | -8.9 | -6.4 |

Table 4 CG-UCI simulation results for MCS 20

| MCS | SCS | Mapping type | IoffsetCG-UCI | CG-UCI SNR@BLER=1% |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | impairment |
| MCS 20 | 15 kHz | A | 8 | 2.8 | 5.3 |
|  |  | B | 8 | 3.3 | 5.8 |
|  | 30 kHz | A | 8 | 3.3 | 5.8 |
|  |  | B | 8 | 4.2 | 6.7 |

Conclusion

In this contribution we provided updated simulation results for interlaced PUCCH format 3 and CG-UCI multiplexed on PUSCH for the operation in unlicensed bands.

References

- R4-2108515, Way forward on NR-U BS demodulation requirements, Huawei, HiSilicon

- R4-2106010, WF on NR-U BS demodulation requirements, Huawei

- R4-2103989, Way forward on PUCCH demodulation requirements, 3GPP TSG-RAN WG4 Meeting #98-e, Ericsson

- R4-2103988, Way forward on NR-U BS demodulation requirements for general part and PUSCH, Huawei, HiSilicon