3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113270

# Electronic Meeting, August 16-27, 2021

Agenda item:	9.8.2.1.2

Source:	OPPO

Title:	Inter-frequency measurement requirements for Rel17 FR1 HST

Document for:	Discussion

- Introduction

In last RAN4 meeting, a WF on FR1 HST RRM was approved [1]. In this contribution, we will provide our views on the remaining issues.

- Discussion

- Whether define the enhancement for inter-frequency measurement in idle mode for HST - Option 1 (HW, OPPO, QC, Ericsson, Xiaomi, CATT, CMCC, vivo): Yes - Option 2 (MTK, Nokia): No - Option 3 (Apple): introduce a dedicated UE capability indicating the support of inter-frequency measurement in idle mode for HST

As RAN4 has agreed on RRC connected state inter-frequency measurement enhancements, it would be also fine with IDLE mode inter-frequency enhancement. We suggest it is also optional UE capability to be introduced for supporting HST inter-frequency measurement enhancement, including RRC connected and Idle state.

Proposal 1: Introduce HST inter-frequency measurement enhancement for both RRC Connected and Idle state.

As agreed, for inter-frequency measurement without MG in RRC connected, the same enhancement for intra-frequency measurement without MG in R16 HST could be reused. For idle state, the R16 enhanced EUTRA-NR inter-RAT measurement requirements could be reused for NR inter-frequency measurements in idle mode.

Proposal 2: If proposal 1 was agreed, the R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements in idle state.

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

For measurement delay, the requirement for inter-frequency measurement with MG in HST in connected state for HST, the enhanced requirements specified for LTE-NR inter-RAT measurements in R16 HST could be also used as baseline.

Proposal 3: The measurement delay requirements for LTE-NR inter-RAT measurements in R16 HST could be also used as baseline for inter-frequency measurement in connected state in R17 HST.

- Principle on the requirements if there are HST inter-frequency layers and non-HST inter-frequency layers to be measured - Option 1 (HW, QC, Ericsson, Apple, CATT, vivo): If RAN4 decided to specify inter-frequency measurement enhancement in FR1 HST, the requirements shall follow the principle: NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf - Option 2 (OPPO, MTK, Nokia): FFS

Both HST and non-HST inter-frequency layers to be measured should be considered. If RAN4 decided to specify inter-frequency measurement enhancement in FR1 HST, the requirements shall follow the principle:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

Proposal 4: Agree the principle of requirements for nter-frequency measurement enhancement in FR1 HST:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

- Conclusion

In this contribution, we provide our views on R17 HST enhancement in FR1.

Proposal 1: Introduce HST inter-frequency measurement enhancement for both RRC Connected and Idle state.

Proposal 2: If proposal 1 was agreed, the R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements in idle state.

Proposal 3: The measurement delay requirements for LTE-NR inter-RAT measurements in R16 HST could be also used as baseline for inter-frequency measurement in connected state in R17 HST.

Proposal 4: Agree the principle of requirements for nter-frequency measurement enhancement in FR1 HST:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

References

- R4-2108341, WF on FR1 HST RRM, CMCC

- R4-2110060, RRM requirement for Rel17 FR1 HST, OPPO