3GPP TSG-RAN WG4 #100-e	R4-2113336

Electronic meeting, 16th August – 27th August 2021

Source: 	Ericsson

Title:  	Interruption requirements for extending NR operation to 71GHz

Agenda Item:	9.16.7.3

Document for:	Discussion

# Introduction

The WF for extending current NR operation to 71 GHz [1] was approved in RAN#99-e meeting.  In this contribution, we present Interruption requirements for extending NR operation to 71GHz.

# Disucssion

According to [1], below issues of impact of higher SCS on RRM requirements were agreed:

- Define new RRM requirements due to higher data/SSB SCS for at least the following topics: - Timing - UE transmit timing - Timing advance (TA) - Interruptions - Active BWP switching delay - Measurement gaps - Interruption time - Study impact on RRM requirements due to higher SSB SCS for at least the following topics: - Intra-frequency measurement - Inter-frequency measurement - Other RRM requirements, if identified are not precluded.

Interruption requirements:

Observation 1: The SCS (e.g., 480 kHz and 960 kHz) applicable for 52.6 – 71 GHz and their corresponding interruption lengths can be included in the tables on the interruption requirements.

Taking Table 8.2.1.2.1-1 as example, 480 kHz and 960 kHz are added following same rules before.

Table 8.2.1.2.1-1: Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |
| 4 | 0.03125 | 17 |  |
| 5 | 0.015625 | 33 |  |

Proposal 1: Below three interruptions are prioritised as follows, taking possible application scenarios into account we proposed:

- EN-DC Interruption

Add SCS (e.g., 480 kHz and 960 kHz)

- SA: Interruptions with Standalone NR Carrier Aggregation

Add SCS (e.g., 480 kHz and 960 kHz)

- NR-DC: Interruptions

Add SCS (e.g., 480 kHz and 960 kHz)

# Conclustion

Observation 1: The SCS (e.g., 480 kHz and 960 kHz) applicable for 52.6 – 71 GHz and their corresponding interruption lengths can be included in the tables on the interruption requirements.

Proposal 1: Below three interruptions are prioritised as follows, taking possible application scenarios into account we proposed:

- EN-DC Interruption

Add SCS (e.g., 480 kHz and 960 kHz)

- SA: Interruptions with Standalone NR Carrier Aggregation

Add SCS (e.g., 480 kHz and 960 kHz)

- NR-DC: Interruptions

Add SCS (e.g., 480 kHz and 960 kHz)

# References

[1] R4-2108354, WF on NR Ext to 71GHz – RRM, Qualcomm