3GPP TSG-RAN WG4 Meeting # 100-e                             R4-2113404

Electronic Meeting, 16– 27 August, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on DC_8A-20A_n28A

Agenda Item:	8.6.1

Document for:	Approval

Introduction

Based on the approved WF [1] in RAN4#99, companies are encouraged to provide the estimation the IMD3 caused by Tx band 20 + band n28 which may fall into Rx of band 8. Besides, the feasibility of 3 low bands in smart phone or FWA is FFS. In this paper, we’d like to provide some analysis for these issues.

Discussion

This inter-band ENDC combination is subjected to the IMD3 of Tx band 20 + band n28 which may fall into Rx of band 8. We use antenna topology as RF front-end architecture in our analysis, as shown in figure 1. The component linearity assumptions are listed in table 1, and the component attenuation and isolation values are listed in table 2.

Figure 1 The RF front-end architecture

Table 1 General linearity parameters

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer/Duplexer | 74 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

Table 2 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| n28 Tx attenuation at Band 8 Rx | 27 |  |
| B20 Tx attenuation at B8 RX | 43 |  |
| The Triplexer isolation between band 20 Tx and band n28A Tx | 50 |  |

Table 3 and Table 4 summarize the MSD analysis results for DC_8A-20A_n28A.

Table 3 MSD calculation for victim Rx of band 8

| Component | IMD3(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| n28 PA Forward mixing interference | -98.4 | -98.4 |
| B20 PA Forward mixing interference | -78.2 | -78.2 |
| n28 PA Reserve mixing interference | -122 | -122 |
| B20 PA Reserve mixing interference | -86.5 | -86.5 |
| n28 Triplexer | -195 | -195 |
| B20 Triplexer | -140 | -140 |
| B8 LNA via filters | -77 | -77 |
| B8 LNA via PCB coupling | -86 | -86 |
| Diplexer (L-H) | -121 | -121 |
| Antenna switch | -83 | -83 |
| Total | -73.47 | -73.47 |

Table 4 MSD requirements for DC_8-20_n28 with UL band 20 and n28

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A-20A_n28A | 8 | 899 | 5 | 25 | 946 | 14.8 | IMD3 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 773 | N/A | N/A |

Proposal: It’s proposed to specify band 8 MSD due to IMD3 of Tx band 20 + band n28 as table 4.

- References

[1]	RP-211371, “New WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, HiSilicon