3GPP TSG-RAN WG4 Meeting #100-e	R4-2113457

Electronic Meeting, 16 August – 27 August 2021

Agenda Item:	9.9.5.2

Source:	Ericsson

Title:	UE demodulation requirements for HST FR2

Document for:	Discussion

# 1	Introduction

RAN4#99-e agreed with the way forward on the UE/BS demodulation requirements for HST FR2 [1]. This contribution discusses the open issues related to UE demodulation requirements.

# 2	Discussion

## 2.1	Deployment scenario

PDSCH requirement for Uni/Bi-directional scenario in scenario A and scenario B​ - Option 1: Define PDSCH requirement with Uni/Bi-directional scenario for both A and B, Define the test applicability rule to reduce the test effort​ - Option 2: Define requirements for both scenario A/B, and Uni/Bi-directional deployment, and not define any applicability between​ - Option 3: Consider output of FR2 HST deployment scenario discussion whether to cover scenario A ​ - Option 4: RAN4 define two test cases for HST FR2​ - Test 1: HST single tap (Uni-directional) with scenario A​ - Test 2: DPS (Uni-directional) with scenario B​ - If RAN4 agree to consider both Uni-directional and Bi-directional deployment, either test 1 or 2 apply Bi-directional model

Firstly, option 4 is not valid because RAN4 agreed not to define PDSCH requirement with HST single-tap channel​ for FR2 HST WI, according to WF [1].

To narrow down the option, we have compared the PDSCH demodulation performance of HST-DPS with uni-directional deployment model and bi-directional deployment model. Figure 1 shows our simulation results based on the simulation assumption and channel model in Appendix. From the simulation results, it is observed that Scenario B shows slight better performance compared with Scenario A in higher SNR for both uni-directional and bi-directional. However we should point out there is no significant difference with regard of the required SNR to achieve 70% of the maximum throughput.

Observation: No performance difference with regard to the 70% of maximum throughput among the uni-directional HST-DPS deployment scenario and bi-directional HST-DPS deployment scenario regardless of Scenario A and B.

| (a) HST-DPS Uni-directional deployment | (b) HST-DPS bi-directional deployment |
| --- | --- |

Figure 1	PDSCH simulation results for HST-DPS for FR2 SCS=120kHz.

RAN4 also discusses the deployment scenario in another sub agenda in this WI. Since the deployment scenario affects to both UE and BS demodulation requirements, we prefer option 3 – we should follow the conclusion in the deployment scenario discussion.

Proposal 1: Consider the output of FR2 HST deployment scenario discussion for the decision of PDSCH requirement for Uni/Bi-directional scenario in scenario A and/or scenario B.

## 2.2	Applicable deployment scenario for DPS transmission schemes

DPS transmission scheme​ - Option 1: - Define PDSCH requirement with DPS scheme 1a in Uni-directional scenario for scenario A. FFS scheme 1b​ - Define PDSCH requirement with DPS scheme 1a and 1b in Uni-directional scenario for scenario B, FFS the number of TCI state configured​ - Define PDSCH requirement with DPS scheme 1a in Bi-directional scenario for scenario A and scenario B. FFS scheme 1b​ - Option 2: Define both DPS transmission scheme 1a and 1b for both Bi-directional and Uni-directional deployment​ - Option 2a: RAN4 define both scheme 1a and 1b if the performance is same, but define the same applicability rule as Rel-16 HST, i.e., if a UE declared supporting > 1 TCI states, the UE will pass scheme 1b and skipped scheme 1a test cases, and if a UE only support 1 TCI state, the UE need to pass scheme 1a and skip scheme 1b test cases.​

As we discussed in 2.1, RAN4 is still discussing which deployment scenario(s) (i.e., Uni-directional and/or Bi-directional deployment with Scenario A and/or B) is adopted. We should wait for the discussion which DPS Scheme(s) are adopted for which deployment scenario(s).

## 2.3	Test setup for PDSCH demodulation requirements

CBW - Option 1: 100MHz CBW - Option 2: 200MHz CBW - Option 3: Align the CBW configuration for PDSCH and PUSCH

RAN4 UE demodulation requirements usually set CBW 100MHz for FR2 SCS=120kHz and this is one of the deployment scenarios used in many commercial networks. TS38.101-4 specifies FR2 UE demodulation requirements with CBW 200MHz also, but it is only one test case for rank 2. We think CBW 100MHz is the important configuration for FR2 SCS=120kHz and we don’t see any reason to configure CBW 200MHz for FR2 HST scenario.

Proposal 2: Configure CBW=100MHz with SCS=120kHz for UE demodulation requirements in HST FR2.

# 3	Summary

Observation: No performance difference with regard to the 70% of maximum throughput among the uni-directional HST-DPS deployment scenario and bi-directional HST-DPS deployment scenario regardless of Scenario A and B.

Proposal 1: Consider the output of FR2 HST deployment scenario discussion for the decision of PDSCH requirement for Uni/Bi-directional scenario in scenario A and/or scenario B.

Proposal 2: Configure CBW=100MHz with SCS=120kHz for UE demodulation requirements in HST FR2.

# References

- R4-2108637, “WF for FR2 HST​ Demodulation”, Samsung.

- R4-2108661, “Way Forward on Channel Modeling for FR2 HST”, Nokia, Nokia Shanghai Bell.

# Appendix

## A.1	Simulation assumption

| Parameters | Value |
| --- | --- |
| Maximum Doppler | 9722Hz |
| CBW/SCS | 100MHz/120kHz |
| PDCSH mapping | Type A, start symbol 1, duration 13 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Antenna configuration | 2x2 |
| MCS | 13 |
| Rank | 1 |
| Dmin | Scenario A: 10m Scenario B: 150m |
| Ds | 700m |
| Velocity | 350km/h |

## A.2	HST-DPS channel model

### A.2.1	Uni-directional deployment [2]

fdt=fmaxcosθt

cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2,  0≤t≤Dsvcosθt mod Dsv,  t>Dsv

### A.2.2	Bi-directional deployment Option 2a  [2]

fdt=fmaxcosθt

cosθt=Ds-vtDmin2+Ds-vt2,  0≤t≤Ds2v-vtDmin2+vt2,  Ds2v≤t≤Dsvcosθt mod Dsv,  t>DSv