| 3GPP TR 38.717-04-01 V0.6.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	TR maintenance	10

5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Par	10

5.1	CA_n3-n28-n41-n78	10

5.1.1	Operating bands for CA	10

5.1.2	Channel bandwidths per operating band for CA	10

5.1.3	∆TIB,c and ∆RIB,c values	10

5.1.4	REFSENS requirements	11

5.2	CA_n25-n41-n66-n71	11

5.2.1	Channel bandwidths per operating bands for CA	11

5.2.2	∆TIB,c and ∆RIB,c values	12

5.2.3	REFSENS requirements	12

5.3	CA_n3-n28-n41-n77	12

5.3.1	Operating bands for CA	12

5.3.2	Channel bandwidths per operating band for CA	12

5.3.3	∆TIB and ∆RIB values	14

5.3.4	REFSENS requirements	14

5.4	 CA_n1-n77-n79-n257	14

5.4.1	Operating bands for CA	14

5.4.2	Channel bandwidths per operating band for CA	15

5.4.3	∆TIB,c and ∆RIB,c values	17

5.4.4	REFSENS requirements	17

5.5	 CA_n1-n78-n79-n257	17

5.5.1	Operating bands for CA	17

5.5.2	Channel bandwidths per operating band for CA	18

5.5.3	∆TIB,c and ∆RIB,c values	20

5.5.4	REFSENS requirements	20

5.6	CA_n3-n5-n7-n78	20

5.6.1	Channel bandwidths per operating bands for CA	20

5.6.2	∆TIB,c and ∆RIB,c values	21

5.6.3	REFSENS requirements	21

5.7	CA_n41-n66-n71-n77	22

5.7.1	Channel bandwidths per operating bands for CA	22

5.7.2	∆TIB,c and ∆RIB,c values	22

5.7.3	REFSENS requirements	22

5.8	CA_n25-n41-n71-n77	23

5.8.1	Operating bands for CA	23

5.8.2	Channel bandwidths per operating band for CA	23

5.8.3	∆TIB,c and ∆RIB,c values	23

5.8.4	REFSENS requirements	24

5.9	CA_n25-n66-n71-n77	24

5.9.1	Operating bands for CA	24

5.9.2	Channel bandwidths per operating band for CA	24

5.9.3	∆TIB,c and ∆RIB,c values	24

5.9.4	REFSENS requirements	25

5.10	CA_n25-n41-n66-n77	25

5.10.1	Operating bands for CA	25

5.10.2	Channel bandwidths per operating band for CA	26

5.10.3	∆TIB,c and ∆RIB,c values	26

5.10.4	REFSENS requirements	27

5.11	CA_n7-n25-n66-n77	27

5.11.1	Operating bands for CA	27

5.11.2	Channel bandwidths per operating bands for CA	27

5.11.3	∆TIB,c and ∆RIB,c values	29

5.11.4	REFSENS requirements	29

5.12	CA_n1-n8-n78-n79	29

5.12.1	Operating bands for CA	29

5.12.2	Channel bandwidths per operating band for CA	30

5.12.3	∆TIB,c and ∆RIB,c values	30

5.12.4	REFSENS requirements	30

5.13	CA_n5-n25-n66-n78	31

5.13.1	Operating bands for CA	31

5.13.2	Channel bandwidths per operating bands for CA	31

5.13.3	∆TIB,c and ∆RIB,c values	31

5.13.4	REFSENS requirements	32

5.14	CA_n13-n25-n66-n77	32

5.14.1	Operating bands for CA	32

5.14.2	Channel bandwidths per operating bands for CA	32

5.14.3	∆TIB,c and ∆RIB,c values	32

5.14.4	REFSENS requirements	33

5.15	CA_n2-n5-n30-n66	33

5.15.1	Operating bands for CA	33

5.15.2	Channel bandwidths per operating bands for CA	33

5.15.3	∆TIB,c and ∆RIB,c values	33

5.15.4	REFSENS requirements	34

5.16	CA_n2-n14-n30-n66	34

5.16.1	Operating bands for CA	34

5.16.2	Channel bandwidths per operating bands for CA	34

5.16.3	∆TIB,c and ∆RIB,c values	34

5.16.4	REFSENS requirements	35

5.17	CA_n3-n28-n77-n79	35

5.17.1	Operating bands for CA	35

5.17.2	Channel bandwidths per operating bands for CA	36

5.17.3	∆TIB,c and ∆RIB,c values	36

5.17.4	REFSENS requirements	36

5.18	CA_n3-n28-n79-n257	37

5.18.1	Operating bands for CA	37

5.18.2	Channel bandwidths per operating bands for CA	37

5.18.3	∆TIB,c and ∆RIB,c values	37

5.18.4	REFSENS requirements	38

5.19	CA_n3-n77-n79-n257	38

5.19.1	Operating bands for CA	38

5.19.2	Channel bandwidths per operating bands for CA	39

5.19.3	REFSENS requirements	40

5.20	CA_n28-n77-n79-n257	40

5.20.1	Operating bands for CA	40

5.20.2	Channel bandwidths per operating bands for CA	41

5.20.3	∆TIB,c and ∆RIB,c values	41

5.20.4	REFSENS requirements	42

5.21	CA_n1A-n3-n8-n77	42

5.21.1	Operating bands for CA	42

5.21.2	Channel bandwidths per operating band for CA	42

5.21.3	Co-existence studies	43

5.21.4	∆TIB,c and ∆RIB,c values	43

5.21.5	REFSENS requirements	43

5.22	CA_n1A-n3-n8-n257	43

5.22.1	Operating bands for CA	43

5.22.2	Channel bandwidths per operating band for CA	44

5.22.3	Co-existence studies	44

5.22.4	∆TIB,c and ∆RIB,c values	45

5.22.5	REFSENS requirements	45

5.23	CA_n1A-n3-n77-n257	45

5.23.1	Operating bands for CA	45

5.23.2	Channel bandwidths per operating band for CA	45

5.23.3	Co-existence studies	47

5.23.4	∆TIB,c and ∆RIB,c values	47

5.23.5	REFSENS requirements	48

5.24	CA_n1A-n8-n77-n257	48

5.24.1	Operating bands for CA	48

5.24.2	Channel bandwidths per operating band for CA	48

5.24.3	Co-existence studies	50

5.24.4	∆TIB,c and ∆RIB,c values	50

5.24.5	REFSENS requirements	50

5.25	CA_n3A-n8-n77-n257	50

5.25.1	Operating bands for CA	50

5.25.2	Channel bandwidths per operating band for CA	51

5.25.3	Co-existence studies	52

5.25.4	∆TIB,c and ∆RIB,c values	52

5.25.5	REFSENS requirements	53

5.26	CA_n41-n66-n71-n78	53

5.26.1	Operating bands for CA	53

5.26.2	Channel bandwidths per operating bands for CA	53

5.26.3	∆TIB,c and ∆RIB,c values	53

5.26.4	REFSENS requirements	54

5.27	CA_n1-n3-n5-n78	54

5.27.1	Channel bandwidths per operating bands for CA	54

5.27.2	∆TIB,c and ∆RIB,c values	54

5.27.3	REFSENS requirements	55

5.28	CA_n1-n5-n7-n78	55

5.28.1	Channel bandwidths per operating bands for CA	55

5.28.2	∆TIB,c and ∆RIB,c values	55

5.28.3	REFSENS requirements	56

5.29	CA_n1-n7-n28-n78	56

5.29.1	Channel bandwidths per operating bands for CA	56

5.29.2	∆TIB,c and ∆RIB,c values	56

5.29.3	REFSENS requirements	56

5.30	CA_n25-n41-n66-n78	57

5.30.1	Channel bandwidths per operating bands for CA	57

5.30.2	∆TIB,c and ∆RIB,c values	57

5.30.3	REFSENS requirements	57

Annex A - Change history	58

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report on inter-band CA for 4 bands DL with 1 band UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address 4 bands DL/1 band UL Inter-band Carrier Aggregation requirements for the Rel-17 band combinations requested by proponents and captured in the WID...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200665, “New WID: NR inter-band CA for 4 bands DL with 1 band UL”, RAN#88-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for 4 bands DL/1 band UL Inter-band Carrier Aggregation under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Par

<Editor’s note: The requirements for specific band combinations shall be described according to the same manner as specified in TS38.101-3.>

## 5.1	CA_n3-n28-n41-n78

### 5.1.1	Operating bands for CA

Table 5.1.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n28-n41-n78 | n3 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
|  | n28 | 703 MHz | - | 748 MHz | 758 MHz | - | 803 MHz | FDD |
|  | n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
|  | n78 | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | TDD |

### 5.1.2	Channel bandwidths per operating band for CA

Table 5.1.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A-n78A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.1.3	∆TIB,c and ∆RIB,c values

For four DLs of Band n3, n28, n41and n78, the same TIB,c and RIB,c values specified for DC_3-28-41_n78 are used as below..

Table 5.1.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41-n78 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41-n78 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 5.1.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.2	CA_n25-n41-n66-n71

### 5.2.1	Channel bandwidths per operating bands for CA

Table 5.2.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A-n71A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n66A-n71A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n66A-n71A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.2.2	∆TIB,c and ∆RIB,c values

For CA_n25-n41-n66-n71 the TIB,c and RIB,c  values are shown in table 5.2.2-1 and  table 5.2.2-2, respectively. Values are derived from DC_2-7-66_n71.

Table 5.2.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |

Table 5.2.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n71 | n25 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |

### 5.2.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.3	CA_n3-n28-n41-n77

### 5.3.1	Operating bands for CA

Table 5.3.1-1: 4DL inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n28-n41-n77 | n3 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
|  | n28 | 703 MHz | - | 748 MHz | 758 MHz | - | 803 MHz | FDD |
|  | n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
|  | n77 | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | TDD |

### 5.3.2	Channel bandwidths per operating band for CA

Table 5.3.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A-n77A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.3.3	∆TIB and ∆RIB values

For four DLs of Band n3, n28, n41and n77, the TIB,c and RIB,c values are given as below.

Table 5.3.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41-n77 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.3.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n41-n77 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.3.4	REFSENS requirements

There are no additional MSD requirements for this band combination

## 5.4		CA_n1-n77-n79-n257

### 5.4.1	Operating bands for CA

Table 5.4.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n77-n79-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.4.2	Channel bandwidths per operating band for CA

Table 5.4.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A-n79A-n257A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n1A-n77A-n79A-n257G | CA_n257G | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A-n79A-n257H | CA_n257G CA_n257H | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A-n79A-n257I | CA_n257G CA_n257H CA_n257I | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.4.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band n1, n77, n79 and n257, the TIB,c and RIB,c values are shown in table 5.4.3-1 and table 5.4.3-2, respectively.

Table 5.4.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77-n79-n257 | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
|  | n257 | 0 |

Table 5.4.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77-n79-n257 | n1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

### 5.4.4	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.5		CA_n1-n78-n79-n257

### 5.5.1	Operating bands for CA

Table 5.5.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78-n79-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.5.2	Channel bandwidths per operating band for CA

Table 5.5.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A-n257A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n1A-n78A-n79A-n257G | CA_n257G | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A-n79A-n257H | CA_n257G CA_n257H | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A-n79A-n257I | CA_n257G CA_n257H CA_n257I | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.5.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band n1, n78, n79 and n257, the TIB,c and RIB,c values are shown in table 5.5.3-1 and table 5.5.3-2, respectively.

Table 5.5.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78-n79-n257 | n1 | 0.3 |
|  | n78 | 0.8 |
|  |  | 1.58 |
|  | n79 | 0 |
|  |  | 1.58 |
|  | n257 | 0 |
| NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 5.5.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78-n79-n257 | n1 | 0 |
|  | n78 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

### 5.5.4	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.6	CA_n3-n5-n7-n78

### 5.6.1	Channel bandwidths per operating bands for CA

### Table 5.6.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n5A-n7A-n78A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n5A-n7B-n78A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.6.2	∆TIB,c and ∆RIB,c values

### For CA_n3-n5-n7-n78 the TIB,c and RIB,c  values are shown in table 5.6.2-1 and  table 5.6.2-2, respectively. Values are derived from DC_3-5-7_n78.

Table 5.6.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n5-n7-n78 | n3 | 0.6 |
|  | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 5.6.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n5-n7-n78 | n3 | 0.2 |
|  | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

### 5.6.3	REFSENS requirements

### MSD requirements are captured in lower order combinations.

## 5.7	CA_n41-n66-n71-n77

### 5.7.1	Channel bandwidths per operating bands for CA

Table 5.7.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n71A-n77A | - | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n66A-n71A-n77A | - | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41(2A)-n66A-n71A-n77A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.7.2	∆TIB,c and ∆RIB,c values

For CA_n41-n66-n71-n77 the TIB,c and RIB,c  values are shown in table 5.7.2-1 and  table 5.7.2-2, respectively.

The values for n41, n66 and n71 are reused from CA_n41-n66-n71, and the values for n77 are reused from CA_n3-n28-n41-n77.

Table 5.7.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n71-n77 | n41 | 0.81/1.32 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 5.7.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n71-n77 | n41 | 0.51/12 |
|  | n66 | 0.5 |
|  | n71 | 0 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 5.7.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.8	CA_n25-n41-n71-n77

### 5.8.1	Operating bands for CA

Table 5.8.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n71-n77 | n25 | 1850 MHz | - | 1915 MHz | 1930 MHz | - | 1995 MHz | FDD |
|  | n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
|  | n71 | 663 MHz | - | 698 MHz | 617 MHz | - | 652 MHz | FDD |
|  | n77 | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | TDD |

### 5.8.2	Channel bandwidths per operating band for CA

Table 5.8.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41C-n71A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41(2A)-n71A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) bandwidth combination set 1 in Table 5.5A.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.8.3	∆TIB,c and ∆RIB,c values

For CA_n25-n41-n71-n77 following TIB,c and RIB,c values specified based on  CA_n25-n41-n71.

Table 5.8.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n71-n77 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |

Table 5.8.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n71-n77 | n25 | 0 |
|  | n41 | 0 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |

### 5.8.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.9	CA_n25-n66-n71-n77

### 5.9.1	Operating bands for CA

Table 5.9.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n66-n71-n77 | n25 | 1850 MHz | - | 1915 MHz | 1930 MHz | - | 1995 MHz | FDD |
|  | n66 | 1710 MHz | - | 1780 MHz | 2110 MHz | - | 2200 MHz | FDD |
|  | n71 | 663 MHz | - | 698 MHz | 617 MHz | - | 652 MHz | FDD |
|  | n77 | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | TDD |

### 5.9.2	Channel bandwidths per operating band for CA

Table 5.9.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n66A-n71A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.9.3	∆TIB,c and ∆RIB,c values

For CA_n25-n66-n71-n77 following TIB,c and RIB,c values are specified based on  CA_n25-n66-n71.

Table 5.9.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n66-n71-n77 | n25 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |

Table 5.9.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n66-n71-n77 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |
|  | n77 | 0.5 |

### 5.9.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.10	CA_n25-n41-n66-n77

### 5.10.1	Operating bands for CA

Table 5.10.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n66-n77 | n25 | 1850 MHz | - | 1915 MHz | 1930 MHz | - | 1995 MHz | FDD |
|  | n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
|  | n66 | 1710 MHz | - | 1780 MHz | 2110 MHz | - | 2200 MHz | FDD |
|  | n77 | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | TDD |

### 5.10.2	Channel bandwidths per operating band for CA

Table 5.10.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41C-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41(2A)-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) bandwidth combination set 1 in Table 5.5A.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41A-n66A-n77(2A) | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.10.3	∆TIB,c and ∆RIB,c values

For CA_n25-n41-n66-n77 following TIB,c and RIB,c values specified based on CA_n25-n41-n66

Table 5.10.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n77 | n25 | 0.5 |
|  | n41 | 0.85/1.36 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.10.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n77 | n25 | 0.3 |
|  | n41 | 0.55/1.06 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

### 5.10.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.11	CA_n7-n25-n66-n77

### 5.11.1	Operating bands for CA

Table 5.11.1-1: Inter-band CA operating bands of CA_n7-n25-n66-n77

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n7-n25-n66-n77 | n7, n25, n66, n77 |

### 5.11.2	Channel bandwidths per operating bands for CA

Table 5.11.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25A-n66A-n77A | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n25A-n66A-n77A | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25(2A)-n66A-n77A | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25A-n66(2A)-n77A | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25A-n66A-n77(2A) | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66A-n77A | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n77A | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n25A-n66A-n77(2A) | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25(2A)-n66(2A)-n77A | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25(2A)-n66A-n77(2A) | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n66(2A)-n77(2A) | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n77A | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n25A-n66(2A)-n77(2A) | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66A-n77(2A) | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25(2A)-n66(2A)-n77(2A) | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66(2A)-n77(2A) | - | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.11.3	∆TIB,c and ∆RIB,c values

For CA_n7-n25-n66-n77, the TIB,c and RIB,c values can reuse CA_n7-n25-n66-n78 defined in TS 38.101-1.

Table 5.11.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66-n77 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 5.11.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66-n77 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

### 5.11.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.12	CA_n1-n8-n78-n79

### 5.12.1	Operating bands for CA

Table 5.12.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n8-n78-n79 | n1 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
|  | n8 | 880 MHz | - | 915 MHz | 925 MHz | - | 960 MHz | FDD |
|  | n78 | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | TDD |
|  | n79 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | TDD |

### 5.12.2	Channel bandwidths per operating band for CA

Table 5.12.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA config | UL config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n8A-n78A-n79A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n8A-n78(2A)-n79A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.12.3	∆TIB,c and ∆RIB,c values

For four DLs of Band n1, n8, n78 and n79, the TIB,c and RIB,c values can be specified as below..

Table 5.12.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78-n79 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 5.12.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78-n79 | n1 | 0.3 |
|  | n8 | 0.3 |
|  | n78 | 0.5 |
|  | n79 | 0 |

### 5.12.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.13	CA_n5-n25-n66-n78

### 5.13.1	Operating bands for CA

Table 5.13.1-1: Inter-band CA operating bands of CA_n5-n25-n66-n78

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n5-n25-n66-n78 | n5, n25, n66, n78 |

### 5.13.2	Channel bandwidths per operating bands for CA

Table 5.13.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66A-n78A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25(2A)-n66A-n78A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66(2A)-n78A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66A-n78(2A) | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A)-n78A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25(2A)-n66A-n78(2A) | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n66(2A)-n78(2A) | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A)-n78(2A) | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.13.3	∆TIB,c and ∆RIB,c values

For CA_n5-n25-n66-n78, the TIB,c and RIB,c values can be specified as below.

Table 5.13.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25-n66-n78 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 5.13.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25-n66-n78 | n5 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |

### 5.13.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.14	CA_n13-n25-n66-n77

### 5.14.1	Operating bands for CA

Table 5.14.1-1: Inter-band CA operating bands of CA_n5-n25-n66-n78

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n13-n25-n66-n77 | n13, n25, n66, n77 |

### 5.14.2	Channel bandwidths per operating bands for CA

Table 5.14.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n25A-n66A-n77A | - | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.14.3	∆TIB,c and ∆RIB,c values

For CA_n13-n25-n66-n77, the TIB,c and RIB,c values can be specified as below.

Table 5.14.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25-n66-n77 | n13 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 5.14.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25-n66-n77 | n13 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 5.14.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.15	CA_n2-n5-n30-n66

### 5.15.1	Operating bands for CA

Table 5.15.1-1: Inter-band CA operating bands of CA_n2-n5-n30-n66

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n2-n5-n30-n66 | n2, n5, n30, n66 |

### 5.15.2	Channel bandwidths per operating bands for CA

Table 5.15.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n5A-n30A-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 5.15.3	∆TIB,c and ∆RIB,c values

For CA_n2-n5-n30-n66, the TIB,c and RIB,c values can reuse E-UTRA CA_2-5-30-66 defined in TS 36.101.

Table 5.15.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2A-n5A-n30A-n66A | n2 | 0.5 |
|  | n5 | 0.3 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |

Table 5.15.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2A-n5A-n30A-n66A | n2 | 0.4 |
|  | n5 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |

### 5.15.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.16	CA_n2-n14-n30-n66

### 5.16.1	Operating bands for CA

Table 5.16.1-1: Inter-band CA operating bands of CA_n2-n14-n30-n66

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n2-n14-n30-n66 | n2, n14, n30, n66 |

### 5.16.2	Channel bandwidths per operating bands for CA

Table 5.16.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n14A-n30A-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 5.16.3	∆TIB,c and ∆RIB,c values

For CA_n2-n14-n30-n66, the TIB,c and RIB,c values can reuse E-UTRA CA_2-14-30-66 defined in TS 36.101.

Table 5.16.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2A-n14A-n30A-n66A | n2 | 0.5 |
|  | n14 | 0.3 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |

Table 5.16.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2A-n14A-n30A-n66A | n2 | 0.4 |
|  | n14 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |

### 5.16.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.17	CA_n3-n28-n77-n79

### 5.17.1	Operating bands for CA

Table 5.17.1-1: Inter-band CA operating bands of CA_n3-n28-n77-n79

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n3-n28-n77-n79 | n3, n28, n77, n79 |

### 5.17.2	Channel bandwidths per operating bands for CA

Table 5.17.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| CA_n3A-n28A-n77A-n79A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |
| CA_n3A-n28A-n77(2A)-n79A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |
| NOTE 3: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.17.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band combinations n3-n28-n77-n79, the TIB,c and RIB,c  values are shown in table 5.17.3-1 and  table 5.17.3-2, respectively.

Table 5.17.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77-n79 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 5.17.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77-n79 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

### 5.17.4	REFSENS requirements

MSD requirements are captured in the lower order combinations.

## 5.18	CA_n3-n28-n79-n257

### 5.18.1	Operating bands for CA

Table 5.18.1-1: Inter-band CA operating bands of CA_n3-n28-n79-n257

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n3-n28-n79-n257 | n3, n28, n79, n257 |

### 5.18.2	Channel bandwidths per operating bands for CA

Table 5.18.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |  |
| CA_n3A-n28A-n79A-n257A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n3A-n28A-n79A-n257G | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n79A-n257H | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n79A-n257I | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.18.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band combinations n3-n28-n79-n257, the TIB,c and RIB,c  values are shown in table 5.18.3-1 and  table 5.18.3-2, respectively.

Table 5.18.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n79-n257 | n3 | 0.3 |
|  | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n257 | 0 |

### 5.18.4	REFSENS requirements

MSD requirements are captured in the lower order combinations.

## 5.19	CA_n3-n77-n79-n257

### 5.19.1	Operating bands for CA

Table 5.19.1-1: Inter-band CA operating bands of CA_n3-n77-n79-n257

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n3-n77-n79-n257 | n3, n77, n79, n257 |

### 5.19.2	Channel bandwidths per operating bands for CA

Table 5.19.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |  |
| CA_n3A-n77A-n79A-n257A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n3A-n77A-n79A-n257G | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n79A-n257H | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n79A-n257I | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n79A-n257A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n3A-n77(2A)-n79A-n257G | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n79A-n257H | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n79A-n257I | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.19.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band combinations n3-n77-n79-n257, the TIB,c and RIB,c  values are shown in table 5.19.3-1 and  table 5.19.3-2, respectively.

Table 5.19.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n77-n79-n257 | n3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 5.19.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n77-n79-n257 | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

### 5.19.3	REFSENS requirements

MSD requirements are captured in the lower order combinations.

Table 5.18.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n79-n257 | n3 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |
|  | n257 | 0 |

## 5.20	CA_n28-n77-n79-n257

### 5.20.1	Operating bands for CA

Table 5.20.1-1: Inter-band CA operating bands of CA_n28-n77-n79-n257

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n28-n77-n79-n257 | n28, n77, n79, n257 |

### 5.20.2	Channel bandwidths per operating bands for CA

Table 5.20.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |  |  |
| CA_n28A-n77A-n79A-n257A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |  |
| CA_n28A-n77A-n79A-n257G | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n79A-n257H | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n79A-n257I | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n79A-n257A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 |  | 400 |  |  |
| CA_n28A-n77(2A)-n79A-n257G | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n79A-n257H | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n79A-n257I | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 |  |  | 80 |  | 100 |  |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.20.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band combinations n28-n77-n79-n257, the TIB,c and RIB,c  values are shown in table 5.20.3-1 and  table 5.20.3-2, respectively.

Table 5.20.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77-n79-n257 | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
|  | n257 | 0 |

Table 5.20.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77-n79-n257 | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

### 5.20.4	REFSENS requirements

MSD requirements are captured in the lower order combinations.

## 5.21	CA_n1A-n3-n8-n77

### 5.21.1	Operating bands for CA

Table 5.21.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n3-n8-n77 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 5.21.2	Channel bandwidths per operating band for CA

Table 5.21.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| CA_n1A-n3A-n8A-n77A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |
| CA_n1A-n3A-n8A-n77(2A) | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.21.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.21.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n3, n8 and n77, the TIB,c and RIB,c values are shown in table 5.21.4-1 and table 5.21.4-2 refer to DC_1-3-8_n77 in 38.101-3, respectively.

Table 5.21.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n77 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |

Table 5.21.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n77 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |

### 5.21.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

## 5.22	CA_n1A-n3-n8-n257

### 5.22.1	Operating bands for CA

Table 5.22.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n3-n8-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.22.2	Channel bandwidths per operating band for CA

Table 5.22.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band |  |  |  |  | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 |  | 15 |  | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 100 |  |  |
| CA_n1A-n3A-n8A-n257A | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n1A-n3A-n8A-n257G | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257H | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257I | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257J | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257K | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257L | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A-n257M | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.22.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.22.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n3, n8 and n257, the TIB,c and RIB,c values are shown in table 5.22.4-1 and table 5.22.4-2, respectively.

Table 5.22.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n257 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |
|  | n257 | 0 |

Table 5.22.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n257 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.5 |
|  | n257 | 0 |

### 5.22.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

## 5.23	CA_n1A-n3-n77-n257

### 5.23.1	Operating bands for CA

Table 5.23.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n3-n77-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.23.2	Channel bandwidths per operating band for CA

Table 5.23.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band |  |  |  |  | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 |  | 15 |  | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 100 |  |  |
| CA_n1A-n3A-n77A-n257A | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n1A-n3A-n77A-n257G | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257H | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257I | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257J | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257K | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257L | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A-n257M | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257A | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n1A-n3A-n77(2A)-n257G | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257H | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257I | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257J | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257K | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257L | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77(2A)-n257M | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.23.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.23.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n3, n77 and n257, the TIB,c and RIB,c values are shown in table 5.23.4-1 and table 5.23.4-2, respectively.

Table 5.23.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n77-n257 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 5.23.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n77-n257 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

### 5.23.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

## 5.24	CA_n1A-n8-n77-n257

### 5.24.1	Operating bands for CA

Table 5.24.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n8-n77-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.24.2	Channel bandwidths per operating band for CA

Table 5.24.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band |  |  |  |  | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 |  | 15 |  | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 100 |  |  |
| CA_n1A-n8A-n77A-n257A | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n1A-n8A-n77A-n257G | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257H | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257I | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257J | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257K | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257L | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77A-n257M | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257A | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n1A-n8A-n77(2A)-n257G | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257H | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257I | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257J | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257K | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257L | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n77(2A)-n257M | - | n1 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.24.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.24.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n8, n77 and n257, the TIB,c and RIB,c values are shown in table 5.24.4-1 and table 5.24.4-2, respectively.

Table 5.24.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n77-n257 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 5.24.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n77-n257 | n1 | 0 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

### 5.24.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

## 5.25	CA_n3A-n8-n77-n257

### 5.25.1	Operating bands for CA

Table 5.25.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n8-n77-n257 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 5.25.2	Channel bandwidths per operating band for CA

Table 5.25.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band |  |  |  |  | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 |  | 15 |  | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 100 |  |  |
| CA_n3A-n8A-n77A-n257A | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n3A-n8A-n77A-n257G | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257H | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257I | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257J | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257K | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257L | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77A-n257M | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 |  | 15 |  | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257A | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |  |
| CA_n3A-n8A-n77(2A)-n257G | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257H | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257I | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257J | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257K | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257L | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n77(2A)-n257M | - | n3 | 5 | 10 |  | 15 |  | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  | n8 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.25.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.25.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n3, n8, n77 and n257, the TIB,c and RIB,c values are shown in table 5.25.4-1 and table 5.25.4-2, respectively.

Table 5.25.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n8-n77-n257 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 5.25.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n8-n77-n257 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

### 5.25.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

## 5.26	CA_n41-n66-n71-n78

### 5.26.1	Operating bands for CA

Table 5.26.1-1: Inter-band CA operating bands involving FR1(four bands)

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n41-n66-n71-n78 | n41, n66, n71, n78 |

### 5.26.2	Channel bandwidths per operating bands for CA

Table 5.26.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |
| CA_n41A-n66A-n71A-n78A | - | n41 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | 0 |
|  |  | n66 | 5 | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |
| CA_n41A-n66(2A)-n71A-n78A | - | n41 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |
| CA_n41A-n66A-n71A-n78(2A) | - | n41 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | 0 |
|  |  | n66 | 5 | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n71A-n78(2A) | - | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.26.3	∆TIB,c and ∆RIB,c values

For CA_n41-n66-n71-n78, the TIB,c and RIB,c values can reuse CA_n41-n66-n71-n77 defined in TS 38.101-1.

Table 5.26.3-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n71-n78 | n41 | 0.33/0.84 |
|  | n66 | 1 |
|  | n71 | 0.5 |
|  |  |  |  | n78 |  | 0.8 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz |  |  |

Table 5.26.3-2: ΔRIB,c  due to NR CA (four bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n71-n78 | n41 | 03/0.54 |
|  | n66 | 0.5 |
|  | n71 | 0.2 |
|  |  |  |  | n78 |  | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz |  |  |

### 5.26.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.27	CA_n1-n3-n5-n78

### 5.27.1	Channel bandwidths per operating bands for CA

Table 5.27.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n5A-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.27.2	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n5-n78 the TIB,c and RIB,c  values are shown in table 5.27.2-1 and  table 5.27.2-2, respectively. Values are derived from DC_1-3-5_n78.

Table 5.27.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n5-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n5 | 0.3 |
|  | n78 | 0.8 |

Table 5.27.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n5-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 5.27.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.28	CA_n1-n5-n7-n78

### 5.28.1	Channel bandwidths per operating bands for CA

Table 5.28.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n5A-n7A-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n5A-n7B-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.28.2	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n7-n78 the TIB,c and RIB,c  values are shown in table 5.28.2-1 and  table 5.28.2-2, respectively. Values are derived from DC_1-5-7_n78.

Table 5.28.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5-n7-n78 | n1 | 0.6 |
|  | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 5.28.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5-n7-n78 | n1 | 0.2 |
|  | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

### 5.28.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.29	CA_n1-n7-n28-n78

### 5.29.1	Channel bandwidths per operating bands for CA

Table 5.29.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n7A-n28A-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.29.2	∆TIB,c and ∆RIB,c values

For CA_n1-n7-n28-n78 the TIB,c and RIB,c  values are shown in table 5.29.2-1 and  table 5.29.2-2, respectively. Values are derived from DC_1-7-28_n78.

Table 5.29.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n7-n28-n78 | n1 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 5.29.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n7-n28-n78 | n1 | 0.2 |
|  | n7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 5.29.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.30	CA_n25-n41-n66-n78

### 5.30.1	Channel bandwidths per operating bands for CA

Table 5.30.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41A-n66A-n78A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.30.2	∆TIB,c and ∆RIB,c values

For CA_n25-n41-n66-n78 the TIB,c and RIB,c  values are shown in table 5.30.2-1 and  table 5.30.2-2, respectively. Values are derived from CA_n25-n41-n66-n77.

Table 5.30.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n78 | n25 | 0.5 |
|  | n41 | 0.83/1.34 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz |  |  |

Table 5.30.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n78 | n25 | 0.3 |
|  | n41 | 0.55/1.06 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz |  |  |

### 5.30.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2020-08 | 3GPP RAN4#96-e | R4-2010682 | TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e | R4-2011887 | Implemented TP’s from RAN4 #96-e: R4-2010254, “TP for TR 38.717-04-01 CA_n3A-n28A-n41A-n78A”, Samsung, KDDI R4-2011676, “TP to add CA_n25A-n41A-n66A-n71A, CA_n25A-n41(2A)-n66A-n71A, CA_n25A-n41C-n66A-n71A”, Ericsson, T-Mobile US | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015926 | Implemented TP’s from RAN4 #97-e: R4-2014118, “TP for TR 38.717-04-01 CA_n3-n28-n41-n77”, Samsung, KDDI R4-2014816, “TP for CA_n1-n77-n79-n257 4DL/1UL for TR38.717-04-01”, NTT DOCOMO, INC. R4-2014817, “TP for CA_n1-n78-n79-n257 4DL/1UL for TR38.717-04-01”, NTT DOCOMO, INC. R4-2016307, “TP to add CA_n3A-n5A-n7A-n78A, CA_n3A-n5A-n7B-n78A”, Ericsson, Telstra | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2101893 | Implemented TP from RAN4 #98-e: R4-2103082, “TP for TR 38.717-04-01: CA_n41-n66-n71-n77”, Ericsson | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2106706 | Implemented TP’s from RAN4 #98-bis-e: R4-2104420, “TP to TR 38.717-04-01: CA_n25-n41-n71-n77”, Nokia, T-Mobile USA R4-2104421, “TP to TR 38.717-04-01: CA_n25-n66-n71-n77 “, Nokia, T-Mobile USA R4-2105277, “TP to TR 38.717-04-01: CA_n25-n41-n66-n77”, Nokia, T-Mobile USA R4-2105073, “TP for TR 38.717-04-01: CA_n7-n25-n66-n77”, Samsung, TELUS, Bell mobility R4-2105278, “TP for TR 38.717-04-01: CA_n1A-n8A-n78A-n79A/ CA_n1A-n8A-n78(2A)-n79A”, Huawei, HiSilicon | 0.4.0 |
| 2021-05 | 3GPP RAN4#99-e | R4-2111081 | Implemented TP’s from RAN4 #99-e: R4-2110677, TP for TR 38.717-04-01: CA_n5-n25-n66-n78, Huawei, HiSilicon, Bell Mobility, Telus R4-2110681, TP for TR 38.717-04-01: CA_n13A-n25A-n66A-N77A, Huawei, HiSilicon, Bell Mobility, Telus R4-2111101, TP for TR 38.717-04-01 to include CA_n25-n41-n66-n77, Ericsson, Bell Mobility | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2111081 | Implemented TP’s from RAN4 #100-e: R4-2111780, TP to TR 38.717-04-01: Addition of CA_n2-n5-n30-n66, Nokia, AT&T R4-2111781, TP to TR 38.717-04-01: Addition of CA_n2-n14-n30-n66, Nokia, AT&T R4-2112062, TP for TR 38.717-04-01: NR CA_n3-n28-n77-n79, SoftBank Corp. R4-2112063, TP for TR 38.717-04-01: NR CA_n3-n28-n79-n257, SoftBank Corp. R4-2112064, TP for TR 38.717-04-01: NR CA_n3-n77-n79-n257, SoftBank Corp. R4-2112065, TP for TR 38.717-04-01: NR CA_n28-n77-n79-n257, SoftBank Corp. R4-2112168, TP for TR 38.717-04-01 CA_n1A-n3A-n8A-n77A and CA_n1A-n3A-n8A-n77(2A), Huawei,HiSilicon R4-2112169, TP for TR 38.717-04-01 CA_n1A-n3A-n8A-n257GHIJKLMA, Huawei,HiSilicon R4-2112170, TP for TR 38.717-04-01 CA_n1A-n3A-n77A-n257GHIJKLMA and CA_n1A-n3A-n77(2A)-n257GHIJKLMA, Huawei,HiSilicon R4-2112171, TP for TR 38.717-04-01 CA_n1A-n8A-n77A-n257GHIJKLMA and CA_n1A-n8A-n77(2A)-n257GHIJKLMA, Huawei,HiSilicon R4-2112172 TP for TR 38.717-04-01 CA_n3A-n8A-n77A-n257GHIJKLMA and CA_n3A-n8A-n77(2A)-n257GHIJKLMA, Huawei,HiSilicon R4-2113168, TP for TR 38.717-04-01 CA_n41-n66-n71-n78, Samsung, Ericsson,Telus, Bell mobility R4-2113588, TP for TR 38.717-04-01 to include CA_n1-n3-n5-n78, Ericsson, Telstra R4-2113590, TP for TR 38.717-04-01 to include CA_n1-n5-n7-n78, Ericsson, Telstra R4-2113597, TP for TR 38.717-04-01 to include CA_n1-n7-n28-n78, Ericsson, BT plc R4-2113607, TP for TR 38.717-04-01 to include CA_n25-n41-n66-n78, Ericsson, Bell Mobility | 0.6.0 |