3GPP TSG-RAN WG4 Meeting #100-e	R4-2113579

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, Telstra

Title:	TP for TR 38.717-03-02: CA_n3-n5-n7

Agenda item:			8.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include UL configurations to DL CA_n3A-n5A-n7A and CA_n3A-n5A-n7B already defined in 38.101-1.

# 2. Text Proposal

# ---Start of changes---

### 5.1.x	CA_n3-n5-n7

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n5-n7 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n5A-n7A | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A-n5A-n7B | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

IMD3 generated by UL n3-n7 might affect DL n5.

IMD2 and IMD3 generated by UL n3-n5 might affect DL n7.

#### 5.1.x.4	REFSENS requirements

MSD value n5 are derived from DC_3A-18A_n41A.

MSD value n7 are derived from DC_3A-7A_n5A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n5-n7 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n7 | 2505 | 10 | 50 | 2625 | 30.0 | FDD | IMD24 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 19.0 | FDD | IMD3 |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-211060, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE