3GPP TSG-RAN WG4 Meeting #100-e 	R4-2113631

Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for PUSCH demodulation requirements for FR1 256QAM

Agenda item:	9.12.3.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the PUSCH demodulation requirements for FR1 256QAM topic are captured in the summary [1] and WF [2].

In this contribution we collect our simulation results, which are partially re-used in our companion discussion paper.

Simulation setup

Table 1: Main test parameters.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1, pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PTRS | Frequency density (KPT-RS) | 2, Disabled |
| configuration | Time density (LPT-RS) | 1, Disabled |

Table 2: Further simulation parameters.

| Parameter | Value |
| --- | --- |
| Number of Tx | 1, 2 |
| Number of Rx | 2, 4, 8 |
| Number of Layers | 1, 2 |
| MCS index | MCS24, MCS22 (table 2) (MCS 20 and 21 are excluded for now) |
| Carrier frequency (GHz) for simulation alignment | 4 |
| Propagation condition | TDL-A 30ns, 10Hz Low |
| SCS and BW | 15kHz: 5MHz, 10MHz, [20MHz]; 30kHz: 10MHz, [20MHz], 40MHz, [100MHz] |
| Timing offset | 0 |
| Frequency offset | 0 |
| Tx EVM | None |
| Testing metric | SNR @ 70% TPUT |

Simulation results

Following the updates of simulation scenarios.

1T2R

Table 3: Results SNR@70%TPUT, 1T2R. Rank 1

| PUSCH 1T2R |  |  |  |  |  | 70%TPUT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,64 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,26 |
|  |  | [15kHz, 20MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20.78 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 17,01 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,52 |
|  |  | [30kHz, 100MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 22.56 |
|  | FR1 Mapping type B | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,61 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,21 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,98 |
|  |  |  | MCS21 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS22 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,53 |

1T4R

Table 4: Results SNR@70%TPUT, 1T4R. Rank 1

| PUSCH 1T4R |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13,01 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 16,69 |
|  |  | [15kHz, 20MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 16.89 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13,35 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 16,93 |
|  |  | [30kHz, 100MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 18.31 |
|  | FR1 Mapping type B | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13,01 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 16,70 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 13,35 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 16,92 |

1T8R

Table 4: Results SNR@70%TPUT, 1T8R. Rank 1

| PUSCH 1T8R |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9,83 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 13,50 |
|  |  | [15kHz, 20MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 13.65 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10,11 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 13,71 |
|  |  | [30kHz, 100MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 14.44 |
|  | FR1 Mapping type B | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 9,83 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 13,50 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 10,13 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 13,71 |

RAN4#99e Simulation results

For reference.

1TX2/4/8RX Rank 1

Table 5: Results SNR@70%TPUT, 1TX2/4/8RX Rank 1

| Antennas | 1Tx2Rx |  | 1Tx4Rx |  | 1Tx8Rx |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS / BW | 15kHz / 20MHz | 30kHz / 100MHz | 15kHz / 20MHz | 30kHz / 100MHz | 15kHz / 20MHz | 30kHz / 100MHz |
| MCS 24 |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN | 20.78 | 22.56 | 16.89 | 18.31 | 13.65 | 14.4 |
| 2 DMRS Ptrs PN | 22.15 | 23.83 | 18.26 | 19.47 | 14.93 | 15.6 |
| 2 DMRS NoPtrs PN | 21.06 | 22.74 | 17.19 | 18.49 | 13.90 | 14.6 |

2Tx2Rx Rank 1

Table 6: Results SNR@70%TPUT, 2TX2RX Rank 1

| Antennas | 2Tx2Rx Rank1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS / BW | 15kHz / 5MHz | 15kHz / 10MHz | 15kHz / 20MHz | 30kHz / 10MHz | 30kHz / 20MHz | 30kHz / 40MHz | 30kHz / 100MHz |
| MCS 24 |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN | 19.84 | 20.55 | 20.78 | 20.21 | 20.57 | 20.66 | 22.56 |
| 2 DMRS Ptrs PN | 20.95 | 21.76 | 22.18 | 21.34 | 21.77 | 21.88 | 23.82 |
| 3 DMRS NoPtrs NoPN | 19.89 | 20.82 | 21.08 | 20.75 | 20.77 | 20.97 | 22.94 |
| 3 DMRS Ptrs PN | 20.97 | 22.02 | 22.45 | 21.93 | 21.93 | 22.25 | 24.26 |
| 2 DMRS NoPtrs PN | 19.99 | 20.71 | 21.07 | 20.37 | 20.74 | 20.82 |  |

2Tx2RX Rank 2

Table 7: Results SNR@70%TPUT, 2TX2RX Rank 2

| Antennas | 2Tx2Rx Rank2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS / BW | 15kHz / 5MHz | 15kHz / 10MHz | 15kHz / 20MHz | 30kHz / 10MHz | 30kHz / 20MHz | 30kHz / 40MHz | 30kHz / 100MHz |
| MCS 24 |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN | 27.15 | 28.13 | 30.95 | 27.82 | 28.97 | 30.28 | 34.33 |
| 2 DMRS Ptrs PN | 28.48 | 29.76 | 32.84 | 29.18 | 30.52 | 31.89 | 36.59 |
| 3 DMRS NoPtrs NoPN | 27.59 | 28.63 | 31.61 | 28.81 | 29.50 | 30.87 | 35.01 |
| 3 DMRS Ptrs PN | 28.83 | 30.34 | 33.46 | 30.29 | 31.08 | 32.58 | 37.27 |
| 2 DMRS NoPtrs PN | 27.32 | 28.43 | 31.19 | 27.94 | 29.16 | 30.47 |  |
| MCS 22 |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN | 24.79 | 25.85 | 28.48 | 25.85 | 26.56 | 28.02 | 31.07 |
| 2 DMRS Ptrs PN | 25.79 | 26.94 | 29.65 | 26.86 | 27.59 | 29.09 | 32.27 |
| 3 DMRS NoPtrs NoPN | 25.36 | 25.96 | 28.73 | 25.80 | 27.24 | 28.22 | 31.47 |
| 3 DMRS Ptrs PN | 26.34 | 27.14 | 29.84 | 26.79 | 28.38 | 29.30 | 32.64 |

2TX4/8Rx Rank1/2

Table 8: Results SNR@70%TPUT, 2TX4/8Rx Rank1/2

| Antennas | 2Tx4Rx Rank1 |  | 2Tx4Rx Rank2 |  | 2Tx8Rx Rank1 |  | 2Tx8Rx Rank2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS / BW | 15kHz / 20MHz | 30kHz / 100MHz | 15kHz / 20MHz | 30kHz / 100MHz | 15kHz / 20MHz | 30kHz / 100MHz | 15kHz / 20MHz | 30kHz / 100MHz |
| MCS 24 |  |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN | 16.89 | 18.31 | 22.85 | 24.16 | 13.65 | 14.44 | 18.17 | 18.75 |
| 2 DMRS Ptrs PN | 18.24 | 19.52 | 24.26 | 25.49 | 14.95 | 15.58 | 19.46 | 19.97 |
| 3 DMRS NoPtrs NoPN |  |  |  |  |  |  |  |  |
| 3 DMRS Ptrs PN |  |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs PN | 17.19 | 18.5 | 23.13 |  | 13.93 | 14.6 | 18.41 |  |
| MCS 22 |  |  |  |  |  |  |  |  |
| 2 DMRS NoPtrs NoPN |  |  | 20.61 | 21.92 |  |  | 16.03 | 16.71 |
| 2 DMRS Ptrs PN |  |  | 21.68 | 22.91 |  |  | 17.03 | 17.70 |
| 3 DMRS NoPtrs NoPN |  |  |  |  |  |  |  |  |
| 3 DMRS Ptrs PN |  |  |  |  |  |  |  |  |

FDD vs TDD

Table 9: Results SNR@70%TPUT, 2TX2RX Rank 1, FDD vs TDD

| Antennas | 2Tx2Rx Rank1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS / BW | 15kHz / 5MHz | 15kHz / 10MHz | 15kHz / 20MHz | 30kHz / 10MHz | 30kHz / 20MHz | 30kHz / 40MHz | 30kHz / 100MHz |
| MCS 24 FDD |  |  |  |  |  |  |  |
| 2 DMRS Ptrs PN | 20.95 | 21.76 | 22.18 | 21.34 | 21.77 | 21.88 | 23.82 |
| MCS 24 TDD |  |  |  |  |  |  |  |
| 2 DMRS Ptrs PN | 21.00 | 21.44 |  | 21.15 | 21.69 | 21.82 |  |
| DELTA | -0.05 | 0.32 |  | 0.19 | 0.08 | 0.06 |  |

Impact of Phase noise

Table 10: Results SNR@70%TPUT, PN on vs. PN off, without PT-RS compensation

|  |  | SINR[dB] @ 70% TP MCS 24 |  |  |  |  | SINR[dB] @ 70% TP MCS 24 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Antennas | SCS / BW | 2 DMRS NoPtrs NoPN | 2 DMRS NoPtrs PN | Diff. PN | Antennas | SCS / BW | 2 DMRS NoPtrs NoPN | 2 DMRS NoPtrs PN | Diff. PN |
| 1Tx2Rx | 15kHz / 20MHz | 20.78 | 21.06 | -0.28 | 2Tx2Rx Rank2 | 15kHz / 5MHz | 27.15 | 27.32 | -0.17 |
|  | 30kHz / 100MHz | 22.56 | 22.74 | -0.18 |  | 15kHz / 10MHz | 28.13 | 28.43 | -0.30 |
| 1Tx4Rx | 15kHz / 20MHz | 16.89 | 17.19 | -0.30 |  | 15kHz / 20MHz | 30.95 | 31.19 | -0.24 |
|  | 30kHz / 100MHz | 18.31 | 18.49 | -0.18 |  | 30kHz / 10MHz | 27.82 | 27.94 | -0.12 |
| 1Tx8Rx | 15kHz / 20MHz | 13.65 | 13.90 | -0.25 |  | 30kHz / 20MHz | 28.97 | 29.16 | -0.19 |
|  | 30kHz / 100MHz | 14.4 | 14.6 | -0.16 |  | 30kHz / 40MHz | 30.28 | 30.47 | -0.19 |
| 2Tx2Rx Rank1 | 15kHz / 5MHz | 19.84 | 19.99 | -0.15 |  | 30kHz / 100MHz | 34.33 |  |  |
|  | 15kHz / 10MHz | 20.55 | 20.71 | -0.16 | 2Tx4Rx Rank1 | 15kHz / 20MHz | 16.89 | 17.19 | -0.30 |
|  | 15kHz / 20MHz | 20.78 | 21.07 | -0.29 |  | 30kHz / 100MHz | 18.31 | 18.5 | -0.19 |
|  | 30kHz / 10MHz | 20.21 | 20.37 | -0.16 | 2Tx4Rx Rank2 | 15kHz / 20MHz | 22.85 | 23.13 | -0.28 |
|  | 30kHz / 20MHz | 20.57 | 20.74 | -0.17 |  | 30kHz / 100MHz | 24.16 |  |  |
|  | 30kHz / 40MHz | 20.66 | 20.82 | -0.16 | 2Tx8Rx Rank1 | 15kHz / 20MHz | 13.65 | 13.93 | -0.28 |
|  | 30kHz / 100MHz | 22.56 |  |  |  | 30kHz / 100MHz | 14.44 | 14.6 | -0.16 |
|  |  |  |  |  | 2Tx8Rx Rank2 | 15kHz / 20MHz | 18.17 | 18.41 | -0.24 |
|  |  |  |  |  |  | 30kHz / 100MHz | 18.75 |  |  |

Conclusion

In this contribution we have provided our simulation results for PUSCH demodulation requirements for FR1 256QAM.
We have not made any observations or proposals.

- References

- R4-2108698, Email discussion summary for [99-e][332] NR_perf_enh2_Demod_Part3_NWM, Moderator (Huawei, HiSilicon), RAN4#99-e.

- R4-2108667, Way forward on FR1 PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#99-e.